3GPP TSG-RAN WG2 Meeting #127Bis	R2-2408497

Hefei, CN, October 14th – 18th, 2024

Agenda Item:	8.7.5

Source: 	Canon Research Centre France

Title:	Discussion on RLC AM Enhancements

Document for:	Discussion

# Introduction

In the RAN2#126 meeting at Fukuoka ([1]), RAN2 discussed how to avoid unnecessary retransmission by RLC AM and reached the following agreement:

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer.

Tx initiated approach was clarified by following agreement:

For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side

Rx initiated approach was also clarified:

For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned

Also, during the same meeting, timely retransmission by RLC was discussed and the following agreements were reached:

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

In the RAN2#127 meeting at Maastricht ([8]), RAN2 discussed how to avoid unnecessary retransmission by RLC AM and reached the following agreement:

Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first. In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer. In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

In this contribution we discuss both how to avoid unnecessary retransmission (2.1), and how to ensure timely RLC retransmission (2.2).

# Discussion

## Avoid unnecessary retransmission

### TX approach

For TX approach it has been discussed whether there is a risk for TX and RX window desynchronization. At transmitting entity, the TX window may be updated once the discard decision is taken. In that case the TX window is updated prior to the RX window. The RX window will be updated based on the discard notification received from the transmitting entity. The RX and TX windows will be out of sync until the discard notification arrives.

Observation 1: For TX initiated approach, the TX and RX windows may be out of sync for a negligible amount of time

Furthermore, if we consider the case of lost or corrupted discard notification signalling, it is possible to adopt the Move Receiving Window (MRW) command as defined in UMTS ([7]):

An SDU can be discarded with explicit signaling when MaxDAT number of retransmissions is reached or the transmission time exceeds a predefined value (Timer_Discard) for an SDU in acknowledged mode RLC. Move Receiving Window (MRW) command is sent to the receiver so that AMD PDUs carrying that SDU are discarded in the receiver and the receiver window is updated accordingly…

In order to limit the signalling overhead, we propose to merge the MRW command with the discard notification signalling.

Proposal 1: Adopt the following steps for TX initiated approach:

- TX discards SDUs based on PDCP discard timer

- The TX window is not updated but the discarded SDU are not sent

- TX sends discard notification to RX

- RX updates RX window based on discard notification

- RX sends a status report acknowledging new window boundary

- TX advances TX window based on status report

### RX Approach

Contrary to TX side, RX side has no knowledge of PDU Sets at any layer. This is because PDU Sets information, including PDU Set boundary and sequence numbers, are not included inline. How RX side can determine that an SDU is obsolete is therefore a difficult issue.

It has been agreed that the receiving entity will abandon SDUs based on a timer, like already done by PDCP.

It is then likely that the timer would be started upon a sequence number gap detection and would be set to a value related to some delay budget. But the time to detect a sequence number gap is unpredictable, with a worst case when the last PDU of a PDU Set is lost, the sequence number gap will only be detected at the successful transmission of a PDU of a further PDU Set.

Observation 2: The time to detect a sequence number gap is unpredictable and can be as large as a PDU Set period

During the extra time induced by the sequence number gap detection, the transmitting entity keeps retransmitting useless SDUs even after the PDU Set delay budget has elapsed.

Observation 3: RX initiated approach is less efficient than the TX initiated approach

### Combined approach

In the combined approach both the transmitting and receiving entities autonomously discard SDUs based on similar criteria (timers). Eventually, each side will discard the same SDUs so there is no need for discard signalling or window signalling. However, the TX and RX windows will not be exactly synced because of the difference between the RX side and TX side timers. We are more concerned about the time taken by the receiving side to detect a sequence number gap before initializing a timer.

|  | TX side approach | RX side approach | Combined TXRX |
| --- | --- | --- | --- |
| Signalling | Discard Discard ack | Fake ack | None |
| Efficiency | Good | Poor* | Good |
| Window synchro | By signalling / Good | By signalling / Good | No signalling / poor** |

Table 2-1 Discard approaches comparison

(*) Time to detect a missing is a waste of time => some unnecessary transmission cannot be avoided

(**) TX and RX will not discard at the same time (time to detect a missing)

Proposal 2: Adopt TX side approach with window synchronization signalling

## Timely RLC retransmission

### Autonomous retransmission

One way to reduce the retransmission delay is to anticipate the retransmission of PDUs with unknown status. The unknown status is indicative of the PDU having been transmitted to the receiver but not acknowledged as being received or detected lost by the receiver.

This enables earlier and more efficient retransmission of some PDUs, which thereby minimizes the overhead when compared to systematic retransmission.

Proposal 3: Consider autonomous retransmission of PDUs with unknown status.

The overhead may be reduced if the autonomous retransmission of PDUs with unknown status is not systematically triggered.

The autonomous retransmission of PDUs having an unknown status may occur following the first transmission of all SDUs of a PDU Set.

Proposal 4: Trigger autonomous retransmission of an SDU with unknown status if all SDUs of the PDU Set are sent or have unknown status.

The autonomous retransmission of PDUs having an unknown status may occur when the transmit and retransmit buffers are empty.

Proposal 5: Trigger autonomous retransmission of an SDU with unknown status if there are no other SDUS to send or retransmit.

To decrease even more the needed resources the autonomous retransmission trigger can be further extended to a remaining time threshold applied to the transmit window. If the transmit window is stalled up to the remaining time threshold of a PDU Set, then autonomous retransmission may be triggered.

Proposal 6: Trigger autonomous retransmission if the transmit window is stalled below a time threshold.

### Enhanced status report

In [2], the authors suggest to speed-up the transmission of status report by the receiving side by the introduction of new triggers, for example, in [3], the authors suggest using a timer-based trigger.

However, while triggering status report either periodically or based on PDB may improve the transmission responsiveness it will not be sufficient in all cases. For example, when a status report timer elapses while the t-Reassembly timer is still running, there will be no nack information sent to the transmitter.

Observation 4: increasing the status report frequency is not useful when the t-Reassembly timer is still running

t-Reassembly timer can be reduced (or even set to 0), which increases the responsiveness of the RLC receiver once the sequence number gap is detected. But reducing the t-Reassembly timer increases the number of false missing detections, which can then increase the overhead across the network.

Observation 5: Enhanced status report costs extra resources in downlink.

In [4], the authors propose to discard the prohibit timer to insure timely transmission of the status report. But the prohibit timer is set by the network for reasons that are independent from the XR application itself, and therefore we think that it should not be ignored when set.

Proposal 7: Do not alter the prohibit timer when set by the network.

Proposal 8: Do not enhance the status report.

### Enhanced polling

The maximum polling frequency can be configured as one polling bit every 4 packets. But polling frequency cannot match the PDU Sets boundaries because PDU Set boundaries are variable.

In [6], the authors suggest to reduce the t-PollRetransmit timer below 5 ms.

Observation 6: Increasing the polling frequency can result in a large number of non-useful status reports being sent.

In [3] and [5], the authors suggest initiating polling by the transmitter based on remaining time information.

However, if the polling bit triggers a status report based on PDB, but the t-Reassembly timer is still running, there will be no nack information sent back to the transmitter.

Also, the prohibit timer might prevent the receiver to send back a status report.

Observation 7: There is a need for certain polling bits to be interpreted differently at the receiving side.

For example, a polling bit is sent based on remaining time information is enhanced with dedicated signalling that allows the receiving entity to handle differently the reassembly and prohibit timers.

Proposal 9: Enhance the polling bit signalling to indicate a low remaining time condition

# Conclusion

In this contribution, we conclude the following observations and proposals.

Observation 1: For TX initiated approach, the TX and RX windows may be out of sync for a negligible amount of time

Proposal 1: Adopt the following steps for TX initiated approach:

- TX discards SDUs based on PDCP discard timer

- The TX window is not updated but the discarded SDU are not sent

- TX sends discard notification to RX

- RX updates RX window based on discard notification

- RX sends a status report acknowledging new window boundary

- TX advances TX window based on status report

Observation 2: The time to detect a sequence number gap is unpredictable and can be as large as a PDU Set period

Observation 3: RX initiated approach is less efficient than the TX initiated approach

Proposal 2: Adopt TX side approach with window synchronization signalling

Proposal 3: Consider autonomous retransmission of PDUs with unknown status.

Proposal 4: Trigger autonomous retransmission of an SDU with unknown status if all SDUs of the PDU Set are sent or have unknown status.

Proposal 5: Trigger autonomous retransmission of an SDU with unknown status if there are no other SDUS to send or retransmit.

Proposal 6: Trigger autonomous retransmission if the transmit window is stalled below a time threshold.

Observation 4: increasing the status report frequency is not useful when the t-Reassembly timer is still running

Observation 5: Enhanced status report costs extra resources in downlink.

Proposal 7: Do not alter the prohibit timer when set by the network.

Proposal 8: Do not enhance the status report.

Observation 6: Increasing the polling frequency can result in a large number of non-useful status reports being sent.

Observation 7: There is a need for certain polling bits to be interpreted differently at the receiving side.

Proposal 9: Enhance the polling bit signalling to indicate a low remaining time condition

# References

- Draft Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, 20 - 24 May, 2024

- R2-2404197, RLC AM retransmission enhancements, Xiaomi

- R2-2404850, RLC re-transmission enhancements for XR, ITRI

- R2-2405002, RLC enhancements for XR, InterDigital

- R2-2405445, Solutions for RLC AM retransmission enhancement, TCL

- R2-2405032, Discussion on RLC Retransmission Enhancements in XR, CMCC

- UMTS TS 125 322 V4.0.0 (2001-03)

- R2-2407575	Report from session on R18 MBS, R18 QoE and R19 XR	Session chair (Huawei)