3GPP TSG-RAN WG2 Meeting #127bis	                               R2-2408499

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.3

Source: 	OPPO

Title:  	Discussion on event-triggered L1 measurement reporting

Document for:	Discussion and Decision

Introduction

In last meeting, RAN2 has discussed the issues related to event-triggered L1 measurement reporting and the following agreements were reached:

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

In this contribution, we discuss the remaining open issues on event-triggered L1 measurement reporting, including the event configuration as well as the reporting content.

Discussion

Event configuration

In last meeting, RAN2 have an initial discussion on the measurement event configuration, and the following agreements were reached:

5  R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

In Rel-18, the CSI-ResourceConfigid is associated with a LTM-CSI-Reportconfig and each CSI-ResourceConfig is associated with a set of CSI-RS resources. To provide the Rel-19 event configuration in serving cell config, one straightforward way is to reuse R18 CSI report config to associate the event configuration with the CSI-ResourceConfig.

R18 LTM CSI report config is reused as baseline to provide event configuration.

Measurement reporting on leave

There is one FFS on the need of measurement reporting once the leaving condition is met:

Agreements on measurement enhancements for LTM:

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

To assist NW to have a better understanding of the radio condition of candidate beams, we see the need to update the measurement result by triggering another L1 measurement report once the leaving condition is met.

L1 measurement reporting is triggered when the leaving condition is fulfilled

Measurement reporting content

In Rel-18 LTM L1 measurement reporting, CSI is used to contain the L1 measurement result as shown below:

For event-triggered L1 measurement reporting, upon the event triggered, the measurement result will be reported to NW through a MAC CE as agreed last meeting.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

In general, the content in R18 CSI report for LTM candidate cells can be considered as baseline. The RS info and the corresponding measurement result can be included in the L1 measurement report MAC CE.

The content of L1 measurement report MAC CE includes at least the following part:

- RS info (i.e. SSBRI)

- Measurement result

Regarding the reported measurement result, L1-RSRP is used as measurement quantity in Rel-18. For Rel-19, one objective is to study the L1 CSI-RS measurement and reporting for LTM candidate, and RAN1 has started the discussion last meeting with the following achieved agreements.

Event-Triggered Reporting

Agreement

- SSB based L1-RSRP measurements is supported for event triggered reporting

- CSI-RS based L1-RSRP measurements is supported for event triggered reporting

- FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting

According to the RAN1 agreements, it seems RAN1 is now discussing about the measurement report quantity and more than one measurement quantity may be supported for CSI-RS based event triggered reporting. Thus, we prefer to leave the discussion of measurement quantity to RAN1 and RAN2 can focus on other potential contents of the L1 measurement report MAC CE.

Measurement report quantity (e.g. L1-RSRP or L1-SINR) should be decided by RAN1.

Send LS to RAN1 to ask the measurement report quantity to be reported by MAC CE.

R18 LTM CSI report allows UE to report the serving cell quality if it is configured as an LTM candidate cell, which is aiming to allow NW to have a better understanding of the radio condition of both serving cell and candidate cells. For event triggered L1 measurement reporting, similar principle can be followed at least in the case that event triggered L1 reporting is used to assist NW to make a cell switch decision.

L1 measurement report MAC CE can contain the measurement result of serving cell.

Another question is whether the MAC CE can contain the measurement result of the other candidate RS in addition to triggered RS. During early discussion on the applicable scenario for event triggered L1 measurement, companies have reached consensus that the triggered event is used for NW to select the target cell/beam to early sync procedure and LTM cell switch procedure. Both scenarios require NW to have a full knowledge of the candidate cells/beams. Besides triggered RS, we see no reason to prohibit UE to report the measurement result of other candidate beams. And as in Rel-18, the number of reported beam and candidate cells could be configured by NW on demand.

Except the triggered beam, the measurement report can contain the measurement result of other candidate beams.

The number of reported beams per cell and number of candidate cells to be reported are configured by NW.

Subsequent measurement reporting

For L1 event triggered measurement reporting, it is questionable whether the event triggered periodical measurement reporting is needed. In our understanding, it is related to whether the one-shot L1 measurement reporting is sufficient for NW to make the LTM cell switch decision.

The motivation of introducing event triggered L1 measurement reporting is to reduce the signaling overhead caused by frequently reporting, if NW can select the target cell/beam properly with single measurement report, we see no need to support event triggered periodical reporting. One compromised way is to rely on NW to determine whether the subsequent periodical measurement reporting is needed, i.e. NW may trigger the semi-persistence L1 measurement reporting if the received event triggered L1 measurement reporting is not sufficient to assist NW to select a proper target cell.

Event triggered measurement reporting is one-shot report.

UE relies on NW to trigger subsequent measurement reporting.

UL grant acquisition

Upon the event has been triggered and the corresponding measurement report MAC CE has been generated, UE should be able to use any available UL grant to deliver the MAC CE. And if the UL grant is not available, dedicated SR can be triggered to request NW for UL grant allocation.

Dedicated SR is configured to acquire UL grant for event triggered L1 measurement reporting.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	R18 LTM CSI report config is reused as baseline to provide event configuration.

Proposal 2	L1 measurement reporting is triggered when the leaving condition is fulfilled

Proposal 3	The content of L1 measurement report MAC CE includes at least the following part:

a)	RS info (i.e. SSBRI)

b)	Measurement result

Proposal 4	Measurement report quantity (e.g. L1-RSRP or L1-SINR) should be decided by RAN1.

Proposal 5	Send LS to RAN1 to ask the measurement report quantity to be reported by MAC CE.

Proposal 6	L1 measurement report MAC CE can contain the measurement result of serving cell.

Proposal 7	Except the triggered beam, the measurement report can contain the measurement result of other candidate beams.

Proposal 8	The number of reported beams per cell and number of candidate cells to be reported are configured by NW.

Proposal 9	Event triggered measurement reporting is one-shot report.

Proposal 10	UE relies on NW to trigger subsequent measurement reporting.

Proposal 11	Dedicated SR is configured to acquire UL grant for event triggered L1 measurement reporting.

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx