3GPP TSG-RAN WG2 Meeting #127bis	R2-2408503

HEIFEI CHINA 14.10.2024-18.10.2024

Source: 	Vodafone, Ericsson

Title: 		Clarification of Measurement Timing Configuration use

Document for: Decision

Agenda Item: 6.1.3

## Introduction

During Rel 17, 3gpp introduced NonCellDefining SSB (NCD-SSB) for redcap UEs and introduced the support of L1 and RRM measurements for non-RedCap UEs based on NCD-SSB in Rel 18. The NCD-SSB feature   allow the UE to obtain synchronisation without needing to decode coreset#0 and SIBs providing a more simple and power efficient design for synchronisation, which is especially useful if operators use e.g. 100 MHz bandwidth carrier with e.g. 20 MHz bandwidth parts.

The following description can be find in TS 38.300:

## Discussion

As it can be seen above, the NCD-SSB can be used to perform L1 and RRM measurements in connected mode. If the source base station does not know that the target base station has activated NCD-SSBs and on which frequencies those are activated, it might be forced to configure inter frequency measurements as it is blind to its neighbour NCD-SSB configuration. This would lead to unnecessary configuration of measurement gaps,  decrease handover performance and UE throughput. With the increased support by both RedCap and Non-RedCap UEs, this information becomes crucial to assure good network and UE performance.

It is also important to consider that NCD-SSBs can be activated dynamically e.g. based on the load situation in the network and are associated to the same PCI, and therefore PCI cannot be used to differentiate between NCD-SSB and “Normal” SSB.

Observation 1: It is important for the operator that the neighbour base stations know if NCD-SSBs are used at a particular point of time. Because NCD-SSB can be dynamically switched on/off, the neighbor base stations should also inform each other about NCD-SSB status in a dynamic way.

During offline discussions, it was pointed out that Xn specification already support via MeasurementTimingConfiguration the exchange of SSB-MTCs  as seen in TS 38.423:

9.2.2.11        Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID | – |  |
| NR CGI | M |  | 9.2.2.7 |  | – |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code | – |  |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  | – |  |
| //skipped rows// |  |  |  |  |  |  |
| Measurement Timing Configuration | M |  | OCTET STRING | Includes the MeasurementTimingConfiguration inter-node message for the served cell, as defined in TS 38.331 [10]. | – |  |

Moreover, TS 38.331 defines MeasurementTimingConfiguration as the following:

Even it seems that a neighbour cell can send within MeasurementTimingConfiguration a list of SSBs and their parameters (e.g. periodicity) and also indicate which of these SSBs to be used for camping, it is not stated anywhere, that MeasurementTimingConfiguration is intended to be used to transfer the knowledge of CD and NCD-SSB used in the neighbour gNB. It is also a rel 15 information element and that makes it even less obvious that it can be used for CD-SSB/NCD-SSB status transfer.

Proposal 1: It is proposed to clarify if MeasurementTimingConfiguration can be used by the gNB to notify the neighbour gNB about the use of CD-SSB and NCD-SSBs and their characteristics. One way to make this clarification is shown below:

campOnFirstSSB Value true indicates that the SSB indicated in the first instance of MeasTiming in the measTiming list can be used for camping, i.e., CD-SSB, and for a PCell configuration (i.e. in spCellConfigCommon of the masterCellGroup).

And

MeasurementTimingConfiguration The MeasurementTimingConfiguration message is used to convey assistance information for measurement timing and to indicate which neighbour cell SSB is CD-SSB. The other indicated SSB can be considered NCD-SSB in use.

If the changes are agreed, it is proposed to inform RAN WG3 about this decision. Relevant CRs can be found in R2-23xxxx.

Proposal 2. If the proposal 1 is agreed, it is proposed to inform RAN WG3 about decision and to discuss from which release the changes can be made.

## Conclusions

Observation 1: It is important for the operator that the neighbor base stations know if NCD SSBs are used at a particular point of time. Because NCD-SSB can be dynamically switched on/off, the neighbor base stations should also inform each other about NCD SSB status in a dynamic way.

Proposal 1: It is proposed to clarify if MeasurementTimingConfiguration can be used by the gNB to notify the neighbour gNB about the use of CD-SSB and NCD-SSBs and their characteristics. Please see corresponding CR attached

Proposal 2. If the proposal 1 is agreed, it is proposed to inform RAN WG3 about decision and to discuss from which release the changes can be made.