3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408511

Hefei, China, 14 - 28 October 2024

Agenda Item:	 8.12.2

Source:	 Huawei, HiSilicon

Title:	 Initial analysis on RAN2 impact for Rel-19 NR MIMO Ph5

Document for:  Discussion and Decision

1	Introduction

For Rel-19 MIMO, according to the WID as shown below [1] and recent RAN1 inputs [5], objectives relevant to UE-initiated/event-driven beam management (UEIBM) and asymmetric DL/UL TRP deployment are identified to have significant RAN2 impacts in terms of RRC configuration and MAC procedure, which are outlined in the WID.

RP-242394, Revised Work Item: NR MIMO Phase 5 The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this contribution, we present detailed analysis and provide our considerations on how RAN2 start work on the UEIBM and asymmetric DL sTRP/UL mTRP, based on RAN1 agreements so far.

2	UE Initiated/Event-driven Beam Management

In order to reduce the periodic CSI reporting overhead and latency, event-based CSI reporting is agreed by RAN plenary for both Rel-19 MIMO WI and Rel-19 LTM WI, i.e. UE reports CSI only when the event is triggered. In particular, it includes triggering event determination procedure and reporting procedure after the event is triggered. Those points are generally similar between two WIs. However, after some detailed discussion in RAN1 and RAN2 meetings, it seems some diverging agreements on reporting procedure. More specifically, RAN1 agreed in MIMO that the UE shall use UCI to carry the CSI report while RAN2 agreed that the LTM uses MAC CE. Note that based on the latest RAN plenary agreement, each WI can proceed their solution separately.

Although the beam reporting procedure in MIMO and LTM can be different based on the current situation, the remaining components of the UE procedure in terms of e.g. event determinations can be quite similar, which is supposed to be specified in the MAC layer. In order to reduce the complexity of the specification, it’s better to capture this part of MIMO and LTM together. Note that, in RAN2 LTM, it is also raised by several companies that the relevant LTM discussion in RAN2 can be to align with the MIMO progress in RAN1.

Observation 1: MIMO and LTM can be unified under the same UE procedure in MAC specification.

In the following, we provide our RAN2 considerations on RRC configurations aspects, trigger event for event-driven beam management, and beam reporting and transmission procedures. In the last, we intend to evoke the discussion on how to model this in specification, in order to avoid back-and-forth discussions in the future between RAN1 and RAN2.

2.1	RRC Configurations of UEIBM

The parameters described above are part of the RAN1 Liaison Statement to RAN2 (R1-2407285), focusing on RRC impacts for Rel-19 MIMO. Here is the usage of the above parameters.

- The parameter reportTransmissionMode-r19 indicates the mode applied for beam reporting.

- The parameter eventThreshold-r19 defines the RRC-configured threshold value for Event-2.

- The parameters eventDetectionTimeWindowLength-r19 and eventInstanceCount-r19 represents the length of time window and the number M, respectively, used in event evaluation procedure.

- The parameter enabledCurrentBeamReport-r19 determines whether the current beam should be reported while parameter nrofReportedRS-UEIBR-r19 shows the number of RS to be reported.

- The parameter newBeamResourceSetEvent2-r19 is used to configured the resource set of new beams for event-2.

The specific functionality impact of the RAN2 due to above parameters will be analysed in the following section of this context. Nevertheless, it’s up to RAN2 to discuss how to capture in RRC ASN.1. From our perspective, these parameters are the fundamental to the procedure of UE-initiated/event-driven beam reporting, all of which cannot be overlooked.

Observation 2: Introduce RRC parameters: reportTransmissionMode-r19, eventThreshold-r19, eventInstanceCount-r19, eventDetectionTimeWindowLength-r19, enabledCurrentBeamReport-r19, nrofReportedRS-UEIBR-r19, newBeamResourceSetEvent2-r19.

The above parameters are proposed to be under IE structure CSI-Report Config. In the legacy CSI reporting structure, it contains the periodic, semi-persistent, and aperiodic reporting in the report Config type field, where one of these options is selected for each report Config. For Rel-19 MIMO, event-triggered reporting is now introduced as an additional option with corresponding configurations. And each report configuration is associated with a single event type, which should be held regardless report Config type. Furthermore, parameters such as the threshold, time window, and the value M should also be configured for the event-trigger reporting.

Proposal 1: For UEIBM, event-triggered reporting is added as a new type in the report Config type field under the IE CSI-Report Config.

2.2	Triggering Event Determination

In the event triggering procedure, it is understood in RAN1 that the UE should determine whether the event condition is met with the new beam or the current beam, leading to the generation of an event instance. Once one or more event instances are generated, UE initiated beam report occurs. The following agreement was made for event determination in RAN1:

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

According to the RAN1 agreements, the event-2 condition is met when the L1-RSRP of the new beam is a threshold value greater than the current beam. This threshold is configured by the network. However, the procedure are different with respect to UE feature as follows:

- For the basic feature of UE, once the new beam has satisfied the event-2 condition, the UE could send the beam report.

- For the advanced feature, the event triggering procedure consists of a time window, and a number M. During the configured time window, once the new beam meets the event-2 condition, the event instance of the UE will increment by 1. When the event instance reaches the value of M, the UE will initiate the reporting procedure.

Note that, it has been agreed in LTM that the TTT mechanism will be applied, which is similar to the mechanism of the time window and M times as agreed in MIMO in RAN1. Thus, the mechanism of the time window and M times can be applied to both LTM and MIMO. In order to simplify the specification, it would be desired to unify the LTM event evaluation and MIMO event triggering procedures.

Proposal 2: From RAN2 MIMO perspective, it’s recommended to have a common design for the LTM event evaluation and MIMO event triggering procedures at least for Event-2.

Based on the agreement, RRC should configure a threshold, time window length and M for controlling UE on triggering event determination (Event-2). It is likely that this event evaluation procedure can also be applied to other events, such as event-1 and event-7. RAN2 can await further progress from RAN1 for the application of these other events.

Then, we can discuss how to maintain the window. Currently, BFR and RLF procedures uses the time window and value M for the trigger event determination. It is straightforward to consider BFR and RLF as baseline for UEI trigger event. Regardless of BFR and RLF, the timer is started upon the reception of the first event instance. Thus it can be applicable to UEI trigger event determination. However, regarding how to restart the window, there can be different options:

- Option-1: Similar to the BFR procedure, the timer is restarted upon the reception of the next event instance that meets the specified conditions, before the current timer expires. Upon the timer expires, the UE also restart the timer. When the number of event instance = the value M, beam reporting occurs.

- Option-2: Similar to RLF procedure, the timer is only restarted upon expiration. At the point of timer expiration, if the number of event instance ≥ the value M, beam reporting occurs.

Proposal 2a: For UEIBM. The UE maintains a timer and counter for triggering event determination. This timer can be started upon the first event instance where the condition is met. FFS for the restart of the timer.

Then we can discuss how to maintain counter. Basically, when the timer is started, the UE needs to count the subsequent event instance where the condition is met. However, whether to count for all the beams or for each beam needs further discussion. Here we show two different understandings:

- Option-1: only event instances for the same beam as the first event instance are counted.

Start the timer

√means the beam satisfies the event condition

Event instance

Event instance

Beam 1√

Beam 1√

Beam 1√ √

Beam 1√

→ Counter: 4

Start the timer

Option-2: any event instances are counted; the beam may be different between first event instance and subsequent event instance.

Event instance

Event instance

→ Counter: 4

Counter: 4

Beam 3√

Beam 1√

Beam 2√

Beam 1√

In the context of Option-1, the time window is defined per beam. In Option-2, the time window is defined per event type, any beam which satisfies the event conditions can be considered as event instance. Compared to the Option-1, the benefit of the Option-2 is that the network is able to receive the beam report in time. For this part, MIMO and LTM should also be captured together to simplify the specification.

Proposal 2b: For UEIBM, RAN2 to discuss how to maintain the counter for trigger event determination.

- Option-1: only event instances for the same beam as the first event instance are counted.

- Option-2: any event instances are counted; the beam may be different between first event instance and subsequent event instance.

2.3	Beam Reporting and UL Transmission

Following the discussion on the beam evaluation procedure, the next step involves beam reporting. The UE is required to send the L1 CSI report to the network using UCI. Two modes (Mode-A and Mode-B) have been agreed upon by RAN1. The detailed procedures for these two modes will be discussed individually in the following context.

During the RAN1 discussion, an alternative option of transmitting the beam report via a MAC CE (Option-1) was considered but ultimately excluded. This decision was guided by a WID which explicitly assumes the reuse of the legacy CSI measurement and reporting framework, where CSI reports are transmitted via UCI. Given that UCI serves as a suitable container for these reports, the reuse of UCI is considered a more reasonable approach.

For the UCI-based beam reporting, three options were listed during the discussions:

•	Option-2 (Mode-A): beam report is transmitted in a UCI in dynamically allocated UL resource.

•	Optoin-3 (Mode-B): beam report is transmitted in a UCI in pre-configured UL resources.

•	Optoin-4: beam report is transmitted in a UCI in pre-configured UL resources not specifically dedicated for UEI beam report.

Initially, Option-3 (Mode-B) was accepted due to several advantages. This mode significantly reduces the complexity in NW side, due to the reason that NW does not need to monitor whether UEI beam report carried in the pre-configured resource all times. Additionally, from the NW perspective, Mode-B offers better resource efficiency, as the pre-configured resources can be shared among UEs.

Then, the option-2 (Mode-A) was also discussed. One key advantage of Mode-A is that the NW can schedule flexibility to avoid collision issue on the second channels. It has very high resource utilization. However, the dynamic resource allocation of step-2 in Mode-A could introduce some latency. Therefore, the Mode-A is also agreed by the RAN1 to work as a fundamental feature for the UE.

Regarding option-4, the uplink resource to carry the UEIBM reporting is pre-configured the absence of pre-indication requires the network to monitor all pre-configured uplink resources for potential UE beam reporting. This could result in unused resources or it may lead to collision of resources if multiple UEs share the same pre-configured resources. Therefore, from the perspective of NW complexity and resource utilization, option-4 should not be considered any more.

In conclusion, there are two modes are used for beam reporting procedure: Mode-A (option-2) and Mode-B (option-3).

2.3.1	Mode-A Reporting procedures

Mode-A reporting have the following procedure, as shown in Fig.1.

Figure.1 Mode-A for UE-initiated/event-driven beam reporting

On the Mode A configuration, RAN1 has the following agreement.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following. - Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID Above applies at least for the single CC case.

Based on the above agreement, two options are considered for mode A configuration, i.e. dedicated SR or new UCI type. From our perspective, the first PUCCH in Mode-A shows a very similar functionality to the legacy SR, as it requests resources for PUSCH and is followed by the network response (DCI) that indicates the allocated PUSCH resources for beam reporting. Reusing the dedicated SR configuration and procedure would simplify the MAC procedure design.

Proposal 3a: For Mode A. RAN2 assumes to re-use the existing SR RRC configuration and MAC procedure for first PUCCH channel.

2.3.2	Mode-B Reporting procedures

Mode-B reporting have the following procedure, as illustrated in Fig.2.

Figure.2 Mode-B for UE-initiated/event-driven beam reporting

On the Mode B configuration, RAN1 has the following agreement.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following - Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above is at least applied to a single CC case.

The definition of SR is to request DG resource for new transmission. In mode-B, UCI is used to indicate a CG resource for beam report. The purpose is different somehow. Modelling this UCI as an SR could appear somewhat strange, given the differing objectives. However, on the other hand, the MAC procedure would be simple if SR is modelled for mode B. The final decision is subject to RAN1's determination. Regardless of whether the SR or a new UCI type is selected, the modelling specifications will be within the MAC layer.

Proposal 3b: For Mode B, it is up to RAN1 to decide for the first PUCCH channel configuration.

2.4	Spec Modelling

It is noted that RAN1 is under discussion on where to capture the UEIBM in the spec. From RAN2 perspective, we have similar practice for BFR. For the BFR procedure, two critical components are used for beam failure detection, an internal counter as BFI_COUNTER and a timer as beamFailureDetectionTimer. Once beam failure is detected, UE can trigger BFR through random access or MAC CE with beam failure and candidate beam information. All such procedures are captured in MAC procedure.

UEIBM contains the event evaluation and reporting procedure, the event evaluation is also based on the counter and timer. The whole procedure is quite similar with BFR procedure and we thus think the UEIBM should be captured in MAC layer to keep the spec simple.

Proposal 4: From RAN2 perspective, the event evaluation and reporting procedure of UEIBM should be captured in MAC spec in terms of timer and counter.

3	Asymmetric DL sTRP/UL mTRP

In Multiple Transmit/Receive Point (multi-TRP) operation, a serving cell can schedule the UE from two TRPs, providing better coverage, reliability and/or data rates for PDSCH, PDCCH, PUSCH, and PUCCH. In legacy mTRP scenario, each TRP have downlink and uplink, as shown in Fig.1. In Rel-19, a new scenario (asymmetric DL sTRP/UL mTRP) is considered, in this scenario, one TRP have downlink and uplink, another TRP only have uplink, as shown in Fig.2.

Figure.3 Legacy multi-TRP scenario

Figure.4 Asymmetric DL sTRP/UL mTRP scenario

3.1	Pathloss

In legacy mTRP scenario, each TRP can transmit the DL RS and UE can measure the PL directly based on each DL RS from the mTRP. In Rel-19, a new scenario (asymmetric DL sTRP/UL mTRP) is considered, in this scenario, only one TRP transmit the DL RS, i.e. there is TRP who does not transmit the DL RS. For the first TRP with DL RS transmission, UE calculate the first PL based on DL RS directly. However, for second TRP without DL RS transmission, some new mechanism is needed, e.g. the second PL is calculated based on the first PL and a PL offset. On PL offset determination, RAN1 agreed that different PL offset can be configured to different TCI states of a TRP since UE is not visible to the TRP deployment as:

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

Based on the above agreement, RAN2 confirms that RRC configures a PL offset for each TCI state configuration and a new MAC CE with eLCID is introduced for updating PL offset for TCI states after RRC configuration.

Proposal 5: RAN2 assumes that RRC configures an PL offset for each TCI state configuration. A new MAC CE with eLCID is introduced to update PL offset for TCI states associated.

However, it is still unclear what format details this new MAC CE looks like. To finalize MAC CE format, it would be necessary to send RAN1 LS to ask some questions for details, for instance:

- whether this new MAC CE applies to CA and DC case.

- Which channel and signal this new MAC CE with PL offset update applies to.

Proposal 6: RAN2 send RAN1 LS to ask for MAC CE format details:

- Whether this new MAC CE applies to CA and/or DC case.

- Which channel and signal this new MAC CE with PL offset update applies to.

3.2	PHR

3.2.1	Triggering

Based on the TS 38.321, the following PHR triggering is defined as:

A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission; NOTE 1: The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5].

In legacy mTRP scenario, different TRPs can transmit different DL RSs and UE can measure the PLs separately based on each DL RS from the mTRP. In asymmetric DL sTRP/UL mTRP scenarios, the PL for one UL TRP is derived based on the DL TRP and the PL for the other UL TRP is derived based on the DL TRP and PL offset as mentioned in above. Though the PL calculation method of each mTRP has been modified, the PHR is still triggered according to the change of PL. Since the triggering mechanism has not been changed, such PL determination may not impact the PHR triggering.

Proposal 7: RAN2 to discuss the PHR triggering impact on pathloss regarding asymmetric DL sTRP/UL mTRP scenarios.

3.2.1	Format

Based on the TS 38.213, PH is calculated by physical layer and indicated the calculated PH to MAC layer. Then MAC layer includes it in the PHR MAC CE in respect to UL resource after PHR triggering is met. In Rel-19, RAN1 made the following agreement on PH calculation for asymmetric DL sTRP/UL mTRP scenarios.

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH

Based on the above agreement, RAN2 can assume physical layer will indicate calculated PH for asymmetric DL sTRP/UL mTRP scenarios to MAC layer, and as in legacy, the MAC layer includes it in the existing MTRP PHR MAC CE, i.e. no new MTRP PHR MAC CE is introduced.

Proposal 8: RAN2 assumes no new MTRP PHR MAC CE is introduced, i.e. the existing MTRP PHR MAC CE can be reused.

# 4	Conclusion

In this paper, we have addressed the objective regarding initial analysis on RAN2 impact for MIMO. There are the following observations and proposals:

UE Initiated Beam Management

Observation 1: MIMO and LTM can be unified under the same UE procedure in MAC specification.

Observation 2: Introduce RRC parameters: reportTransmissionMode-r19, eventThreshold-r19, eventInstanceCount-r19, eventDetectionTimeWindowLength-r19, enabledCurrentBeamReport-r19, nrofReportedRS-UEIBR-r19, newBeamResourceSetEvent2-r19.

Proposal 1: For UEIBM, event-triggered reporting is added as a new type in the report Config type field under the IE CSI-Report Config.

Proposal 2: From RAN2 MIMO perspective, it’s recommended to have a common design for the LTM event evaluation and MIMO event triggering procedures at least for Event-2.

Proposal 2a: For UEIBM. The UE maintains a timer and counter for triggering event determination. This timer can be started upon the first event instance where the condition is met. FFS for the restart of the timer.

Proposal 2b: For UEIBM, RAN2 to discuss how to maintain the counter for trigger event determination.

- Option-1: only event instances for the same beam as the first event instance are counted.

- Option-2: any event instances are counted; the beam may be different between first event instance and subsequent event instance.

Proposal 3a: For Mode A. RAN2 assumes to re-use the existing SR RRC configuration and MAC procedure for first PUCCH channel.

Proposal 3b: For Mode B, it is up to RAN1 to decide for the first PUCCH channel configuration.

Proposal 4: From RAN2 perspective, the event evaluation and reporting procedure of UEIBM should be captured in MAC spec in terms of timer and counter.

Asymmetric DL sTRP/UL mTRP

Proposal 5: RAN2 assumes that RRC configures an PL offset for each TCI state configuration. A new MAC CE with eLCID is introduced to update PL offset for TCI states associated.

Proposal 6: RAN2 send RAN1 LS to ask for MAC CE format details:

- Whether this new MAC CE applies to CA and/or DC case.

- Which channel and signal this new MAC CE with PL offset update applies to.

Proposal 7: RAN2 to discuss the PHR triggering impact on pathloss regarding asymmetric DL sTRP/UL mTRP scenarios.

Proposal 8: RAN2 assumes no new MTRP PHR MAC CE is introduced, i.e. the existing MTRP PHR MAC CE can be reused.

# 5	Reference

- RP-242394, WID revision: NR MIMO Phase 5;

- RAN1 #108 chairman notes;

- TS 3GPP 38.331 V18.2.0

- TS 3GPP 38.321 V18.3.0

- R1-2407285 LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5