3GPP TSG-RAN2 Meeting #127bis	R2-2408521

Hefei, China, Oct 14th –18th, 2024

Source: 			ZTE Corporation

Title: 	Discussion on measurement event prediction

Agenda item:	8.3.3

Document for: 	Discussion and Decision

- Introduction

In the RAN2 125bis meeting, the following agreements about measurement event prediction were achieved [1]:

1. At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction is also allowed. 2. Clarifications on what is being as input should be provided with results 3. Start with A3 as a baseline. 4. Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

Since companies have studied the RRM measurement prediction in the past a few meetings and provided the initial simulation results for RRM measurement prediction, we can re-start the study of measurement event prediction. In this paper, we will share our views on the measurement event prediction.

- Discussion

## 2.1 Usage of Measurement event prediction

For normal mobility, the network usually triggers handover procedure based on the received measurement events. For instance, A3/A5 can be used to trigger coverage-based mobility and A4 can be used to trigger load balance-based mobility. Regarding measurement event prediction, the intention is to predict when/what event will be fulfilled. So, it seems the event prediction mainly considers temporal-domain prediction. However, in our understanding, even for spatial-domain or frequency-domain prediction, measurement event prediction is needed, because for measurement prediction in UE side model, the UE is not expected to report the predicted measurement results periodically to network, this will increase the signalling burden over Uu interface, So, a simple solution is to evaluate the predicted measurement results and send report to network if the predicted measurement results fulfill some criteria, e.g. like measurement events.

Observation 1: In UE side model of RRM measurement prediction, the UE is not expected to transmit the predicted results to the network periodically because this increases signalling overhead.

Therefore, in our view, the usage of measurement event prediction is to help the UE to deliver the predicted measurement results to the network, and inform the network when/whether the predicted measurement results can be used to trigger handover. Specifically, for temporal-domain prediction, the UE evaluates at which future time instance the criteria of measurement event are met based on the predicted future cell results of serving cell and neighbor cells, and report them to the network before the channel quality deteriorates. For frequency domain or spatial domain measurement prediction, the UE evaluates whether the criteria of measurement event are met based on the predicted cell results.

Proposal 1: Measurement event prediction is a method that helps UE to deliver the UE’s predicted RRM measurement results to the network.

Proposal 2: Spatial-domain, frequency-domain and temporal-domain RRM measurement prediction can be considered to provide the inputs (e.g. predicted measurement results) for measurement event prediction.

In the previous meeting, some companies proposed that the measurement event prediction can be used to evaluate whether the event is stable. The general idea is that: The UE evaluates how long the event will keep, and if the time that the event keeps is short, the measurement event is considered as non- stable, and the UE may not report the measurement results to the network, as shown in Fig. 1:

Figure 1: Diagram for prediction of measurement event length proposed by some companies

However, there are some issues to be clarified: (1) how to determine the time that event keeps is short or not, the threshold is defined by the UE or the network? If the threshold is defined by the UE, it is possible for the network to not receive any measurement report for a long time, since the UE think the current event is not stable. In this way, the network can not perceive channel quality. If the threshold is defined by the network, why not directly set a longer TTT for measurement event prediction, which is straighter and simpler. (2) Since the prediction window is sliding, before the UE predicts that the measurement event is not stable, the UE can predict the measurement event will be triggered. Taking Fig. 2 as an example, when the UE is at t-4, the UE can predict that the measurement event will be triggered at t5. So, at t-4, the UE already can report the measurement results to the network according to the current measurement scheme. But only after t-1, the UE can know that the measurement event is not stable. (3) The prerequisite of event time of stay prediction is that the length of the prediction window is larger than TTT and threshold which is used to determine whether the event is stable. So, the prediction accuracy for event stay-of-time may be not good because the prediction accuracy will decrease with the increase of prediction window length. Based on the analysis above, we think the usage of predicting time that event keeps is not clear.

Figure 2: The timing of measurement event prediction

Observation 2: For evaluating whether measurement event is stable, the following issues need to be clarified:

- How to determine whether measurement event is stable or not;

- Before the UE evaluates whether the measurement event is stable or not, the UE already can predict whether the measurement event is triggered or not, and report the results to the network.

- The length of prediction window needs to be larger than TTT and the threshold, which may impact the prediction accuracy.

Proposal 3: The usage of predicting how long the measurement event will stay are unclear.

- measurement event simulation assumption

It is agreed that the simulation assumptions of RRM measurement prediction are the baseline for use cases other than RRM measurement prediction. So, the measurement event prediction can reuse the simulation assumptions of RRM measurement prediction. However, there are still some measurement event prediction specific simulation assumptions that have not been determined yet. In this section, we’d like to share our views on them.

In the TR 36.839, the A3 time to trigger (TTT) and A3 offset are set to different values in different simulations. For example, in the Hotspot calibration simulation, the candidate TTT values include 480ms, 160ms, 80ms and 40ms. It’s difficult to evaluate which candidate value is most suitable for measurement event prediction scenario. So, our suggestion is to use the values that widely used in real deployment, i.e. 3dB A3 offset and 320ms TTT. And for simplicity, we suggest not consider hysteresis, same as in TR 36.839.

One may concern that with 320ms TTT, there is only one measurement result within TTT in FR2 scenario since the measurement period is 400ms, which impacts the evaluation of measurement event. Actually, in reality, 400ms measurement period is the minimum requirement, and the UE may perform measurement more frequently during TTT. So, in reality, 320ms TTT is feasible in FR2 scenario. Considering the measurement behavior related to TTT evaluation is strongly related to UE (or chipset)'s implementation. In the simulation, our suggestion is it’s up to company implementation to evaluate whether the criteria of measurement event are met at the end of TTT.

Proposal 4: Take the following parameter values as baseline in the measurement event simulation:

- 3 dB for A3 baseline offset;

- 320ms for A3 baseline time-to-trigger;

- Not consider A3 hysteresis.

Proposal 5: It’s up to company implementation to evaluate whether the criteria of measurement event are met at the end of TTT.

## 2.3 Performance KPI

Considering measurement event is mainly used to trigger handover, the time of predicted event is important for handover performance, for example, if the declaration of event is later than actual event-triggered-time, too-late handover may occur. On the contrary, if the declaration of event is earlier than actual event-triggered-time, too-early handover may happen. So, for measurement event prediction, we suggest to consider to the time difference of actual event-triggered time and predicted event-triggered time as KPI.

Besides, we also need to evaluate the prediction accuracy, e.g. whether the actual measurement event is predicted in advance or whether the predicted measurement event is the actual measurement event. Similar to RLF prediction, the missed measurement event detection, the false measurement event detection and F1 score can also be considered as metric to evaluate prediction accuracy.

Proposal 6: For indirect measurement event prediction, consider the following KPI:

    - The time difference of actual event-triggered time and predicted event-triggered time;

    - Missed measurement event detection, false measurement event detection, F1 score.

- Conclusion and proposals

In this contribution, we have the following observations and proposals:

Observation 1: In UE side model of RRM measurement prediction, the UE is not expected to transmit the predicted results to the network periodically because this increases signalling overhead.

Observation 2: For evaluating whether measurement event is stable, the following issues need to be clarified:

- How to determine whether measurement event is stable or not;

- Before the UE evaluates whether the measurement event is stable or not, the UE already can predict whether the measurement event is triggered or not, and report the results to the network.

- The length of prediction window needs to be larger than TTT and the threshold, which may impact the prediction accuracy.

Proposal 1: Measurement event prediction is a method that helps UE to deliver the UE’s predicted RRM measurement results to the network.

Proposal 2: Spatial-domain, frequency-domain and temporal-domain RRM measurement prediction can be considered to provide the inputs (e.g. predicted measurement results) for measurement event prediction.

Proposal 3: The usage of predicting how long the measurement event will stay is unclear.

Proposal 4: Take the following parameter values as baseline in the measurement event simulation:

- 3 dB for A3 baseline offset;

- 320ms for A3 baseline time-to-trigger;

- Not consider A3 hysteresis.

Proposal 5: It’s up to company implementation to evaluate whether the criteria of measurement event are met at the end of TTT.

Proposal 6: For indirect measurement event prediction, consider the following KPI:

    - The time difference of actual event-triggered time and predicted event-triggered time;

    - Missed measurement event detection, false measurement event detection, F1 score.

- Reference

- Report of RAN2 125bis meeting.