3GPP TSG-RAN WG2 Meeting #127bis                                 R2-2408527

Hefei, China, Oct. 14 – 18, 2024

Agenda item:		8.1.2.3

Source:				Samsung

Title:					Discussion on LCM for POS use case

Document for:		Discussion & Decision

1	Introduction

This contribution provides our view on LCM for POS use case according to the following objective in WID [1].

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

2	Discussion

According to WID [1], there are five positioning sub use cases, which can be summarized as below.

| Case | Positioning type | Model entity | AI/ML for Pos type |
| --- | --- | --- | --- |
| 1 (1st priority) | UE-based positioning | UE-side model | Direct |
| 2a (2nd priority) | UE-assisted/LMF-based positioning | UE-side model | assisted |
| 2b (2nd priority) | UE-assisted/LMF-based positioning | LMF-side model | Direct |
| 3a (1st priority) | NG-RAN node assisted positioning | gNB-side model | assisted |
| 3b (1st priority) | NG-RAN node assisted positioning | LMF-side model | Direct |

Until now, the following agreements were made for the use case 3a/b with gNB/LMF-side model.

For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3. RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

According to the agreement above, we do not see any potential RAN2 spec. impact for the case 3a/b (with gNB/LMF-side model). Thus, from RAN2 perspective, the case 3a/b can be deprioritized compared to the case 1 (with UE-side model). Also, since the case 2a/b already have 2nd priority according to WID [1], RAN2 can focus on the use case 1 for now and further discuss other cases later.

Proposal. 1: For POS use cases, RAN2 can prioritize the discussion on the case 1 over other cases.

Based on the observation above, we discuss the LCM aspect mainly considering the case 1 with UE-side model in the following part.

2.1 UE capability/applicability reporting

Regarding the UE capability reporting for POS, the following agreements were made in RAN2 until now.

(In RAN2#125bis) 1. Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress. “supported” means that the UE is capable of supporting the functionality and doesn’t mean neccesarily that the UE has the model available. FFS what functionality refers to. 2. Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability. (In RAN2#126) 1 The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. 2 wait for RAN1 for associate ID discussion

For the POS case 1, we think the functionality can refer to UE location estimate based on the output of AI/ML model for direct positioning. Also, according to the agreement above, the UE can indicate the supported AI/ML positioning functionalities to LMF via LPP Capability Transfer procedure (RequestCapabilities/ProvideCapabilities). Then the next issue to discuss is how to handle dynamic capabilities. From our understanding, the wording “dynamic capabilities” in the previous agreement refers to UE’s report on applicable functionalities.

On top of the capability, the requirement on the AI/ML model could be another factor to impact whether the AI/ML-enabled functionality is actually applicable or not, e.g., an AI/ML model may require certain air interface resource to obtain the measurement/assistance information to facilitate the model operation or have some validity conditions (e.g., applicable area/scenario/environment), or certain computation/storage/power/processing time requirement is needed. Thus, for the UE-side model case, the UE should be able to report the applicable functionalities to LMF. Regarding this aspect, RAN2 made the following agreement until now.

(In RAN2#125bis) 1 Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message. 2 Support reactive reporting of UE-sided applicable functionality. The NW configures AI/ML functionalities via RRC/LPP message. FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality 3 FFS how the two approaches will be specified and whether we can combine them into one procedure. FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed) (In RAN2#126) 2 We will work offline on the definitions for functionality types and define what is availability. 3 The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable. For a functionality to be applicable at least there should at least one model available within it. FFS other details on what is applicability/non-applicability. 4 For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training. RAN2 will wait for RAN1 input for further details. FFS if the same applies to positioning 5 For BM use case, As a baseline the UE determines whether a functionality is applicable. Existing UAI framework is used at least for proactive reporting of applicable functionality. FFS reactive (In RAN2#127) 2 Applicable functionalities refers to functionalities that the UE is ready to apply for inference

First, according to the pink-colored agreement that applicable functionalities refer to “functionalities that the UE is ready to apply for inference”, it seems reasonable to us that UE can determine the applicability of a certain functionality. Therefore, the yellow-colored agreement for BM use case can also apply to POS use case.

Proposal. 2: For POS Case 1, the UE determines whether a functionality is applicable as a baseline.

For applicability of a functionality, the UE should have at least one model available to use for UE-side condition. In addition, there can be other UE-side conditions for the applicability check, but they must be transparent to NW (i.e., LMF) and up to UE implementation without any spec. impact. Meanwhile, as in the green-colored agreement, there can be some NW-side additional conditions that NW needs to provide to UE for the applicability check. Even though it is still unclear what specific information can be NW-side additional conditions even in RAN1, we can assume that NW provides it to UE if needed.

In legacy positioning procedure, LMF controls the overall positioning procedure. For UE-assisted/based mode, LMF interacts with UE via LPP to determine the POS method to use, provide assistance information (e.g., DL-PRS configuration), request measurement/estimation, and determine UE’s location based on the report from UE. Also, LMF interacts with gNBs/TRPs via NRPPa to obtain the information (e.g., TRP/PRS configuration) to assist UE’s positioning. Considering that NW-side additional conditions for POS are most likely related to the NW configuration that LMF can obtain from gNB/TRP and provide to UE, we can assume that LMF provides the NW-side additional conditions to UE via LPP signalling as a baseline.

Proposal. 3: For POS Case 1, LMF provides NW-side additional conditions to UE via LPP signaling (e.g., LPP ProvideAssistanceData) as a baseline if needed. 

According to the red-colored agreement above, two different approaches (i.e., proactive, reactive) should be supported for applicability report. Potential procedures are illustrated in Figure 1 to support both proactive and reactive approaches.

Figure 1. Potential procedure for proactive/reactive approach

- Step 1: LMF sends LPP RequestCapabilities message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends LPP ProvideCapabilities message to network, containing supported functionalities at the UE side.

- Step 3: LMF sends LPP ProvideAssistanceData message to UE. Following configurations are provided from LMF to UE:

- LMF may provide NW-side additional condition.

- LMF may implicitly request applicable functionality report.

- (Before “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

- Step 4: UE reports applicable functionality in the following scenarios:

1) Upon change of applicable functionality

2) As response to NW-side additional condition requesting applicable functionality reporting in step 3

- Step 5:

1) LMF configures inference configuration to UE.

In step 3, the LMF provides NW-side additional conditions (e.g., DL-PRS configuration, associate ID, …) for the UE who supports the concerned functionalities if needed. This can be done via LPP ProvideAssistanceData message as in the legacy POS procedure. UE can determine the actual applicable functionalities and report applicability related information at step 4. In our view, the UE can report the applicability information via LPP ProvideCapabilities message. In BM use case, we assume use of UAI for applicability reporting since RRC UE capability exchange procedure does not support dynamic capability reporting from UE. However, in POS use case, LPP UE capability exchange procedure already allows UE to dynamically report its capability whenever needed. For example, the UE can provide unsolicited capabilities to LMF by sending LPP ProvideCapabilities message without receiving LPP RequestCapabilities message from LMF, to report the dynamically changing UE’s status (e.g., remoteUE-Indication-r18). Based on applicability related information, the LMF configures applicable functionalities. More specifically, in the POS Case 1, the LMF can request UE to estimate its location using AI/ML model via LPP RequestLocationInformation message.

Observation. 1: LPP capability indication procedure allows UE to provide unsolicited capabilities to LMF. I.e., UE dynamically report its capability without any request from LMF.

Proposal. 4: For POS Case 1, UE reports its applicable AI/ML functionalities via LPP ProvideCapabilities message.

2.2 Model Determination/Selection

For UE-side model case, the UE can have multiple AI/ML models that are applicable for the same functionality. In that case, one of the models should be selected to use when the AI/ML functionality is configured/applied. For the model selection, we can consider the following options:

- Option 1: The UE decides which model to use among the applicable models.

- Option 2: The UE decides which model to use among the applicable models and indicates it to the LMF.

- Option 3: The LMF decides which model to use among the applicable models and indicates it to the UE.

For the option 1, the UE can decide one model to use among the applicable models. For applicability check, the LMF can provide some assistance information (e.g., additional condition like DL-PRS configuration) or some criteria (e.g., POS QoS requirement on accuracy/latency) to UE and the UE checks whether each model is applicable or not for the functionality.

The option 2 is almost same with the option 1 except the point that the UE can indicate the selected model to the LMF. However, we don’t see any reason why the LMF should be aware of the model used at the UE side. In our view, the model used at UE side can be transparent to the LMF as in the case of option 1 unless the LMF decides which model to use as in the option 3.

For the option 3, the LMF can decide which model to use among the applicable models and indicates it to the UE. In this case, the LMF should be able to check whether each model at the UE side is applicable or not for the functionality first. For the applicability check, the LMF needs to know not only the NW-side additional conditions but also the UE-side conditions. However, it seems ineffective/impractical for UE to provide the UE-side additional conditions (e.g., requirement on computation/storage/power/processing) for the LMF.

Based on the discussion above, the option 1 seems most reasonable. However, we can wait for input from RAN1 since this issue is still under discussion in RAN1.

Proposal. 6: For POS Case 1, RAN2 assume UE decides/selects which model to use among applicable models for the configured functionality.

2.3 Performance monitoring

For UE-side model case, the UE who deploy the model can do performance monitoring as baseline, but it may not be the only entity for performance monitoring. From our understanding, RAN1 is still considering at least the LMF as another monitoring entity for UE-side model case.

The discussion on which entity to perform performance monitoring for UE-side model case is actually associated with the discussion on what monitoring metric to use for the performance monitoring.

- Model output based: This type is most straightforward one which is based on the comparison between model output and label. For this, the monitoring entity should be able to have both model output (e.g., estimated location from AI/ML) and label (e.g., known location or estimated location from other methods).

- Model input based: This type is using the feature of input data matching or not to decide whether the AI model is still applicable to the current situation. This is more like stochastic way to monitor the model, which may require the AI model shows some certain connection with the input data characteristic, e.g., certain distribution etc.

- Other (measurement) based: This type is usually consisting of side information which associates the AI model. Such side information could be reflected by other measurement instead of the measurement for data input. For example, if certain SNR level or RSRP level shows for an AI model characters, based on the measurement of the non-AI used reference signals can also represents the current SNR or RSRP level, which could be used for the monitoring.

In RAN1#116bis, it is agreed to study the following options for label-based (model output based) monitoring.

- Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF.

Since those options are still under discussion in RAN1, we can just wait for RAN1’s conclusion on this to avoid any potential duplicated work.

Proposal. 7: For AI/ML POS Case1, RAN2 wait for RAN1’s progress on model performance monitoring.

2.4 Functionality management

In RAN2#125bis, RAN2 agreed the following for common LCM framework/signaling.

Agreements: 1 For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). 2 “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

Open issue is whether/how we support “UE autonomous, decision reported to the network” and “Network decision, UE-initiated”. In our understanding, one difference between “UE autonomous, decision reported to the network” and “Network decision, UE-initiated” would be who makes final decision. In case of “UE autonomous, decision reported to the network”, UE applies decision and report to gNB i.e. UE makes final decision, while in case of “Network decision, UE-initiated”, UE requires to get gNB decision to update functionality.

For “UE autonomous, decision reported to the network”, we can consider the following scenario in the POS case 1.

- The LMF can request UE to estimate its location using AI/ML model via LPP RequestLocationInformation message.

- The UE performs the location estimation using AI/ML model, but the performance monitoring result can be ‘fail’.

- The UE can determine to fall back to the legacy positioning method (i.e., “UE autonomous decision”) and reports the estimated location with the fall back indication (i.e., “report to the network”) via LPP ProvideLocationInformation.

For “Network decision, UE-initiated”, we can consider the following scenario for the POS case 1.

- The LMF can request UE to estimate its location using AI/ML model via LPP RequestLocationInformation message.

- The UE performs the location estimation using AI/ML model, but the performance monitoring result can be ‘fail’.

- The UE reports some error cause (e.g., performanceMonitoringFail) via LPP ProvideLocationInformation (i.e., “UE-initiated”).

- The LMF can determine to fall back to some legacy positioning method (i.e., “NW decision”). The LMF requests UE to estimate its location using the legacy method via LPP RequestLocationInformation message.

- The UE reports the estimated location via LPP ProvideLocationInformation.

In the expected signalling flow above, it is assumed that the UE performs the monitoring since both UE autonomous decision and initiation from UE make sense only if the performance monitoring is done at UE side. On the other hand, for “network decision, network-initiated”, it is more practical with LMF-side performance monitoring.

For the UE-side monitoring case, both “UE autonomous, decision reported to the network” and “Network decision, UE-initiated” can work, but the latter can bring additional delay from step4/5 in POS procedure. Thus, in our view, “UE autonomous, decision reported to the network” can be prioritized over “Network decision, UE-initiated”.

Meanwhile, according to the latest FL summary on POS [2], RAN1 is still discussing the following options for functionality/model management for label-based model performance monitoring.

For label-based model performance monitoring of AI/ML positioning Case 1, study the following options: - Option 1: the target UE makes the functionality/model management decision/recommendation (e.g., activate or deactivate a model, update a model, fallback to legacy positioning methods) as a result of model performance monitoring. - The target UE notifies the LMF of its functionality/model management decision/recommendation. - Option 1 is at least applicable if Option A (The target UE side performs monitoring metric calculation) is supported. - Option 2: LMF makes the functionality/model management decision (e.g., model deactivation, fallback) as a result of model performance monitoring, and notifies the target UE. - Option 2 is at least applicable if Option B (The LMF performs monitoring metric calculation) is supported.

Based on the options on the table in RAN1, it is likely that both Option 1 (“UE autonomous, decision reported to the network”) and Option 2 (“network decision, network-initiated”) can be studied further. Meanwhile, “Network decision, UE-initiated” seems to be excluded. Since RAN1 is still studying the feasibility of those options and do not make any conclusion/agreement yet, it is too early for RAN2 to discuss the detailed signaling procedure for functionality management for Case 1.

Proposal. 8: For AI/ML POS Case1, RAN2 wait for RAN1’s progress on functionality management.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

General aspect:

Proposal. 1: For POS use cases, RAN2 can prioritize the discussion on the case 1 over other cases.

UE capability/applicability reporting:

Proposal. 2: For POS Case 1, the UE determines whether a functionality is applicable as a baseline.

Proposal. 3: For POS Case 1, LMF provides NW-side additional conditions to UE via LPP signaling (e.g., LPP ProvideAssistanceData) as a baseline if needed.

Observation. 1: LPP capability indication procedure allows UE to provide unsolicited capabilities to LMF. I.e., UE dynamically report its capability without any request from LMF.

Proposal. 4: For POS Case 1, UE reports its applicable AI/ML functionalities via LPP ProvideCapabilities message.

Model selection:

Proposal. 6: For POS Case 1, RAN2 assume UE decides/selects which model to use among applicable models for the configured functionality.

Performance monitoring:

Proposal. 7: For AI/ML POS Case1, RAN2 wait for RAN1’s progress on model performance monitoring.

Functionality management:

Proposal. 8: For AI/ML POS Case1, RAN2 wait for RAN1’s progress on functionality management.

4	Reference

- RP-240774, ‘Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface’, Qualcomm

- R1-2407269, ‘Summary #3 of specification support for positioning accuracy enhancement’, Ericsson