3GPP TSG-RAN2#127bis									         R2-2408530

Hefei, China, Oct 14th – 18th, 2024

Source: 			ZTE Corporation, Sanechips

Title: 	LCP enhancements for XR

Agenda item:	8.7.4.1

Document for: 	Discussion and Decision

# Introduction

Rel-19 XR WI in RP-240791 has an objective for RAN2 to study and if justified specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity.

In RAN2#125bis meeting, the following related agreements has been made [2]:

- RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied: - Alternative 1: Enhance LCP restrictions/LCH selection. - Alternative 2: Enhance LCH prioritization. - RAN2 should consider potential impact on traffic from SRBs.

In RAN2#126 meeting, the following related agreements has been made [3]:

LCP enhancements – LCH prioritization - Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. - The solution should consider impact on UE complexity (as already indicated in SI objective description) - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data - FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH - We try to avoid RAN1 impacts. - RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. - For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial - The solutions should not disallow non-delay critical data from using an UL grant. - LCP prioritization within a logical channel will not be considered in RAN2 discussions - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). DSR enhancements – multiple thresholds and grouping of data - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included

In RAN2#127 meeting, the following related agreements has been made [4]:

LCP Enhancement - RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. - FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3). Enhanced DSR contents - Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. - For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In this contribution we will further investigate the possible scheduling enhancements per this objective.

# Discussion

# LCP operation and scheduling enhancements

Logical channel prioritisation in NR relies on controlling the uplink rate for logical channels using the following:

- A priority: Logical channels are served in the order of priority in general

- A prioritised bit rate (PBR):  Is used to ensure relevant logical channel is served up to its PBR. Ensures that logical channels with lower priority are not starved.

- The time elapsed since Bj was last incremented (e.g. resource allocated for the logical channel)

- Logical channel restrictions: Ensures that certain physical resources are reserved for a given set of logical channels

In normal circumstances (stable radio conditions, no congestion), the existing tools in NR MAC are sufficient to ensure guaranteed quality of service for GBR services whilst ensuring best effort service for other logical channels. However, during temporary congestion situations and/or bad radio conditions, there may be situations where the current mechanisms may be not sufficient. Specifically, since the time elapsed is considered, the LCHs with small delay budget cannot be scheduled with higher priority than the LCHs with large delay budget (e.g. the smaller the delay budget is, the smaller the remaining time is. But the delay budget cannot be expressed by the elapsed time). The consequence of this is that in such situations, the data with low remaining time may be essentially discarded even before the actual situation causing congestion or bad radio conditions gets better. This leads to poor quality of service and may have overall negative impact to the applications. Applications may react to this temporary packet discards by reducing the overall throughput and/or overall codec mode which may not necessarily be needed in the end.

In Rel-18, the delay status report mechanism is specified to inform the network about such situations. However, whilst the gNB scheduler may react to a DSR by providing grants to the logical channels that are starving (e.g. scheduling grant with higher priority for the LCHs with less remaining time), it is important to ensure that discarding of packets unnecessarily is avoided (e.g. UE should be scheduled with higher priority for the LCHs with less remaining time). It should be noted that mechanisms such as DSR and/or packet discard etc are best to deal with temporary dip in radio conditions or congestion. If in general, there is a (more permanent) problem of capacity over the air interface or if the radio conditions deteriorate to an extent where current bit rates cannot be sustained, it is best for the RAN to eventually let packets to be discarded such that upper layers reduce the offered throughput. However, such upper layer mechanisms should not be invoked for temporary situations.

Observation 1: Under normal conditions current LCP mechanism in NR works generally well to ensure that the quality of service requirements for XR applications are met.

Observation 2: If the radio conditions deteriorate over longer term and/or there is a larger capacity problem in the network, other means such as indication to higher layers to reduce the offered through put etc should be adopted instead.

Observation 3: When temporary congestion and/or poor radio conditions are encountered, avoiding unnecessary packet discard by minor modifications to the LCP procedure may be useful.

When studying modifications to LCP, it is important to ensure that this doesn’t result in unforeseen consequences to other services. Specifically, it should be noted that LCP is used also to ensure that SRBs are prioritised over DRBs and certain MAC CEs are sent with higher priority than data. It should be kept in mind that during poor radio conditions there may be RAN level signalling data and/or MAC CEs which may be triggered (e.g. measurements/handover signalling etc). Allowing this signalling to go through unimpeded is important because this can resolve the poor radio conditions which actually cause the problems in the first place. Any delay to such signalling will prolong the adverse conditions at radio level and will only have overall negative consequences. In order to avoid impacts to signalling connection between UE and network it is important to ensure that any enhancements to LCP do not impact the priorities of SRBs over DRBs

Observation 4: During temporary deterioration of radio conditions, additional RAN level signalling may be generated (e.g. measurements/HO signalling) and this signalling is essential to quickly alleviate the problems in the first place.

In RAN2#127 meeting, it has been agreed that:

- RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. - FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3).

To improve the transmission for delay critical data(e.g. data with remaining time below a threshold), enhancements can be considered for the resource allocation stage of LCP: e.g. treat the priority of the logical channel with delay critical data (e.g. the data that the remaining time is below a threshold) is higher than the priority of the logical channel without delay critical data, and then allocate the resource based on LCH priority. This impacts specification less and does not impact the scheduling priority for non delay critical data.

Proposal 1: Introduce a remaining time threshold for LCP enhancement.

Proposal 2: At the stage for allocation of resources in LCP, treat the priority of the logical channel with delay critical data (e.g. the data that the remaining time is below the remaining time threshold) is higher than the priority of the logical channel without delay critical data; and then allocate the resource based on LCH priority.

# Conclusion and proposals

The following observations/proposals are made:

Observation 1: Under normal conditions current LCP mechanism in NR works generally well to ensure that the quality of service requirements for XR applications are met.

Observation 2: If the radio conditions deteriorate over longer term and/or there is a larger capacity problem in the network, other means such as indication to higher layers to reduce the offered through put etc should be adopted instead.

Observation 3: When temporary congestion and/or poor radio conditions are encountered, avoiding unnecessary packet discard by minor modifications to the LCP procedure may be useful.

Observation 4: During temporary deterioration of radio conditions additional RAN level signalling may be generated (e.g. measurements/HO signalling) and this signalling is essential to quickly alleviate the problems in the first place.

Proposal 1: Introduce a remaining time threshold for LCP enhancement.

Proposal 2: At the stage for allocation of resources in LCP, treat the priority of the logical channel with delay critical data (e.g. the data that the remaining time is below the remaining time threshold) is higher than the priority of the logical channel without delay critical data; and then allocate the resource based on LCH priority.

# References

- RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#103

- Draft_R2_125bis_Meeting_Report_v2

- Draft_R2_126_Meeting_Report_v2

- Draft_RAN2_127_Meeting_Report_v1