3GPP TSG-RAN2#127bis									         R2-2408531

Hefei, China, Oct 14th – 18th, 2024

Source: 			ZTE Corporation, Sanechips

Title: 	DSR enhancements for XR

Agenda item:	8.7.4.2

Document for: 	Discussion and Decision

# Introduction

Rel-19 XR WI in RP-240791 has an objective for RAN2 to study and if justified specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity.

In RAN2#125bis meeting, the following related agreements has been made [2]:

- RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied: - Alternative 1: Enhance LCP restrictions/LCH selection. - Alternative 2: Enhance LCH prioritization. - RAN2 should consider potential impact on traffic from SRBs.

In RAN2#126 meeting, the following related agreements has been made [3]:

LCP enhancements – LCH prioritization - Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. - The solution should consider impact on UE complexity (as already indicated in SI objective description) - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data - FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH - We try to avoid RAN1 impacts. - RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. - For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial - The solutions should not disallow non-delay critical data from using an UL grant. - LCP prioritization within a logical channel will not be considered in RAN2 discussions - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). DSR enhancements – multiple thresholds and grouping of data - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included

In RAN2#127 meeting, the following related agreements has been made [4]:

LCP Enhancement - RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. - FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3). Enhanced DSR contents - Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. - For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In this contribution we will further investigate the possible DSR enhancements per this objective.

# Discussion

# DSR enhancements

It has been agreed in RAN2#127 meeting that network should be able to configure multiple remaining time thresholds for each LCG to report multiple pairs of remaining time and buffer sizes per LCG, but there will be a single triggering threshold. The value range of remainingTimeThreshold-r18   is INTEGER (1..64) ms. Very small remaining times (e.g. 1ms, 2ms etc ) are not useful as there is not enough time to report these such that network can receive the DSR, process this and respond with scheduling. Considering the remaining useful range of the remaining times, at most three remaining time thresholds seem a reasonable choice of split and this. Only these configured remaining times along with the buffered data associated with these remaining times will be reported. For each of the portions reported, the eDSR should report the corresponding buffer size and the shortest remaining time among the PDCP SDUs in this portion, which is shown in the following figure. And for flexibility, variable number of remaining time thresholds (e.g.1, 2 or 3 thresholds) should be allowed. The total  number of remaining time thresholds to be used can be configurable.

Proposal 1: For DSR enhancement, at most three remaining time thresholds can be configured for one LCG, and variable number of remaining time thresholds are allowed.

Figure 1: Example for 3 pairs of remaining time and related Data volume in eDSR

It has been agreed in RAN2#127 meeting that there will be a single eDSR triggering threshold. Considering that although the multiple pairs of remaining time and buffer size information can provide a more comprehensive data volume picture to the network, which is helpful for radio resource allocation from long term point of view, only the buffer size with remaining time below the smallest threshold is critical for scheduler. Thus, the smallest threshold should be used to trigger the eDSR.

Proposal 2: The smallest threshold should be used to trigger the eDSR.

Since the delay critical data definition is used to identify the data volume to be reported in (e)DSR, the usage is not changed after introducing multiple remaining time thresholds, the delay critical data definition is not impacted due to multiple reporting thresholds in the DSR, except the parameter name update of the remaining timer threshold(s), if necessary.

Proposal 3: The delay critical data definition is not impacted due to multiple reporting thresholds in the DSR, except the parameter name update of the remaining timer threshold(s), if necessary.

Considering that the non-delay critical data can be reported by BSR, it is not necessary to report the non-delay critical data in eDSR.

Proposal 4: The non-delay critical data ahead of delay critical data is not included in the buffer size calculation for eDSR.

To report with multiple pairs of remaining time and buffer size for one LCG, DSR MAC CE should be enhanced to report multiple pairs of remaining time and buffer size for one LCG. Considering that there is not enough “R” bits to extend the DSR MAC CE to report multiple pairs of remaining time and buffer size for one LCG, one of the following options can considered to report multiple pairs of remaining time and buffer size for one LCG:

Option 1: Use multiple DSR MAC CEs simultaneously to report multiple pairs of remaining time and buffer size, e.g.

| DSR MAC CE 1 | DSR MAC CE 2 | ... |
| --- | --- | --- |

DSR MAC CE 1 includes pair of remaining time and buffer size with remaining time is less than the remaining time threshold 1

DSR MAC CE 2 includes pair of remaining time and buffer size with remaining time is less than the remaining time threshold 2 but equal to or larger than the remaining time threshold 1

...

Option 2: define new DSR MAC CE as follows:

| R | LCG ID |  | F4 | F3 | F2 | F1 | Oct 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BT1 | R | Remaining Time 1 |  |  |  |  | Oct 2 |
| Buffer size1 |  |  |  |  |  |  | Oct 3 |
| BT2 | R | Remaining Time 2 |  |  |  |  | Oct 4 |
| Buffer size2 |  |  |  |  |  |  | Oct 5 |
| BT3 | R | Remaining Time 3 |  |  |  |  | Oct 6 |
| Buffer size3 |  |  |  |  |  |  | Oct 7 |
| BT4 | R | Remaining Time 4 |  |  |  |  | Oct 8 |
| Buffer size4 |  |  |  |  |  |  | Oct 9 |

Enhanced DSR MAC CE for reporting multiple pairs of remaining time and buffer size

In which:

-	LCG ID: This field identifies the group of logical channel(s) whose DSR is being reported.

-	Fi: This field indicates whether the pair of remaining time i and buffer size i are present. Fi is set to 0 to indicate that the Oct including BTi, remaining time i and buffer size i are absent; Fi is set to 1 to indicate that the Oct including BTi, remaining time i and buffer size i are present;

-	Remaining Time i: This field indicates the shortest remaining value of running PDCP discardTimer among all PDCP SDUs that are buffered for an LCG, with remaining time less than the remaining time threshold i but equal to or larger than the remaining time threshold i-1, but have not been transmitted in any MAC PDU, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. The length of this field is 6 bits. This field is present only if the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the value r in this field indicates a remaining time within the range of (r, r + 1] msec;

-	BT i: This field is present only if the corresponding LCG is configured with additionalBS-TableAllowed and the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the BT field set to 1 indicates that the buffer sizes with finer granularity are used to set the value of the Buffer Size field, while the BT field set to 0 indicates that the buffer sizes specified in legacy Table are used instead;

-	Buffer Size i: The Buffer Size i field indicates the total amount of UL data with remaining time less than the remaining time threshold i but equal to or larger than the remaining time threshold i-1 for an LCG after the MAC PDU has been built.

-	R: Reserved bit, set to 0.

It can been seen that using multiple DSR MAC CE simultaneously in one MAC PDU to report multiple pairs of remaining time and buffer impacts specification less.

Observation 1: Using multiple DSR MAC CEs simultaneously in one MAC PDU to report multiple pairs of remaining time and buffer has less specification impact than defining a new DSR MAC CE with multiple remaining times.

UE should report it’s capability on supporting enhanced DSR. Only if the UE reports it’s capability, multiple remaining time threshold for one LCG can be configured for the UE.

Proposal 5: To enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG, the following aspects are specified:

- Using multiple DSR MAC CEs simultaneously in one MAC PDU to report multiple pairs of remaining time and buffer, e.g. the first DSR MAC CE reports the pair of remaining time and buffer size corresponds to the first remaining time threshold, the second DSR MAC CE reports the pair of remaining time and buffer size corresponds to the second remaining time threshold, etc.

- Introduce UE capability on support enhanced DSR.

# Conclusion and proposals

The following observations/proposals are made:

Observation 1: Using multiple DSR MAC CEs simultaneously in one MAC PDU to report multiple pairs of remaining time and buffer has less specification impact than defining a new DSR MAC CE with multiple remaining times.

Proposal 1: For DSR enhancement, at most three remaining time thresholds can be configured for one LCG, and variable number of remaining time thresholds are allowed.

Proposal 2: The smallest threshold should be used to trigger the eDSR.

Proposal 3: The delay critical data definition is not impacted due to multiple reporting thresholds in the DSR, except the parameter name update of the remaining timer threshold(s), if necessary.

Proposal 4: The non-delay critical data ahead of delay critical data is not included in the buffer size calculation for eDSR.

Proposal 5: To enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG, the following aspects are specified:

- Using multiple DSR MAC CEs simultaneously in one MAC PDU to report multiple pairs of remaining time and buffer, e.g. the first DSR MAC CE reports the pair of remaining time and buffer size corresponds to the first remaining time threshold, the second DSR MAC CE reports the pair of remaining time and buffer size corresponds to the second remaining time threshold, etc.

- Introduce UE capability on support enhanced DSR.

# References

- RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#103

- Draft_R2_125bis_Meeting_Report_v2

- Draft_R2_126_Meeting_Report_v2

- Draft_RAN2_127_Meeting_Report_v1