3GPP TSG-RAN2#127bis										             R2-2408532

Hefei, China, Oct 14th – 18th, 2024                            Revision of R2-2406456

Source: 			ZTE Corporation, Sanechips

Title: 	RLC enhancements for XR

Agenda item:	8.7.5

Document for: Discussion and Decision

# Introduction

Rel-19 XR WI[1] has an objective for RAN2 to specify RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. .

In RAN2#125bis meeting, the following agreements have been made[2]:

- We focus on RLC AM - RAN2 will analyse solutions to ensure timely RLC retransmission(s) for XR - RAN2 will analyse how to avoid unnecessary retransmissions (e.g. to avoid reTx of out-dated packets)

In RAN2#126 meeting, the following agreements have been made[3]:

- For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach - For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side - For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned - To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism - Impact on capacity should be considered - RAN2 focuses on the enhancements for UL traffic

In RAN2#127 meeting, the following agreements have been made[4]:

Unnecessary retransmissions – Rx and Tx approach clarifications - Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first. - In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer. - In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

In this contribution, we will further discuss the RLC re-transmission related enhancements and give our proposals.

# Discussion

Issue 1: Timely retransmissions on RLC layer for XR traffic
When an RLC PDU containing an RLC SDU segment is lost, the receiving RLC entity waits for the lost PDU for RLC SDU reassembly, and only the reassembled RLC SDU can be delivered to upper layer (e.g. PDCP layer and Application layer).

For the RLC AM mode, if an RLC PDU(e.g. RLC SDU segment) is missed, RLC status PDU will be used to trigger the re-transmission, but the current RLC Status PDU can only be triggered based on “poll” bit or based on t-reassembly timer, it cannot be triggered in a timely manner for delay critical service. E.g. if “poll” bit is set too frequently but there is no PDU missing, frequent RLC status PDUs will waste unnecessary radio resource; if “poll” bit is not set frequently but there is PDU missing, the lost PDU cannot be retransmitted timely. And the value range of t-reassembly timer is not finer(e.g. ms0, ms5, ms10, ms15, ms20, ms25, ms30,...) enough to trigger the RLC status PDU timely.

Observation 1.  The current RLC retransmission mechanism cannot provide feedback in a timely manner for delay critical service.

In the RAN2#126 meeting, the following agreements have been made:

- To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism.

For the autonomous retransmission, the following solutions have been proposed:

- blind re-transmission of RLC PDU

- RLC transmission triggered by HARQ NACK

For blind re-transmission of RLC PDU: if the transmitting entity does not know whether a RLC PDU has been transmitted unsuccessfully and always re-transmission of every RLC PDU, the re-transmission of the RLC PDU that has already been transmitted successfully will waste the radio resource. Especially for XR service will large data rate, the blind re-transmission of every PDU will impact the network capacity.

Observation 2: Blind re-transmission of RLC PDU will waste radio resource and impact network capacity.

For RLC transmission triggered by HARQ NACK: since the HARQ NACK is explicitly indicated by NDI in PDCCH for UL HARQ procedure, and based on NDI in PDCCH, UE cannot determine that the PDU is transmitted unsuccessfully, or the HARQ re-transmission times reaches the maximum HARQ retransmission limitation. Thus, based on UL HARQ NACK, UE cannot determine whether a UL PDU is transmitted unsuccessfully and can not be used to trigger RLC retransmission timely.

Observation 3: Based on UL HARQ NACK, UE cannot determine whether a UL PDU is transmitted unsuccessfully and can not be used to trigger RLC retransmission in a timely manner.

To improve the RLC AM PDU re-transmission efficiently, the following mechanism can be considered:

Option 1: When the receiving RLC entity detects a missing RLC PDU (e.g. an RLC segment gap is detected), it immediately triggers the RLC status PDU to trigger the retransmission of the missing RLC PDU.

Option 2: When the receiving RLC entity detects an RLC PDU (e.g. RLC SDU or RLC SDU segment) missing, it starts a timer for waiting for the missed RLC PDU (e.g. a prohibit timer in the receiving side of an AM RLC entity in order to prohibit transmission of a STATUS PDU for a duration to wait for the missed RLC PDU). When the timer expires, the RLC STATUS PDU can be triggered.

Option 2 can trigger the RLC PDU retransmission in a timely fashion for delay critical service and can avoid unnecessary RLC STATUS PDUs based on network configuration (of the prohibit timer). In fact, Option 1 can be realized with option2 by setting the prohibit timer to zero. Thus, we prefer option 2 which is more flexible.

For option 2, the t-Reassembly timer can be reused, but some finer values(e.g. ms2, ms3, ms4, ms6, ...) should be extended.

Proposal 1: Timer with finer value range triggering RLC STATUS report is supported. e.g. when the receiving RLC entity detects an RLC PDU missing, it starts a timer, when the timer expires and the missing PDU has not been received yet, an RLC STATUS PDU will be triggered.

When there are a large number of data PDUs to be transmitted in the receiving RLC entity, the Data PDUs may block the STATUS PDU transmission, in which case the STATUS PDU can will not be transmitted in a timely manner and retransmissions cannot be triggered. To overcome this issue, RLC STATUS PDU should be prioritized over RLC data PDUs during LCP in MAC layer.

Proposal 2: RLC STATUS PDU should be prioritized over RLC data PDUs during LCP in MAC layer.

Issue 2: For avoiding unnecessary RLC AM retransmissions

In some cases, the transmitting RLC entity may discard some RLC PDUs, , e.g.

Case 1: Discard based on indication from PDCP layer when the corresponding PDCP SDUs are discarded for the discardTimer or discardTimerForLowImportance expiring.

Case 2:  Discard based on the maxRetxThreshold, e.g. the RLC retransmission times have reached the maxRetxThreshold

In these cases, the receiving RLC entity cannot know that the TX RLC entity has discarded a PDU in timely manner, which may cause the RLC RX window stalling and may trigger unnecessary RLC status report. To deal with this issue, the following agreements have been made in RAN2#126:

- For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach - For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side - For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned

For the case 1, the possible solutions are as follows:

- Rx initiated approach: the transmitting PDCP entity sends the PDCP SN gap report to the receiving PDCP entity, and the receiving PDCP entity can notify the receiving RLC entity about the obsolete SDUs based on existing mechanism.

- Tx initiated approach: the transmitting PDCP entity indicates the transmitting RLC entity about the obsolete SDUs based on existing mechanism, and the transmitting RLC entity notifies the obsolete SDUs to the receiving RLC entity using RLC SDUs GAP report, similar as the PDCP SN gap report.

For the case 2, the possible solutions as follows:

- Rx initiated approach: When the receiving RLC entity determines that the retransmission times of some RLC SDU segment reachs the maxRetxThreshold , it indicates to upper layers(the transmitting PDCP entity) that max retransmission has been reached, the transmitting PDCP entity sends the PDCP SN gap report to the  receiving PDCP entity, and the receiving PDCP entity can notify the receiving RLC entity about the obsolete SDUs based on existing mechanism.

- Tx initiated approach: the transmitting RLC entity notifies the obsolete SDUs to the receiving RLC entity using RLC SDUs GAP report, similar as the PDCP SN gap report.

Observation 4: Both the Rx initiated approach and the Tx initiated approach can work for avoiding unnecessary RLC AM retransmissions and notifying the receiving RLC entity to update RLC RX window.

Considering that Rx initiated approach depends on the PDCP SN gap report, and the PDCP SN gap report is triggered only when there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s), that means if there is no new PDCP SDU coming, the PDCP SN gap report will not be triggered even a RLC SDUs is discarded, which is not timely for the receiving RLC entity to update RLC RX window.

Observation 5: The Rx initiated approach cannot notify the receiving RLC entity in a timely manner to update RLC RX window.

In RAN2#127 meeting, it has been agreed that  Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach, but what type of a local timer is used should be considered,  the possible  local timer in the RX side may be one of the follows:

Option 1: PDB like Timer is used for the  local timer in the RX side, based on which to stop receiving an obsolete SDU. Considering that the RX side cannot determine the time when the PDU arrives at the Tx PDCP, and cannot know when to start the PDB like Timer, PDB like Timer is not suitable be used in Rx side.

Option 2: t-Reassembly like timer is used for the  local timer in the RX side, based on which to stop receiving an obsolete SDU. Considering that t-Reassembly is already used to trigger RLC STATUS report for RLC A AM mode, and RLC STATUS report is essential for RLC A AM mode, which is the main difference from RLC UM mode and whould not be touched.

Observation 6: both the  PDB like Timer or t-Reassembly like timer is not suitable as a local timer for Rx side to stop receiving an obsolete SDU.

Proposal 3: When the transmitting RLC entity discards some RLC segments, it notifies the obsolete SDUs information to the receiving RLC entity.

Issue 3: About RRC re-establishment/Release triggered by UP
In TS 38.322, the maximum number of retransmissions (e.g. maxRetxThreshold) to determine the maximal retransmission number; and if the retransmission count reaches the maximum number of retransmissions, the RLC entity will indicate this to upper layer (see the following context in TS 38.322).

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall: - if the RLC SDU or RLC SDU segment is considered for retransmission for the first time: - set the RETX_COUNT associated with the RLC SDU to zero. - else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU: - increment the RETX_COUNT. - if RETX_COUNT = maxRetxThreshold: - indicate to upper layers that max retransmission has been reached.

And based on the TS 38.331, when the indication from SCG RLC that the maximum number of retransmissions has been reached is received, RLC failure report, SCG radio link failure report or RRC re-establishment procedure will be triggered; when the indication from MCG RLC that the maximum number of retransmissions has been reached, RLC failure report or RRC connection reestablishment will be triggered; The RRC re-establishment procedure or RRC connection release may impact the performance of delay critical services.

In XR, multiple QoS flows may be used simultaneously, some flows may provide delay critical service, and some flow may provide non-delay critical / best effort services. To guarantee the user experience, some relaxation for non-delay critical service can be enabled for triggering the RLC failure. e.g. a low priority logical channel may not trigger RRC re-establishment procedure or RRC connection release when the re-transmission number reaches the maxRetxThreshold.

Proposal 4: When delay critical services are running concurrently with non-delay critical/best effort services, it should be possible for the RLC bearers for non-delay critical services to be configured such that connection reestablishment is not triggered even if maxRetxThreshold is reached.

# Conclusion

The following observations/proposals are made:

Observation 1.  The current RLC retransmission mechanism cannot provide feedback in a timely manner for delay critical service.

Observation 2: Blind re-transmission of RLC PDU will waste radio resource and impact network capacity.

Observation 3: Based on UL HARQ NACK, UE cannot determine whether a UL PDU is transmitted unsuccessfully and cannot be used to trigger RLC retransmission in a timely manner.

Observation 4: Both the Rx initiated approach and the Tx initiated approach can work for avoiding unnecessary RLC AM retransmissions and notifying the receiving RLC entity to update RLC RX window.

Observation 5: The Rx initiated approach cannot notify the receiving RLC entity timely to update RLC RX window.

Observation 6: Both the PDB like Timer or t-Reassembly like timer is not suitable as a local timer for Rx side to stop receiving an obsolete SDU.

Proposal 1: Timer with finer value range triggering RLC STATUS report is supported. e.g. when the receiving RLC entity detects an RLC PDU missing, it starts a timer, when the timer expires and the missing PDU has not been received yet, an RLC STATUS PDU will be triggered.

Proposal 2: RLC STATUS PDU should be prioritized over RLC data PDUs during LCP in MAC layer.

Proposal 3: When the transmitting RLC entity discards some RLC segments, it notifies the obsolete SDU information to the receiving RLC entity.

Proposal 4: When delay critical services are running concurrently with non-delay critical/best effort services, it should be possible for the RLC bearers for non-delay critical services to be configured such that connection reestablishment is not triggered even if maxRetxThreshold is reached.

# References

- RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#103

- Draft_R2_125bis_Meeting_Report_v1

- Draft_R2_126_Meeting_Report_v2

- Draft_RAN2_127_Meeting_Report_v1