3GPP TSG-RAN2#127bis										        R2-2408534

Hefei, China, Oct 14th – 18th, 2024

Source: 			ZTE Corporation, Sanechips

Title: 	Initialization of DRX_SFN_COUNTER at handover

Agenda item:	7.5.3

Document for: Discussion and Decision

# Introduction

- At RAN2#126 it was agreed the UE will initialise the DRX_SFN_COUNTER to 1 if the drx-TimeReferenceSFN is included in the RRC (re-)configuration which is received during the first half of hyper frame and to 0 otherwise. At RAN2#127, there were some proposals to optimize the DRX_SFN_COUNTER initialization in HO case, and it was agreed that we leave this up to network implementation, unless a solution that works in all scenarios (CHO/LTM/Normal HO/Reconfiguration etc) and does not require network to send an extra RRC reconfiguration message can be agreed at next meeting.

- In this tdoc we will discuss this issue and propose a way forward.

# Discussion

For the CHO and LTM procedure, the DRX parameter and drx-referenceSFN will have been configured to UE before the HO execution, and the configuration applies to the source serving cell. In these cases:

- For intra gNB-DU LTM and CHO, since the traffic pattern is unchanged and the timing between target cell and the source cell is not changed, both UE and gNB can maintain DRX-SFN-COUNTER, and DRX-SFN-COUNTER re-initialization is not necessary in these cases.

- For inter gNB-DUs LTM and CHO:  if there is timing mis-alignment between source cell and target cell, DRX-SFN-COUNTER re-initialization will be necessary. But the drx-referenceSFN configuration and LTM/CHO execution may across one or more SFN boundary (e.g. SFN wraparound may have be occurred already),  use of the drx-referenceSFN configured to initialize the  DRX-SFN-COUNTER during the LTM/CHO execution procedure is not suitable, and initializing the  DRX-SFN-COUNTER in this case is unclear.

Observation 1: if there is timing mis-alignment between source cell and target cell, initialization of the DRX-SFN-COUNTER during the LTM/CHO execution procedure is insufficient since the traffic periodicity may be mis-aligned with the DRX cycle after HO anyway.

For legacy HO triggered by RRCReconfiguration, since the DRX parameters and drx-referenceSFN are configured by target gNB, the target gNB and UE can initialize the DRX-SFN-COUNTER based on the target gNB's SFN.

Observation 2: For legacy HO triggered by RRCReconfiguration (i.e. no CHO/LTM), the target gNB and UE can initialize the DRX-SFN-COUNTER based on the target gNB's SFN.

Thus, for normal HO case (i.e. no LTM/CHO) there is no issue as long as the DRX-SFN-COUNTER is initialized using the target cell SFN and for all other HO cases, it seems the best thing is to leave this up to network implementation since there will any way be a reconfiguration needed to align the traffic periodicity with the DRX timing in the new target cell. Based on this, it is proposed to decide between the following options:

Proposal: RAN2 to agree one of the following options:

- Option 1:  In case of handover, the UE uses the target cell SFN as reference for initializing the DRX_SFN_COUNTER and if there is still any ambiguity left (e.g. CHO/LTM cases), the handling is left to network implementation.

- Option 2: No changes are done and everything is left to network implementation

# Conclusion

In this contribution the following observations and proposals are made:

Observation 1: if there is timing mis-alignment between source cell and target cell, initialization of the DRX-SFN-COUNTER during the LTM/CHO execution procedure is insufficient since the traffic periodicity may be mis-aligned with the DRX cycle after HO anyway.

Observation 2: For legacy HO triggered by RRCReconfiguration (i.e. no CHO/LTM), the target gNB and UE can initialize the DRX-SFN-COUNTER based on the target gNB's SFN.

Proposal: RAN2 to agree one of the following options:

- Option 1:  In case of handover, the UE uses the target cell SFN as reference for initializing the DRX_SFN_COUNTER and if there is still any ambiguity left (e.g. CHO/LTM cases), the handling is left to network implementation – CR in R2-2408528

- Option 2: No changes are done and everything is left to network implementation

# References

- Draft_RAN2_127_Meeting_Report_v1