3GPP TSG-RAN WG2#127bis											    R2-2408542

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.2

Source: 	Quectel

Title:	        Energy status report for Ambient-IoT

Document for:	 Discussion

- Introduction

The following agreements have been made in RAN2 meetings [1][2]:

RAN2#126 1 RAN2 will study the following cases for AIoT paging message: • a message containing an ID of a single A-IoT device. • a message containing a group ID that maps to multiple A-IoT devices. • a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. • a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details. RAN2#127 Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices.

In this contribution, we will discussion the energy status report based on these agreements.

- Discussion

  - The necessary of energy status report

Once receiving paging request from CN, the AIoT reader shall multiple AIoT paging message to device(s), and every paging message indicate information from which the device can determine resources to be used for response. If target device could not receive paging, or could not respond to paging because of power level, the reserved wireless resource for this device is wasted. Therefore, it has been agreed in RAN2 #126 meeting that device can receive as long as there is enough energy. Furthermore, from the AIoT reader point of view, if target device could respond paging as long as there is enough energy, the wireless resource reserved in paging shall be saved.

Proposal 1: Target device shall respond paging as long as there is enough energy.

The other way of saving wireless resources is due to AIoT reader. If AIoT reader could acquire the information when the target AIoT has enough energy to respond the paging, it could select the proper occasion to transmit the paging. For this proper paging occasion, the target device has enough energy to respond the paging.

Figure 1: AIot reader select the paging occasion based on the energy level of target device

- In the example of figure 1, AIoT reader could select the paging occasion based on the energy status report of target device, and avoid to transmit in the former 3 paging occasions unnecessarily.

- Another case is data transmission. If AIoT reader find there are many downlink packets in the buffer, and the energy of AIot device will be exhausted based on the energy status report, it shall choose some packets with high priority to transmit firstly, and inform the related CN entities to decrease the coming packet.

- In these scenarios above, the energy status report is very useful, so it is proposed:

- Proposal 2: the energy status report shall be introduced.

  - The content of energy status report

Since the object of energy status report is to help the AIoT reader to choose the proper paging occasion, and the packet of high priority to transmit firstly, the two types of information shall be included in energy status report:

- How many bits could be transmitted based on the current energy level

- When the energy is enough for paging responding

In the example of figure 2, the AIoT device indicates the energy status report to reader duration the data transmission. Based on this report, AIot reader determines the amount of data to be transmitted, and stops to the transmission at the proper occasion. When the AIot device accumulates the enough energy to receive the paging and respond, the paging message shall be triggered.

Figure 2: the content of energy status report

Proposal 3: the content of energy status report includes:

  - How many bits could be transmitted based on the current energy level

  - When the energy is enough for paging responding

  - The occasion of the energy status report

Based on figure 2, there are two triggering condition for the energy status report. Firstly. if the remaining energy is lower than a threshold, AIoT device shall trigger this energy status report, and help the AIoT reader to schedule the data transmission graciously. Secondly, if AIoT device will enter the idle state, this energy status report shall be triggered to indicate the available paging occasion to AIoT reader.

- Proposal 4: the triggering condition of the energy status report includes:

  - the remaining energy of AIoT device is lower than a threshold

  - AIoT device will enter the idle state

- Conclusion

Proposal 1: Target device shall respond paging as long as there is enough energy.

- Proposal 2: the energy status report shall be introduced.

Proposal 3: the content of energy status report includes:

  - How many bits could be transmitted based on the current energy level

  - When the energy is enough for paging responding

- Proposal 4: the triggering condition of the energy status report includes:

  - the remaining energy of AIoT device is lower than a threshold

  - AIoT device will enter the idle state

- References

[1] Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

[2] Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands, ETSI MCC