3GPP TSG-RAN WG2 Meeting #127bis	R2-2408544

Hefei, China, 14 – 18 October 2024

Agenda item:	8.1.3

Source:	Nokia

Title:	Data collection for NW-side models

WID/SID:	NR_AIML_Air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This document presents the normative discussion on data collection for network-side models relevant for Rel-19. Both beam management and positioning accuracy enhancement use cases are considered.

# 2	Discussion

## 2.1	Beam management

### 2.1.1	Basic Procedure

RAN2 agreed to enhance Immediate Minimization of Drive Test (MDT) [1] for OAM centric approaches in network-side data collection. The basic procedures of an OAM centric network-side data collection can be realized with the following steps illustrated in Figure 1. These steps will set the basis for the procedure to enable data collection configuration and reporting.

Step 1, Step 8: RAN2 can assume that data collection configuration activation can reuse the existing procedure of Immediate MDT.

Step 2: Check UE capability information (e.g. logging support).

Step 3: Configure CSI reporting framework to measure, log and report L1 measurements via RRC message.

Step 4-5: If triggering of logging is configured, UE will measure L1 measurements and log in AS layer. Otherwise, go to Step 7.

Step 6: If measurements are logged and triggering of reporting is configured, then UE will determine when to report logged data based on the reporting types.

Step 7: Transmit logged data via RRC message.

Figure 1: Basic procedure of an OAM centric data collection.

Proposal 1: RAN2 to adapt the above steps of Figure 1 as a baseline procedure for OAM centric data collection.

### 2.1.2	Framework

In the last RAN2#127 meeting [2], there were several discussions on how to enhance Immediate MDT. However, we have to recall that this enhancement is to enable data collection for network-side models for beam management use cases.  This enhancement would also be applicable to other use cases. The extension of Immediate MDT in beam management use cases, should enable L1 measurements data to be available for offline training data collection. As such, one of two plausible enhancements [3, 4] could be foreseen: a) enhancement of RRM framework; b) enhancement in CSI reporting framework.

The approach a) is based on L3 measurement frameworks and Immediate MDT is uses this framework. Extensions to this approach include, for example, the configuration of CSI resources within MeasConfig, a new logging configuration, and a new L1 measurement report configuration along with report quantity and type configurations.

The approach b), on the other hand, is based on L1 measurements. According to TS 38.331 [5], CSI-MeasConfig is used to configure CSI-RS belonging to the serving cell and report to be transmitted on PUCCH on the serving cell or PUSCH triggered by DCI received on the serving cell. Extensions to this approach may include, for example, a new logging configuration, modification in report quantity, dedicated CSI-RS resources.

Observation 1: To enable data collection at network-side for Beam Management Case 1 and Case 2 both RRM framework and CSI-Reporting framework require enhancement.

Although RRM framework has a clear benefit from mobility perspective, it is yet to be analyzed the impact of the inclusion of physical layer resources. The framework requires association of MeasID, MeasConfig and ReportConfig to enable measurement reports. To place configuration for logged CSI resources in RRM framework, for instance, a new CSI-ResourceConfiguration needs to be added within MeasConfig Information Element. The CSI reporting framework, UE measures the CSI RS resource elements and resource blocks while in RRM framework, the measurement objects are inter-RAT/inter-frequency and intra-RAT/intra-frequency related signals to be measured. Currently, the association of CSI resources with the RRM measurement object is made through servingCellMO, but the relationship neither consider a distinct MeasID nor reporting IDs determining a distinct reporting type for CSI resources.

Observation 2: CSI-Reporting framework associates with RRM measurement object, but this doesn’t support reporting according to RRM framework.

The characteristics of CSI-RS are distinct compared to RRM measurements, and it is not straightforward how these should be collocated in the same measurement configurations, due to complex dependencies of CSI resources, measurements and their reporting configuration from physical layer configuration (e.g. BWP). In order to minimize the specification effort, enhancement in CSI reporting framework would be simpler than that of in RRM framework.

Proposal 2: Reuse the CSI reporting framework for configuration of measurements and configuration of reporting for offline training data collection at network-side for beam management use cases.

### 2.1.3	Measurement Configuration

In measurement configuration, the training of network-side models for beam management will rely on the UE that receives a CSI reporting configuration containing the measurement and reporting configuration for the set of RS resources (e.g. NZP-CSI-RS resources or SSB resources). UE performs measurements on these RS resources specified in CSI-MeasConfig and reports CRI/RSRP or SSBRI/RSRP. For network-side model, in particular, L1-RSRP will be used as training data. Though pending on the addition of any new measurement quantities that may be defined, the legacy CSI reporting framework is sufficient to configure the UE to measure, collect and report beam related measurements for the training of AI/ML network-side models for beam management.

Proposal 3: CSI-MeasConfig can be used as a baseline to configure CSI-RS measurements for training data collection. Detailed configuration parameters applicability or extensions is FFS.

### 2.1.4	Measurement Report Configuration

In order to enable reporting of data collection from UE, UE may provide its capabilities to log L1 measurements and report the logged L1 measurements from UE to network via RRC message. If UE provides logging capability, then gNB can configure UE to log and then send the measurement reports which can be used for data collection for training of network-side model in beam management. In general, Immediate MDT uses an RRCReconfiguration message for activating RRM-related data collection. On the other hand, for beam management, the RRCReconfiguration message already conveys the IE CSI-ReportConfig. Therefore, the legacy CSI-ReportConfig within RRCReconfiguration message serves a good reference and baseline. It is possible to reuse the CSI-ReportConfig to configure the UE to log beam-level L1 measurements. In addition to this, CSI-ReportConfig may need to be enhanced to allow necessary trigger to enable storing these measurements within a report and transmission of this report over L3 layer.

Proposal 4: CSI-ReportConfig is enhanced with necessary information to trigger measurements report storing and measurements delivery over DCCH (L3). Detailed logging and reporting criteria are FFS.

For the beam management related measurements and the reporting, the gNB will configure UE. This configuration can reuse the existing RRCReconfiguration message used for CSI-ReportConfig to measure, log and report L1 measurements by UE with some possible extension. The RRC configurations for Immediate MDT remain agnostic to the MDT purpose, However, the trace activation and trace record reporting procedures [6] for Immediate MDT, that are triggering the gNB and enabling the reports forwarding to TCE, require supporting Trace Procedures extensions for the OAM-centric data collection. The scope of the Immediate MDT procedures extensions is in RAN3 and SA5 realm. From RAN2’s perspective, the measurement configured in UE for Immediate MDT should comply with the transferring and reconfiguration principles for the current measurements configured in UE for CSI reporting purposes.

Observation 3: The scope of the Trace Activation and Reporting extensions for Immediate MDT procedures to enable data collection for beam management is in RAN3 and SA5 realm.

Proposal 5: For data collection, RRC Reconfiguration (through CSI-MeasConfig), is used to configure the UE to log beam-level L1-RSRP measurements when logging capability is available in the UE.

From RAN1 reply LS [7], we note that the data collection for offline model training considers a relaxed latency. In many cases, if report transmission is done in real-time for both training data collection and usual network operations, both network performance and data collection can be affected. Thus, we think it is important to consider a relaxed latency requirement in the solution for report configuration for logged beam level measurement data collection.

Proposal 6: The solution for report configuration for logged beam level measurement data collection should consider relaxed latency requirement, e.g., the delay caused by using low priority SRBs is acceptable.

#### 2.1.4.1	Triggering Conditions for Data Collection Termination

Referring to the RAN2#127 [2] agreement on power state of UE as follows:

4. Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network.

Measuring the resources and logging those measurements can impact UE performance or conversely, could be impacted by the UE power. Any of these situations may relate to detecting performance degradation in the UE, while being involved in data collection for ML purposes. Even though the determination of the UE’s power state is implementation specific, the UE should notify the network about detection of the performance issue to respect the network control and network awareness of the situation. An indication would allow network to decide how to adjust the data collection for the UE (release or modify). The retrieval of this logged data is still up to network to decide.

Observation 4: Power state determination is UE implementation specific, and thus can be generalized to any degradation of the UE performance.

Proposal 7: UE shall not stop autonomously data collection termination.

Proposal 8: Degradation of the UE performance (e.g. due to power state) during data collection for ML purposes is indicated via existing information in UAI to the gNB and its up to gNB to reconfigure UE.

#### 2.1.4.2	Triggering of Reporting

At RAN2#127 [2] the following agreements were made on report triggering:

RAN2 #127 Agreements

5. FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

7. On-demand request from the network is supported.   FFS details on signalling

The support of UE to log L1 measurements would require a buffer. This means UE can provide the buffered measurements to the network in one of the following ways:

Option 1: The UE indicates the availability of the report to the network, and then the network can fetch it (similar procedure as in logged MDT).

Option 2: The UE sends the report to the network according to reporting configuration.

The first option mitigates the overload situation. In other words, the network can delay the retrieval of the report whenever there is an overload situation. The main drawback is that this option requires an additional overhead that is two extra RRC messages. On the other hand, the advantage of the second option is lower overhead. In this 2nd option, the network cannot simply stop the UE of sending a report when there is an overload situation, however, this issue can be mitigated if a low priority SRB is selected for transmission of the report. Moreover, it is a reasonable assumption that the network will not configure data collection in high load situations.

Option 1 is a good choice for UE in RRC IDLE or INACTIVE state. However, it is not well justified the ‘availability indication’ by the UE in the RRC CONNECTED state. The reason is that the network can configure the UE with the periodicity or frequency regarding when to send the report as well as network is aware of UE’s minimum storage capability. If we allow less frequent reporting with a low priority SRB, then option 2 is sufficient.

Proposal 9: RAN2 to assume that UE will be configured by the network when to send the report to the network.

In general, network can configure UE to report based on an event. Defining a new event based on UE buffer does not seem beneficial, as:

1) if the network is not aware of the AS buffer size of the UE (buffer size could be UE implementation specific), then it is unclear how the network can use this event

2) if the network is aware of the AS buffer size of the UE, then it can consider this limitation via other reporting triggers (e.g., how frequently the UE should report)

Proposal 10: AS buffer event based reporting is not useful.

In RAN#127 [2], companies proposed scenario in which network may request logged L1-RSRP measurements reporting on demand. This on-demand network request for reporting is initially a feature of logged-MDT framework in which UE information procedure is used by the network to request the UE to report information [Clause 5.7.10, 5]. Such feature can be introduced in the Immediate MDT framework to support on-demand network request in RRC CONNECTED state, where request-response RRC message pair UEInformationRequest / UEInformationResponse may be reused. However, specification impact on the current framework (e.g., UEInformationResponse message) on how the network enables on-demand L1-RSRP measurement reporting for both BM-Case1 and BM-Case2 (including different alternatives corresponding to each sub use case) needs to be discussed in RAN2.

Proposal 11. Support on-demand network request for L1 measurement reporting based on RRC UEInformationRequest and UEInformationResponse for both BM-Case 1 and BM-Case 2.

#### 2.1.4.3	SRB Selection for Reporting

Regarding to the SRB to be used for data collection reporting, the following agreement was made at RAN#127 [2]:

RAN2 #127 Agreements

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

The agreement above was to support a low priority SRB for transmission of the logged CSI measurements. In general, different SRBs associated with RRC messages enable different prioritization of the signaling messages. Since the prioritization of the signaling message may impact the control and management of overall operations, the data transmitted for offline training purposes has lower priority than the normal operations. For the lower priority SRBs stated in TS 38.331 [5], it is worth noting that SRB2 is used for carrying NAS messages and for RRC messages which include logged measurement information (MDT reports); SRB3 and SRB5 are out of scope here, as being relevant for Dual Connectivity, and SRB4 is used for RRC messages which include application layer measurement report information (QoE reports). RRC procedures considered in the scope of data collection for beam management (TR 38.843 [8]), refers to MDT or UAI frameworks.

Observation 5: SRB3 and SRB5 cannot be used because are configured for Dual Connectivity.

The problem of using an existing SRB is that the current SRBs are used for different purposes, and thus their prioritization cannot be adjusted according to the logged measurement delivery. For example, if SRB2 is selected, then it cannot be heavily deprioritized in overload situations, as SRB2 is also used for sending NAS messages, which are important to be delivered. If SRB4 is selected, then the priority of QoE reports and data collection reports cannot be set independently. Defining a new SRB for this purpose requires some more standardization efforts, but it enables the network the better control the load coming from reporting of logged beam level measurement data.

Proposal 12: RAN2 to discuss whether existing low priority SRB (SRB2, SRB4) or introduce a new SRB to be used for data collection transmission.

### 2.1.5	Multiple configuration

At RAN2#127 [2] the following agreements were made on multiple measurement configuration:

RAN2 #127 Agreements

1. As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

According to the above agreement [2], UE will receive logging configurations, measurement configurations from the network. The network can explicitly configure trigger for data collection process initiation in two different ways: immediately and UE initiated. In this regard, multiple configurations can be understood as a UE is configured with two different data collection initiation processes in RRC CONNECTED state. In addition, a dedicated RSes for data collection or the same RSes as configured for inference can be applied when it comes to configuring and triggering UE with multiple measurement configuration. Same definition could also imply that UE is configured to measure with same or different data collection RSes in parallel. If UE is configured with same data collection RSes (say a subset of resources) with multiple different data collection initiation configurations, then there will be duplicate measurements reporting. If UE is configured with different RSes in multiple configurations, the CSI computation complexity of the UE can increase. Since this will impact UE behaviour at the physical layer which would require input from RAN1. In our understanding, UE shall report what it is being configured by the network which will not require any additional spec impact.

Observation 6: Multiple configurations can be understood as a UE is configured with two different data collection initiation processes in RRC CONNECTED state.

Observation 7: Multiple configurations can be understood as a UE is configured with the same RSes used for inference can be used for multiple data collection triggers.

Observation 8: Multiple configurations can be understood as a UE is configured to measure with same or different data collection RSes in parallel.

Regardless of which configuration framework is adopted (RRM or CSI reporting framework), we observe that each of the considered procedures enable multiple instances of the configurations (of measurement resources and reporting types), with certain allowable maximum values (cf. TS 38.331 [5]).

Observation 9: Multiple instances of configurations are supported by adequate RRC configurations.

Data collection for offline training that will involve data logging will be based on the RRC configuration. Therefore, the applied procedure(s) should ensure total number of CSI-RS resources and applied configurations do not exceed the maximum allowable values (even if shared with other LCM purposes such as inference).

Proposal 13: RAN2 assumes that configurations for data collection share total allowable number of CSI-RS resources and reporting configurations for other purposes (e.g. legacy CSI-RS, Rel-19 CSI-RS for inference) and do not exceed it.

## 2.2	Data Collection for Positioning

Network-side data collection for positioning use cases in the Rel-19 AI/ML work item in RAN2 has so far not progressed much but there are some agreements in other working groups related to data collection and/or model training that can be considered for discussion and decision in RAN2. For AI/ML positioning there are five different cases with differing priority but in RAN#105 [9] it was decided that RAN1 should focus only on 1st priority positioning use cases in the remainder of the RAN1 meetings this year. Although this decision was targeting RAN1, it is good idea for RAN2 also to adopt this guidance, especially since RAN2 did not find much time to discuss positioning use cases. Amongst the 1st priority use cases, Case 1 (UE-based positioning with model on UE-side) is the only case that RAN2 can focus on while Case 3a and Case 3b, which are gNB-assisted positioning with impacts to NRPPa, should be addressed in RAN3. SA2 is also working on Case 3b which should be taken into consideration by RAN3.

Proposal 14: For AI/ML positioning, until RAN#106, RAN2 should focus their specification efforts on positioning Case 1 (UE-based positioning with model on UE-side) while leaving positioning Case 3a and Case 3b to RAN3.

For positioning Case 1, RAN1 had discussed which entities can generate the measurement and its related data (Part A components of the collected data sample for training) and ground truth label and its related data (Part B components of the collected data sample for training). The RAN1 discussions resulted in some working assumptions and some agreements as shown below:

RAN1 had made the following agreements in RAN1#116bis [10]:

Working Assumption For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE.

Working Assumption For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

RAN1 had made the following agreements in RAN1#118 [11]:

Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

Looking at the RAN1 decisions above, for training data generation of AI/ML based positioning Case 1, RAN1 agreed that ‘how’ the collected data sample is transferred is out of scope of RAN1. Also, for model performance monitoring of AI/ML positioning Case 1 RAN1 agreed to study the feasibility, benefits and potential specification impacts of different options for monitoring metric calculation but there is some signalling procedure aspects captured under the different options for monitoring metric calculation.  So, RAN2 could focus their discussion on training data transfer and check for impacts to network side data collection for positioning Case 1 and could help RAN1 by providing feedback about feasibility from signalling perspective for any of the signalling procedure aspects captured under the different options for monitoring metric calculation. The measurements and ground truth label needed for performance monitoring are similar to offline training.

Even though positioning Case 1 is about UE-side model which may involve UE-side data collection for training, since LPP signalling in TS 37.355 [12] already supports LMF forwarding of PRU information to UE (LPP is used to get PRU information from PRU and LPP is used to forward that information to target UE), it is feasible for LMF to forward ground truth label (Part B data) from PRU to UE. RAN1 also has a working assumption that ground truth label can be generated by LMF itself and this also can reuse the LPP signalling from LMF to UE to transfer the ground truth label to UE. Similarly, the channel measurement and its related data (Part A data) from a PRU can be forwarded by the LMF to the target UE.

Proposal 15: For positioning Case 1, LPP signalling is used for transfer of ground truth label and related data (Part B data) and channel measurement and related data (Part A data) from PRU to LMF. FFS: details of LPP signalling.

Proposal 16: For positioning Case 1, LPP signalling is used for transfer of ground truth label and related data (Part B data) from LMF to target UE, for both PRU generated and LMF generated ground truth label and related data. FFS: details of LPP signalling.

Proposal 17: For positioning Case 1, LPP signalling is used by LMF to forward the channel measurement and related data (Part A data), generated by a PRU, to the target UE. FFS: details of LPP signalling.

In the current LPP protocol specification TS 37.355 [12], the PRU information that is forwarded by the LMF to the target UE is provided as position calculation assistance for UE-based DL-TDOA and DL-AoD methods. However, for positioning Case 1 the channel measurement (Part A) and ground truth data (Part B) from PRU can be considered as training data assistance for UE-based AI/ML-based positioning method, but of course RAN1 first needs to agree what an AI/ML-based positioning method is. In our view, AI/ML-based positioning is expected to be a new positioning method. This decision will drive how the LPP signalling is specified for the above proposals in this paper i.e., whether existing position calculation assistance information element can be extended or if new training assistance data information element(s) have to be introduced that are specific to AI/ML-based positioning method.

Regarding the RAN1 study on feasibility, benefits and potential specification impacts of different options for monitoring metric calculation for positioning Case 1, for the signalling procedure aspects highlighted in yellow in the RAN1 agreement above, purely from a signalling perspective, the yellow highlighted signalling requirements are feasible from RAN2 point of view but RAN1 must decide between monitoring metric calculation Option A and Option B.

Proposal 18: Send LS to RAN1 to confirm if AI/ML-based positioning is considered as a new positioning method and to provide feedback to RAN1 about signalling feasibility for training data transfer and signalling feasibility for different options for model performance monitoring of AI/ML positioning Case 1.

# 3	Conclusion

This document has made the following observations:

Observation 1: To enable data collection at network-side for Beam Management Case 1 and Case 2 both RRM framework and CSI-Reporting framework require enhancement.

Observation 2: CSI-Reporting framework associates with RRM measurement object, but this doesn’t support reporting according to RRM framework.

Observation 3: The scope of the Trace Activation and Reporting extensions for Immediate MDT procedures to enable data collection for beam management is in RAN3 and SA5 realm.

Observation 4: Power state determination is UE implementation specific, and thus can be generalized to any degradation of the UE performance.

Observation 5: SRB3 and SRB5 cannot be used because are configured for Dual Connectivity.

Observation 6: Multiple configurations can be understood as a UE is configured with two different data collection initiation processes in RRC CONNECTED state.

Observation 7: Multiple configurations can be understood as a UE is configured with the same RSes used for inference can be used for multiple data collection triggers.

Observation 8: Multiple configurations can be understood as a UE is configured to measure with same or different data collection RSes in parallel.

Observation 9: Multiple instances of configurations are supported by adequate RRC configurations.

And proposed the following:

Proposal 1: RAN2 to adapt the above steps of Figure 1 as a baseline procedure for OAM centric data collection.

Proposal 2: Reuse the CSI reporting framework for configuration of measurements and configuration of reporting for offline training data collection at network-side for beam management use cases.

Proposal 3: CSI-MeasConfig can be used as a baseline to configure CSI-RS measurements for training data collection. Detailed configuration parameters applicability or extensions is FFS.

Proposal 4: CSI-ReportConfig is enhanced with necessary information to trigger measurements report storing and measurements delivery over DCCH (L3). Detailed logging and reporting criteria are FFS.

Proposal 5: For data collection, RRC Reconfiguration (through CSI-MeasConfig), is used to configure the UE to log beam-level L1-RSRP measurements when logging capability is available in the UE.

Proposal 6: The solution for report configuration for logged beam level measurement data collection should consider relaxed latency requirement, e.g., the delay caused by using low priority SRBs is acceptable.

Proposal 7: UE shall not stop autonomously data collection termination.

Proposal 8: Degradation of the UE performance (e.g. due to power state) during data collection for ML purposes is indicated via existing information in UAI to the gNB and its up to gNB to reconfigure UE.

Proposal 9: RAN2 to assume that UE will be configured by the network when to send the report to the network.

Proposal 10: AS buffer event based reporting is not useful.

Proposal 11: Support on-demand network request for L1 measurement reporting based on RRC UEInformationRequest and UEInformationResponse for both BM-Case 1 and BM-Case 2.

Proposal 12: RAN2 to discuss whether existing low priority SRB (SRB2, SRB4) or introduce a new SRB to be used for data collection transmission.

Proposal 13: RAN2 assumes that configurations for data collection share total allowable number of CSI-RS resources and reporting configurations for other purposes (e.g. legacy CSI-RS, Rel-19 CSI-RS for inference) and do not exceed it.

Proposal 14: For AI/ML positioning, until RAN#106, RAN2 should focus their specification efforts on positioning Case 1 (UE-based positioning with model on UE-side) while leaving positioning Case 3a and Case 3b to RAN3.

Proposal 15: For positioning Case 1, LPP signalling is used for transfer of ground truth label and related data (Part B data) and channel measurement and related data (Part A data) from PRU to LMF. FFS: details of LPP signalling.

Proposal 16: For positioning Case 1, LPP signalling is used for transfer of ground truth label and related data (Part B data) from LMF to target UE, for both PRU generated and LMF generated ground truth label and related data. FFS: details of LPP signalling.

Proposal 17: For positioning Case 1, LPP signalling is used by LMF to forward the channel measurement and related data (Part A data), generated by a PRU, to the target UE. FFS: details of LPP signalling.

Proposal 18: Send LS to RAN1 to confirm if AI/ML-based positioning is considered as a new positioning method and to provide feedback to RAN1 about signalling feasibility for training data transfer and signalling feasibility for different options for model performance monitoring of AI/ML positioning Case 1.

# References

[1] TS 37.320, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2 (Release 17), Version 18.3.0, 3GPP, 2024-09.

[2] “RAN2 Chair’s Notes”, 3GPP TSG RAN WG2 #127, Maastricht, NL, August 19 – 23, 2024.

[3] R2-2406673,	Further discussion on NW-sided data collection,	Apple, 3GPP TSG RAN WG2#127, Maastricht, NL, August 19 – 23, 2024.

[4] 	R2-2407436	Further Consideration on NW side Data Collection, ZTE, 3GPP TSG RAN WG2#127, Maastricht, NL, August 19 – 23, 2024.

[5] 	TS 38.331, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) protocol specification (Release 18)”, V18.2.0, 3GPP, 2024-07.

[6] TS 32.422, “3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; Subscriber and equipment trace; Trace control and configuration management (Release 17), Version 19.0.0, 3GPP. 2024-09.

[7] R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1, Qualcomm, 3GPP TSG RAN WG1#114-bis, Xiamen, China, October 9th – 13th, 2023.

[8] TR 38.843 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 3GPP, 2024-01.

[9] RP-242399, “Revised WID: AI/ML for NR air interface”, 3GPP TSG RAN #105, Melbourne, AU, September 09 – 12, 2024.

[10] “RAN1 Chair’s Notes”, 3GPP TSG RAN WG1 #116bis, Changsha, China, April 15 – 19, 2024.

[11] “RAN1 Chair’s Notes”, 3GPP TSG RAN WG1 #118, Maastricht, NL, August 19 – 23, 2024.

[12] TS 37.355, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP) (Release 18)”, V18.3.0, 3GPP, 2024-09.