3GPP TSG-RAN WG2 Meeting #127bis                                    	R2-2408547

Hefei, China, Oct. 14th – 18th, 2024

Agenda item:	8.9.3

Source:	ESA, Eutelsat Group, Viasat, Inmarsat, Novamint, Echostar, Sateliot, Toyota ITC

Title:	Repetitions and Delay Considerations about SA and DSA

Document for:	Discussion and Decision

- Introduction

The WID on NTN IoT Phase-3 [1] has following objective for the EDT enhancements.

Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

- Msg3 transmission without msg1/ Random Access Response (RAR)

In this document, we provide a quick summary of the previous ESA document, R2-2407502, and further discussion about the delay impact when working far from PLR=1% or using repetitions.

- Performance summary for SA / DSA / CRDSA solutions

In the previous ESA contribution [2], a complete set of simulations was reported for comparing different strategies as alternative of a pure SA solution in satellite systems (both GEO and NGSO). For sake of clarity, the general definitions and assumptions have been depicted in the following figure.

Figure 1: Pictorial view of Msg3 transmissions in N consecutive EDT occasions.

The main system assumptions for the capacity evaluation with the new proposed schemes are summarized in the following table.

Table 1: Main system assumptions for the evaluation of SA, DSA, and CRDSA uplink capacity.

| NTN Scenario | GEO / 3GPP Set-1 |
| --- | --- |
| NTN Channel | AWGN |
| Sub-carrier spacing (SCS) | 3.75 kHz |
| Number of sub-carriers ( Nsc ) | 48 |
| Uplink SNR | 4.8 dB for all receive packets Note: From TR 36.763, the GEO UL SNR is 2.6 dB. We removed 2.2 dB margin for scintillations which appears unjustified for a typical (not worst-worst-case) link budget. |
| Msg3-EDT TBS ( Nbitspkt ) | 328 bits |
| Modulation | p/4-QPSK |
| Config#1 |  |
| Single EDT duration ( TEDT ) | 96 ms |
| FEC Coding rate ( r ) | 0.61 (see Appendix) |
| Number of EDT occasions ( Nopp ) | 2 or 3 or 4 |
| Number of replicas ( K ) | 2 or 3 |
| CRDSA SIC iterations ( Niter ) | 4 |
| Config#2 |  |
| Single EDT duration ( TEDT ) | 128 ms |
| FEC Coding rate ( r ) | 0.46 (see Appendix) |
| Number of EDT occasions ( Nopp ) | 2 or 3 |
| Number of replicas ( K ) | 2 or 3 |
| CRDSA SIC iterations ( Niter ) | 4 |

To simplify the uplink capacity evaluation, it is implemented a collision-based assessment of all proposed scheme:

- we assume a transmitted Msg3 is successful decoded if it does not collide with another transmitted message;

- if the Msg3 is successfully decode, all its replicas are removed (or cancelled) from the other EDT occasions;

- the decoding is unsuccessful if it collides with another message (unless all interferers have been cancelled).

It is important to underline that it is a worst-case scenario, because in certain conditions (e.g., large SNR, different FEC coding rates, and received packet power unbalance) the real physical layer turbo decoder might successfully decode a packet even in presence of collisions [4], [5]. In particular, the received packet power unbalance, which is a very realistic working condition due to different UE locations within the satellite coverage, will improve the overall system throughput. This is not the case for the assumptions made here in this example for Config#1 and Config#2.

Finally, for the capacity assessment, it is important to define the NTN UE traffic generation. The traffic is Poisson distributed and the normalized load in bits/symbol is computed as:

Gnorm=Npkr log2MNscNopp

where Npk is the average number of packets, Nsc is the number of NB-IoT subcarriers, Nopp is the number of EDT opportunities, r is the FEC coding rate, and M the modulation order (i.e., 4 for QPSK). The traffic load in terms of bit/s can be computed as

G=NpkNbitspktTEDTtot=GnormNsc NbitspktTEDT r log2M

where Nbitspkt corresponds to the number of bits in the Msg3-EDT packet, TEDTtot=NoppTEDT where TEDT is the single EDT opportunity duration in seconds. The throughput in bits/s is defined as:

T(G)=G[1-PLR(G)]

where the PLR function represents the traffic Packet Loss Ratio derived from the simulation.

## 2.1 Final Performance Comparison

For completeness, Figure 2 and Figure 3 show the comparison among the current legacy SA method and the two proposed solutions for enhancing the NTN IoT uplink capacity. The advantages of DSA and CRDSA are very well evident.

Observation 1: The DSA solution is improving the Msg3-EDT throughput by about a factor of 4.5 without any impacts on the current receiver implementation at the network, and minimal specification impact overall.

Observation 2: The CRDSA solution is improving the Msg3-EDT throughput by about a factor from 20 to 40, depending on the selected configuration.

Proposal 1: RAN2 to specify the support of Msg#3-EDT replicas.

Proposal 2: RAN2 to specify the indexing strategy of the Msg#3-EDT replicas, when multiple copies are transmitted.

Figure 2: Config#1 final performance comparison.

Figure 3: Config#2 final performance comparison.

- Delay consideration and Repetition impacts

It well known in literature [7] the importance of selecting a proper input traffic load, G, to avoid unstable behaviour of the random-access schemes due to the presence of many undesired repetitions of packets. This is well depicted in Figure 4, where a graphical explanation is reported for the Slotted ALOHA (SA) scheme.

Figure 4: Extracted from [7]. It is important to select a stable working point in SA to avoid instability.

The normalized SA throughput is maximized at input traffic load G=1, that corresponds to a maximum throughput of TG=e-1 (i.e., 36.7%) and for the corresponding PLR=(1-e-1) (i.e., 63.2%). Of course, this PLR value is far to be a stable network traffic working point due to the presence of too many retransmissions. As discussed in [7] and shown in Figure 4 (left), the reason is the presence of more users that are still repeating their packets (i.e., nqr) with respect to new packets (i.e., (m-n)qa). In other words, the system is afflicted with several backlogged packets, nqr≫(m-n)qa. The congestion control algorithm must ensure that the SA scheme is working with PLR lower than 50%.

Observation 3: The SA PLR working point must be lower than 50%.

Based on the system parameters presented in the previous section, both the PLR and the offered throughput have been reported in the next figures, for both SA and DSA schemes.

Figure 5: SA and DSA comparison in terms of PLR as a function of the input traffic load.

Figure 6: SA and DSA comparison in terms of the offered throughput [kbps] as a function of the input traffic load.

As expected, the SA outperforms DSA only in the region where the PLR is greater than 40%, as reported in Table 2, without considering any retransmissions.

Table 2: Throughput value, T(G), derived from Figure 6.

| Target PLR | SA Throughput | DSA Throughput |
| --- | --- | --- |
| 1% | 1.8 kbit/s | 8 kbit/s |
| 10% | 15.4 kbit/s | 28.9 kbit/s |
| 40% | 48 kbit/s | 48 kbit/s |
| 50% | 57.6 kbit/s | 50.4 kbit/s |
| 60% | 60.6 kbit/s | 48.9 kbit/s |
| 70% | 59.1 kbit/s | 44.2 kbit/s |

Observation 4: The DSA scheme outperforms SA up to PLR = 40%.

Hereafter, we want to show that anyway for satellite networks it is extremely important to assume low values of PLR (e.g., <10%).

If N repetitions are necessary to reach the desired service error rate, b, it is easy to define that the overall maximum delivery time is:

D=N∙TEDTtot+N-1tRTD

where TEDTtot=NoppTEDT is the single RAO duration, and tRTD is the distance between consecutive RAO. For example, for a GEO system, tRTD=510 ms (e.g., including 10 ms margin on top of twice the GEO RTD), while TEDTtot=384 ms in Config#1 with Nopp=4.

Suppose that the probability that a packet fails N or more times should not be more than b. We wish to find out the maximum normalized throughput that the system can have under the above condition. Again, this is a reasonable requirement for satellite systems where the round-trip propagation delay is very large. The probability of success of a packet in each try is (1-PLR). Assuming independent tries in each RAO, the random variable N representing the number of times a packet must be retransmitted to be successful will have a geometric distribution:

PLR≤β1/N

Assuming that b=1% is an acceptable value, the following relationship applies:

Table 3: Number of retransmissions, N, and overall packet delay as function of the PLR in each try, given the target b=1%.

| PLR | N | D [s] |
| --- | --- | --- |
| 1% | 1 | 0.384 |
| 10% | 2 | 1.278 |
| 22% | 3 | 2.172 |
| 32% | 4 | 3.066 |
| 40% | 5 | 3.960 |

Again, it is evident that working with high PLR values leads to large maximum packet delay (e.g., several seconds) in achieving the desired service acceptance rate, (1-b).

Observation 5: For target PLR greater than 10%, the packet delivery time with 99% successful rate can exceed several seconds in GEO systems.

This analysis has been also carried out in the original DSA paper [8], and the main results showing the maximum normalized throughput, Smax, as a function of b and different values of k (i.e., number of DSA replicas) and n (i.e., number of repetitions N). For b<10%, the Authors in [8] have analytically demonstrated that DSA has always a better throughput compared to SA, and the gain varies from 200% to 500%.

Figure 7: Results extracted from [8].

- Conclusion

Following observations and proposals are made:

Observation 1: The DSA solution is improving the Msg3-EDT throughput by about a factor of 4.5 without any impacts on the current receiver implementation at the network, and minimal specification impact overall.

Observation 2: The CRDSA solution is improving the Msg3-EDT throughput by about a factor from 20 to 40, depending on the selected configuration.

Observation 3: The SA PLR working point must be lower than 50%.

Observation 4: The DSA scheme outperforms SA up to PLR = 40%.

Observation 5: For target PLR greater than 10%, the packet delivery time with 99% acceptance rate can exceed several seconds in GEO systems.

Proposal 1: RAN2 to specify the support of Msg#3-EDT replicas (i.e., introduction of DSA solutions).

Proposal 2: RAN2 to specify the indexing strategy of the Msg#3-EDT replicas, when multiple copies are transmitted.

References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, September 2024.

[2] R2-2407502, “Discussion on DSA and CRDSA Performance”, ESA, August 2024.

[3] E. Casini, R. De Gaudenzi and O. Del Rio Herrero, "Contention Resolution Diversity Slotted ALOHA (CRDSA): An Enhanced Random-Access Scheme for Satellite Access Packet Networks," in IEEE Transactions on Wireless Communications, vol. 6, no. 4, pp. 1408-1419, April 2007.

[4] O. del Rio Herrero and R. De Gaudenzi, “Generalized Analytical Framework for the Performance Assessment of Slotted Random Access Protocols”, in IEEE Transactions on Wireless Communications, vol. 13, pp. 809-821, February 2014.

[5] A. Mengali, R. De Gaudenzi, and P. -D. Arapoglou, “Enhancing the Physical Layer of Contention Resolution Diversity Slotted ALOHA”, in IEEE Transactions on Communications, vol. 65, pp. 4295-4308, Oct. 2017.

[6] ETSI EN 301 545-2, “Digital Video Broadcasting (DVB); Second Generation DVB Interactive Satellite System (DVB-RCS2); Part 2: Lower Layers for Satellite standard”.

[7] D. P. Bertsekas, and R. G. Gallager, “Data Networks (2nd Edition)”, Prentice Hall, 1992, ISBN 0132009161 (https://web.mit.edu/dimitrib/www/datanets.html )

[8] G. L. Choudhury and S. S. Rappaport, “Diversity ALOHA: A Random Access Scheme for Satellite Communications”, IEEE Transactions On Communications, vol. 31, March 1983.

- Appendix

Based on the selection of SCS=3.75 kHz, a single resource unit (RU) is 32 ms, and it is composed of 112 QPSK symbols. Considering the presence of DM-RS, there are 96 data-symbols and 16 pilots for each RU. Therefore, the FEC coding rate for the proposed configurations is

- Config#1 (NRU=3): Total coded bits=96 × 3 × 2= 576. Then, the coding rate r: (328+24 CRC)/576= 0.61

- Config#2 (NRU=4): Total coded bits=96 × 4 × 2= 768. Then, the coding rate r: (328+24 CRC)/768= 0.46

Even though the turbo decoder performance is not relevant in the considered methodology for the capacity assessment (i.e., collision-based), we have reported below the physical layer BLER results. Both configurations achieve BLER<1% well before the selected UL SNR value of 4.8dB. As expected, the lower coding rate has an additional 2dB margin.

Figure 8: Physical layer turbo decoder performance for both configurations.