3GPP TSG-RAN WG2 Meeting #127bis	R2-2408552

Hefei, China, October 14-18, 2024

Agenda Item:	8.1.5

Source:	NEC

Title:	AIML Model Transfer

Document for:	Discussion, Decision

# Introduction

In section 7.2.1.4 Model transfer/delivery within TR38.843 for Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface during Release-18, it is said that whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) is unclear from the outcome of the present study. However the following model transfer methods were considered.

-	Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

-	Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

-	Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

-	Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data.

-	Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP).

-	Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

This paper further discusses the detailed AIML Model Transfer aspects.

# Down-selection of model transfer method

## RRC signalling and DRB based AIML model transfer

Even though, many model transfer method has been identified, we suggest that the discussion at RAN2 should focus on the evaluation of the model transfer method that have air interface impact and leave the discussion on other solutions to other Work Group (e.g. SA2).

From RAN2 perspective, the following two solutions should be further evaluated towards a potential down-selection among all of the model transfer methods on the table, since RRC signalling and user plane (air interface) based approach requires the direct support from the L2/L3 protocol stack managed by RAN2:

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

However, it would be highlighted that the discussion for above two solutions actually assumes the AIML model storage at the gNB. This is like distributed manner based AIML model storage, the stored AIML model can be seen as a sort of data, which can be transmitted from gNB to the UE via SRB or DRB.

Logically, as long as the gNB holds the AIML model, it would be very easy for gNB to initiate AIML model transfer to the UE via the existing RRC message or new RRC message. This solution is at least feasible for the AIML model transfer with small size. Generally speaking, model transfer by signalling (i.e., CP) has more high level reliability and priority, but the size of model is a key disadvantage. When considering the existing RRC segment mechanism, 45KB is the maximum signalling transfer size. This solution may not be suitable for the AIML model, of which the size is up to multiple MB. If the size of AIML model goes up, the AIML transfer requires lots of message segmentation (provided that we can expand the segmentation mechanism at Rel-18) before transmission over the air interface, which may also present overwhelming effect to other RRC messages.

Alternatively, to support big size based AIML model transfer, the data based transfer (i.e., UP based solution) could be considered and in practice, the AIML model data can be carried by data radio bearer. Data based transmission is suitable for big size model transfer, and it can also handle data interruption well. However, for the DRB based model transfer, it is not clear how such transfer path is established. In legacy UP transmission, the path is established involving 5GC/UPF, and gNB cannot directly establish a UP channel to transfer the data generated by itself. There is a QoS management issue, since in legacy traffic transmission, the gNB follows the QoS parameters of QoS flow to provide the corresponding air interface mechanism to support the required reliability and latency. If an isolated DRB is established between gNB and the UE, the QoS parameter for such DRB should be clarified for model transfer. The corresponding work may be subject to SA2 and a LS should be sent for such liaison.

Considering the pros and cons for CP/UP based solution, they both have merit and shortage and may be useful in different scenarios, then we suggest to not exclude these two solutions from each other. Instead, we need also to discuss the co-existence of both solutions. In this case, gNB may be able to download the AIML model to the UE via both approach, and depending on the AIML model size, or other necessary criteria, the gNB may decide one of the approach to initiate a model transfer to the UE. In addition, different gNB with different capability may support different approach, depending on the AIML function it supports.

Proposal-1: RAN2 is suggested to focus on the evaluation on the following air interface based AIML model transfer solutions.

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

Proposal-2: RRC signalling is adopted for small size AIML model transfer (e.g. less than 45KB) and DRB is adopted for big size AIML model transfer.

Proposal-3: LS to SA2 on the QoS parameter setting for the DRB used for AIML model transfer.

Proposal-4: RAN2 to discuss how to determine the AIML model transfer method (RRC signalling or DRB based) for model transfer to the UE.

## Failure handling for Model transfer

When the UE downloads the AIML model from the network but experiences Radio Link Failure, the UE is supposed to reconnect with a cell via RRC reestablishment and then the UE is expected to continue the previous AIML model transfer. In this discussion, we assume the UE be able to recover the radio link with the same cell or same base station, and leave other cases for further study.

Case-1: UE downloads the AIML model via RRC messages

As discussed in the previous session, in case of RRC message AIML model transfer, the RRC segmentation may occur for AIML model transfer. The RRC Segments number for AIML model transfer may be put into the RRC context, which will be known by both UE and gNB.

When the UE experiences RLF, both UE and gNB can be expected to maintain the UE’s RRC context. So then when the UE resumes the RRC connection, the AIML model transfer can recover based on the stored RRC Segment number within the UE’s RRC context.

Alternatively, the UE can report RRC Segment number (and the AIML model ID) as part of the RLF report during RRC reestablishment, and then the network just continue the AIML model download referring to this Segment number. In legacy handling, when the UE experiences RLF, according to clause 5.3.10.3 of TS 38.331, the UE will discard any segments of stored segmented RRC message. In this case, besides reporting RRC Segment number to the network, the UE still needs to ensure that the segmented RRC message will not be discarded.

Proposal-5: The network can continue its AIML model transfer via RRC messages after RLF failure recovery based on RRC Segment numbers.

Case-2: UE downloads the AIML model via DRB

As discussed in the previous session, in case of  DRB based AIML model transfer, the UE downloads the AIML model via a specific DRB established between UE and gNB, where the segmentation may occur at PDCP layer. In this case, the PDCP Segments is governed by PDCP SN.

In PDCP specification, the PDCP data recovery procedure is available, we can extend its usage to this case, to allow the PDCP data recovery to occur during RRC Reestablishment after RLF. This can only apply to the special DRB that carries AIML model transmission. In addition, it is possible to extend the usage of the PDCP status report, i.e., to require the UE to send a PDCP Status Report to network to provide its reception status for AIML Model transfer (Downlink), during RRC Reestablishment after RLF. Then based the report, the network continues its AIML model download referring to the UE reception status for the AIML model as indicated by the PDCP SR, when UE reconnects to the network.

Proposal-6: The network can continue its AIML model transfer via DRB after RLF failure recovery based on PDCP Status Report.

# Area based model transfer

RAN1 discussed the usage of the area specific AIML model. It is assumed that a particular AI/ML model should be applicable to a specific area with the intention that multiple cells may use the same AI/ML model for a given function (e.g., Beam management) and the other area may use a different AIML model. As can be seen, the area to deploy the same AI/ML model for a feature can be expressed by a list of cells. Under a particular network deployment, this area can be a vendor specific network deployment area within the multi-vendor network environment. For example, for a particular radio network deployed by a specific vendor, one area may offer the specific AIML model for its supported AI/ML function, according to the strategy of the vendor deploying the radio network.

Depending on the change of the area during movement, the UE may need to update its AIML model or enable/disable the usage of the AIML functions, since different radio network area may offer diversified AIML capability based on the network deployment. In practice, for a portion of the radio network deployed by the same vendor, the AIML function may be switched on for performance observation before its massive deployment to all of the gNBs, which is especially true for the two-sided AIML model.

From the UE perspective, it would be very important to check the area that the serving cell belongs to and to perform necessary model download. This means the UE needs to update its own model in order to function correctly if the UE intends to use such AIML model to communicate with the network, otherwise the UE and the Network may risk misalignment on the AIML model for the same function e.g., CSI feedback report and then AIML functions may not be able to execute as expected.

The area based model transfer should be applicable to both RRC Connected UE and the UEs in RRC_IDLE or RRC_INACTIVE.

Proposal-7: The area specific AIML model is supported for the AIML use cases

Proposal-8: The area to deploy the same AI/ML model for a feature can be expressed by a list of cells

Proposal-9: The UE needs to update its AIML model upon AIML area change to align with the gNB capability in the area.

Proposal-10: The area based model transfer should be applicable to both RRC Connected UE and the UEs in RRC_IDLE or RRC_INACTIVE.

# RRC State for AIML model transfer

The discussion on AIML model transfer should take an assumption on which RRC state(s) the UE can download the model. The current discussion assumes at least RRC Connected UE can download the AIML model from the network. However it is important to clarify if the UE in RRC Idle or RRC Inactive state can also download the AIML model. Logically, the UE can download the AIML model without the need to go to RRC Connected state. This discussion may be related to which node is responsible for transferring the AIML model to the UE. If the gNB holds the AIML model, the gNB may be able to notify the UEs in RRC Idle or RRC Inactive, and the UE can determine if there is a need for it to download the AIML model from the network during RRC Idle or RRC Inactive. Or otherwise the UE can also download the AIML model it transit from RRC Idle or RRC Inactive to RRC Connected state.

Proposal-11: RAN2 to discuss which RRC state(s) the UE can download the AIML model from the network.

# AIML model transfer during Mobility

## Mobility Scenarios considering AIML support

During handover, there are different possibilities for the AIML capability from the network perspective. For example, the two gNBs (i.e., source gNB and target gNB) may be deployed by two different vendors, each using different AIML model for a AIML use case e.g., beam management. In addition, the source gNB and target gNB may be deployed by two different vendors, one is equipped with AIML technology, and the other is not, in which case, the Base station capabilities are different. The source gNB and target gNB may be deployed with different version of software by the same vendor, in which case, the Base station capabilities to run AIML algorithm are different.

From the UE perspective, the UE may move from one cell to another cell without AI/ML model change for a particular feature, or vice versa. The UE may stop the applicability of an AIML model for such feature when moving from one cell to another, or change to a different AI/ML model for the feature.

During UE mobility, the following scenarios should be supported by the UE:

- from non-AIML supporting gNB to AIML supporting gNB;

- from AIML supporting gNB to non AIML supporting gNB

- from one AIML supporting gNB to another AIML supporting gNB

In any case, in order to maximize the efficiency of the utilization of AIML algorithm, it is expected that the UE and the network should align the utilization of the AIML model for a particular feature.

Proposal-12: During UE mobility, the following scenarios should be supported by the UE:

- from non-AIML supporting gNB to AIML supporting gNB;

- from AIML supporting gNB to non AIML supporting gNB;

- from one AIML supporting gNB to another AIML supporting gNB.

In the following discussion, the third scenario is firstly assumed for this paper, but the first and the second scenarios should be also considered later.

## Requirements for AIML operation during handover

It is important to clarify the requirements for AIML operation during handover, in order to allow the UE to continue to use the AIML algorithm for a particular feature as much as possible. This may be very useful to ensure a consistent UE performance during the UE movement. Meanwhile, as the UE is expected to align with the network (gNB) on the operation of AIML algorithm, it would be quite essential to clarify if the UE is required to be ready to run the AIML algorithm as supported by the target gNB. For example, if the target gNB uses a different AIML for beam management from the source gNB, should the UE be ready to run the corresponding AIML model immediately after its access to the target gNB due to handover operation. This requirement should be clarified by RAN2.

Proposal-13: RAN2 to discuss if the UE is required to be ready to run the corresponding AIML model as supported by the target gNB immediately after its access to the target gNB due to handover operation.

## AIML capability exchange for mobility

In order to prepare the handover between source gNB and target gNB for the UE, source gNB and target gNB can exchange its AIML capabilities before the handover (e.g., during Xn setup). This may be carried by new Xn interface based message. With this information exchange, the source gNB will know if the target gNB will continue the AIML algorithm for a particular feature after the handover and may decide if there is a need to make necessary context transfer. In some scenarios, both source gNB and target gNB may support the AIML based operation for the same feature but may run different model. In this case, source gNB and target gNB may need to exchange its supported AIML model for such feature.

Proposal-14: RAN2 to assume that AIML capability exchange between the source gNB and target gNB is performed for UE mobility before handover and confirm with RAN3.

## AIML model transfer during handover

During the handover procedure, the target gNB may hold the AIML model by itself. It is also possible that the target gNB does not hold the AIML model, but there is a third party entity that hosts the AIML model. This third party entity can be another network entity or OTT server, depending on the AIML architecture assumptions. For example, in some centralized AIML model deployment architecture, a specific server may be deployed to train the AIML model and to host and maintain the AIML model, which is to be used within the whole wireless network.

In this case, RAN2 and RAN3 needs to discuss how the target gNB indicates to the UE the source of the AIML model, since the UE should download the corresponding AIML model as the target gNB before that can be used between the UE and the gNB.

Proposal-15: RAN2 to discuss how the UE downloads the AIML model (as supported by target gNB) from a (centralized) entity during handover.

In addition, we need to also consider the failure case for AIML transfer during mobility. For example, the UE is downloading the AIML model from the source gNB, but at the same time the UE is directed by the source gNB to handover to a target gNB. In this case, the AIML model transfer may be interrupted by the handover procedure. RAN2 should study the mechanism for the continuation of the AIML model transfer for such case. When there is a need, RAN3 may be liaisoned to design a solution jointly.

Proposal-16: RAN2 to discuss how to handle AIML model transfer interruption during UE’s handover from source gNB to target gNB.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: RAN2 is suggested to focus on the evaluation on the following the air interface based AIML model transfer solutions.

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

Proposal-2: RRC signalling is adopted for small size AIML model transfer (e.g. less than 45KB) and DRB is adopted for big size AIML model transfer.

Proposal-3: LS to SA2 on the QoS parameter setting for the DRB used for AIML model transfer.

Proposal-4: RAN2 to discuss how to determine the AIML model transfer method (RRC signalling or DRB based) for model transfer to the UE.

Proposal-5: The network can continue its AIML model transfer via RRC messages after RLF failure recovery based on RRC Segment numbers.

Proposal-6: The network can continue its AIML model transfer via DRB after RLF failure recovery based on PDCP Status Report.

Proposal-7: The area specific AIML model is supported for the AIML use cases

Proposal-8: The area to deploy the same AI/ML model for a feature can be expressed by a list of cells

Proposal-9: The UE needs to update its AIML model upon AIML area change to align with the gNB capability in the area.

Proposal-10: The area based model transfer should be applicable to both RRC Connected UE and the UEs in RRC_IDLE or RRC_INACTIVE.

Proposal-11: RAN2 to discuss which RRC state(s) the UE can download the AIML model from the network.

Proposal-12: During UE mobility, the following scenarios should be supported by the UE:

- from non-AIML supporting gNB to AIML supporting gNB;

- from AIML supporting gNB to non AIML supporting gNB;

- from one AIML supporting gNB to another AIML supporting gNB.

Proposal-13: RAN2 to discuss if the UE is required to be ready to run the corresponding AIML model as supported by the target gNB immediately after its access to the target gNB due to handover operation.

Proposal-14: RAN2 to assume that AIML capability exchange between the source gNB and target gNB is performed for UE mobility before handover and confirm with RAN3.

Proposal-15: RAN2 to discuss how the UE downloads the AIML model (as supported by target gNB) from a (centralized) entity during handover.

Proposal-16: RAN2 to discuss how to handle AIML model transfer interruption during UE’s handover from source gNB to target gNB.

# Reference

- R2-2301440  Summary of RAN2 post-meeting discussion on AIML on model transfer/delivery

- TR37.817

- TR38.843