3GPP TSG-RAN WG2 Meeting #127bis													     R2-2408557

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.3.3

Source:	Apple

Title:	On Measurement Event Prediction Evaluation

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide our views on measurement event prediction simulation assumptions, use cases, KPIs and everything else that needs to be ironed out to provide evaluation results in the following meetings.

# 2   	Discussion

### 2.1 	Background

For reference, here are the agreements reached so far on the topic of measurement event prediction:

1. At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction is also allowed. 2. Clarifications on what is being as input should be provided with results 3. Start with A3 as a baseline. 4. Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

### 2.2 	Use cases and general assumptions

In theory, A3 event can be configured for three measurement quantities: RSRP, RSRQ or SINR. Since RSRP is what’s used for mobility and because we’ve already agreed to focus on RSRP for RRM measurement prediction, it makes sense to limit measurement event prediction to RSRP.

Proposal 1: for measurement event prediction, RSRP is the only measurement quantity.

With this, it is perhaps obvious that all the simulation assumptions agreed for measurement predictions should be re-used for measurement event prediction. It makes littles sense to deviate from those.

Proposal 2: all the simulation assumptions agreed for measurement predictions should be re-used for measurement event prediction.

Before diving into the use cases discussion, there is an observation to make – already in RAN2#127 companies began to realize that the scope of the evaluations in this study is rather large and we need to limit the evaluation effort to something reasonable (see, for example, the agreement to limit the speeds to only 30km/h for the inter frequency case). We hope this trend continues and with this in mind we propose to focus on the following use cases for the measurement event prediction case.

Regarding the events to evaluate, we’ve already agreed to start with A3. This event covers the most frequently used case for mobility. All the other events from the A-series are essentially the subsets of the A3 case and all the other events are not directly related to the current study to begin with. Initially, there were proposals to consider A2, for example, but that would have only made sense if the evaluation is limited to A2. Once A3 has been agreed, there is little motivation to consider other measurement events.

Proposal 3: only A3 is evaluated.

Naturally, we need to consider both intra-frequency and inter-frequency cases, not only because both are used in practical deployment but mainly because the AI/ML models would be different for those.

Proposal 4: consider both intra-frequency and inter-frequency.

Then with regards to FR1 and FR2, once again our preference is to focus on FR1 as this is what’s commonly used in practice and if this study is to have any practical impact beyond an academic exercise, FR1 must be considered. We do acknowledge that there is a lot of [academic] interest from many companies in RAN2 to also study FR2 and therefore we accept to cover FR2 as well, with lower priority (if prioritization is needed).

Proposal 5: focus on FR1, can consider FR2 with lower priority.

In summary, we came up with 4 cases listed in the table below:

| Case number | Inter vs intra frequency | FR1 vs FR2 | Priority |
| --- | --- | --- | --- |
| 1 | Intra-frequency | FR1 | ? |
| 2 | Inter-frequency | FR1 | ? |
| 3 | Intra-frequency | FR2 | ? |
| 4 | Inter-frequency | FR2 | ? |

If we limit the evaluation to these 4, prioritization may not be needed. If, however, prioritization is required, FR1 must be higher priority.

Finally, with regards to “direct” vs. “indirect”, considering a very similar discussion we had at RAN2#127 on a related topic for RLF prediction, we suggest allowing companies to choose whatever approach they prefer and of course report which approach has been used. No need to repeat this discussion which leads nowhere.

Proposal 6: companies can choose whether to do “direct” or “indirect” prediction and report which approach has been used. No need to discuss this issue any longer.

### 2.3 	RRC configuration parameters

RRC parameters agreed for measurement prediction naturally apply to the case of event prediction as well. Therefore, in this section we only describe the RRC parameters of relevance specific to measurement event configuration.

Obviously, we need to align on all the parameters that go into the A3-1 and A3-2 inequality calculations defined in TS 38.331 clause 5.5.4.4, which are listed below.

- Ofn is the measurement object specific offset of the reference signal of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

- Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

- Ofp is the measurement object specific offset of the SpCell (i.e. offsetMO as defined within measObjectNR corresponding to the SpCell).

- Ocp is the cell specific offset of the SpCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the SpCell), and is set to zero if not configured for the SpCell.

- Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

- Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event).

Here we note that while cell-specific and measurement object-specific offets are a very useful tool for allowing flexibility in deployments, it is hardly needed in simulations. Furthermore, there is no way to agreed on specific values for each cell and the best we can do is to agree on a distribution of such offsets, which would only complicate the evaluation. Therefore, our proposal is not to consider cell-specific and measurement object-specific offets.

Proposal 7: no need to consider cell-specific offsets (i.e. Ocn and Ocp) and measurement object-specific offsets (i.e. Ofn and Ofp).

We don’t have a strong preference for specific values, the values provided below are just suggestions and we are open to discuss any values within reasonable limits of what is normally used in deployments.

| Parameter | Value |
| --- | --- |
| hysteresis | 2 or 3 |
| a3-Offset | 2 or 3 |
| timeToTrigger | 320 |

Proposal 8: agree on values for the RRC parameters as follows (some deviations from the values provided can be discussed): hysteresis: 2 or 3, a3-Offset: 2 or 3, timeToTrigger: 320.

### 2.4 	KPIs

While prediction and observation window ranges are not of course KPIs, we need to agree on these first. As we have stated before, there is no need to limit model inputs in the study – companies should be allowed to use whatever inputs they find useful, which also applies to the observation window. Naturally, observation window length would impact complexity, so companies would be naturally incentivised to use the shortest window that provides adequate results. As for the prediction window, this being a study intended among other things to explore the limits of ML algorithms we suggest using a liberal prediction window length, and therefore window length(s) of at least up to 1s should be allowed.

Proposal 9: no need to limit observation window length, prediction window lengths of at least up to 1s should be allowed.

Most machine learnings output a probability estimate, i.e. in our case that would be x% likelihood that a measurement event happens within a prediction window.

Proposal 10: model output is the likelihood that measurement event A3 happens within a prediction window.

In order for this to be useful, though, a classification threshold needs to be agreed upon so that a predicted likelihood about that threshold would be considered an event prediction and below it would be considered no event prediction. We propose to use 95% threshold, but we are open to discuss alternative values.

Proposal 11: 95% classification threshold should be applied to measurement event prediction accuracy estimate to decide whether an event is predicted or not.

Then, naturally, the usual precision, recall, and F1-score should be used.

Precision measures the proportion of true positive predictions among all positive predictions made by the model. It is calculated as the ratio of true positives to the sum of true positives and false positives.

Precision indicates how many of the predicted positive instances are actually positive. A high precision indicates that the model is making few false positive predictions.

Precision = True PositivesTrue Positives+False Positives

Recall, sometimes referred to as Sensitivity, measures the proportion of true positive predictions among all actual positive instances in the dataset. It is calculated as the ratio of true positives to the sum of true positives and false negatives.

Recall indicates how many of the actual positive instances the model is able to correctly identify. A high recall indicates that the model is capturing most of the positive instances in the dataset.

Recall = True PositivesTrue Positives+False Negatives

F1-score is the harmonic mean of precision and recall. It provides a single metric that combines both precision and recall, balancing the trade-off between the two. It is calculated as:

F1-score considers both false positives and false negatives, making it a useful metric for imbalanced datasets where precision and recall may be skewed.

F1-score=2*Precision*RecallPrecision+Recall

Proposal 12: precision, recall, and F1-score should be used as KPIs.

# 3	Proposals

Proposal 1: for measurement event prediction, RSRP is the only measurement quantity.

Proposal 2: all the simulation assumptions agreed for measurement predictions should be re-used for measurement event prediction.

Proposal 3: only A3 is evaluated.

Proposal 4: consider both intra-frequency and inter-frequency.

Proposal 5: focus on FR1, can consider FR2 with lower priority.

Proposal 6: companies can choose whether to do “direct” or “indirect” prediction and report which approach has been used. No need to discuss this issue any longer.

Proposal 7: no need to consider cell-specific offsets (i.e. Ocn and Ocp) and measurement object-specific offsets (i.e. Ofn and Ofp).

Proposal 8: agree on values for the RRC parameters as follows (some deviations from the values provided can be discussed): hysteresis: 2 or 3, a3-Offset: 2 or 3, timeToTrigger: 320.

Proposal 9: no need to limit observation window length, prediction window lengths of at least up to 1s should be allowed.

Proposal 10: model output is the likelihood that measurement event A3 happens within a prediction window.

Proposal 11: 95% classification threshold should be applied to measurement event prediction accuracy estimate to decide whether an event is predicted or not.

Proposal 12: precision, recall, and F1-score should be used as KPIs.

# 4	References

[1]		 RAN2#127 and RAN#127-bis meeting minutes