# 3GPP TSG RAN WG2 Meeting #127b      	                           R2-2408560
Hefei, China, 14th – 18th Oct., 2024

Agenda item:	8.5.2

Source:	Apple

Title:	Further Discussion on on-demand SSB for SCell

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

The Rel-19 WID of Network Energy Savings (WID RP-242354) was updated in RAN#105 [1], the WI objective on on-demand SSB (OD-SSB) SCell operation is copied below:

OD-SSB was discussed in RAN2#127 [2] and below agreements were made.

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Based on these agreements, we discuss below issues in this contribution.

- OD-SSB transmission indication

  - RRC indication

  - MAC-CE indication

- L3 measurement based on OD-SSB

# 2 Discussion

RAN2#127 agreed to first study Scenario 2/2A and wait RAN1 conclusion on Scenario 3A/3B [2].

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

We provide Figure.1 to illustrate Scenario 2/2A for ease of followed discussion.

Figure.1 Illustration of considered scenarios (scenario 2/2A) of on-demand SSB

## 2.1 OD-SSB transmission indication

### 2.1.1 RRC indication

RAN2#127 [2] made below agreement on RRC indication of initial state (i.e. activation/deactivation) of OD-SSB. Whether to allow dynamic reconfiguration of OD-SSB state is not clear.

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

We don’t support the dynamic reconfiguration due to below considerations:

- Irrespective of whether it is supported, scenario 2A (i.e. simultaneous SCell and OD-SSB activation) can’t be supported because existing RRC indication doesn’t support dynamic reconfiguration of SCell activation/deactivation.

  - According to below copy of TS 38.331 [3], RRC can only configure the initial state of SCell.

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Need S

]],

omit

}

sCellState Indicates whether the SCell shall be considered to be in activated state upon SCell configuration. If the field is included for an SCell configured with TRS for fast activation of the SCell, such TRS is not used for the corresponding SCell.

  - TS 38.133 only specified requirement of RRC based SCell activation upon SCell addition (i.e. initial state) in Section 8.3.4 [4].

- It can’t bring benefit of either signalling overhead reduction or signalling latency reduction, compared with RRC de-configuration.

  - Signalling overhead: dynamic reconfiguration needs to include one bit in RRC message, but RRC de-configuration can also use one bit to release OD-SSB configuration (e.g. via SetupRelease structure).

  - Signalling latency: both dynamic reconfiguration and RRC de-configuration have the same latency of 19ms according to TS 38.331 [3] because the main latency contributor is the UE’s processing time for the RRC message.

- It will be duplicated with MAC-CE indication.

  - In Rel-16, RRC based SCell activation was introduced for the purpose of reducing initial SCell activation latency upon SCell addition / resume with RRC message. RAN2 didn’t introduce dynamic reconfiguration because it was duplicated with MAC-CE.

Observation 1: Dynamic RRC reconfiguration of OD-SSB state (activation/deactivation) will bring below issues:

- Scenario 2A anyway can’t be supported because RRC can only configure initial state of SCell in legacy.

- It can’t bring either signalling overhead or latency benefit, compared with RRC de-configuration.

- It will be duplicated with MAC-CE indication.

Thus, we propose:

Proposal 1: Don’t support RRC reconfiguration of OD-SSB state (activation/deactivation), i.e. RRC based indication is only used to indicate the initial activation/deactivation state of OD-SSB.

### 2.1.2 MAC-CE indication

RAN2#127 [2] agreed to introduce a new MAC-CE to indicate OD-SSB state. It is FFS whether to optimize for scenario 2A.

OD-SSB transmission indication:

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

We now prefer not to optimize scenario 2A due to below considerations:

- RAN1#118 has agreed to include SSB periodicity in MAC-CE and other SSB configurations are under discussion [5]. It already makes the new MAC-CE of OD-SSB indication quite complicated. The further optimization to include SCell activation state for scenario 2A may make the UE behavior even more complex.

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

- RAN2#127 discussed another alternative of MAC-CE format with variable size which is proposed in our contribution [6]. It is mainly intended to handle the case that OD-SSB indication and SCell indication are sent in different time. However, after carefully checking definition of Scenario 2A, we are aware that scenario 2A requires OD-SSB indication and SCell indication being sent simultaneously, and it is scenario 3A that OD-SSB indication and SCell indication are sent in different time. As it is FFS whether RAN1 will support scenario 3A, we think that MAC-CE format design doesn’t need to consider different timing at this stage.

Observation 2: It is scenario 3A that OD-SSB indication and SCell indication are sent in different time. As it is FFS whether RAN1 will support scenario 3A, MAC-CE format design doesn’t need to consider it at this stage.

Based on above discussion, we think one single new MAC-CE can handle both scenarios:

- Scenario 2: the UE only receives the new MAC-CE to indicate OD-SSB state (activation / deactivation).

- Scenario 2A: the UE receives both the new MAC-CE of OD-SSB indication and the legacy SCell activation MAC-CE simultaneously, where the simultaneous reception of the both MAC-CE can only be achieved by NW multiplexing them in one single MAC PDU.

Observation 3: Scenario 2A requires the UE to receive both new OD-SSB MAC-CE and legacy SCell activation MAC-CE simultaneously, which can be achieved only by NW multiplexing them in one single MAC PDU.

Thus, according to Observation 3 and 4, we propose:

Proposal 2: Don’t introduce separate MAC-CE for scenario 2A, where the UE expects to receive the new OD-SSB MAC-CE and legacy SCell activation MAC-CE being multiplexed in one single MAC PDU.

Please note that legacy SCell A/D MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells, according to TS 38.321 [7]. Similarly, the new OD-SSB MAC-CE should also support the two formats.

Proposal 3: The new OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

Finally, as RAN1 is still discussing whether to include other SSB configuration in MAC-CE, we think that RAN2 can’t discuss detailed MAC-CE format until RAN1 make conclusion.

Proposal 4: RAN2 postpone the discussion on detailed MAC-CE format design until RAN1 make conclusion on all the required SSB configurations in the MAC-CE.

## 2.2 L3 measurement based on on-demand SSB

In RAN2#127 [2], it was agreed to consider L3 RRM for both case 1 and case 2:

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Meanwhile, RAN1#118 has agreed to include SSB periodicity in MAC-CE [5]:

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

As we discussed in RAN2#127 [2], we think the above RAN1 / RAN2 agreements require spec changes on SMTC in L3 measurement. As follows, we first elaborate gaps of L3 measurement framework, and then discuss solution for L3 RRM based on OD-SSB covering both case 1 and case 2.

### 2.2.1 Gaps of existing L3 measurement framework

For the existing L3 measurement framework, we identify two gaps to enable L3 RRM for OD-SSB case 1 and case 2.

Gap 1: Legacy SMTC is periodical pattern which can’t well adapt to dynamics of OD-SSB in both case 1 and case 2

In legacy L3 RRM based on SSB, the NW need to configure UE SMTC in MeasObjectiveNR, as highlighted below:

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                   OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

omit unrelated part

}

SMTC is a periodical window because it is based on the assumption that SSB is always transmitted in periodical way. Furthermore, according to Section 5.5.2.10 of TS 38.331 [3], the UE shall not consider SSB outside the SMTC occasion for RRM measurements:

#### 5.5.2.10	Reference signal measurement timing configuration

…omit unrelated part...

On the indicated ssbFrequency, the UE shall not consider SS/PBCH block transmission in subframes outside the SMTC occasion for RRM measurements based on SS/PBCH blocks and for RRM measurements based on CSI-RS except for SFTD measurement (see TS 38.133 [14], clause 9.3.8).

It means that the UE only performs L3 measurements based on SSB within the SMTC window. However, one single SMTC may not cover all the required SSB(s) to perform L3 measurements because the assumption of periodic SSB transmission is NOT valid anymore for OD-SSB in both case 1 and case 2:

- Case 1: OD-SSB is transmitted only during one period (i.e. absence outside of the period). And its periodicity may be dynamically changed via MAC-CE during the transmission period.

- Case 2: OD-SSB is expected to use different periodicity from always-on SSB (e.g. denser than always-on SSB). And its periodicity may be dynamically changed via MAC-CE.

Observation 4: Legacy SMTC is a periodical pattern which can’t well adapt to dynamics of OD-SSB in both case 1 and case 2:

- Case 1: OD-SSB is transmitted only during one period, and its periodicity may be dynamically changed via MAC-CE during the transmission period.

- Case 2: OD-SSB is expected to use different periodicity from always-on SSB, and its periodicity may be dynamically changed via MAC-CE.

We take an example of Case 2 illustrated in Figure.2, where the UE is configured with one SMTC whose window covers the always-on SSB. However, when OD-SSB is triggered after T1, the configured SMTC window can’t cover OD-SSB. Thus, although the UE may know when OD-SSB starts transmission via activation MAC-CE, it can’t perform L3 measurements towards OD-SSB because OD-SSB is not covered within its configured SMTC. One may argue that NW implementation may resolve this issue (e.g., via proper configuration of SMTC by considering OD-SSB pattern). However, please note that SMTC is configured per frequency rather than per cell, which makes NW difficulty to configure one single SMTC window to cover all SSBs without causing negative impacts to legacy UE. For example, NW may configure a SMTC window with short periodicity to cover both always-on SSB and OD-SSB, but it will lead to UE extra power consumption, especially for the legacy UEs.

Figure.2 Illustration of issue scenario: one single SMTC can only cover always-on SSB but can’t cover OD-SSB. So, UE can’t perform L3 measurements towards OD-SSB, although it knows when OD-SSB is triggered.

Observation 5: As SMTC is configured per frequency rather than per cell, it is difficulty for NW to configure one single SMTC window to cover both OD-SSB and always-on SSB without causing negative impacts to legacy UE (e.g. extra power consumption if NW configures a dense SMTC to cover both types of SSBs).

Gap 2: The UE is mandated to perform serving cell measurement when servingCellMO is configured, which is not aligned with agreement of case 1

From Section 5.3.3.1 of TS 38.331

omit unrelated part

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

omit unrelated part

According to Section 5.3.3.1 of TS 38.331 [3], the UE is mandated to perform serving cell measurement when servingCellMO is configured (even if no ReportConfigNR is linked with the servingCellMO):

This is a basic principle ever since from LTE L3 RRM which means that the dynamic activation/deactivation for serving cell measurement is not allowed. We think it is conflicted with below RAN2#127 agreement on case 1 [2], which means that the UE needs to stop performing measurement when OD-SSB is not transmitted.

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

Observation 6: In legacy RRC, the UE is mandated to perform serving cell measurement when servingCellMO is configured (even if no ReportConfigNR is linked with the servingCellMO). It is conflicted with RAN2#127 agreement on case 1 which means that the UE needs to stop measurement when OD-SSB is not transmitted.

Thus, we can summarize the two gaps of legacy L3 RRM as:

- Gap 1: Legacy SMTC is periodical pattern which can’t well adapt to dynamics of OD-SSB in both case 1 and case 2.

- Gap 2: The UE is mandated to perform serving cell measurement when servingCellMO is configured, which is not aligned with agreement of case 1.

### 2.2.2 Solution of RRM based on on-demand SSB

To fill the two gaps identified in Section 2.2.2, we propose the following unified solution for both case 1 and case 2:

- Introduce a new optional IE (e.g. ODSSB-ConfigMobility) within MeasObjectNR to configure L3 measurement of OD-SSB, which includes at least a list of SMTC for different OD-SSB patterns (e.g. different periodicity).

  - In case 1, smtc1 is absent because no always-on SSB. Upon reception of OD-SSB MAC-CE, the UE applies the corresponding SMTC from the configured SMTC list in ODSSB-ConfigMobility.

    - Note that the UE will regard all SSBs are outside of SMTC when smtc1 is absent, and thereby it will not perform L3 measurements.

  - In case 2, smtc1 is set according to pattern of always-SSB. Upon reception of OD-SSB MAC-CE, the UE applies the corresponding SMTC from the configured SMTC list in ODSSB-ConfigMobility.

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                          OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                 OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

omit unrelated part

...,

omit unrelated part

[[

odssb-ConfigMobility             ODSSB-ConfigMobility                                      OPTIONAL,   -- Need R

]]

}

ODSSB-ConfigMobility::=.       SEQUENCE {

smtcList                          SEQUENCE (SIZE (1..maxNODSSB)) OF SSB-MTC                             OPTIONAL,   -- Need N

}

We believe that this is the solution with the least specification impacts, and we also provide an example in Figure. 3 to help illustration.

Figure.3 Illustration of unified solution for case 1 and case 2 with example of only one OD-SSB periodicity

During RAN2#127 [2], the other two candidate solutions were proposed, but we are not convinced.

- Other solution 1: UE autonomously adjusts SMTC / measurements according to OD-SSB pattern of the SCell.

  - It will result in at least 3 issues: 1) It may cause misalignment between UE and NW on the actually measured SSB(s) by the UE; 2) The autonomous SMTC adjustment according to OD-SSB of SCell may challenge the principle that MO is per frequency configured; 3) It may result in significant spec changes in TS 38.133 where all L3 measurement requirements are specified with SMTC periodicity (e.g. SCell activation latency) [4]. The autonomous SMTC adjustment may essentially make these requirements become UE implementation.

- Other solution 2: Rely on NW implementation (e.g. NW reconfigures or de-configures servingCellMO when OD-SSB is activated or deactivation, respectively).

  - Please note that servingCellMO is RRC signaling. Then, such NW implementation will imply that NW always needs to send one RRC message for each state change of OD-SSB between activation and deactivation. It will make MAC-CE signaling pointless.

Based on above analysis, we propose:

Proposal 5: Introduce a configuration within MeasObjectNR to include at least a list of SMTC for different OD-SSB patterns (e.g. different periodicity), which the UE applies upon reception of the OD-SSB indication.

- In case 1, smtc1 is absent because no always-on SSB. Upon reception of OD-SSB indication, the UE applies the corresponding SMTC from the configured SMTC list in ODSSB-ConfigMobility.

- In case 2, smtc1 is set according to pattern of always-SSB. Upon reception of OD-SSB indication, the UE applies the corresponding SMTC from the configured SMTC list in ODSSB-ConfigMobility.

## 2.3 UL signalling to trigger on-demand SSB transmission

Agreement (RAN1#116)

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement (RAN1#116b)

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

RAN1#116 [3] and RAN1#116b [4] made below related agreement:

Thus, RAN1 didn’t conclude whether to support on-demand SSB SCell operation triggered by UE. We think whether to support it depends on the use case of on-demand SSB. For example, if Scenario 3B (i.e., for SSB-less operation after on-demand SSB is terminated) is finally precluded, we think it is unnecessary to introduce UL signaling as legacy SCell activation / deactivation procedure. In this case, it is left to the NW implementation to determine when to activate / deactivate one SCell.

Due to such dependency, RAN2 can wait RAN1 on whether to introduce on-demand SSB SCell operation triggered by UE and related signaling.

Proposal 6: RAN2 wait RAN1 on whether to introduce on-demand SSB SCell operation triggered by UE and related signaling.

# 3 Conclusion

In this contribution, we further discuss RAN2 specification impacts of on-demand SSB for SCell. Our observations are:

Observation 1: Dynamic RRC reconfiguration of OD-SSB state (activation/deactivation) will bring below issues:

- Scenario 2A anyway can’t be supported because RRC can only configure initial state of SCell in legacy.

- It can’t bring either signalling overhead or latency benefit, compared with RRC de-configuration.

- It will be duplicated with MAC-CE indication.

Observation 2: It is scenario 3A that OD-SSB indication and SCell indication are sent in different time. As it is FFS whether RAN1 will support scenario 3A, MAC-CE format design doesn’t need to consider it at this stage.

Observation 3: Scenario 2A requires the UE to receive both new OD-SSB MAC-CE and legacy SCell activation MAC-CE simultaneously, which can be achieved only by NW multiplexing them in one single MAC PDU.

Observation 4: Legacy SMTC is a periodical pattern which can’t well adapt to dynamics of OD-SSB in both case 1 and case 2:

- Case 1: OD-SSB is transmitted only during one period, and its periodicity may be dynamically changed via MAC-CE during the transmission period.

- Case 2: OD-SSB is expected to use different periodicity from always-on SSB, and its periodicity may be dynamically changed via MAC-CE.

Observation 5: As SMTC is configured per frequency rather than per cell, it is difficulty for NW to configure one single SMTC window to cover both OD-SSB and always-on SSB without causing negative impacts to legacy UE (e.g. extra power consumption if NW configures a dense SMTC to cover both types of SSBs).

Observation 6: In legacy RRC, the UE is mandated to perform serving cell measurement when servingCellMO is configured (even if no ReportConfigNR is linked with the servingCellMO). It is conflicted with RAN2#127 agreement on case 1 which means that the UE needs to stop measurement when OD-SSB is not transmitted.

Based on above observations, our proposals are:

RRC indication

Proposal 1: Don’t support RRC reconfiguration of OD-SSB state (activation/deactivation), i.e. RRC based indication is only used to indicate the initial activation/deactivation state of OD-SSB.

MAC-CE indication

Proposal 2: Don’t introduce separate MAC-CE for scenario 2A, where the UE expects to receive the new OD-SSB MAC-CE and legacy SCell activation MAC-CE being multiplexed in one single MAC PDU.

Proposal 3: The new OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

Proposal 4: RAN2 postpone the discussion on detailed MAC-CE format design until RAN1 make conclusion on all the required SSB configurations in the MAC-CE.

L3 measurement of on-demand SSB

Proposal 5: Introduce a configuration within MeasObjectNR to include at least a list of SMTC for different OD-SSB patterns (e.g. different periodicity), which the UE applies upon reception of the OD-SSB indication.

- In case 1, smtc1 is absent because no always-on SSB. Upon reception of OD-SSB indication, the UE applies the corresponding SMTC from the configured SMTC list in ODSSB-ConfigMobility.

- In case 2, smtc1 is set according to pattern of always-SSB. Upon reception of OD-SSB indication, the UE applies the corresponding SMTC from the configured SMTC list in ODSSB-ConfigMobility.

UL signaling

Proposal 6: RAN2 wait RAN1 on whether to introduce on-demand SSB SCell operation triggered by UE and related signaling.

# 4 References

[1] RP-242354, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#127, Chair Notes.

[3] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[4] TS 38.133-v18.4.0, Requirements for support of radio resource management, 2024-1.

[5] RAN1#118, Chair Notes.

[6] R2-2406669, Discussion on RAN2 work of on-demand SSB for Scell, Apple.

[7] TS 38.321-v18.0.0, Medium Access Control (MAC) protocol specification, 2023-12.

[8] RAN1#116, Chair Notes.

[9] RAN1#116b, Chair Notes.