# 3GPP TSG RAN WG2 Meeting #127b      	                           R2-2408561
Hefei, China, 14th – 18th Oct., 2024

Agenda item:	8.5.3

Source:	Apple

Title:	Discussion on on-demand SIB1

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

The Rel-19 WID of Network Energy Savings (WID RP-242354) was updated in RAN#105 [1]. The study objective on on-demand SIB1 (OD-SIB1) for UEs in IDLE/INACTIVE state becomes normative WI objective as copied:

In this contribution, we share our view on essential issues from below aspects:

- UL WUS configuration and signaling

- UL WUS configuration update mechanism

- Detailed procedure of OD-SIB1 acquisition

- Access restriction for legacy UEs

# 2 Discussion

## 2.1 UL WUS configuration and signalling

### 2.1.1 Issue 1: Whether UL WUS configuration is included in SIB1 or new SIB of Cell A

In RAN1#117 [2], below table was agreed as starting point of required UL WUS configuration.

Agreement For further study of on-demand SIB1 in idle/inactive mode, use the following Table I (from R1-2405106, Ericsson) as a starting point to discuss the required parameters/contents inside the UL WUS configuration. Table I. Purpose; Parameters. To which cell does the config applies; NES-CellId; PhysCellId. ARFCN-ValueNR. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResource; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. rsrp-ThresholdSSB. prach-RootSequenceIndex. msg1-SubcarrierSpacing. restrictedSetConfig. tdd-UL-DL-ConfigurationCommon. frequencyInfoUL; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz. SIB1 reception; pdcch-ConfigSIB1; ssb-SubcarrierOffset. controlResourceSetZero. searchSpaceZero. RAR Reception; pdcch-ConfigOD-SIB1-RAR; controlResourceSet. monitoringSlotPeriodicityAndOffset. Duration. monitoringSymbolsWithinSlot. aggregationLevels.

Although this is just a starting point of discussion, it is sufficient to estimate payload size of UL WUS configuration. Below is our analysis on approximate payload size of each part:

| Purpose | Approximate payload size analysis | Note | Total payload size |
| --- | --- | --- | --- |
| Identify NES cell(s) | PhysCellId (10bit) ARFCN-ValueNR (22bit) | This is only value of one NES cell. Please note that RAN2 agreed multiple NES cells’ UL WUS config can be included in on Cell A. | 32bit |
| RACH resource parameter for SIB1 request | rach-ConfigGeneric (40bit) - prach-ConfigurationIndex (8bit) - msg1-FDM (2bit) - msg1-FrequencyStart (9bit) - zeroCorrelationZoneConfig (4bit) - preambleReceivedTargetPower (8bit) - preambleTransMax (4bit) - powerRampingStep (2bit) - ra-ResponseWindow (3bit) ssb-perRACH-Occasion (3bit) si-RequestPeriod (3bit) SI-RequestResources (14bit) - ra-PreambleStartIndex (6bit) - ra-AssociationPeriodIndex (4bit) - ra-ssb-OccasionMaskIndex (4bit) Other RACH config (22bit) - msg1-SubcarrierSpacing (3bit) - prach-RootSequenceIndex (10bit) - rsrp-ThresholdSSB (7bit) - n-TimingAdvanceOffset (2bit) - TDD-UL-DL-ConfigCommon (~30bit) - ss-PBCH-BlockPower (7bit) - restrictedSetConfig (1bit) | - Assume up to 275 PRBs. - n-TimingAdvanceOffset is missed. | 120bit |
| BWP info of initial BWP | BWP (~20bit), including locationAndBandwidth, SCS, cyclicPrefix. | It is necessary to identify PRACH and RAR frequency location because PRACH / CORESET / SS is configured per BWP. | 20bit |
| UL frequency info | FrequencyInfoUL-SIB (~133bit) - MultiFrequencyBandListNR (80bit) - absoluteFrequencyPointA (22bit) - scs-SpecificCarrierList (~24bit) - p-Max (6bit) - frequencyShift7p5khz (1bit) | - It is necessary to obtain carrier information of PRACH transmission and related setting (e.g. UL power). - Assume up to 8 bands - Assume only 1 SCS. | 133bit |
| SIB1 reception | pdcch-ConfigSIB1 (8bit) | It seems conflicted with latest RAN1 agreement | 8bit (TBD) |
| RAR reception | RA-SearchSpace (6bit) monitoringSlotPeriodicityAndOffset (16bit) duration (12bit) monitoringSymbolsWithinSlot (14bit) nrofCandidates (6bit) | - Assume CORESET#0 is used for RAR reception (i.e. no extra common CORESET configuration). - Assume only one search space is configured for RAR. | 52bit |
| Total payload size: 369bit×Num of NES Cell (e.g. If 8 NES Cells’ UL WUS config are included, 2952 bit is required) * * Optimization of area-specific configuration is not considered, i.e. one NES cell has one UL WUS config in this analysis. |  |  |  |

Table.1 Analysis on required payload size of UL WUS configurations in Cell A

As can be observed from Table.1, at least 369 bit is required if only one NES cell’s UL WUS configurations is included, and it can be up to 2952bit if 8 NES cells’ UL WUS configurations are included. This number may increase because some configurations under discussion (e.g. window to detect OD-SIB1 in RAN1) are not included.

However, according to TS 38.331 [3], SIB1 or a SI message is limited to 2976 bit.

NOTE 2:	The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

Observation 1: At least 369bit is required if only one NES cell’s UL WUS configurations is included, and it can be up to 2952 bit if 8 NES cells’ UL WUS configurations are included. However, according to TS 38.331, SIB1 or a SI message is limited to 2976 bit.

Considering payload size of current SIB1 is already very large and it is increasing with each release, we think NES cell(s)’ UL WUS configuration can only be included in a new SIB of Cell A. The maximum number of NES cells can be further studied.

Proposal 1: One or multiple NES cell(s)’ UL WUS configuration are included in a new SIB of Cell A and NES cell. FFS maximum number of NES cells.

### 2.1.2 Issue 2: Detailed UL WUS configuration

Compared our analysis in Table.1 with RAN1 agreed baseline, we have below differences:

- n-TimingAdvanceOffset is missed in RAN1 agreed baseline.

  - It is necessary for the UE to calculate initial TA.

- BWP info of initial BWP (i.e. BWP including locationAndBandwidth, SCS, cyclicPrefix) is missed in RAN1 agreed baseline.

  - It is necessary for the UE to identify PRACH and RAR frequency location because PRACH/ CORESET / SS is configured per BWP

- SIB1 reception parameters (pdcch-ConfigSIB1 to indicate CORESET#0 and SS#0) seems to be conflicted with below RAN1#118 agreement [4] because it only falls intro Option 2. We can wait RAN1 update.

Agreement For Case-2: For type 0 PDCCH monitoring occasions for on demand SIB1, on how the search space zero configuration is provided, RAN1 to down select from the following options: - Option 1: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell - Option 2: searchSpaceZero for on-demand SIB1 is provided from UL WUS configuration - Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS. Combination of multiple options is not precluded.

Thus, we propose:

Proposal 2: On top of RAN1#117 agreed UL WUS configuration, at least n-TimingAdvanceOffset and initial BWP info (i.e. BWP including locationAndBandwidth, SCS, cyclicPrefix) are also required.

## 2.2 UL WUS configuration update mechanism

RAN2#127 made below agreement on UL WUS configuration update mechanism [5]:

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

We think it is clear the camped NES cell can update its own UL WUS configuration. However, it is not clear whether Rel-19 UEs can update UL WUS configuration of neighbor NES cells as well. This issue was also discussed in RANP#105 [6], but companies had different understanding. We think RAN2 should align the understanding.

We prefer to allow the Rel-19 UEs to update UL WUS configuration of neighbor NES cells. Otherwise, once cell reselection is triggered, the Rel-19 UEs have to return to Cell A to obtain the latest UL WUS configuration of neighbor cells. It is unnecessary because it introduces extra cell reselection latency. Meanwhile, following our Proposal 1, we can reuse the same design to include neighbor cells’ UL WUS configuration in camping NES cell’s new SIB, which doesn’t need extra specification work. And we should not restrict NES cell to include neighbor cells’ UL WUS configuration in its new SIB.

Observation 2: If UL WUS config of neighbor NES cells can’t be updated in camping NES cell, the Rel-19 UEs have to return to Cell A to obtain their latest UL WUS configuration once cell reselection is triggered. It is unnecessary because it introduces extra cell reselection latency.

Observation 3: The same mechanism can be reused to include neighbor cells’ UL WUS configuration in camping NES cell’s new SIB, which doesn’t need extra specification work.

Thus, we propose:

Proposal 3: Once Rel-19 NES UE camps on the NES cell, it may receive updated UL WUS configuration of both the camping NES cell and neighbor NES cells.

## 2.3 Procedure of OD-SIB1 acquisition

### 2.3.1 Trigger condition of OD-SIB1 acquisition

The related study agreements in RAN2#126b [7] can be found below:

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

Meanwhile, whether a new RSRP threshold is needed was also discussed in RAN2#126 [7], but no conclusion was made:

Proposal 9: In addition to the already agreed conditions (i.e. UE doesn’t have a valid SIB1, the cell doesn’t broadcast SIB1 and supports on-demand SIB1) introduce a new RSRP triggering condition for the NES UE to be allowed to send WUS to trigger on-demand SIB1. FFS what is the triggering condition and whether the triggering condition is a new threshold.

[Samsung, Xiaomi, OPPO, NEC, Lenovo]: Think the existing cell reselection criterion is enough. [Nokia]: May be useful, but reluctant to agree with them now. We can discuss it in stage 3. [ZTE]: Support the proposal. It is good for NW is allowed to configure separate threshold than cell reselection criterion threshold. [LG]: Current cell reselection criterion is not absolute threshold, it’s relative to the serving cell’s radio condition. Support the proposal. [IDC]: If we have new threshold and the threshold is not met, anyway the existing cell reselection criterion should be used to trigger WUS transmission. [Vivo]: With the new threshold, it will make UE behaviour more complicated. We need to consider many combination cases, e.g. new threshold is {met, not met} + existing cell reselection criterion is {met, not met}. [Samsung]: Rather discussing whether new threshold is required or not, it’s better to discuss whether we’ll allow earlier or later WUS transmission triggering (than cell reselection criterion). The UE may achieve it w/o new threshold. [Apple]: Seems each company has different understanding on the use of the threshold. It’s good to see overall companies’ understanding on how the threshold is used or works.

Our preference is:

- On the need of new RSRP threshold: we think its benefit is not clear. In addition, considering legacy cell reselection criterion already uses RSRP threshold (e.g. ThreshX, HighP, ThreshX, LowP), new RSRP threshold will bring more complicated UE behavior during cell reselection (e.g. if new threshold is met but legacy threshold is not). Thus, we prefer to reuse legacy cell reselection criterion as OD-SIB1 trigger condition.

- Whether UE is mandate to trigger OD-SIB1 acquisition when one NES cell meets condition: we think it should NOT be mandate (i.e. it is up to UE implementation) at least due to below cases:

  - If the selected NES cell is broadcasting SIB1 (e.g. due to request of other UE before), it is obvious that the Rel-19 UE doesn’t need to trigger OD-SIB1 acquisition procedure.

  - Besides one NES cell, it is possible that there are normal cell or other NES cell also meeting the trigger condition (i.e. legacy cell reselection criterion). For example, one normal cell in another frequency with same frequency priority may also meet the cell reselection criterion. In this case, we think the UE should be allowed to reselect to the normal cell rather than triggering OD-SIB1 acquisition in the NES cell, which will incur extra access latency.

Observation 4: When one NES cell meets trigger condition (i.e. legacy cell reselection criterion), the UE may not trigger OD-SIB1 procedure at least in below cases:

- The selected NES cell is broadcasting SIB1 (e.g. due to request of other UE before).

- Besides one NES cell, there are normal cell or other NES cell also meeting the trigger condition.

Thus, we propose:

Proposal 4: If one NES cell meets the existing intra-frequency / inter-frequency cell reselection criterion, the UE may trigger OD-SIB1 acquisition procedure towards this NES cell. No need to specify other conditions.

### 2.3.2 UE behaviour after sending OD-SIB1 request

According to RAN1 / RAN2 agreements made in study phase, our understanding on the UE behaviour after sending OD-SIB1 request is illustrated in Figure. 1.

Figure. 1 Illustration of UE behaviour after sending OD-SIB1 request based on recent RAN1/RAN2 agreements

We think the main remaining issues are:

- OD-SIB1 monitoring window, including how it is configured, offset and UE behavior if window expires.

- How the UE obtains COREST#0 and SearchSpace#0 to receive OD-SIB1

Please note RAN1#118 has made related agreements [4]:

Agreement (window mechanism) At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration Agreement (window mechanism) At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options: - Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission - FFS: Definition of RAR reception time - Option 2: The reference time point is defined based on the UL-WUS transmission time - Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission Agreement (SS#0 to receive OD-SIB1) For Case-2: For type 0 PDCCH monitoring occasions for on demand SIB1, on how the search space zero configuration is provided, RAN1 to down select from the following options: - Option 1: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell - Option 2: searchSpaceZero for on-demand SIB1 is provided from UL WUS configuration - Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS. Combination of multiple options is not precluded.

Since RAN1 has started the discussion, we think RAN2 can wait RAN1 conclusion and then identify any RAN2 spec impacts.

Proposal 5: RAN2 wait RAN1 conclusion on below aspects:

- OD-SIB1 monitoring window, including how it is configured, offset and UE behavior if window expires.

- How the UE obtains COREST#0 and SearchSpace#0 to receive OD-SIB1

### 2.3.3 Failure case and UE behaviour

RAN2#126b made below agreements [7]:

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

We think that 2nd agreement is not clear in below aspects:

- Detailed condition of “random access procedure failure of OD-SIB1 request”

- Whether the behavior is specified or leave to UE implementation

From Section 5.1.4 of TS 38.321:

Omit unrelated part

1>	if ra-ResponseWindow configured in RACH-ConfigCommon expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received:

2>	consider the Random Access Response reception not successful;

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

2>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

3>	if the Random Access Preamble is transmitted on the SpCell:

4>	indicate a Random Access problem to upper layers;

4>	if this Random Access procedure was triggered for SI request:

5>	consider the Random Access procedure unsuccessfully completed.

3>	else if the Random Access Preamble is transmitted on an SCell:

4>	consider the Random Access procedure unsuccessfully completed.

For 1), we think it is similar to existing Msg1 based OD-SIB procedure, whose random access procedure failure can be found in TS 38.321 [8]:

As indicated, the random access procedure fails when UE transmits Msg1 for preambleTransMax times but still doesn’t receive RAR. We think it can be reused for OD-SIB1, i.e. “random access failure” means when UE transmits the OD-SIB1 request for configured maximum times but still doesn’t receive RAR. Meanwhile, one difference from legacy OD-SIB1 procedure is the OD-SIB1 window mechanism. As the window mechanism is not clear yet, we think whether OD-SIB1 window expires without reception of SIB1 is also regarded as procedure failure can be further discussed.

For 2), we prefer to specify UE behavior rather than leaving to UE implementation. In legacy OD-SIB procedure, it is up to UE implementation when to retry the SI request because it is not essential to acquire other SIB(s). However, SIB1 is most essential information, and thereby RAN2#126b has agreed that the UE bars the NES cell if the procedure fails. As the cell barring will cause significant impact to system performance, the UE behavior should be clearly specified.

Observation 5: In legacy OD-SIB procedure, it is up to UE implementation when to retry the SI request because it is not essential to acquire other SIB(s). However, SIB1 is most essential information, and RAN2#126b has agreed that the UE bars the NES cell if the procedure fails. Thus, the UE behavior should be clearly specified.

Thus, we propose:

Proposal 6: Update RAN2#126 agreement on UE behavior on RACH failure of OD-SIB1 as below:

When UE transmits the OD-SIB1 request for configured maximum times but still doesn’t receive RAR, UE is specified to regard OD-SIB1 can’t be acquired in the NES cell and considers it as barred. FFS whether OD-SIB1 window expiry without reception of SIB1 is also regarded as procedure failure.

### 2.3.4 Paging interruption during OD-SIB1 acquisition

This issue was raised by some company contribution in study phase [9], but it was not treated. In details, after the UE sends OD-SIB1 request in NES cell, it can’t monitor paging in NES Cell until OD-SIB1 is acquired. This is an extra paging interruption compared with legacy cell reselection. We think it is a valid issue because TS 38.133 has specified requirement of paging interruption during cell reselection (i.e. “The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms”) [10]:

#### 4.2.2.6	Maximum interruption in paging reception

UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW.

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc.

At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms.

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell.

TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell.

These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

Observation 6: TS 38.133 has specified requirement of paging interruption during cell reselection (i.e. “The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms”).

We think there are two alternatives:

- Alt-1: leave it to UE implementation. But RAN2 need to send LS to RAN4 to check whether the paging interruption requirement needs change.

- Alt-2: include PCCH-Config of NES cell in UL WUS configuration provided by Cell A, so that Rel-19 UEs can monitor paging from NES cell during OD-SIB1 acquisition.

Since RAN2 has not discussed this issue before, we suggest RAN2 to discuss these 2 alternatives.

Proposal 7: On paging interruption issue during OD-SIB1 acquisition, RAN2 discuss the following 2 solutions:

- Alt-1: leave it to UE implementation. RAN2 send LS to RAN4 to check whether the paging interruption requirement during cell reselection specified in TS 38.133 needs change.

- Alt-2: include PCCH-Config of NES cell in UL WUS configuration provided by Cell A, so that Rel-19 UEs can monitor paging from NES cell during OD-SIB1 acquisition.

## 2.4 Access restriction for legacy UEs

The following candidate solutions of access restriction for legacy UEs were identified in RAN2#127 [5]:

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Our preference is:

- Option 3 should be deprioritized because it can only be a complementary solution for Option 1/2. In another wording, it is an enhancement on top of Option 1/2. Thus, it should be discussed after down-selection between Option 1 and Option 2.

- Between Option 1 and Option 2, we prefer to wait RAN1 conclusion on whether / how to specify “no SIB1 indication via ssb-SubcarrierOffset in MIB”. In detail, if RAN1 agree to use a specifical / reserved value of ssb-SubcarrierOffset for NES cell, it means that legacy UE will ignore NES cells during cell reselection and RAN2 can specify to allow Rel-19 UEs to access NES cells which are identified via ssb-SubcarrierOffset. In this case, Option 1 is not necessary.

- Meanwhile, to avoid legacy impact, some companies proposed to restrict NES cells to use NCD-SSB in RANP#105 [6]. It can also be one candidate solution, but we may not add them in the list for now because it anyway needs to be discussed in RAN1 first.

Observation 7: If RAN1 agree to use a specifical / reserved value of ssb-SubcarrierOffset for NES cell, it means that legacy UE will ignore NES cells during cell reselection and Rel-19 UEs can be specified to access NES cells which are identified via ssb-SubcarrierOffset.

Thus, we propose to postpone the discussion in RAN2:

Proposal 8: On access restriction for legacy UEs, RAN2 wait RAN1 conclusion on whether / how to specify “no SIB1 indication via ssb-SubcarrierOffset in MIB” and whether to restrict NES cells to use NCD-SSB.

# 3 Conclusion

In this contribution, we discuss on-demand SIB1 for IDLE/INACTIVE UEs. Our observations are:

Observation 1: At least 369bit is required if only one NES cell’s UL WUS configurations is included, and it can be up to 2952 bit if 8 NES cells’ UL WUS configurations are included. However, according to TS 38.331, SIB1 or a SI message is limited to 2976 bit.

Observation 2: If UL WUS config of neighbor NES cells can’t be updated in camping NES cell, the Rel-19 UEs have to return to Cell A to obtain their latest UL WUS configuration once cell reselection is triggered. It is unnecessary because it introduces extra cell reselection latency.

Observation 3: The same mechanism can be reused to include neighbor cells’ UL WUS configuration in camping NES cell’s new SIB, which doesn’t need extra specification work.

Observation 4: When one NES cell meets trigger condition (i.e. legacy cell reselection criterion), the UE may not trigger OD-SIB1 procedure at least in below cases:

- The selected NES cell is broadcasting SIB1 (e.g. due to request of other UE before).

- Besides one NES cell, there are normal cell or other NES cell also meeting the trigger condition.

Observation 5: In legacy OD-SIB procedure, it is up to UE implementation when to retry the SI request because it is not essential to acquire other SIB(s). However, SIB1 is most essential information, and RAN2#126b has agreed that the UE bars the NES cell if the procedure fails. Thus, the UE behavior should be clearly specified.

Observation 6: TS 38.133 has specified requirement of paging interruption during cell reselection (i.e. “The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms”).

Observation 7: If RAN1 agree to use a specifical / reserved value of ssb-SubcarrierOffset for NES cell, it means that legacy UE will ignore NES cells during cell reselection and Rel-19 UEs can be specified to access NES cells which are identified via ssb-SubcarrierOffset.

Based on observations, our proposals are:

UL WUS configuration and signaling

Proposal 1: One or multiple NES cell(s)’ UL WUS configuration are included in a new SIB of Cell A and NES cell. FFS maximum number of NES cells.

Proposal 2: On top of RAN1#117 agreed UL WUS configuration, at least n-TimingAdvanceOffset and initial BWP info (i.e. BWP including locationAndBandwidth, SCS, cyclicPrefix) are also required.

UL WUS configuration update mechanism

Proposal 3: Once Rel-19 NES UE camps on the NES cell, it may receive updated UL WUS configuration of both the camping NES cell and neighbor NES cells.

Procedure of OD-SIB1 acquisition

Proposal 4: If one NES cell meets the existing intra-frequency / inter-frequency cell reselection criterion, the UE may trigger OD-SIB1 acquisition procedure towards this NES cell. No need to specify other conditions.

Proposal 5: RAN2 wait RAN1 conclusion on below aspects:

- OD-SIB1 monitoring window, including how it is configured, offset and UE behavior if window expires.

- How the UE obtains COREST#0 and SearchSpace#0 to receive OD-SIB1

Proposal 6: Update RAN2#126 agreement on UE behavior on RACH failure of OD-SIB1 as below:

When UE transmits the OD-SIB1 request for configured maximum times but still doesn’t receive RAR, UE is specified to regard OD-SIB1 can’t be acquired in the NES cell and considers it as barred. FFS whether OD-SIB1 window expiry without reception of SIB1 is also regarded as procedure failure.

Proposal 7: On paging interruption issue during OD-SIB1 acquisition, RAN2 discuss the following 2 solutions:

- Alt-1: leave it to UE implementation. RAN2 send LS to RAN4 to check whether the paging interruption requirement during cell reselection specified in TS 38.133 needs change.

- Alt-2: include PCCH-Config of NES cell in UL WUS configuration provided by Cell A, so that Rel-19 UEs can monitor paging from NES cell during OD-SIB1 acquisition.

Access restriction for legacy UEs

Proposal 8: On access restriction for legacy UEs, RAN2 wait RAN1 conclusion on whether / how to specify “no SIB1 indication via ssb-SubcarrierOffset in MIB” and whether to restrict NES cells to use NCD-SSB.

# 4 References

[1] RP-242354, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN1#117, Chair Notes.

[3] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[4] RAN1#118, Chair Notes.

[5] RAN2#127, Chair Notes.

[6] RANP#105, Chair Notes.

[7] RAN2#126, Chair Notes.

[8] TS 38.321-v18.0.0, Medium Access Control (MAC) protocol specification, 2023-12.

[9] R2-2406470, Remaining details to enable on-demand SIB1, Intel Corporation.

[10] TS 38.133-v18.4.0, Requirements for support of radio resource management, 2024-1.