# 3GPP TSG RAN WG2 Meeting #127b      	                           R2-2408563
Hefei, China, 14th – 18th Oct., 2024

Agenda item:	8.1.2.2

Source:	Apple

Title:	Remaining issues on LCM procedure of UE-sided model for AI/ML based beam management

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The updated Rel-19 WID of AI/ML for air interface (WID RP-242399) was agreed in RAN#105 [1]. The WI objective on general LCM framework, including applicable conditions, is led by RAN2 which is copied below:

In RAN2#125b [2] RAN2#126 [3], LCM procedure of UE-sided model of beam management was discussed, and several agreements were made with below FFS unaddressed:

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

Then, RAN2#127 had post-meeting email discussion on LCM procedure [4][5]. The basic procedure of applicability functionality reporting was agreed in RAN2#127 [6] and some questions with potential RAN1 impacts were identified to be included in the LS for RAN1 (R2-2407848) [7]. However, we believe there are still some RAN2 remaining issues (except the questions for RAN1 in [7]) need further discussion.

In this contribution, we further discuss above leftover from below aspects

- Remaining issues on RAN2#126 agreement

- Remaining issues on applicable functionality reporting

# 2 Discussion

## 2.1 Remaining issues on RAN2#126 agreement

Below agreements were made in RAN2#126 [3] with one FFS left.

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

We don’t think it is an essential issue. In our understanding, “functionality deactivation” just means one AI/ML functionality is disabled and fallback to legacy. We think one functionality should at least per a sub-use case. Then, it is confusing what is “functionality switching” means. For example, we think the switching from “temporal prediction in beam management” to “spatial prediction in beam management” will only introduce more confusions. Meanwhile, we think it is just a clarification and don’t think it needs to be captured in any specifications. Thus, we propose:

Proposal 1: RAN2 clarify “functionality deactivation” refers to “functionality is disabled and fallback to legacy”. And it doesn’t need to be captured in any specifications.

## 2.2 Remaining issues on applicable functionality reporting

Based on agreements in RAN2#127 [6], the basic applicable functionality reporting framework can be illustrated in Figure. 1 with FFSs highlighted to address in this section:

Figure.1: Illustration of basic LCM procedure of UE-sided model for beam management with FFS labelled

### 2.2.1 FFS1: signalling of data collection for training in step 3a

This is to address below issue listed in agenda desciption:

8.1.2.2  LCM for UE-sided model for Beam Management use case

Including functionality identification, additional conditions and further reporting of applicable functionalities.  Contributions should focus on issues not dependent on RAN1 (i.e. on questions we sent to RAN1) and issues we haven’t yet discussed (e.g. necessary signalling/protocols to configure the UE for training, etc)

Agreement

For UE sided model in beam management, support associated ID

- [Working Assumption]

  - The associated ID at least can be configured within CSI framework

    - FFS on details

    - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

- UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID

  - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

According to RAN1 progress, we understand at least CSI framework can configure data collection for UE-sided model training. Whether other signaling can work can wait RAN1 progress. Meanwhile, RAN1#118 [8] has confirmed the WA of associated ID and further agreed that associated ID at least can be configured within CSI framework, where the wording “at least” is captured due to RAN2 discussion on applicability reporting (i.e. whether it can be configured in OtherConfig of RRCReconfiguration in step 4).

Thus, we propose:

Proposal 2: RAN2 assume at least CSI framework can configure data collection and associated ID(s) for UE-sided model training. Whether other signaling can work waits RAN1 progress.

Furthermore, RAN1 discussed whether associated ID can be applicable to multiple cells, but they can’t conclude:

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

We think that it is an important issue to determine whether LCM procedure of UE-sided model can be deployed in practical system. So, we want to share our view from RAN2 perspective.

We think the associated ID should be applied to multiple cells because of below considerations:

- If associated ID is cell specific, it means that the UE needs to train cell specific AI/ML model because UE is not aware the underlaying NW implementation behind the ID (e.g. NW’s beam design). Thus, once the UE moves to another cell, it will have to re-initiate the training procedure of AI/ML model or re-download required AI/ML model. It will make solution quite complicated and not scalable.

- In AI/ML based positioning, it is LMF to configure the data collection, and thereby the associated ID included in data collection configuration from LMF can only be applied to multiple cells.

Observation 1: If associated ID is cell specific, the UE needs to train cell specific AI/ML model because UE is not aware the underlaying NW implementation behind the ID (e.g. NW’s beam design). Thus, once the UE moves to another cell, it has to re-initiate the training procedure of AI/ML model or re-download required AI/ML model.

Meanwhile, we also think it is feasible to specify either PLMN specific associated ID or area specific associated ID from signaling perspective because 3GPP already specify multiple similar signaling solutions before:

- PLMN specific ID:

  - Rel-16 RACS has introduced PLMN assigned UE Capability ID.

- Area specific ID:

  - Rel-15 has introduced area specific system information.

  - Rel-17 positioning has introduced area ID for assistance data.

Observation 2: 3GPP had introduced multiple signaling for ID applicable to multiple cells, including PLMN assigned UE Capability ID, area specific system information and area ID for assistance data in positioning.

Thus, we propose:

Proposal 3: It is feasible to apply associated ID in multiple cells from signaling perspective (either PLMN specific ID or Area specific ID).

### 2.2.2 FFS2: if report “available functionalities” in step 4

This FFS is to address the following issue in summary report [4].

Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

In our understanding, the intention to introduce available functionality is to allow the UE to notify NW the case that one functionality has available model, but it is not applicable (e.g. due to un-satisfaction of NW-side additional condition or un-satisfaction of UE-side additional condition). However, we are not convinced it is necessary because the NW can’t differentiate whether it is due to un-satisfaction of NW-side additional condition or un-satisfaction of UE-side additional condition and thereby doesn’t have clue how to react.

Observation 3: Even if available functionality is introduced, the NW can’t differentiate whether it is due to un-satisfaction of NW-side additional condition or un-satisfaction of UE-side additional condition and thereby doesn’t have clue how to react.

Thus, we propose:

Proposal 4: Don’t introduce available functionalities separately from applicable functionalities.

### 2.2.3 FFS3: whether/how to handle handover in step 4

This issue was raised in post-meeting email discussion [5]:

- Whether neighboring cell’s applicability needs to be considered in applicable functionality reporting to serving cell?

We think this issue is important because it determines whether the UE’s AI/ML operation can continue after handover. As illustrated in Figure.2, if associated ID is cell specific, it means that the same value of associated ID in two cells may represent different NW implementation (e.g. beam pattern). Thus, the UE needs to train cell specific AI/ML model because UE is not aware the underlaying NW implementation behind the ID. Then, if the UE can’t report applicable functionalities of neighbor cells, its AI/ML operation may be interrupted when the UE performs handover to another cell. Specifically, for each UE handover, the UE may have to re-train / re-download applicable AI/ML model and autonomously trigger the applicable functionality reporting, which will significantly degrade AI/ML performance.

Observation 4: If associated ID is cell specific and UE can’t report applicable functionalities of neighbor cells, its AI/ML operation may be interrupted during handover. Specifically, for each handover, the UE may have to re-train / re-download applicable AI/ML model and autonomously trigger the applicable functionality reporting.

Figure.2 Illustration of AI/ML operation interruption issue during handover (assume that associated ID is cell specific, i.e. Cell A and B may use same associated ID but NW implementation behind the same ID is different)

If the associated ID is applicable to multiple cells (e.g. PLMN specific or area specific), we still have the issue of AI/ML operation interruption. But we may resolve it from NW side rather than rely on UE reporting applicability of neighbor cells. For example, the source cell can exchange associated ID(s) with neighbor cells via inter-node message and configure the associated ID(s) of neighbor cells for UE to report their applicability.

Observation 5: If associated ID is applicable to multiple cells, the issue of AI/ML operation interruption can be resolved from NW side. For example, the source cell can exchange associated ID(s) with neighbor cells via inter-node message and configure the associated ID(s) of neighbor cells for UE to report their applicability.

According to above analysis, we can see that the issue of AI/ML operation interruption needs to be resolved irrespective of whether associated ID is applicable to multiple cells. But the specific solution may depend on whether associated ID is cell specific, which is up to RAN1 to decide. Thus, we propose RAN2 to confirm to resolve this issue.

Proposal 5: Irrespective of whether associated ID is applicable to multiple cells, RAN2 discuss how to guarantee the continuity of the AI/ML operation during handover (e.g. allow UE to report applicability of neighbor cells, or source cell exchanges associated ID with neighbor cells via inter-node message).

### 2.2.4 FFS4: UAI or RRCReconfigurtionComplete as response in step 4

This FFS was captured in agreement of RAN2#127 [6]:

-	“Step 4”: UE reports applicable functionality in the following scenarios:

1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

We prefer to use RRCReconfigurationComplete to response NW due to below reasons:

- The NW may ignore the UE’s applicable functionality reporting if UAI is used

According to Section 7.9 of TS 38.300 [9], it is up to gNB whether to accommodate the request in UAI.

7.9 UE Assistance Information

…

In the second case, the UE can express a preference for temporarily reducing the number of maximum secondary component carriers, the maximum aggregated bandwidth and the number of maximum MIMO layers. In all cases, it is up to the gNB whether to accommodate the request.

Thus, we have concern that NW ignore the UE’s applicable functionality reporting in Step 4 and configures / activates functionality which is non-applicable for the UE.

- UAI don’t mandate the UE to report upon configuration

According to Section 5.7.4.2 of TS 38.331 [10], it is up to UE implementation whether / when to initiate UAI reporting because “may” is used in all UAI reporting initiation statements.

5.7.4.2	Initiation

A UE capable of providing delay budget report in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide delay budget report and upon change of delay budget preference.

A UE capable of providing overheating assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting internal overheating, or upon detecting that it is no longer experiencing an overheating condition.

…

Thus, it is possible that some UEs supporting AI/ML never report applicable functionalities (e.g. due to the UE’s preference not to use AI/ML). It may result in failure of LCM procedure in Rel-19.

- UAI may not work in some scenarios (e.g. Handover, neighbour measurement)

We take Handover as example. In legacy, target cell configuration is included in HO command and the UE response RRCReconfiguraitonComplete message to target cell. However, as UAI message can’t be sent directly to target cell, it means existing UAI framework can’t work for handover case.

- UAI will introduce extra signalling overhead

As applicable functionality reporting is configured via RRCReconfiguraiton message, the UE will always need to respond with RRCReconfiguraitonComplete message. It is unnecessary and redundant to use another RRC message (e.g. UAI) to report them.

Observation 6: As response to NW-side additional condition, RRCReconfigurationComplete is preferred over UAI due to below reasons:

- The NW may ignore the UE’s applicable functionality reporting if UAI is used

- UAI don’t mandate the UE to report upon configuration, which is not aligned with RAN2#127 agreement

- UAI may not work in some scenarios (e.g. Handover, neighbour measurement)

- UAI will introduce extra signalling overhead

Thus, we propose to use RRCReconfigurationComplete.

Proposal 6: As response to NW-side additional condition, RRCReconfigurationComplete is used to report applicable functionalities in Step 4.

### 2.2.5 FFS5: whether to introduce “configured functionalities” in step 5

This FFS is to address the following issue in summary report [4].

Proposal 2: RAN2 discuss further whether definition on configured functionalities is needed.

Proposal 4: RAN2 discuss further relationship between configured functionalities and applicable functionalities as a part of discussion on the need of defining configured functionalities.

For above Proposal 2, we don’t think it is necessary to introduce the definition of “configured functionalities” due to below reasons:

- The definition is too obvious because anyway gNB/LMF will configure for functionalities.

- At least stage 3 specification (either RRC or MAC) doesn’t need this definition because what NW configured to UE is always clearly specified in RRC and MAC spec. Thus, whether it is configuration for training or inference, or monitoring should be crystal clear from text before and after in stage 3 specification.

Then, for above Proposal 4, we think RAN2 only need to clarify that the functionalities configured in Step 5 is a subset of applicable functionalities reported by the UE in Step 4. It should be RAN2 common understanding.

Thus, we propose:

Proposal 7: No need to introduce definition of “configured functionalities”. RAN2 clarify that the functionalities configured in Step 5 is a subset of applicable functionalities reported by the UE in Step 4.

### 2.2.6 FFS6: how to handle on-going functionality becoming non-applicable

This is one issue which has not been discussed in RAN2 yet. Please note that RAN2#127 agreed that the UE can decide whether one functionality is applicable or not according to UE-side additional condition internally known by UE [6].

-	UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration).  FFS  how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

Although specific examples of UE-side additional conditions are not explicitly listed in the agreement, they can be UE’s power state and/or remaining memory for AI/ML operation. Thus, when the UE is performing inference operation configured/activated by NW in Step 6, it is possible that the on-going functionality may become non-applicable due to insufficient memory or insufficient battery for inference operation. In this case, the UE behaviour is not clear (e.g. whether the inference operation can be stopped when NW still configure the UE to perform inference).

Observation 7: When the UE is performing inference operation configured/activated by NW in Step 6, it is possible that the on-going functionality may become non-applicable due to insufficient memory or insufficient battery for inference operation. The followed UE behaviour is not clear (e.g. whether inference can be stopped).

Because UE’s memory and battery status may change dynamically, we don’t think it is a corner case. Thus, the UE behaviour needs to be clearly specified.

Proposal 8: RAN2 specify the UE behaviour on how to handle the functionality which is performing inference operation but becomes non-applicable (e.g. due to insufficient memory or battery to continue inference).

# 3 Conclusion

In this contribution, we discuss remaining issues on LCM procedure of UE-sided model for beam management. Our observations are:

Observation 1: If associated ID is cell specific, the UE needs to train cell specific AI/ML model because UE is not aware the underlaying NW implementation behind the ID (e.g. NW’s beam design). Thus, once the UE moves to another cell, it has to re-initiate the training procedure of AI/ML model or re-download required AI/ML model.

Observation 2: 3GPP had introduced multiple signaling for ID applicable to multiple cells, including PLMN assigned UE Capability ID, area specific system information and area ID for assistance data in positioning.

Observation 3: Even if available functionality is introduced, the NW can’t differentiate whether it is due to un-satisfaction of NW-side additional condition or un-satisfaction of UE-side additional condition and thereby doesn’t have clue how to react.

Observation 4: If associated ID is cell specific and UE can’t report applicable functionalities of neighbor cells, its AI/ML operation may be interrupted during handover. Specifically, for each handover, the UE may have to re-train / re-download applicable AI/ML model and autonomously trigger the applicable functionality reporting.

Observation 5: If associated ID is applicable to multiple cells, the issue of AI/ML operation interruption can be resolved from NW side. For example, the source cell can exchange associated ID(s) with neighbor cells via inter-node message and configure the associated ID(s) of neighbor cells for UE to report their applicability.

Observation 6: As response to NW-side additional condition, RRCReconfigurationComplete is preferred over UAI due to below reasons:

- The NW may ignore the UE’s applicable functionality reporting if UAI is used

- UAI don’t mandate the UE to report upon configuration, which is not aligned with RAN2#127 agreement

- UAI may not work in some scenarios (e.g. Handover, neighbour measurement)

- UAI will introduce extra signalling overhead

Observation 7: When the UE is performing inference operation configured/activated by NW in Step 6, it is possible that the on-going functionality may become non-applicable due to insufficient memory or insufficient battery for inference operation. The followed UE behaviour is not clear (e.g. whether inference can be stopped).

Based on observations, our proposals can be found below:

Proposal 1: RAN2 clarify “functionality deactivation” refers to “functionality is disabled and fallback to legacy”. And it doesn’t need to be captured in any specifications.

Proposal 2: RAN2 assume at least CSI framework can configure data collection and associated ID(s) for UE-sided model training. Whether other signaling can work waits RAN1 progress.

Proposal 3: It is feasible to apply associated ID in multiple cells from signaling perspective (either PLMN specific ID or Area specific ID).

Proposal 4: Don’t introduce available functionalities separately from applicable functionalities.

Proposal 5: Irrespective of whether associated ID is applicable to multiple cells, RAN2 discuss how to guarantee the continuity of the AI/ML operation during handover (e.g. allow UE to report applicability of neighbor cells, or source cell exchanges associated ID with neighbor cells via inter-node message).

Proposal 6: As response to NW-side additional condition, RRCReconfigurationComplete is used to report applicable functionalities in Step 4.

Proposal 7: No need to introduce definition of “configured functionalities”. RAN2 clarify that the functionalities configured in Step 5 is a subset of applicable functionalities reported by the UE in Step 4.

Proposal 8: RAN2 specify the UE behaviour on how to handle the functionality which is performing inference operation but becomes non-applicable (e.g. due to insufficient memory or battery to continue inference).

# 4 References

[1] RP-242399, New WID: New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#125b, Chair Notes.

[3] RAN2#126, Chair Notes.

[4] R2-2407165	, Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 1, Samsung.

[5] R2-2406381, Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2, Intel Corporation.

[6] RAN2#127, Chair Notes.

[7] R2-2407848, LS on applicable functionality reporting for beam management UE-sided model.

[8] RAN1#118, Chair Notes.

[9] TS 38.300-v18.0.0, NR and NG-RAN Overall Description; Stage 2, 2023-12.

[10] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.