# 3GPP TSG RAN WG2 Meeting #127b      	                           R2-2408564
Hefei, China, 14th – 18th Oct., 2024

Agenda item:	8.1.2.3

Source:	Apple

Title:	LCM procedure of AIML based positioning

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The updated Rel-19 WID of AI/ML for air interface (WID RP-242399) was agreed in RAN#105 [1]. The WI objective on general LCM framework, including applicable conditions, is led by RAN2 which is copied below:

In RAN2#126 [2], the agreements below on the LCM of AI/ML based positioning were made with one highlighted FFS on dynamic capability that needs further discussion:

Agreements for NWside model

- RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

Agreements

- The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities.  FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality.

- wait for RAN1 for associate ID discussion

- At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation.  FFS further details on signaling enhancements

The summary report of [POST126][032] provided the proposal below on applicability reporting [3], but was postponed:

Proposal 7: For positioning Case 1, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling (unsolicited information transfer). UE can also provide applicable functionality reporting as a response to LMF request (solicited information transfer). FFS on the order of inference configuration and applicable functionality reporting, FFS on functionality activation.

In this contribution, we further discuss below the issues of the LCM procedure of AI/ML based positioning.

- Dynamic capability and signaling

- Applicable functionality reporting

# 2 Discussion

## 2.1 Dynamic capability and signalling

We address below the highlighted FFS of RAN2#126 agreement [2]:

Agreements

- The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities.  FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality.

From LS R2-2108960:

Regarding the question from SA2:

"Whether the UE positioning capability is variable or not? If yes, in which situation it is changed?"

RAN2 would like to provide the following response:

The UE positioning capability can be variable. Example situations where the UE positioning capability may change include:

-	Radio configuration dependency: Positioning capabilities based on current/active radio configuration are not static (e.g., the srs-PosResourceConfigCA-BandList in TS 37.355 is provided for the current configured CA band combination).

-	Power Savings: A (e.g., IoT) UE whose battery level is low may switch off positioning support in order to conserve battery power for more important tasks such as communicating with an external server or may report lower processing capabilities.

-	Processing Resources Constraints: The available processing resources (processors, memory, etc.) may be shared between "communication" and "positioning operations".  If the "communication operation" requires increased processing resources, the resources allocated to the "positioning operation" may temporarily be reduced.

RAN2 would like to point out that the above are examples and there may be other situations in which the UE positioning capability may change. Also, the examples may not be supported on all UEs and may not always need to be supported.

As we see, RAN2 has agreed to reuse LPP message RequestCapabilities/ProvideCapabilities message to indicate supported functionalities of AI/ML based positioning. And it is FFS how to handle dynamic capabilities. Actually, RAN2 had discussed a similar issue in Rel-17 positioning and agreed that the existing LPP message RequestCapabilities/ProvideCapabilities is allowed to dynamically update UE’s positioning capability, which is included in the below copy of reply LS to SA2 (R2-2108960) [4]:

As highlighted above, the existing LPP capability signaling already satisfies the requirement of dynamic capability in LCM (i.e. applicable functionality reporting):

- As highlighted part shows, dynamic capability changes on “Radio configuration dependency” are essentially NW-sided condition (e.g. NW implementation changes reflected via associated ID).

- As highlighted part shows, dynamic capability changes on “Power Savings” and “Processing Resources Constraints” are essentially UE-sided conditions (e.g. UE power state and UE memory).

Observation 1: According to LS R2-2108960, existing LPP capability signaling RequestCapabilities / ProvideCapabilities already satisfies the requirement of dynamic capability in LCM (i.e. applicability reporting):

- Dynamic capability changes on “Radio configuration dependency” are essentially NW-sided condition.

- Dynamic capability changes on “Power Savings” and “Processing Resources Constraints” are essentially UE-sided condition.

Thus, we propose RAN2 to confirm existing LPP capability signaling can already handle dynamic capability reporting.

Proposal 1: RAN2 confirm that the existing LPP capability signaling (RequestCapabilities / ProvideCapabilities) already support dynamic capability reporting, i.e. no extra specification work is foreseen to support dynamic capability reporting for AI/ML based positioning.

## 2.2 Applicable functionality reporting

A summary report of [POST126][032] provided the below proposal on applicability reporting [3], but it was postponed:

Proposal 7: For positioning Case 1, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling (unsolicited information transfer). UE can also provide applicable functionality reporting as a response to LMF request (solicited information transfer). FFS on the order of inference configuration and applicable functionality reporting, FFS on functionality activation.

### 2.2.1 Issue 1: signalling of applicability reporting

We think above Proposal in [3] is aligned with our Proposal 1, i.e., the existing LPP capability signaling (RequestCapabilities / ProvideCapabilities) can be reused for dynamic applicable functionality reporting (i.e. it is not necessary to use UAI for it). Meanwhile, as captured in TS 37.355 [5], please note that existing LPP capability signaling already supports both solicited reporting in Section 5.1.1 and unsolicited reporting in Section 5.1.2.

Observation 2: In TS 38.355, existing LPP capability signaling already supports both solicited reporting in Section 5.1.1 and unsolicited reporting in Section 5.1.2.

Thus, for the above Proposal 7, we think the text before FFS can be confirmed.

Proposal 2: For UE-side model of AI/ML positioning Case 1, existing solicited LPP capability reporting via RequestCapabilities / ProvideCapabilities in Section 5.1.1 of TS 37.355 and unsolicited LPP capability reporting via ProvideCapabilities in Section 5.1.2 of TS 37.355 can be reused for applicable functionality reporting.

### 2..2 Issue 2: whether associated ID is used in AI/ML positioning

Although RAN2#126 agreed the applicability reporting procedure with associated ID, some company contributions challenged its necessity [7]. Their two main arguments are listed below:

- Some NW-side additional conditions (e.g. TRP/ARP locations, DL-PRS beam information, synchronization information) can be regarded to be already included in existing LPP message ProvideAssistanceData. Additional Assistance Data ("network-side additional conditions") can be easily added to the existing framework, if needed.

- Any associated ID for the "Network-side Additional Conditions" would imply a "hard-decision" and thus, suboptimal. A simple "associated ID" change indication does not provide any information to what degree the information has changed, and therefore, corresponds to a "hard decision" not to use a trained model for inference in the given scenario. Providing the (normal) assistance data as "Network-side Additional Conditions" leaves the decision on whether a trained model is still suitable for inference in a given situation up to implementation.

We are not convinced by these two arguments:

- For argument 1), we agree that some NW-side additional conditions are already included in existing LPP ProvideAssistanceData. However, existing Assistance Data may not cover all NW-side additional conditions required for AI/ML positioning. For example, the association between PRS configuration used for data collection and PRS configuration used for inference. We disagree that these new NW-side additional conditions can be easily added to existing framework because each new assistance information needs to be agreed and then specified in 3GPP. If some assistance information has risk of exposure of NW implementation, they are difficulty to be agreed in 3GPP. At least, such standardization work will unnecessarily delay the deployment of AI/ML positioning. On the contrary, associated ID is a solution with better forward compatibility because the NW implementation behind associated ID is transparent to the UE.

Observation 3: It is not always easy to add new NW-side additional condition(s) to LPP Assistance Data. At least, such standardization work will unnecessarily delay the deployment of AI/ML positioning. On the contrary, associated ID solution has better forward compatibility as NW implementation behind is transparent to the UE.

- For argument 2):

  - Firstly, we disagree that associated ID always implies a "hard-decision". According to below agreement of RAN2#127 [8]:

UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration).  FFS  how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

it allows the UE to determine applicable functionality based on its internal information (i.e. UE-side additional condition) which already leaves room for flexible UE implementation.

  - Secondly, we don’t think associated ID and legacy way are mutual exclusive. In our understanding, if associated ID is provided in LPP ProvideAssistanceData, the UE can still request new DL PRS configuration via LPP message LPP RequestAssistanceData when it determines current functionality is not applicable.

Observation 4: According to RAN2#127 agreement, applicability reporting based on associated ID already leaves room for flexible UE implementation, i.e. it is not “hard-decision”.

Based on above analysis, we propose that associated ID is also applicable to AI/ML positioning Case 1, and it is straight forward to include it in LPP RequestCapabilities (solicited) and LPP ProvideAssistanceData (unsolicited).

Proposal 3: For UE-side model of AI/ML positioning Case 1, RAN2 assume that associated ID can be included in LPP RequestCapabilities (solicited report) and LPP ProvideAssistanceData (unsolicited report).

Proposal 4: If associated ID is provided in LPP ProvideAssistanceData, the UE can still request new DL PRS configuration via LPP RequestAssistanceData when it determines current functionality is not applicable.

Then, similar to the discussion in AI/ML based beam management, a followed question is whether the associated ID in AI/ML positioning is applicable to multiple cells. In AI/ML based beam management, it is agreed that it is applied at least in one single cell and the case of multiple cells is FFS.

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

However, different from AI/ML based beam management, it is LMF (as a CN entity) to provide LPP Assistance Data in AI/ML positioning, and thereby the associated ID included in it should be applied to multiple cells. Meanwhile, please note that existing LPP message ProvideAssistanceData already supports AreaID-CellList to indicate a particular network area where the associated assistance data are valid, as copied from TS 37.355 [6].

#### –	AreaID-CellList

The IE AreaID-CellList provides the NR Cell-IDs of the TRPs belonging to a particular network area where the associated assistance data are valid. Each cell is included in only one area.

-- ASN1START

AreaID-CellList-r17 ::= SEQUENCE (SIZE(1..maxCellIDsPerArea-r17)) OF NR-Cell-IDs-r17

NR-Cell-IDs-r17 ::= SEQUENCE {

nr-CellGlobalID-r17				NCGI-r15					OPTIONAL,	-- Need ON

nr-PhysCellID-r17				NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-ARFCN-r17					ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

Observation 5: In AI/ML positioning, it is LMF to provide LPP Assistance Data, and thereby the associated ID included in it should be applied to multiple cells. Meanwhile, existing LPP ProvideAssistanceData already supports AreaID-CellList to indicate a particular network area where the associated assistance data are valid.

Thus, we propose that the associated ID included in LPP ProvideAssistanceData and LPP RequestCapabilities is also area specific.

Proposal 5: Similar to existing AreaID-CellList, the associated ID(s) included in LPP ProvideAssistanceData and LPP RequestCapabilities are area specific.

### 2.2.3 Issue 3: FFS on the order of inference configuration and applicable functionality reporting

Please note that existing TS 38.305 has captured below principle for LPP procedures [7]:

#### 7.1.2.5	Sequence of procedures

LPP procedures are not required to occur in any fixed order, in order to provide greater flexibility in positioning. Thus, a UE may request assistance data at any time in order to comply with a previous request for location measurements from the LMF; an LMF may instigate more than one request for location information (e.g., measurements or a location estimate) in case location results from a previous request were not adequate for the requested QoS; and the target device may transfer capability information to the server at any time if not already performed.

Despite the flexibility allowed by LPP, it is expected that procedures will normally occur in the following order:

1. Capability Transfer;

2. Assistance Data Transfer;

3. Location Information Transfer (measurements and/or location estimate).

We see no reason to challenge above principle of LPP in AI/ML based positioning. Thus, we propose:

Proposal 6: The step order of applicable functionality reporting procedure follows existing principle on sequence order of LPP procedures captured in TS 38.305, i.e. no need to specify fixed step order (e.g. order between inference configuration and applicable functionality reporting).

### 2.2.4 Issue 4: FFS on functionality activation

First, please note that RAN2#126 [2] has agreed to reuse existing LPP Location Information Transfer procedure for the UE to provide inference results via RequestLocationInformation / ProvideLocationInformation messages.

- At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation.  FFS further details on signaling enhancements

The existing LPP Location Information Transfer procedure can be found in Section 5.3.1 of 37.355 [6]:

### 5.3.1 Location Information Transfer procedure The Location Information Transfer procedure is shown in Figure 5.3.1-1. Figure 5.3.1-1: LPP Location Information transfer procedure 1. The server sends a RequestLocationInformation message to the target to request location information, indicating the type of location information needed and potentially the associated QoS. 2. The target sends a ProvideLocationInformation message to the server to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless the server explicitly allows additional location information. If step 3 does not occur, this message shall set the endTransaction IE to TRUE. 3. If requested in step 1, the target sends additional ProvideLocationInformation messages to the server to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless the server explicitly allows additional location information. The last message shall include the endTransaction IE set to TRUE.

As we can see, the UE immediately sends response via ProvideLocationInformation to LMF upon reception of RequestLocationInformation. Thus, it can be regarded that the functionality is activated when the LPP message RequestLocationInformation (i.e. configuration) is received, i.e. no extra activation procedure after reception of configuration.

Observation 6: In existing LPP Location Information Transfer procedure, the UE activates the response procedure when LPP message RequestLocationInformation is received, i.e. no extra activation procedure after reception of configuration.

We see no reason to introduce a new activation procedure on top of the existing procedure for AI/ML based positioning. Thus, we propose:

Proposal 7: For UE-side model of AI/ML positioning Case 1, the functionality is activated upon reception of LPP message RequestLocationInformation as legacy LPP Location Information Transfer procedure, i.e. no need to introduce extra procedure of functionality activation.

Finally, we provide Figure.1 to implement all the Proposals in this contribution. And we also propose RAN2 to agree this Figure as starting point of followed study on LCM procedure of AI/ML based positioning.

Proposal 8: RAN2 agree Figure.1 as starting point for the followed study on LCM procedure of AI/ML based positioning.

Figure.1: Starting point of LCM procedure of UE-sided model for AI/ML positioning Case 1

# 3 Conclusion

In this contribution, we share our view on LCM procedure of AI/ML based positioning. Our observations are:

Observation 1: According to LS R2-2108960, existing LPP capability signaling RequestCapabilities / ProvideCapabilities already satisfies the requirement of dynamic capability in LCM (i.e. applicability reporting):

- Dynamic capability changes on “Radio configuration dependency” are essentially NW-sided condition.

- Dynamic capability changes on “Power Savings” and “Processing Resources Constraints” are essentially UE-sided condition.

Observation 2: In TS 38.355, existing LPP capability signaling already supports both solicited reporting in Section 5.1.1 and unsolicited reporting in Section 5.1.2.

Observation 3: It is not always easy to add new NW-side additional condition(s) to LPP Assistance Data. At least, such standardization work will unnecessarily delay the deployment of AI/ML positioning. On the contrary, associated ID solution has better forward compatibility as NW implementation behind is transparent to the UE.

Observation 4: According to RAN2#127 agreement, applicability reporting based on associated ID already leaves room for flexible UE implementation, i.e. it is not “hard-decision”.

Observation 5: In AI/ML positioning, it is LMF to provide LPP Assistance Data, and thereby the associated ID included in it should be applied to multiple cells. Meanwhile, existing LPP ProvideAssistanceData already supports AreaID-CellList to indicate a particular network area where the associated assistance data are valid.

Observation 6: In existing LPP Location Information Transfer procedure, the UE activates the response procedure when LPP message RequestLocationInformation is received, i.e. no extra activation procedure after reception of configuration.

Based on observations, our proposals can be found below:

Proposal 1: RAN2 confirm that the existing LPP capability signaling (RequestCapabilities / ProvideCapabilities) already support dynamic capability reporting, i.e. no extra specification work is foreseen to support dynamic capability reporting for AI/ML based positioning.

Proposal 2: For UE-side model of AI/ML positioning Case 1, existing solicited LPP capability reporting via RequestCapabilities / ProvideCapabilities in Section 5.1.1 of TS 37.355 and unsolicited LPP capability reporting via ProvideCapabilities in Section 5.1.2 of TS 37.355 can be reused for applicable functionality reporting.

Proposal 3: For UE-side model of AI/ML positioning Case 1, RAN2 assume that associated ID can be included in LPP RequestCapabilities (solicited report) and LPP ProvideAssistanceData (unsolicited report).

Proposal 4: If associated ID is provided in LPP ProvideAssistanceData, the UE can still request new DL PRS configuration via LPP RequestAssistanceData when it determines current functionality is not applicable.

Proposal 5: Similar to existing AreaID-CellList, the associated ID(s) included in LPP ProvideAssistanceData and LPP RequestCapabilities are area specific.

Proposal 6: The step order of applicable functionality reporting procedure follows existing principle on sequence order of LPP procedures captured in TS 38.305, i.e. no need to specify fixed step order (e.g. order between inference configuration and applicable functionality reporting).

Proposal 7: For UE-side model of AI/ML positioning Case 1, the functionality is activated upon reception of LPP message RequestLocationInformation as legacy LPP Location Information Transfer procedure, i.e. no need to introduce extra procedure of functionality activation.

Proposal 8: RAN2 agree Figure.1 as starting point for the followed study on LCM procedure of AI/ML based positioning.

Figure.1: Starting point of LCM procedure of UE-sided model for AI/ML positioning Case 1

# 4 References

[1] RP-242399, New WID: New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#126, Chair Notes.

[3] R2-2406381, Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2, Intel Corporation.

[4] R2-2108960, Response LS on storage of UE Positioning Capabilities.

[5] 3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[6] 3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[7] R2-2406643, On LCM for UE-sided models, Qualcomm Incorporated.

[8] RAN2#127, Chair Notes.