# 3GPP TSG RAN WG2 Meeting #127b      	                        R2-2408565
Hefei, China, 14th – 18th Oct., 2024

Agenda item:	8.1.3

Source:	Apple

Title:	Further discussion on NW-sided data collection

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The updated Rel-19 WID of AI/ML for air interface (WID RP-242399) was agreed in RAN#105 [1]. One of its sub-objectives is to specify data collection for NW-sided model, as highlighted copied below:

In RAN2#126 [2] and RAN2#127 [3], NW-sided data collection was discussed. It was agreed to design a unified measurement framework, and immediate MDT was agreed to support L1 measurement logging and reporting with some leftover issues. In this contribution, we will address the key remaining issues.

Meanwhile, RAN2 didn’t discuss NW-sided data collection of AI/ML based positioning and required data type for both NW-side model and UE-side model, which are included in the guidance of agenda item:

8.1.3 NW side data collection

Contributions should focus on the mechanisms and principles identified for data collection for network side model training during rel-18.   Contributions should discusss type of data required to be collected for NW sided model and UE sided model (common to NW sided and different).  Question to RAN1 should also be identified.

In this contribution, we further discuss NW-sided data collection from below aspects:

- NW-sided data collection for AI/ML based beam management

- NW-sided data collection for AI/ML based positioning

- Collected data type of NW-sided data collection

- Collected data type of UE-sided data collection

# 2 Discussion

## 2.1 NW-sided data collection for AI/ML based beam management

Based on latest agreements, our understanding on the whole procedure, including OAM centric and gNB centric data collection, is illustrated in Figure .1.

Figure. 1 Overall procedure of OAM centric and gNB centric NW-sided data collection

In the following, we discuss the below remaining issues based on Figure.1.

- Overall procedure

- AS procedure step 1: RRC configuration of L1 measurement logging

- AS procedure step 2: How UE logs L1 measurement

- AS procedure step 3: Trigger reporting of logged L1 measurement

- AS procedure step 4: Reporting of logged L1 measurement

### Overall procedure

As illustrated in Figure.1, our understanding is:

- CN procedure (i.e. procedure between RAN and CN/OAM/UDM) is labelled in blue font.

  - We think RAN2 can assume this procedure reuses existing procedure of immediate MDT.

  - One possible exception is that maybe a new server instead of TCE can be introduced to store training data, but it is up to RAN3/SA5 to decide.

- AS procedure (i.e. procedure between RAN and UE) is labelled in orange font.

  - This procedure includes 4 steps as illustrated in Figure. 1 and is common to both OAM centric and gNB centric data collection.

  - Difference between OAM centric and gNB centric data collection is which NW entity (OAM vs gNB) to trigger data collection and generate data collection configuration.

Observation 1: Difference between OAM centric and gNB centric data collection is which NW entity (OAM vs gNB) to trigger data collection and generate data collection configuration. Their AS procedure is same.

We have UE privacy concern on gNB centric data collection because the collected data is generated by the UE which may include UE’s privacy sensitive information (e.g., the UE’s location information). Please note that the UE privacy requirement is already satisfied by OAM centric data collection because user consent for MDT is required before NW configures the UE to perform immediate MDT, according to TS 32.422 [4]:

4.9.1 Signalling based MDT

In case of signalling based MDT getting user consent before activating the MDT functionality is required because of privacy and legal obligations. It is the Operator responsibility to collect user consent before initiating an MDT for a specific IMSI, IMEI number or SUPI.

Collecting the user consent shall be done via customer care process. The user consent information availability should be considered as part of the subscription data and as such this shall be provisioned to the UDM database.

Observation 2: UE privacy requirement is preserved in OAM centric data collection due to UE consent mechanism in MDT, but it is not clear how to satisfy UE privacy requirement in gNB centric data collection.

Since RAN2 agreed to aim the same measurement framework for both gNB-centric data collection and OAM-centric data collection, we think similar UE consent mechanism in MDT should also be supported in gNB-centric data collection, i.e. user consent is required before NW configures the UE with gNB-centric data collection. The detailed signaling can be further discussed.

Proposal 1: To strive for a common measurement framework, similar UE consent mechanism in MDT is supported in gNB-centric data collection, i.e. UE consent is required before NW configures the UE gNB-centric data collection. FFS signaling details.

### Step 1: RRC configuration of L1 measurement logging

#### 2.1.2.1 RRC framework

In legacy immediate MDT and L3 RRM, measurement configuration (i.e. MeasConfig) is included in RRCReconfiguration message. And whether it is immediate MDT or L3 RRM is transparent to the UE. The existing RRC structure is illustrated in Figure.2, where each MeasID links one instance of MeasObjectNR with one instance of ReportConfigNR.

Meanwhile, according to below RAN1#116b agreement [5], RAN1 has agreed to reuse existing CSI framework for data collection of beam management.

Agreement

For network-sided AI/ML model for BM-Case1 and BM-Case2,

- support using existing CSI framework for configuration of Set A as the starting point

- support using existing CSI framework for configuration of Set B as the starting point

- Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specifications

Thus, RAN2 should integrate existing L1 CSI framework into L3 measurement framework illustrated in Figure.2. The existing RRC structure of CSI framework is illustrated in Figure.3, where each csi-ResourceConfigId links one instance of csi-ResourceConfig and one instance of csi-ReportConfig.

Figure. 2 Illustration of existing RRC structure of L3 RRM and immediate MDT (note: some unrelated IEs, e.g., offsetMO, are omitted)

Figure. 3 Illustration of existing RRC structure of L1 CSI framework (note: some unrelated IEs, e.g., csi-SSB-ResourceSet, are omitted)

Based on analysis of existing RRC structure, our initial consideration on how to integrate L1 measurement logging into L3 measurement framework can be found in Figure.4 with below explanation:

- New L1 CSI resource configuration (e.g. list of NZP CSI-RS resource sets) and logging configuration (e.g. logging interval) are introduced within MeasObjectNR.

- New L1 measurement reporting configuration is introduced within ReportConfigNR to indicate at least the reporting quantity configuration (e.g. L1 RSPR and/or beam index) and reporting type (e.g. periodic or on-demand, FFS event trigger).

Figure. 4 An example of how to integrate logged L1 measurement into L3 RRM framework (green modules are introduced for logged L1 measurement/reporting)

So, we propose:

Proposal 2: MeasConfig in RRCReconfiguration is reused to configure logged L1 measurements with below specification changes to integrate L1 measurement logging:

- Within MeasObjectNR: introduce L1 CSI resource configuration and logging configuration.

- Within ReportConfigNR: introduce L1 measurement reporting configuration to indicate at least the reporting quantity configuration and reporting type (e.g. periodic or on-demand reporting).

Then, we address below two FFSs of RAN2#127 agreements [3]:

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

#### 2.1.2.2 FFS#1: if multiple configurations can be provided to the UE

In our understanding, the intention of multiple measurement configurations is to allow different L1 CSI-RS resource sets follow different measurement / logging configurations (e.g. different L1 CSI resource sets are measured with different periodicities). This is a valid use case because datasets collected from different L1 CSI resource sets may be used to train different AI/ML models by the NW. However, please note that existing L1 CSI framework already supports this feature. As illustrated in Figure. 3, multiple NZP-CSI-RS-ResourceSet and multiple CSI-ReportConfig can be configured where one NZP-CSI-RS-ResourceSet can be associated with one CSI-ReportConfig via CSI-ResourceConfigId.

Observation 3: Existing L1 CSI framework already supports multiple measurement configurations: multiple NZP-CSI-RS-ResourceSet and multiple CSI-ReportConfig can be configured where one NZP-CSI-RS-ResourceSet can be associated with one CSI-ReportConfig via CSI-ResourceConfigId.

We think it is straight forward to reuse this signaling framework. As illustrated in Figure. 4, under MeasObjectNR. multiple L1 CSI resource sets and multiple logging configuration can be configured, where one L1 CSI resource set is associated with one logging configuration via CSI-ResourceConfigId.

Proposal 3: Similar to existing L1 CSI framework, multiple L1 CSI resource sets and multiple logging configuration (e.g. with different logging periodicities) can be configured in MeasObjectNR, whether one L1 CSI resource set is associated with one logging configuration via CSI-ResourceConfigId.

#### 2.1.2.3 FFS#2: if dynamic activation/deactivation is support

We think it is not reasonable to support dynamic activation/deactivation. Enhanced MDT is intended to support data collection for model training, and thereby multiple logged L1 measurement samples are reported via a single RRC message. Thus, it is a L3 procedure and only large data volume makes sense. If NW needs to reconfigure the data collection (e.g. change the logging interval), it can use RRC reconfiguration. We don’t see benefit to support dynamic activation / deactivation.

Proposal 4: Do not support dynamic activation/deactivation of immediate MDT with L1 measurement logging.

### 2.1.3 Step 2: How UE logs L1 measurement

RAN2#127 has made below agreements on how UE logs L1 measurements [3]. We address these FFSs respectively.

Agreements

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

- FFS on event based data collection/logging

#### 2.1.3.1 FFS#1: FFS on the memory size

We need to determine the minimum AS layer memory size. In legacy NR, logged MDT and QoE measurements (including paused measurements and IDLE/INACTIVE measurements) had specified the minimum AS layer memory size, which are all 64kB according to TS 38.306 [6]:

Logged MDT (From Section 4.2.18 of TS 38.306) loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of MDT logged measurements is 64KB. For eRedCap UE supporting this feature, the minimum memory size of MDT logged measurements is 16KB.; UE; No; No; No. QoE (from Section 6 of TS 38.306) AS layer memory size for QoE paused measurement reports; It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17.. AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE; It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports".

Observation 4: In legacy NR, logged MDT, QoE measurements in IDLE/INACTIVE state and QoE paused measurements had specified the minimum AS layer memory size, which are all 64kB according to TS 38.306.

Meanwhile, RAN1 Reply LS (R1-2310681) [7] provided typical data size for model training in beam management. According to the highlighted parts, we can estimate that the typical training data size stored in AS layer memory is 100 bits8 instances (Set B) + 500 bits4 instances (Set A) = 2800 bits=0.35KB.

Observation 5: Based on the typical training data size of beam management provided in RAN1 Reply LS (R1-2310681), the typical training data size stored in AS layer memory is 100 bits8 instances (Set B) + 500 bits4 instances (Set A) =0.35KB.

From RAN1 reply LS (R1-2310681) LCM purpose; UE-side/NW-side models; Data content; Typical data size (per data sample); Typical latency requirement; Notes. Training; UE-side, NW-side; L1-RSRPs and/or beam-IDs; See Note 1 for L1-RSRPs; Relaxed. ==omit table rows on “Inference” and “Monitoring” === Note 1: There is no agreement on the data size of L1-RSRPs for Set A or Set B, but the following typical data size is provided as guidance for RAN2 discussion. Based on existing L1-RSRP reporting methodology, i.e., 7 bits for the strongest beam and 4 bits for the remaining beams, for Set B = 16 as an example, the typical data size would be 67 (hence up to ~100 bits), and for Set A = 128 as an example, the typical data size would be 515 (hence up to ~500 bits) if all beams in Set A were to be collected. For BM Case 2, the data size L1-RSRPs for Set A and Set B represents the data size per predicted future time instance and per history measurement time instance, respectively. Payload size may not be fixed. Note 2: There are no agreements on the reporting type. Note 4: Feasibility and necessity of the monitoring schemes listed in the table are under discussion. Note 5: For BM Case 2, the typical value of the number of history measurement time instance used in evaluations is up to 8 and typical value of the number of predicted future time instance is 1~4.

Based on above analysis, we think it is sufficient to reuse the same minimum AS layer memory size as logged MDT and QoE. Meanwhile, to allow the NW to know when each sample of reported L1 measurement is measured, the UE can include the time stamp when the L1 measurement sample is logged. Thus, we propose:

Proposal 5: The minimum AS layer memory size for L1 measurement logging is same as existing logged MDT and QoE (i.e. 64KB).

Proposal 6: The stored data in UE’s AS buffer at least includes L1 measurements (L1 RSRP and/or beam index) and the logging time stamp.

#### 2.1.3.2 FFS#2: (upon low power) whether the UE stops autonomously or report to the network

We have concern on both solutions listed in the FFS:

- Alt-1: UE autonomous stopping: Please note that RAN2#127 has agreed it is up to UE implementation to determines power state [3], which means there is no specified requirement on low power state. Thus, we think the UE autonomous stopping seems to be too risky because the UE and NW may not be aligned on the understanding of “low power state”.

- Alt-2: UE reports to the Network: If this solution is adopted, it is quite likely that UAI is used to report “low power state”. However, we have concern that the NW will ignore the UE’s reporting without any followed action because Section 7.9 of TS 38.300 [8] has captured “it is up to gNB whether to accommodate the request in UAI”. It will make this feature useless.

RAN2#123b agreement:

We will introduce ‘wait timer’ for the reactive approach

    - The UE starts the timer when the UE requests a temporary restriction to the network if the timer is configured. We assume network configures the length for this timer.

    - Stop: if UE receives reconfiguration that does not exceed the capabilities that UE suggested via capability restriction report

    - Expiry: UE can apply the temporary UE capability restriction upon the timer expiry.

From Section 7.1.1 of TS 38.331:

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T348 | Upon transmission of MUSIM temporary restriction of musim-CapRestriction for serving cell(s) with capabilities restricted, release of SCell or release of SCG. | Upon reception of RRCReconfiguration message that does not exceed UE temporary capability restriction transmitted via musim-CapRestriction. | UE may apply the temporary UE capability restriction in accordance with the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction. |

To resolve above issues, we prefer to specify a solution in-between above Alt-1 and Alt-2 which is similar to reactive temporary restriction reporting specified in Rel-18 MUSIM. In detail, the UE starts T348 upon sending UAI with temporary restriction request, and the UE may autonomously apply the requested temporary capability restriction if T348 expires without reception of NW reaction. The detailed RAN2 agreement and its specification in TS 38.331 [9] can be found below:

Observation 5: Rel-18 MUSIM had specified reactive temporary restriction reporting. Specifically, the UE starts T348 upon sending UAI with temporary capability restriction request, and it may autonomously apply the requested temporary restriction if T348 expires without reception of NW reaction.

We believe similar solution can be introduced to harmonize Alt-1 and Alt-2. Specifically, the UE may send UAI to notify the NW on its low power state and starts a wait timer. If the wait timer expires without reception of NW reaction, it may stop the data collection.

Proposal 7: On the low power state issue during data collection, introduce the following solution similar to reactive temporary restriction reporting of Rel-18 MUSIM:

- Upon low power state, the UE may send UAI to notify the NW.

- The UE starts a wait timer when sending the UAI. If the wait timer expires without reception of NW reaction, it may stop the data collection.

#### 2.1.3.3 FFS3: whether to support event based data collection/logging

We think it makes sense to support event triggered data logging. Please note that NW typically adopt different MIMO solutions when the UE is in cell edge (e.g. exploring robust) and cell center (e.g. exploring high throughput). Thus, we believe that NW may train different AI/ML models for cell center case and cell edge case, respectively. As a consequence, if the radio condition based event is not introduced, the UE may mix the data in cell center and cell edge together, which may make NW can’t train separate AI/ML models. Furthermore, existing logged MDT has supported two events to trigger logging:

- Event L1: when serving cell RSRP/RSRQ < threshold (similar to A2)

EventType-r16 ::= CHOICE {

outOfCoverage     NULL,

eventL1           SEQUENCE {

l1-Threshold      MeasTriggerQuantity,

hysteresis        Hysteresis,

timeToTrigger     TimeToTrigger

},

...

}

eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state.

Event L2: when UE enters any cell selection.

As the starting point, we think RAN2 can consider simple event based on serving cell RSRP (i.e. similar to A1/A2).

Proposal 8: Support event triggered data logging based on radio condition (e.g. when serving cell RSRP > threshold or RSRP < threshold).

### 2.1.4 Step 3: Trigger reporting of logged L1 measurement

In RAN2#127 [3], below agreements on reporting was agreed. We address the 3 FFSs on event based reporting and on-demand reporting.

Agreements

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- On-demand request from the network is supported.   FFS details on signalling

#### 2.1.4.1 FFS#1: whether AS buffer event based reporting is supported

We support to introduce event based reporting when the AS buffer is full. The motivation is mainly to avoid misalignment between UE and the NW. As agreed in RAN2#127 [3], the UE autonomously stops measurements for data collection and logging when its AS buffer is full. Please note that NW is not aware of the UE’s autonomous stopping. Such misalignment will cause waste of valuable radio resource for NW to send L1 CSI-RS and NW may not have sufficient measurement to complete model training. If the NW can know the UE has stopped the data collection, it may reconfigure data collection to let the UE continue it.

Observation 6: As agreed in RAN2#127, the UE autonomously stops measurements for data collection when its AS buffer is full. As NW is not aware of the UE’s autonomous stopping, such misalignment will cause waste of valuable radio resource and NW may not have sufficient measurement to complete model training.

Thus, we don’t think the AS buffer based event reporting is an optimization, and it is necessary to avoid misaligned understanding between the UE and NW.

Proposal 9: To avoid misalignment between UE and NW, support event based reporting when AS buffer is full.

#### 2.1.4.2 FFS#2: whether send availability indication or full report when AS buffer is full

During online discussion of RAN2#127 [3], there were divergent view on whether to send availability indication or full report, as illustrated in Table 1.

| Solution | Pros | Cons |
| --- | --- | --- |
| Alt-1: send availability indication | Avoid UL congestion due to large volume data reporting | - It introduces extra report latency due to waiting NW request. - It will lead to missing of the UE’s measurements between the time when the event condition is met (i.e. AS buffer is full) and the time receiving NW request. |
| Alt-2: send full report | - No extra report latency - The UE can continue measurement and logging after full reporting. | It may cause UL congestion if multiple UEs report simultaneously. |

Table.1 Comparison between sending availability indication and full report

With analysis in Table.1, we think both alternatives can be supported because they are target for different scenarios:

- Alt-1 is more suitable for the cell with a large number of UEs and the trained AI/ML model is not sensitive to missing some continuous data samples.

- Alt-2 is more suitable for the cell with a small number of UEs and the trained AI/ML model is sensitive to missing some continuous data samples.

Thus, we should allow NW flexibility by introduce configurable reporting type.

Proposal 10: When the event condition is met (i.e. AS buffer is full), whether sending availability indication or full report can be configured by the Network in RRC.

#### 2.1.4.3 FFS#3: signaling of on-demand reporting

Existing logged MDT uses the RRC message pair UEInformationRequest / UEInformationResponse for NW requested reporting. We think it can be reused for on-demand reporting of logged L1 measurement.

Proposal 11: Reuse existing RRC message pair UEInformationRequest / UEInformationResponse for NW requested reporting of logged L1 measurement.

As one use-case of the on-demand reporting, the UE may notify NW on the availability of stored L1 measurement when it returns to RRC_CONNECTED state, and the NW can retrieve the stored measurements in the UE.

Proposal 12: If the UE enters RRC_IDLE/RRC_INACTIVE state (e.g. upon RLF) and still has logged L1 measurement stored in its AS buffer, it notifies NW on the availability of stored L1 measurement when it returns to RRC_CONNECTED state.

### 2.1.5 Step 4: Reporting of logged L1 measurement

RAN2#127 made below agreements, and the FFS is which SRB is used for reporting.

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

In legacy:

- SRB1/2 have the highest priority which are used to send control signaling (RRC and NAS) and measurements (including RRM, immediate MDT and logged MDT).

- SRB4 has lower priority than SRB1/2 which is used to send QoE measurements.

On one hand, we agree that data collection reporting is not urgent, and it may cause latency of message with SRB1/2. On the other hand, the data collection reporting is similar to logged MDT which is specified to use SRB2. Furthermore, considering enhanced MDT should be a general framework for NW-sided data collection, it should be designed forward compatible. Thus, we prefer to introduce a new SRB with configurable priority.

Proposal 13: Considering data collection reporting framework should be forward compatible, introduce a new SRB with configurable priority.

## 2.2 NW-sided data collection for AI/ML based positioning

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

  - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

First, please note that the updated Rel-19 WID [1] has clearly indicated RAN1 will not discuss any proposals on 2nd priority issues.

Following the guidance from the updated WID, we think RAN2 can also postpone the NW-side data collection discussion on case 2a/2b which are 2nd priority issues.

Proposal 14: Following guidance of updated WID, RAN2 postpone the discussion on data collection of Case 2a)/2b) which are 2nd priority issues.

Then, for case 3a)/3b), Section 7.2.1.3.1 of TR 38.843 captured below conclusion [10]:

From Section 7.2.1.3.1 of TR 38.843

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

Based on above conclusion, it seems to be straight forward for RAN2 to assume NPPRa will be used for exchange of dataset / measurement / label / inference outcome in case 3a)/3b). Thus, we propose:

Proposal 15: For NW-sided model of case 3a)/3b) of AI/ML based positioning, RAN2 assume NPPRa is used to exchange dataset / measurement / label / inference outcome between gNB to LMF.

## 2.3 Collected data type of NW-sided data collection

First, we understand the specific collected data type should be decided by RAN1. Thus, RAN2 can only make some assumption for RAN1 confirmation. Then, according to RAN1 Reply LS (R1-2310681) [7], the required data type is summarized in Table. 2.

| Use case | Training | Inference | Performance monitoring |
| --- | --- | --- | --- |
| Beam management | L1-RSRP, beam ID | Prediction for Set B | L1-RSRP, beam ID |
| Positioning 2b) | Measurements based on PRS Label: timing, LOS/NLOS | Measurement: timing, power and/or phase info | FFS |
| Positioning 3b) | Measurements based on SRS Label: timing, LOS/NLOS | Measurement: timing, power and/or phase info | FFS |
| Positioning 3a) | Measurements based on SRS Label: Location coordinates | Location coordinates | FFS |

Table.2 Summary of required data type for NW-sided model

According to Table 2, we have the below observation:

- Beam management: at least L1-RSRP and beam ID are required data type.

- Positioning Case 2b): the detailed measurements and label information are not clear.

- Positioning Case 3a)/3b): the required measurement and label information is between gNB and LMF, which should be studied in RAN1/RAN3 rather than RAN1.

Meanwhile, similar to Proposal 14, RAN2 should follow guidance of updated WID to postpone discussion on positioning Case 2a)/2b). Thus, we propose:

Proposal 16: On required data type for NW-sided data collection:

- RAN2 assume that at least L1-RSRP and beam ID are required for AI/ML based beam management. Send LS to RAN1 for confirmation.

- RAN2 leave the work of positioning Case 3a)/3b) to RAN1/RAN3.

- Following guidance of updated WID, RAN2 postpone the discussion on positioning Case 2b).

## 2.4 Collected data type of UE-sided data collection

UE-sided data collection includes UE-side model for AI/ML based beam management and positioning case 1/2a). As the UE-side model is deployed at UE side, the basic assumption is that UE-side model is trained by UE side, and thus the required data type corresponding to AI/ML model input is naturally known at UE side. Therefore, when UE uses UE-side AI/ML model for inference, the UE can generate the measurement for AI/ML model inference using the same type or format of the measurement as training phase by implementation. 3GPP should not restrict the data type used for input of AI/ML model training. Thus, we do not think it is necessary to specify the data type of AI/ML model input. 3GPP only need to specify model output for UE-side model, and reference signal (RS) used for model input measurement and its configuration.

Observation 7: As the UE-side model is deployed at UE side, UE-side model is trained by UE side, and thus the required data type corresponding to AI/ML model input is naturally known at UE side. 3GPP should not restrict the data type used for input of AI/ML model training.

Thus, we propose:

Proposal 17: On model training and inference for UE-sided model, RAN2 assume that 3GPP only specify model output, reference signal (RS) used for model input measurement and its configuration. The specific data used for model input and their intermediate processing is up to UE implementation.

Meanwhile, we agree that data type needs to be specified for performance monitoring, especially when the NW performs the monitoring. But it is RAN1 expertise and RAN1 is working on it for both AI/ML based management and AI/ML based positioning. Thus, to avoid potential duplicated work, we propose RAN2 to leave the work to RAN1.

Proposal 18: On performance monitoring for UE-sided model, RAN2 leaves to RAN1 on the required data type.

Finally, RAN2 should send LS to RAN1 on Proposal 16 and Proposal 17.

Proposal 19: RAN2 send LS to RAN1 to check any issue for assumption in Proposal 17 and Proposal 18.

# 3 Conclusion

In this contribution, we further discuss NW-sided data collection. Our Observation are:

Observation 1: Difference between OAM centric and gNB centric data collection is which NW entity (OAM vs gNB) to trigger data collection and generate data collection configuration. Their AS procedure is same.

Observation 2: UE privacy requirement is preserved in OAM centric data collection due to UE consent mechanism in MDT, but it is not clear how to satisfy UE privacy requirement in gNB centric data collection.

Observation 3: Existing L1 CSI framework already supports multiple measurement configurations: multiple NZP-CSI-RS-ResourceSet and multiple CSI-ReportConfig can be configured where one NZP-CSI-RS-ResourceSet can be associated with one CSI-ReportConfig via CSI-ResourceConfigId.

Observation 4: In legacy NR, logged MDT, QoE measurements in IDLE/INACTIVE state and QoE paused measurements had specified the minimum AS layer memory size, which are all 64kB according to TS 38.306.

Observation 5: Based on the typical training data size of beam management provided in RAN1 Reply LS (R1-2310681), the typical training data size stored in AS layer memory is 100 bits8 instances (Set B) + 500 bits4 instances (Set A) =0.35KB.

Observation 6: As agreed in RAN2#127, the UE autonomously stops measurements for data collection when its AS buffer is full. As NW is not aware of the UE’s autonomous stopping, such misalignment will cause waste of valuable radio resource and NW may not have sufficient measurement to complete model training.

Observation 7: As the UE-side model is deployed at UE side, UE-side model is trained by UE side, and thus the required data type corresponding to AI/ML model input is naturally known at UE side. 3GPP should not restrict the data type used for input of AI/ML model training.

Based on observations, our proposals can be found below:

NW-sided data collection for AI/ML based beam management

Proposal 1: To strive for a common measurement framework, similar UE consent mechanism in MDT is supported in gNB-centric data collection, i.e. UE consent is required before NW configures the UE gNB-centric data collection. FFS signaling details.

Proposal 2: MeasConfig in RRCReconfiguration is reused to configure logged L1 measurements with below specification changes to integrate L1 measurement logging:

- Within MeasObjectNR: introduce L1 CSI resource configuration and logging configuration.

- Within ReportConfigNR: introduce L1 measurement reporting configuration to indicate at least the reporting quantity configuration and reporting type (e.g. periodic or on-demand reporting).

Proposal 3: Similar to existing L1 CSI framework, multiple L1 CSI resource sets and multiple logging configuration (e.g. with different logging periodicities) can be configured in MeasObjectNR, whether one L1 CSI resource set is associated with one logging configuration via CSI-ResourceConfigId.

Proposal 4: Do not support dynamic activation/deactivation of immediate MDT with L1 measurement logging.

Proposal 5: The minimum AS layer memory size for L1 measurement logging is same as existing logged MDT and QoE (i.e. 64KB).

Proposal 6: The stored data in UE’s AS buffer at least includes L1 measurements (L1 RSRP and/or beam index) and the logging time stamp.

Proposal 7: On the low power state issue during data collection, introduce the following solution similar to reactive temporary restriction reporting of Rel-18 MUSIM:

- Upon low power state, the UE may send UAI to notify the NW.

- The UE starts a wait timer when sending the UAI. If the wait timer expires without reception of NW reaction, it may stop the data collection.

Proposal 8: Support event triggered data logging based on radio condition (e.g. when serving cell RSRP > threshold or RSRP < threshold).

Proposal 9: To avoid misalignment between UE and NW, support event based reporting when AS buffer is full.

Proposal 10: When the event condition is met (i.e. AS buffer is full), whether sending availability indication or full report can be configured by the Network in RRC.

Proposal 11: Reuse existing RRC message pair UEInformationRequest / UEInformationResponse for NW requested reporting of logged L1 measurement.

Proposal 12: If the UE enters RRC_IDLE/RRC_INACTIVE state (e.g. upon RLF) and still has logged L1 measurement stored in its AS buffer, it notifies NW on the availability of stored L1 measurement when it returns to RRC_CONNECTED state.

Proposal 13: Considering data collection reporting framework should be forward compatible, introduce a new SRB with configurable priority.

NW-sided data collection for AI/ML based positioning

Proposal 14: Following guidance of updated WID, RAN2 postpone the discussion on data collection of Case 2a)/2b) which are 2nd priority issues.

Proposal 15: For NW-sided model of case 3a)/3b) of AI/ML based positioning, RAN2 assume NPPRa is used to exchange dataset / measurement / label / inference outcome between gNB to LMF.

Collected data type of NW-sided data collection

Proposal 16: On required data type for NW-sided data collection:

- RAN2 assume that at least L1-RSRP and beam ID are required for AI/ML based beam management. Send LS to RAN1 for confirmation.

- RAN2 leave the work of positioning Case 3a)/3b) to RAN1/RAN3.

- Following guidance of updated WID, RAN2 postpone the discussion on positioning Case 2b).

Collected data type of UE-sided data collection

Proposal 17: On model training and inference for UE-sided model, RAN2 assume that 3GPP only specify model output, reference signal (RS) used for model input measurement and its configuration. The specific data used for model input and their intermediate processing is up to UE implementation.

Proposal 18: On performance monitoring for UE-sided model, RAN2 leaves to RAN1 on the required data type.

Proposal 19: RAN2 send LS to RAN1 to check any issue for assumption in Proposal 17 and Proposal 18.

# 4 References

[1] RP-234039, New WID: New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#126, Chair Notes.

[3] RAN2#127, Chair Notes.

[4] TS 32.422-v17.8.0, Trace control and configuration management, 2022-9.

[5] RAN1#116b, Chair Notes.

[6] TS 38.306-v18.0.0, User Equipment (UE) radio access capabilities, 2023-12.

[7] R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1.

[8] TS 38.300-v18.0.0, NR and NG-RAN Overall Description; Stage 2, 2023-12.

[9] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[10] TR 38.843-v2.0.1, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 2023-12.