3GPP TSG-RAN WG2 Meeting #127bis	R2-2408579

Hefei, China, 14 – 18 October 2024

Agenda item:	8.7.6

Source:	Apple

Title:	Views on MAC Signalling for XR Rate Control

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

During RAN #105, the following new objective has been approved for Rel-19 WI on XR for NR Phase 3 [1]:

-	Specify uplink congestion signalling [RAN2]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

This paper presents our views on this topic.

# Discussions

## RAN-Aware XR Rate Control

The XR rate control signalling allows the Application to adjust the source coding scheme based on the instantaneous RAN status (e.g. channel quality, network loading etc), in order to maintain the operation without severely impacting user experience. The objective of this WI focusses on DL signalling, which targets to enable adaptation of Application at the UE side.

This intention is essentially very similar to Recommended Bit Rate MAC CE in 5G NR, as shown in Figure 1:

Figure 1 Recommended Bit Rate MAC CE in TS 38.321.

The Recommended Bit Rate MAC CE enables the gNB to indicate a suitable data rate for traffics corresponding to a logical channel (LCH). To recap, this MAC CE provides the following information:

- An ID of a Logical Channel (LCID) that this Recommended Bit Rate MAC CE is applicable

- A recommended Bit Rate for the concerned Logical Channel, where the value is chosen from Table 6.1.3.20-1 of TS 38.321, along with a potential multiplier.

At the first glance, it seems Recommended Bit Rate MAC CE can be reused to serve the purpose of XR rate control. For instance, when UL congestion is severe, the gNB may indicate a low value of Recommended Bit Rate via the MAC CE, and the UE Application may reduce the XR frame rate. Nonetheless, as the existing Recommended Bit Rate MAC CE was designed mainly to support Voice Communications instead of XR use cases, the Recommended Bit Rate values may not be sufficient to support the targeted Applications. Currently, the highest Recommended Bit Rate value in Table 6.1.3.20-1 of TS 38.321 is 8000 kbits/s, which is insufficient to support many typical XR use cases with a much higher data rate.

Furthermore, XR services may comprise multiple different traffic flows, including video, audio, and haptic feedback. It is possible that some of these traffic flows are mapped to the same DRB (and hence the same LCH) due to their similar QoS requirements, but the recommended bit rates for these traffic flows may be different. Consequently, we may end up increasing or decreasing the data rate for some of the traffic flows inappropriately, as the existing Recommended Bit Rate MAC CE is unable to differentiate multiple QoS flows mapped to this LCH.

Observation 1: The purpose of XR rate control signalling is very similar to Recommended Bit Rate MAC CE in NR.

Observation 2: For XR use cases, a higher range of Recommended Bit Rate is needed. Moreover, granularity based on QoS flow is more suitable.

## New MAC CE for XR Rate Control

Based on the Observations made in the preceding section, we think RAN2 can consider introducing a new DL MAC CE to enable XR rate control, which has a similar purpose as the legacy Recommended Bit Rate MAC CE. Specifically, this new MAC CE should indicate the Recommended Bit Rate for one or more QoS flows (in contrast to per LCH), with a possibility to indicate data rate higher than the values that are currently available in Table 6.1.3.20-1 of TS 38.321. To enable indication of higher Recommended Bit Rates, RAN2 may either introduce a new table, or utilize the Reserved entries that are currently available in Table 6.1.3.20-1 of TS 38.321. In addition, the MAC CE may also include some information that is specifically applicable to this XR traffic flow.

Proposal 1: RAN2 should introduce a new MAC CE with similar purpose as Recommended Bit Rate MAC CE:

- The new MAC CE can indicate Recommended Bit Rate for one or more QFIs, and

- The new MAC CE can indicate Recommended Bit Rate with higher values than the legacy.

- Other additional information is not precluded

# Conclusions

In this paper, we presented our views on XR rate control signalling in DL, which indicates how XR Applications should adjust their rates for UL. We have made the following observations:

Observation 1: The purpose of XR rate control signalling is very similar to Recommended Bit Rate MAC CE in NR.

Observation 2: For XR use cases, a higher range of Recommended Bit Rate is needed. Moreover, granularity based on QoS flow is more suitable.

Based on these observations, we propose that:

Proposal 1: RAN2 should introduce a new MAC CE with similar purpose as Recommended Bit Rate MAC CE:

- The new MAC CE can indicate Recommended Bit Rate for one or more QFIs, and

- The new MAC CE can indicate Recommended Bit Rate with higher values than the legacy.

- Other additional information is not precluded

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.