3GPP TSG-RAN2 Meeting #127bis	R2-2408587

# Hefei, China, October 14th – 18th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5021 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on UE behaviour upon reception of RRCReconfigCompleteSL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core, NR_SL_Relay-Core, NR_SL_relay_enh-Core, NR_SL_enh2 |  |  |  |  |  | Date: |  |  | 2024-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current procedure specified in 5.8.9.1.9 is quite insufficient. Although the UE shall “consider the configurations in the corresponding RRCReconfigurationSidelink message to be applied” in this clause, the only specified processing in leve-2 bullet under this sentence is to process sl-DRX-ConfigReject. It gives wrong impression to readers that there is nothing significant to be triggered in the Tx UE side upon the reception of this message, except handing SL-DRX configuration reject. Actually, the “to be applied” means the TX UE assuming the peer RX UE has applied the configuration, but the Tx UE behaviour is still unspecified except the handing of SL-DRX reject. Literally, there are many actions to be taken by the UE when the Tx UE of RRCReconfigruaitonSidleink message receives the “RRCReconfigruaitonSidleinComplete” message, such as: - Addition/modification/release of SL-DRB - Addition/modification/release of PC5 Relay RLC channel - Triggering the QoS split for end-to-end SL DRBs added/modified. - Addition/modification/release of SL RLC bearer for SL CA - Addition/modification/release of SL carrier - Stop Timer T421 for R18 L2 U2N Remote UE In the current RRC spec, the above Tx UE actions are spread in many different procedures, with the text like “ ….. after receiving the RRCReconfigurationCompleteSidelink message, …“ appearing in different subclauses, such as 5.3.5.3, 5.8.9.1a.1, 5.8.9.1a.2, 5.8.9.1a.5, 5.8.9.1a.6, 5.8.9.1b and 5.8.9.7. It is worth nothing that the UE actions according to those sections have been elaborated in 5.8.9.1.3 and referred to the above sections for the RX UE side, but there is no corresponding text in 5.8.9.1.9 for the TX UE side. Based on the above analysis, we think the UE behaviour in 5.8.9.1.9 should also be elaborated to match the procedures in other sections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 5.8.9.1.9, added “by the peer UE” in “consider the configurations in the corresponding RRCReconfigurationSidelink message to be applied“. - In 5.8.9.1.9, added the missing SL-DRB add/modification/release actions. Also, for end-to-end SL-DRB addition/modification, added triggers for UE information transfer procedure for QoS split. - In 5.8.9.1.9, added the missing PC5 Relay RLC channel add/modification/release actions. - In 5.8.9.1.9, added the missing additional SL RLC bearer add/modification/release actions. - In 5.8.9.1.9, added the missing SL Carrier addition/release actions. - In 5.8.9.1.9, added the missing “stop T421” behaviour. Impact analysis Impacted functionality: UE Reception of RRCReconfigurationSidelinkComplete. Inter-operability: - If Sidelink UE implements this change and gNB does not, there is no inter-operability issue between UE and gNB. - If Sidelink TX UE implements this change, but the Sidelink RX UE is not implemented this change, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong UE implementation of processing RRCReconfigurationCompleteSL message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.1.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

BEGIN OF CHNAGES

##### 5.8.9.1.9	Reception of an RRCReconfigurationCompleteSidelink by the UE

The UE shall perform the following actions upon reception of the RRCReconfigurationCompleteSidelink:

1>	stop timer T400 for the destination, if running;

1>	consider the configurations in the corresponding RRCReconfigurationSidelink message to be applied by the peer UE;

2>	if the RRCReconfigurationCompleteSidelink message includes the sl-DRX-ConfigReject:

3>	consider no sidelink DRX to be applied for the corresponding sidelink unicast communication;

- if the configurations in the corresponding RRCReconfigurationSidelink message includes the slrb-ConfigToReleaseList:

2>	for each entry value included in the slrb-ConfigToReleaseList:

3>	perform the sidelink DRB release procedure, according to clause 5.8.9.1a.1;

- if the configurations in the corresponding RRCReconfigurationSidelink message includes the slrb-ConfigToAddModList:

2>	for each slrb-PC5-ConfigIndex value included in the slrb-ConfigToAddModList:

3>	perform the sidelink DRB addition/modification procedure, according to clause 5.8.9.1a.2;

3>	if the UE is a L2 U2U Remote UE and the added/modified sidelink DRB is an end-to-end DRB:

4>	triggers the UE information transfer procedure to inform end-to-end QoS information to the connected L2 U2U relay UE, as specified in 5.8.9.11.

1>	if the configurations in the corresponding RRCReconfigurationSidelink includes the sl-RLC-ChannelToReleaseListPC5:

2>	for each SL-RLC-ChannelID value included in the sl-RLC-ChannelToReleaseListPC5:

3>	perform the PC5 Relay RLC channel release procedure, according to clause 5.8.9.7.1;

1>	if the configurations in the corresponding RRCReconfigurationSidelink includes the sl-RLC-ChannelToAddModListPC5:

2>	for each sl-RLC-ChannelID-PC5 value included in the sl-RLC-ChannelToAddModListPC5:

3>	perform the PC5 Relay RLC channel addition procedure, according to clause 5.8.9.7.2;

1>	if the configurations in the corresponding RRCReconfigurationSidelink message includes the sl-RLC-BearerToReleaseList:

2>	for each entry value included in the sl-RLC-BearerToReleaseList;

3>	perform the additional sidelink RLC bearer release procedure, according to clause 5.8.9.1a.5;

1>	if the configurations in the corresponding RRCReconfigurationSidelink message includes the sl-RLC-BearerToAddModList:

2>	for each SL-RLC-BearerConfigIndex value included in the sl-RLC-BearerToAddModList:

3>	perform the additional sidelink RLC bearer addition/modification procedure, according to clause 5.8.9.1a.6;

1>	if the configurations in the corresponding RRCReconfigurationSidelink message includes the sl-CarrierToReleaseList:

2>	for each entry value included in the sl-CarrierToReleaseList;

3>	perform the sidelink carrier release procedure, according to clause 5.8.9.1b.1;

1>	if the configurations in the corresponding RRCReconfigurationSidelink message includes the sl-CarrierToAddModList:

2>	for each sl-CarrierId value included in the sl-CarrierToAddModList:

3>	perform the sidelink carrier addition procedure, according to clause 5.8.9.1b.2;

1>	if the UE is a L2 U2N remote UE receiving a RRCReconfigurationCompleteSidelink message from L2 U2N relay UE:

2>	if split SRB1 with duplication is not configured:

3>	stop timer T421, if running.

END OF CHNAGES