3GPP TSG-RAN WG2 Meeting #127bis		R2-2408596

Hefei, China, 14th – 18th Oct. 2024

Agenda Item: 	8.6.2

Source:	Apple

Title:	Open items for inter-CU LTM in SA and DC cases

Document for:		Discussion and Decision

# Introduction

In this document we focus on the remaining aspects of inter-CU LTM in both SA and DC cases that are more stage-2 oriented, as we think resolving stage-2 issues are critical for moving on to stage 3 aspects.

# Inter-CU in SA

## Identification of CU by the UE

In RAN2-127 and prior meetings, we agreed that it would RRC configuration that informs the UE about the CU to which each candidate LTM cell belongs to. LTM cell switch MAC CE would not need to have any additional information about this.

RAN2-127: 6. If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

In Rel-18, RAN2 already have devised such RRC configuration for informing the UE about whether to reset RLC or not based on source and target LTM configurations, sort of indication whether the resulting LTM switch is between DUs or was intra-DU.

Observation 1: ltm-NoResetID-r18 provide the UE with information based on which the UE can decide intra or inter-DU LTM switch during the time of the switch, in order to reset RLC or not.

Observation 2: With inter-CU LTM, we can have more than one DU as the candidates for each CU the UE is configured to have LTM candidates with. In other words, the intra or inter-DU derivation by the UE is now scoped per CU.

Observation 3: Any RRC configuration that informs the UE about CU information should work with the existing information provided with ltm-NoResetID-r18

Observation 4: It would be better to design stage-3 CU configuration to follow the ltm-NoResetID-r18 logic, but ensure that the UE interpretation still allows for multiple DUs for each of the involved CUs.

Proposal 1: When configured with inter-CU LTM where CU information is provided by RRC, Rel-19 UEs should treat the information derived from ltm-NoResetID-r18 on a per-CU basis.

In we agree to proposal 1, then the easier form of signalling the CU information in RRC would be to follow the ltm-NoResetID-r18 logic as shown below.

LTM-Candidate-r18 ::= SEQUENCE { ltm-CandidateId-r18 LTM-CandidateId-r18, ltm-CandidatePCI-r18 PhysCellId OPTIONAL, -- Need M ltm-SSB-Config-r18 LTM-SSB-Config-r18 OPTIONAL, -- Need M ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL, -- Need M ltm-ConfigComplete-r18 ENUMERATED {true} OPTIONAL, -- Need R ltm-EarlyUL-SyncConfig-r18 OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18) OPTIONAL, -- Need R ltm-EarlyUL-SyncConfigSUL-r18 OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18) OPTIONAL, -- Need R ltm-TCI-Info-r18 LTM-TCI-Info-r18 OPTIONAL, -- Need M ltm-NoResetID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ltm-UE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ..., [[ ltm-CU-ID-r19 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ]] } LTM-Config-r18 ::= SEQUENCE { ltm-ReferenceConfiguration-r18 CHOICE { release NULL, setup ReferenceConfiguration-r18 } OPTIONAL, -- Need M ltm-CandidateToReleaseList-r18 SEQUENCE (SIZE(1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18 OPTIONAL, -- Need N ltm-CandidateToAddModList-r18 SEQUENCE (SIZE(1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 OPTIONAL, -- Need N ltm-ServingCellNoResetID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need N ltm-CSI-ResourceConfigToAddModList-r18 SEQUENCE (SIZE(1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18 OPTIONAL, -- Need N ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE(1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18 OPTIONAL, -- Need N attemptLTM-Switch-r18 ENUMERATED {true} OPTIONAL, -- Cond LTM-MCG ltm-ServingCellUE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need N ... , [[ ltm-ServingCell-CU-ID-r19 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ]] }

# Inter-CU in DC

## Discussion on inter-CU LTM in DC

In RAN2-127 very good progress has been made on inter-CU LTM in DC cases on several aspects including:

| Who initiates SN LTM | Conclusion: SN |
| --- | --- |
| Who controls SN LTM | Conclusion: SN |
| For inter-CU SN LTM, whether MCG part configuration can be changed during SN LTM | Conclusion: yes |
| For inter-CU MN LTM, whether SCG configuration can be updated | Conclusion: yes, same as legacy procedure |
| Security key handling uses SCPAC as baseline | Conclusion: agreed as baseline (unless SA3 has issues) |

In this chapter, we analyzed several leftover issues for inter-CU LTM with DC configured.

### Co-existence of inter-MN LTM with intra-SN LTM

It is observed that there is considerable interest in supporting it. If this scenario can be supported, we think both source LTM MN cell and candidate LTM MN cell may configure UE with intra-SN LTM. And SN LTM configuration is contained in RRCReconfiguration for SCG part configuration.

Fig 1a - Source MN configures intra-SN LTM                Fig 1b - Candidate MN configures intra-SN LTM

Observation 5: Intra-SN LTM can be provided either by SN of source MN or by SN of candidate MN.

From UE point of view, the handling of the co-existence of two LTM-Config(s) is straightforward. Once UE performs inter-CU LTM, UE would apply the new configuration from that candidate PCell. At the mean time, UE would either add or release the intra-SN LTM-Config based on candidate MN’s configuration.

Proposal 2: Support coexistence of inter-MN LTM and intra-SN LTM and no special handling is required.

### Inter-SN LTM

Even though SCPAC can be used as baseline for preparation phase and security handling, we think necessary changes are inevitable. Below we illustrate the flow chart of inter-SN LTM in Fig. 2.

Figure 2 – Inter-SN LTM flow chart

For example the following are needed to be discussed on top of what have been agreed:

- Step 1 (from figure 2): Source SN provides a list of proposed LTM PSCell candidates, and Reference SCG configuration;

- Step 4 (from figure 2): The MN sends SCG LTM-Config to all involved SN(s), since SCG LTM-Config should be known by source/target SN.

- Step 8 (from figure 2): SN transmits LTM command MAC CE to UE. We think legacy LTM Cell Switch Command MAC CE can be reused since we decided that UE by itself selects the sk-counter.

Proposal 3: The MN shares SCG LTM-Config to all involved SN(s) during Inter-SN LTM preparation.

Proposal 4: Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

# Conclusion

Observation 1: ltm-NoResetID-r18 provide the UE with information based on which the UE can decide intra or inter-DU LTM switch during the time of the switch, in order to reset RLC or not.

Observation 2: With inter-CU LTM, we can have more than one DU as the candidates for each CU the UE is configured to have LTM candidates with. In other words, the intra or inter-DU derivation by the UE is now scoped per CU.

Observation 3: Any RRC configuration that informs the UE about CU information should work with the existing information provided with ltm-NoResetID-r18

Observation 4: It would be better to design stage-3 CU configuration to follow the ltm-NoResetID-r18 logic, but ensure that the UE interpretation still allows for multiple DUs for each of the involved CUs.

Observation 5: Intra-SN LTM can be provided either by SN of source MN or by SN of candidate MN.

Proposal 1: When configured with inter-CU LTM where CU information is provided by RRC, Rel-19 UEs should treat the information derived from ltm-NoResetID-r18 on a per-CU basis.

Proposal 2: Support coexistence of inter-MN LTM and intra-SN LTM and no special handling is required.

Proposal 3: The MN shares SCG LTM-Config to all involved SN(s) during Inter-SN LTM preparation.

Proposal 4: Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.