3GPP TSG-RAN WG2 #127bis	R2-2408600

Hefei, China, 14 – 18 October 2024

Agenda item:		8.5.3	On-demand SIB1

Title:	Discussion on On-demand SIB1 signalling details

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

During RAN2 #127, the following agreements were made:

OD-SIB1 Validity: 2. We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell). 3. Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement). WUS configuration of NES cell from NES cell: 4. Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures. Access restriction for legacy UEs: 6. Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded. - Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB. - Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB. - Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Furthermore, it was agreed to pursue standardisation of “Case 2” during RAN #107

In this contribution, we discuss further considerations on UL WUS configuration and SIB1 update when for UEs camping on an NES cell, and related legacy barring UEs issues.

# 2. Discussion

2.1	UL WUS configuration signalling details for cell A and update at NES cell

There have been discussions about having the UL WUS configuration in Cell A’s SIB1. This has the advantage of UEs only interested in the NES cell to be able to get the UL WUS configuration more easily.
However, a single Cell A may provide UL WUS configurations for many NES cells, so any change in one of the neighbouring NES cell would require to update the Cell A’s SIB1, which would negatively impact all UEs not interested in a given NES cell.

Instead, it would be better to have UL WUS configuration available on demand, or at least provided as other SI.

Proposal 1: UL WUS configuration is not provided by Cell A’s SIB1 and is part of Other SI, either periodically broadcast, broadcast on-demand, or sent in a dedicated manner.

A UE camping on the OD-SIB1 NES cell will have its SIB1 expire after 3 hours and may need to reacquire it using UL WUS request. Thus, it was agreed that UL WUS update is provided by the NES Cell.
However, there are still some issues when the SIB1 is being updated by the NES cell itself.

If there is a SIB1 update, then all UEs on the NES cell will monitor the SIB1 update and can assume that the NW will broadcast the updated SIB1 to all UEs, as in legacy, i.e. UEs are not expected to do UL WUS request.

Proposal 2: In an OD-SIB1 NES cell, when the SIB1 is updated, legacy SIB1 update procedure is used and the UEs will expect the updated SIB1 to be broadcast by the NES cell, i.e. UEs do not perform UL WUS request to acquire the updated SIB1.

2.2	Access restriction for legacy UEs

The following was agreed in RAN2#127:

Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Firstly, Option 3 is compatible with Options 1 and 2 and may be used in addition to them. However, this will not be applicable for initial cell selection, so an additional barring mechanism may still be necessary.

Observation: Option 3 (Network includes cells supporting OD-SIB1 to list of excluded cells) can be implemented regardless of additional barring mechanisms at the NES cell.

Different options for legacy UE barring are being discussed in RAN1. Typically, there are some issues regarding UEs that can bypass MIB barring (i.e. NTN UEs) or different options to signal SIB1 location from pdcch-ConfigSIB1.

Proposal 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Proposal 4: RAN2 needs further input from RAN1 to decide on legacy UE barring.

# 3. Conclusion

In this contribution we discussed support of on-demand SIB1 for UEs in Idle/Inactive and made the following proposals.

Proposal 1: UL WUS configuration is not provided by Cell A’s SIB1 and is part of Other SI, either periodically broadcast, broadcast on-demand, or sent in a dedicated manner.

Proposal 2: In an OD-SIB1 NES cell, when the SIB1 is updated, legacy SIB1 update procedure is used and the UEs will expect the updated SIB1 to be broadcast by the NES cell, i.e. UEs do not perform UL WUS request to acquire the updated SIB1.

Proposal 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Proposal 4: RAN2 needs further input from RAN1 to decide on legacy UE barring.

# References

[1] R2-2403731, Report from session on V2X/SL, R19 NES and MOB

[2] RAN1 Chair’s Notes (RAN1#116bis)