3GPP TSG-RAN WG2 Meeting #127bis                                 R2-2408605

Hefei, China, 14th Oct. – 18th Oct, 2024

Agenda Item	: 8.4.4

Source	: LG Electronics Inc.

Title	: LP-WUS operation in RRC_CONNECTED Mode

Document for	: Discussion and Decision

Introduction

In the RAN1#105 meeting, the following is left as the remaining open issue for the RAN2-related aspects of LP-WUS in RRC_CONNECTED mode:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

In this contribution, we would like to clarify and show our views on the RAN2-related aspects of LP-WUS in RRC_CONNECTED mode.

Discussion

Regarding LP-WUS operation in RRC_CONNECTED mode, RAN1 had the agreement in RAN1#118 [3] as follows:

Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18 - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration - LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions. R1-2407516 FL summary #3 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement Select one of the following alternatives in RAN1#118bis Alt 1: For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value Alt2: For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value - Different minimum time gaps correspond to different sleep states - Companies are encouraged to details on how the reported values are to be used by the network Agreement LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC Agreement For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS - FFS applicable QCL type(s)

In the RAN1#118 meeting [3], it was agreed to support Option 1-2 as a working assumption, disregarding the previous separate classifications of Option 1-2-1 and Option 1-2-2. Accordingly, in the last RAN2#127 meeting [4], the following was agreed for Option 1-2, and further discussion was postponed.

Agreement For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

As a continuation of our last meeting, we believe it would be beneficial to confirm companies' understanding of redefined Option 1-2, and then discuss which option or combination of options is most appropriate.

Procedures to allow UE MR PDCCH monitoring triggered by LP-WUS

Option 1-2

Option 1-2, as defined by RAN1, is 'LP-WUS monitoring outside at least legacy C-DRX active time, according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.' Since the definition alone for Option 1-2 may lead to different interpretations among companies, we believe it is essential for RAN2 to first establish a common understanding of Option 1-2 before delving into detailed discussions.

The first topic regarding the common understanding of Option 1-2 is whether PDCCH monitoring is performed solely by the LP-WUS indication, solely by the existing DRX timer, or by both.

RAN1 had already agreed that 'UE is configured with legacy C-DRX configurations as Rel-18' and 'UE PDCCH monitoring behaviors related to other legacy DRX timers (drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL) are not affected.' Therefore, RAN2 can further discuss whether PDCCH monitoring in Option 1-2 is performed solely based on LP-WUS indication or if it is also influenced by the drx-onDurationTimer.

We believe that continuously performing PDCCH monitoring during all DRX OnDuration periods is highly detrimental from a power-saving perspective. Moreover, since receiving an LP-WUS indication to perform PDCCH monitoring can also include the DRX OnDuration period, having multiple factors determining PDCCH monitoring during the DRX OnDuration period can be redundant and wasteful.

Thus, the legacy DRX OnDuration should not be configured for PDCCH monitoring but for DRX-related operations other than PDCCH monitoring (e.g., CSI/SRS reporting). This means that PDCCH monitoring operations are conducted only when LP-WUS is received, regardless of the DRX OnDuration.

Proposal 1. For Option 1-2, RAN2 confirms that PDCCH monitoring is only triggered by LP-WUS indication.

The second topic regarding the common understanding of Option 1-2 is, ‘Which timer should be initiated when PDCCH monitoring is triggered by an LP-WUS indication?’. In the previous RAN2#127 meeting, it was left as FFS whether the timer that the UE starts after the LP-WUS indication triggers PDCCH monitoring in Option 1-2 is a newly defined timer or a legacy timer.

We believe that a new timer should be initiated when PDCCH monitoring is triggered by an LP-WUS indication. The reason is that if a legacy timer is used for PDCCH monitoring triggered by an LP-WUS indication, it would likely be the drx-InactivityTimer, which is unsuitable.

According to the current MAC specification [5], the drx-InactivityTimer can only be started or restarted when a new transmission is scheduled during the PDCCH monitoring period. This is because the drx-InactivityTimer was introduced to ensure continuous PDCCH monitoring, as there could be subsequent scheduling once a transmission is scheduled. Starting the drx-InactivityTimer to perform PDCCH monitoring upon receiving an LP-WUS indication while only LP-WUS monitoring is being performed does not align with the original purpose of the drx-InactivityTimer.

For this reason, we believe that it is more appropriate to introduce a new timer for this operation, ensuring that each timer is used according to its intended purpose. Specifically, when the UE receives an LP-WUS indication outside the DRX OnDuration period, a new timer should be started to perform PDCCH monitoring during the new DRX active time. If a new timer is introduced, discussions will also be needed regarding the additional configuration, including the introduction of new DRX active time and parameters for the new DRX active time.

Proposal 2. For Option 1-2, RAN2 confirms that a new timer will be started upon receiving an LP-WUS indication to perform PDCCH monitoring.

The third topic regarding the common understanding of Option 1-2 is whether LP-WUS monitoring should be performed during the legacy DRX OnDuration period.

The DRX configuration is designed to monitor PDCCH during DRX OnDuration, anticipating data reception based on the characteristics of the data. Therefore, if no monitoring is performed during DRX OnDuration, there will be an increased risk of missing data transmission, leading to potential data loss and increased latency. This will result in the degradation of overall performance and reliability. To mitigate this issue, it is essential to ensure that at least LP-WUS monitoring should be conducted not only during the DRX OnDuration interval but also in all other periods.

Proposal 3. For Option 1-2, LP-WUS monitoring is performed not only outside but also inside the legacy C-DRX OnDuration interval.

About supporting both Option1-1 and Option1-2 simultaneously

In the RAN1#118 meeting, whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE is left as FFS.

The operations of Option 1-1 and Option 1-2 are entirely different. According to the RAN2#126 agreement, for Option 1-1, if an LP-WUS indication is received on an LP-WUS monitoring occasion that is located at a configured time offset before the start of drx-onDurationTimer, the drx-onDurationTimer starts, and PDCCH monitoring is performed while drx-onDurationTimer is running. In contrast, our understanding of Option 1-2 is that the LP-WUS configuration is unrelated to the DRX configuration. When an LP-WUS indication is received on one of the LP-WUS monitoring occasions configured by the LP-WUS configuration, a new timer starts, and PDCCH monitoring is performed while the new timer is running.

When Option 1-1 and Option 1-2 are configured simultaneously, and an LP-WUS indication is received during an LP-WUS occasion outside the legacy C-DRX active time, two timers (both the drx-onDurationTimer and the new timer) are started. Both timers aim to ensure that the UE performs PDCCH monitoring while the timer runs. Running multiple timers for the same purpose not only results in timer duplication but also leads to unnecessary power consumption, increased latency, potential conflicts, and complex implementation issues. This negatively impacts efficiency and reliability.

Consequently, we believe it is preferable to configure a single option well rather than configuring both Option 1-1 and Option 1-2 simultaneously.

Proposal 4. Simultaneous configuration of Option 1-1 and Option 1-2 is not supported.

Comparison between Option1-1 and Option 1-2

Option 1-1 and Option 1-2 operate differently, each with its own set of advantages and disadvantages. Nevertheless, we believe that Option 1-1 can achieve performance similar to Option 1-2 in terms of power consumption and data latency. This can be done by shortening the length of the DRX cycle without specification impact, as in the figure below.

Moreover, Option 1-2 requires additional configuration for PDCCH monitoring behavior to receive data outside the legacy DRX active time. This includes the introduction of new DRX active time and parameters for the new DRX active time, which could lead to specification changes. In addition, if the PDCCH monitoring triggered by the LP-WUS indication is not aligned with the DRX OnDuration, the distinction between ‘legacy DRX behavior besides PDCCH monitoring’ and ‘PDCCH monitoring behavior’ would lead to a significant specification impact as well as increased power consumption. For example, if the UE's configuration for periodic CSI/L1-RSRP reporting is ‘Periodic CSI/L1-RSRP is periodically reported regardless of whether the UE is indicated to wake up or not’, the UE would need to wake up the MR not only for PDCCH monitoring upon receiving the LP-WUS but also periodically for CSI/L1-RSRP reporting. This would require more power consumption than the legacy operation, making it unsuitable. Therefore, to prevent such situations, restrictions on each function will be necessary, which, in turn, would lead to a specification impact.

In conclusion, since similar performance to Option 1-2 can be achieved through DRX configuration in Option 1-1, there seems to be no reason to choose Option 1-2 while enduring the spec impact. We believe that a well-configured DRX configuration setup in Option 1-1 (with a short-cycled DRX cycle) is sufficient to provide reliable, low-latency data reception without any specification changes.

Proposal 5. The option that LP-WUS replaces DCP is the optimal option for the procedure to allow UE MR PDCCH monitoring triggered by LP-WUS.

Conclusion

Based on the above discussion, we made the following proposal.

Proposal 1. For Option 1-2 (LP-WUS monitoring outside at least legacy C-DRX active time, according to the LP-WUS monitoring configuration to trigger PDCCH monitoring), RAN2 confirms that PDCCH monitoring is only triggered by LP-WUS indication.

Proposal 2. For Option 1-2, RAN2 confirms that a new timer will be started upon receiving an LP-WUS indication to perform PDCCH monitoring.

Proposal 3. For Option 1-2, LP-WUS monitoring is performed not only outside but also inside the legacy C-DRX OnDuration interval.

Proposal 4. Simultaneous configuration of Option 1-1 and Option 1-2 is not supported.

Proposal 5. The option that LP-WUS replaces DCP is the optimal option for the procedure to allow UE MR PDCCH monitoring triggered by LP-WUS.

References

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- TR 38.869 v18.0.0, Study on low-power Wake-up Signal and Receiver for NR

- Chair notes RAN1#118

- R2_127 MIMOevo MUSIM LPWUS SBFD (Erlin) 2024-08-23 0900

- 3GPP TS 38.321, NR; Medium Access Control (MAC) protocol specification