3GPP TSG-RAN WG2 Meeting #127bis	R2-2408607

Hefei, China, Oct 14th – 18th, 2024

| Agenda Item: | 8.6.2 (NR_Mob_Ph4-Core) |
| --- | --- |
| Source: | LG Electronics Inc. |
| Title: | Discussion on inter-CU LTM with DC |
| Document for: | Discussion and Decision |

1.	Introduction

In this paper, we discuss SN security update for inter-CU LTM cell switch, where CU is acting as MN and SN is unchanged.

2.	Discussion

2.1.	SN security update for inter-CU LTM cell switch where CU is acting as MN and SN is unchanged

In RAN2#127 meeting, RAN2 agreed the following:

For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

This means that the legacy SN security update procedure is re-used for the case where CU is acting as MN and SN is unchanged. Note that the legacy SN security update procedure is always performed if sk-Counter is included in the RRCReconfiguration message.

Specifically in legacy, the network sends to the UE the MN RRCReconfiguration message including the new MN security configuration and the single counter value (i.e., sk-Counter) for updating the SN security if the MN security update is needed due to, e.g, MN handover while the SN connection is maintained before/after MN handover. To this end, the SN RRCReconfiguration message including sk-Counter is embedded in the MN RRCReconfiguration message. Upon receiving the RRCReconfiguration message, the UE updates the MN security based on the new MN security configuration (i.e., new KgNB). Subsequently, the UE updates the SN security based on the new KgNB and sk-Counter (i.e. new S- KgNB). To sum up, the SN security key update is based only on the provision of sk-Counter in the case of MN handover with keeping the existing SN connection.

For LTM cell switch, one or more candidate cell configurations are provided to the UE beforehand. Upon receiving an LTM cell switch command (CSC) MAC CE from the network, the UE performs the LTM cell switch toward the candidate cell indicated by the LTM CSC MAC CE, i.e., the UE first applies the stored candidate cell configuration and subsequently the UE accesses the candidate cell based on the stored candidate cell configuration and the LTM CSC MAC CE. In order to update the SN security with legacy Rel-15 RRC reconfiguration with sync procedure in the case of inter-CU MCG LTM while the SN connection is maintained, the pre-configured candidate cell configuration of MCG LTM target cell should include the single counter value, i.e., sk-Counter, for updating SN security.

In RAN2#125bis meeting, RAN2 agreed the following:

Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

Based on the above agreement, the following two scenarios (i.e., Figure 1 and Figure 2) may occur.

Figure 1. Subsequent intra-CU MCG LTM cell switches after inter-CU MCG LTM cell switch in case where the SN connection via a specific cell is maintained.

Figure 1 describes a scenario that subsequent intra-CU MCG LTM cell switches are performed after inter-CU MCG LTM cell switch in case where the SN connection via a specific cell is maintained.

Scene#0: The UE is configured with DC, where Cell#0 belonging to CU#0 is PCell and Cell#5 belonging to CU#5 is PSCell. The MN security key and the SN security key are KgNB#0 and S-KgNB#0, respectively. Cell#1 and Cell#2 belonging to CU#1 are configured with candidate cells for MCG LTM cell switch with keeping the SN connection via the current PSCell (i.e., Cell#5).

Scene#1: Inter-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#0 to Cell#1. Due to PCell change, the MN security is updated, i.e., the MN key is updated from KgNB#0 to KgNB#1. Subsequently, the SN security is updated based on KgNB#1 and sk-Counter included in the candidate cell configuration for Cell#1, where sk-Counter value is 1. Consequently, the SN security is updated to S-KgNB#1.

Scene#2: Intra-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#1 to Cell#2. Since CU for MCG (i.e., MN) is not changed due to PCell change, the MN security update is not performed, i.e., the MN key is maintained as KgNB#1. On the other hand, for the SN key, the SN security update is performed because the candidate cell configuration for Cell#2 includes sk-Counter. Consequently, the SN key is updated to S-KgNB#2 based on KgNB#1 and sk-Counter value 2. Since the MN security is not updated, the SN security update is redundant and unnecessary, which can result in data interruption at SCG due to the random access procedure for SN security update.

Based on the legacy Rel-15 RRC reconfiguration with sync procedure, whether to perform the SN security update during MN RRC reconfiguration (e.g., MN handover) does not depend on whether the MN security update is performed but rather on whether sk-Counter is provided. This means that the legacy Rel-15 RRC reconfiguration with sync procedure cannot prevent redundant SN security update during subsequent intra-CU MCG LTM cell switches. Hence, as described in scene#2, a redundant SN security update may be performed during subsequent intra-CU MCG cell switches.

Observation 1. During subsequent intra-CU MCG LTM cell switches, a redundant SN security update may be performed due to the provision of sk-Counter into candidate cell configurations.

Scene#3: Intra-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#2 to Cell#1. Since CU for MCG (i.e., MN) is not changed due to PCell change, the MN security update is not performed, i.e., the MN key is maintained as KgNB#1. On the other hand, for the SN key, the SN security update is performed because the candidate cell configuration for Cell#1 includes sk-Counter. Consequently, the SN key is updated to S-KgNB#1 based on KgNB#1 and sk-Counter value 1. Here, the SN key that were used in the past (i.e. S-KgNB#1) is re-used, which violates the security requirement that the security key that were used in the past should not be re-used. This violation can incur a severe security related problem, e.g., leakage of the SN key may occur due to repetitive intra-CU MCG LTM cell switches with keeping the existing SN connection.

Observation 2. During subsequent intra-CU MCG LTM cell switches, the SN key that were used in the past may be re-used.

It seems that the problem incurred by the observation 2 is a consequence of the problem incurred by the observation 1. This implies that a violation of security requirement due to duplicated SN security key use during subsequent intra-CU MCG LTM cell switches can be prevented by blocking redundant SN security update during subsequent intra-CU MCG LTM cell switches. To this end, an SN security update during subsequent MCG LTM cell switches with DC (i.e., SN is not changed) needs to be performed only if MN security update is performed during an MCG LTM cell switch. For example, the new Rel-19 RRC ID introduced in the last meeting (i.e., RAN2#127) can be used for UE to perform SN security update if MN security update is performed based on the new Rel-19 RRC ID. Other options given in [1] based approaches are also possible.

Proposal 1. SN security update during subsequent MCG LTM cell switches with DC (i.e., SN is not changed) is performed only if MN security update is performed during an MCG LTM cell switch.

Figure 2. Subsequent inter-CU LTM cell switches in case where the SN connection via a specific cell is maintained.

Figure 2 describes a scenario that subsequent inter-CU MCG LTM cell switches are performed in case where the SN connection via a specific cell is maintained.

Scene#0: the UE is configured with DC, where Cell#0 belonging to CU#0 is PCell and Cell#5 belonging to CU#5 is PSCell. The MN security key and the SN security key are KgNB#0 and S-KgNB#0, respectively. Cell#1 belonging to CU#1 and Cell#2 belonging to CU#2 are configured with candidate cells for MCG LTM cell switch with keeping the SN connection via the current PSCell (i.e., Cell#5).

Scene#1: Inter-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#0 to Cell#1. Due to PCell change, the MN security is updated, i.e., the MN key is updated from KgNB#0 to KgNB#1. Subsequently, the SN security is updated based on KgNB#1 and sk-Counter included in the candidate cell configuration for Cell#1, where sk-Counter value is 1. Consequently, the SN security is updated to S-KgNB#1.

Scene#2: Inter-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#1 to Cell#2. Due to PCell change, the MN security is updated, i.e., the MN key is updated from KgNB#1 to KgNB#2. Subsequently, the SN security is updated based on KgNB#2 and sk-Counter included in the candidate cell configuration for Cell#2, where sk-Counter value is 2. Consequently, the SN security is updated to S-KgNB#2.

Scene#3: Inter-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#2 to Cell#1. Due to PCell change, the MN security is updated, i.e., the MN key is updated from KgNB#2 to KgNB#3. Subsequently, the SN security is updated based on KgNB#3 and sk-Counter included in the candidate cell configuration for Cell#1, where sk-Counter value is 1. Consequently, the SN security is updated to S-KgNB#3. Here, sk-Counter value 1 is re-used for updating the SN key.

Observation 3. During subsequent inter-CU MCG LTM cell switches, the same sk-Counter value may be repetitively used.

In [2], there is the note saying that the SN Counter value (i.e., sk-Counter) input is never reused within or across DC and SCPAC (see the yellow-highlighted part below).

A.16 Derivation of KSN for dual connectivity This input string is used when the MN and UE derive KSN during dual connectivity. The following input parameters shall be used: - FC =0x79, - P0 = Value of the SN Counter as a non-negative integer, - L0 = length of the SN Counter value (i.e. 0x00 0x02). The input key KEY shall be KeNB when the MN is an ng-eNB and KgNB when the MN is a gNB. NOTE: The same input string is used for both DC and SCPAC. Therefore, it is crucial that the SN Counter value input is never reused within or across DC and SCPAC.

Based on the note, repeated use of the same sk-Counter during subsequent inter-CU MCG LTM cell switches may be problematic. For example, the repeated use of the same sk-Counter value may incur a security concern, e.g., limited randomization for generating a new security key by using the same sk-Counter value. In RAN2 perspective, it is difficult to judge whether or not it is problematic to repeatedly use the same sk-Counter during subsequent inter-CU MCG LTM cell switches, as it is outside the scope of RAN2. Therefore, RAN2 needs to consult with SA3 whether the repeated use of the same sk-Counter during subsequent inter-CU MCG LTM cell switches is problematic or not.

Proposal 2. Send LS to SA3 to ask whether or not it is problematic to repeatedly use the same sk-Counter during subsequent inter-CU MCG LTM cell switches.

3.	Conclusion

In this paper, we have made the following observations and proposals:

Observation 1. During subsequent intra-CU MCG LTM cell switches, a redundant SN security update may be performed due to the provision of sk-Counter into candidate cell configurations.

Observation 2. During subsequent intra-CU MCG LTM cell switches, the SN key that were used in the past may be re-used.

Proposal 1. SN security update during subsequent MCG LTM cell switches with DC (i.e., SN is not changed) is performed only if MN security update is performed during an MCG LTM cell switch.

Observation 3. During subsequent inter-CU MCG LTM cell switches, the same sk-Counter value may be repetitively used.

Proposal 2. Send LS to SA3 to ask whether or not it is problematic to repeatedly use the same sk-Counter during subsequent inter-CU MCG LTM cell switches.

4.	References

[1] R2-2403929, LS on security handling for inter-CU LTM in non-DC cases, Apple, 3GPP TSG RAN WG2#125bis, Changsha, China, 15th - 19th April 2024

[2] TS 33.501 Security architecture and procedures for 5G system (Release 18)

5.	Annex

5.1.	Agreements

5.1.1.	RAN1 agreements

RAN1#118:

Agreement

-       Support L1-RSRP measurement based on CSI-RS

-       FFS: Support L1-SINR measurement based on CSI-RS

Agreement

-        Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported

Agreement

-        CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting

-        FFS: CSI-RS based L1-SINR report is supported for gNB scheduled measurement reporting

-        Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting

Agreement

-        SSB based L1-RSRP measurements is supported for event triggered reporting

-        CSI-RS based L1-RSRP measurements is supported for event triggered reporting

-        FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting

Agreement

For gNB scheduled reporting and event triggered reporting

-        At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell

o   FFS: aperiodic and semi-persistent CSI-RS

-        At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell

o   FFS: CSI-RS for mobility

Agreement

-        For the identification of the serving cell RS for event evaluation,

o   At least the following options are further studied in RAN1, where different options could apply to different LTM event

§  Option. 1: Derived from QCL (type-D) RS(s) of the indicated joint/DL TCI state for the serving cell

§  Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell

·       QCL RS or SSB is configured by the network

§  Option. 3: Measurement RS(s) is/are explicitly configured

§  Option. 4: Derived from QCL RSs of activated TCI states with the best quality, or SSB which is QCLed with the QCL RSs of activated TCI states with the best quality.

§  Option 6: Derived from QCL RSs of activated TCI states, or SSB which is QCLed with the QCL RSs of activated TCI states

-        The RSs of the candidate cell(s) for event evaluation are explicitly configure

-        Note: Companies are encouraged to take into account the RAN2 agreement (i.e current beam rather than best beam) for their further study.

5.1.2.	RAN2 agreements

RAN2#125bis:

Agreements on scenarios:

- RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios.

- Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

- UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

Agreements on latency analysis:

- Mobility latency analysis of rel-18 intra-CU LTM is reused for Rel-19 inter-CU LTM.

Agreements on early sync phase:

- Early DL and UL sync is also supported for inter-CU LTM.  Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval.

- PDCCH ordered early RACH is supported for inter-CU LTM.

- For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option.

Agreements on LTM cell switch execution phase:

- Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

- FFS if there is an inter-CU LTM w/o security key change.

Agreements on measurements:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

RAN2#126:

Agreements on inter-CU LTM:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

- For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

- Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

- FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

- Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

- RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

- For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

RAN2#127:

Agreements on inter-CU LTM

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

- The source CU is responsible to generate the common CSI resource configuration.

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

Inter-CU LTM switch:

- For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch:

- As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

DRB/PDU session mismatch in subsequent inter-CU LTM:

- RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421.

LTM and L3 HO:

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

- MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

5.1.3.	RAN3 agreements

RAN3#123bis:

- Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

- Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

- Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

- Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

- Early data forwarding can be supported for inter-CU LTM.

RAN3#124:

- Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

- Introduce a new procedure on Xn to transfer the TA information.

- A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

- Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

- The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

- Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

- Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

- A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own.

RAN3#125:

- Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

- Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

- Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

- Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation.

- TP R3-244836 for TS38.300 is agreed.