3GPP TSG-RAN WG2 Meeting #127bis	                                      R2-2408610

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.7.3 RRM measurement gaps/restrictions related enhancements

Source:	NEC

Title:	Discussion on RRM measurement gaps/restrictions related enhancements

Document for:	Discussion, Decision

Introduction

One objective in the approved WID on XR (eXtended Reality) for NR Phase 3 (RP-240719) on RRM measurement gaps/restrictions related enhancements is

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc.). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

This contribution provides our views on the above objective，mainly focus on RAN2 impacts from solutions considered by RAN1.

Discussion

RAN1 started their discussion on enabling TX/RX for XR during RRM measurements from RAN1#116 meeting. To enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements, three alternatives, dynamic indication, semi-persistent and semi-static solutions were discussed in RAN1. To skip a particular gap/restriction, dynamic indication assumes to send an explicit indication by DCI whereas semi-persistent and semi-static solutions assume to send activation command via MAC CE or configure a pattern(s) via RRC in the RAN1 discussion. It is obviously that dynamic indication solution itself falls into RAN1 scope and has no RAN2 impact.

The latest RAN1#118 meeting agreed following work assumption [1] to select dynamic indication among three alternatives and sent an LS (R1-2407561) to RAN4.

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Final LS in R1-2407561.

Among three alternatives, dynamic indication, semi-persistent and semi-static solutions, the latest RAN1#118 meeting agreed the work assumption to select dynamic indication to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements.

In the current MAC specification, the handling of measurement gap of an activated measurement gap is specified in Clause 5.14. To ensure the MAC entity operating correctly, MAC layer needs to know the status of a measurement gap, i.e., either “activated” or “deactivated”. When an explicit indication by DCI to skip a particular gap/restriction in the dynamic indication solution is transmitted, it is reasonable to assume there would be a notification of the skipped measurement gap from the lower layer to MAC layer which can be left to implementation. With this notification, MAC entity is able to categorize the skipped measurement gas as a deactivated measurement. Then no RAN2 impact is foreseen for supporting RAN1 agreed work assumption.

RAN2 confirm that no RAN2 impact is foreseen for supporting RAN1 agreed work assumption, i.e., dynamic indication to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements.

Another issue discussed in RAN1 was whether to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. In details, the following UE assistance information were studied/discussed in RAN1:

•	FFS: UE assistance information related to measurement occasions;

•	FFS: UE assistance information related to channel conditions;

•	FFS: UE assistance information related to traffic.

•	FFS: UE assistance information related to UE mobility.it is need to support UE

The following conclusion was summarized in RAN1#117 meeting.

Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.

UE assistance information is a RRC layer procedure for the UE to inform the network of its desired configuration or assistance information. It may be beneficial for semi-persistent and semi-static solutions discussed in RAN1; however, no gain is foreseen for dynamic indication. Considering agreed RAN1 work assumption to select dynamic indication to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements, we think it is no need to discuss it again in RAN2.

RAN2 confirm that it is no need to discuss UE assistance information for supporting dynamic indication solution.

In the TR38.835 [2], following adaptation of MG period schemes are also evaluated in B.1.7 Scheduling restrictions due to RRM measurements：

Scheme 7.5.1: Adaptation of MG period. The MG period is adapted to be 2 times when PCell RSRP is 2dB better than the best neighbour cell and 4 times when PCell RSRP is 4dB better than the best neighbour cell. The UE speed is 30km/h.

-	Scheme 7.5.2 Adaptation of MG period. The MG period is adapted to be 4 times when PCell RSRP is 2dB better than the best neighbour cell and 8 times when PCell RSRP is 4dB better than the best neighbour cell. The UE speed is 30km/h.

Although simulation results in B.1.7 showed gains of adaptation of MG period schemes comparing to schemes with SMTC/measurement gap scheduling restrictions, similar performance gains can also be achieved by using explicit indication by DCI to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements. We do not see strong motivations to further discuss solutions for supporting adaptation of MG period.

RAN2 confirm that it is no need to further discuss solutions for supporting adaptation of MG period considering RAN1 agreed the work assumption to select dynamic indication to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements.

Summary

This contribution provides our views on the RRM measurement gaps/restrictions related enhancements objective, mainly focus on RAN2 impacts from solutions considered by RAN1.

Observation 1	Among three alternatives, dynamic indication, semi-persistent and semi-static solutions, the latest RAN1#118 meeting agreed the work assumption to select dynamic indication to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements.

- Proposal 1	 RAN2 confirm that no RAN2 impact is foreseen for supporting RAN1 agreed work assumption, i.e., dynamic indication to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements.

Proposal 2	RAN2 confirm that it is no need to discuss UE assistance information for supporting dynamic indication solution.

Proposal 3	RAN2 confirm that it is no need to further discuss solutions for supporting adaptation of MG period considering RAN1 agreed the work assumption to select dynamic indication to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements.

- References

[1] RAN1#118 meeting chair note

[2] TR38.835