3GPP TSG-RAN WG2 #127bis	R2-2408612

Hefei, China, 14 – 18 October 2024

Agenda item:		8.5.2	On-demand SSB SCell operation

Title:		Further discussion on On-demand SSB for SCell

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#127, the discussion on On-demand SSB for SCell has started and reached the following agreements with some open issues as FFS [1].

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

In addition, RAN2 has received the LS from RAN1 which captures the RAN1 agreements so far [2]. In this contribution, we discuss the open issues and provide our views.

# 2. Discussion

2.1	Signalling aspect

In the last meeting, RAN2 confirmed that both RRC-based and MAC-based signalling are supported for the OD-SSB transmission indication on top of the RAN1 agreements. For both ways, there are open issues as FFS and we discuss them below.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

RRC-based

The RRC signalling can be used at least to indicate initial state of OD-SSB configuration. Later (if necessary), the MAC CE can be used for the OD-SSB transmission indication. For the reconfiguration case, there are some divergent views among companies in the last meeting. Some companies who want to limit to the initial state only consider that if the network wants to deactivate via RRC siganlling, the network can directly de-configure the OD-SSB configuration, as it is anyway the same meaning as long as it is done by RRC reconfiguration. Some other companies who want to use RRC also for later state control consider that some network may perform activation/deactivation via RRC without supporting or using MAC CE. Both arguments would be valid and understandable.

Given that companies have different views, we assume RAN2 should decide which way to go by considering a generic policy as RAN2 does usually, i.e. to avoid functional duplication. In this case, the RRC signalling is necessary for the initial state control and the MAC CE is necessary for more dynamic control. As the MAC CE is already available, it would be sufficiently covering the case where the network wants to make it deactivated later on. It would be simpler and sufficient to support the RRC-based signalling only for the initial state control of OD-SSB configuration.

Proposal 1: RRC-based OD-SSB transmission indication is used to indicate only the initial activation/deactivation state of OD-SSB configuration.

MAC-based

For the MAC-based, it is FFS whether to introduce further optimization for the scenario 2A. As per the previous agreement, a new MAC CE to trigger the OD-SSB transmission is introduced at least intended for the scenario 2, and the legacy SCell activation/deactivation MAC CE will not be changed. For the scenario 2A, the network can send the new MAC CE and the legacy MAC CE in one signalling. We assume one octet or four octets MAC CE format for the new MAC CE without including SCell index. There is no issue for this approach basically.

Then, the open issue is whether any optimization is necessary for the scenario 2A. A possible optimization may be to combine the indication of OD-SSB (de)activation and the indication of the SCell (de)activation in one MAC CE. However, even if we do this optimization, gain will be to reduce 1 bye for one of the MAC sub-header. We still do not see need of optimization, unless any critical issue is found.

Proposal 2: RAN2 to confirm that an optimization is not supported for MAC-based OD-SSB transmission indication for the scenario 2A.

2.2	L3 RRM measurements

In the last meeting, it was agreed to study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration. As per the latest RAN1 agreements, at least the L1 measurement based on OD-SSB is supported with periodic, semi-persistent and aperiodic manner. In addition, current RAN1 agreements support the two types of OD-SSB:

For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

Based on this information, we assume that the network configures RRM measurement based on always-on CD-SSB for a UE. In addition, the network may configure RRM measurement based on OD-SSB. As the current RAN1 assumption does not include CD-SSB on sync-raster as OD-SSB, it would be suitable to assume additional measurement object is configured specific to the OD-SSB. From UE perspective, the UE performs the RRM measurements based on OD-SSB, only when the OD-SSB transmission is activated (as confirmation of the previous agreement). The network should ensure this condition.

Proposal 3: RAN2 to assume that the network configures a measurement object specific to the OD-SSB, which is not located on synchronization raster or is the NCD-SSB, for L3 RRM measurements.

One open issue is how the UE should handle the RRM measurements performed for different OD-SSB transmissions bursts, i.e. sets of RRM measurements results are obtained temporally separated. This is the RAN4 scope and RAN2 should wait for RAN4 progress on that. Thus, RAN2 should focus on the signalling aspects and expected UE behaviour based on the configurations for now.

Proposal 4: RAN2 to discuss signalling aspects for RRM measurements based on the OD-SSB (e.g. additional MO, start/stop of the measurements along with the OD-SSB transmission bursts, etc) and wait for RAN4 progress on detailed requirements.

# 3. Conclusion

In this contribution we discussed the signalling aspect of OD-SSB status indication and L3 RRM measurement. We reached the following proposals.

Proposal 1: RRC-based OD-SSB transmission indication is used to indicate only the initial activation/deactivation state of OD-SSB configuration.

Proposal 2: RAN2 to confirm that an optimization is not supported for MAC-based OD-SSB transmission indication for the scenario 2A.

Proposal 3: RAN2 to assume that the network configures a measurement object specific to the OD-SSB, which is not located on synchronization raster or is the NCD-SSB, for L3 RRM measurements.

Proposal 4: RAN2 to discuss signalling aspects for RRM measurements based on the OD-SSB (e.g. additional MO, start/stop of the measurements along with the OD-SSB transmission bursts, etc) and wait for RAN4 progress on detailed requirements.

# References

[1] R2-2407571, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung)

[2] R1-2407438=R2-2407914, LS to RAN2 for on-demand SSB SCell operation, RAN1

[3] RAN1#118, RAN1 Chair’s Notes

[4] RP-242354, Revised WID

# Annex for reference

RAN1#116 agreements

Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations. Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission Agreement Support on-demand SSB SCell operation triggered by gNB. - FFS Details of associated signaling/indication/configuration provided to UE Agreement - For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. - FFS: The combination of above options - FFS: How to define time instance A/B and the value of N per option - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

RAN1#116bis agreements

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB Agreement - For a cell supporting on-demand SSB SCell operation, - L1 and/or L3 measurement based on on-demand SSB is supported for the cell. - FFS further details on L1 and/or L3 measurement Agreement The following agreement from RAN1#116 is modified (in red) - For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. - FFS: The combination of above options - FFS: How to define time instance A/B and the value of N per option - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement) Agreement For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. - FFS: Details on On-demand SSB transmission indication

RAN1#117 agreements

Agreement (Signalling) For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement (Contents) (amended as shown in red in Friday session) For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused Agreement - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB. Agreement - For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - FFS: Whether the value of T is predefined or indicated/configured by gNB - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE. - Sub-carrier spacing of the on-demand SSB - Physical Cell ID of the on-demand SSB - Location of on-demand SSB burst - Downlink transmit power of on-demand SSB - FFS: Other parameters - FFS: Whether each of above parameters is configured/indicated explicitly or not

RAN1#118 agreements

Agreement - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster Agreement Support L3 measurement based on on-demand SSB - Further work on L3 measurement is up to RAN2/RAN4 R1-2407437 DRAFT LS to RAN2 for on-demand SSB SCell operation Moderator (LG Electronics) Agreement LS to RAN2 for on-demand SSB SCell operation is agreed. Final LS in R1-2407438. Agreement The previous RAN1 agreement made in RAN1#117 is revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - FFS: Whether the value of T is predefined or indicated/configured by gNB - (Working assumption) T=T_min - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement LS on timeline for On-demand SSB operation on SCell is agreed in R1-2407565.