3GPP TSG-RAN WG2 #127bis	R2-2408613

Hefei, China, 14 – 18 October 2024

Agenda item:		8.6.3	L1 event triggered measurement reporting

Title:		Details of L1 event triggered measurement reporting

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#126 and #127, there was good progress on the event triggered L1 measurement reporting [1][2]. In RAN2#127, there were important agreements on how to perform the L1 event evaluations:

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. 	MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

In this contribution, we discuss some more details on the L1 event evaluations and provide our views.

# 2. Discussion

2.1	Event evaluation

In the last meeting, it was agreed to use the current beam in the serving cell for event evaluation in L1 event triggered measurement reporting. We understood that one of the reasons why it is not the best beam is because unlike the cell level quality in the legacy mobility, L1 beam quality may not be so stable and can be changed more dynamically. So, keep using the current beam is simpler and easier to be managed.

Having assumed that, we see one open issue in TTT handling. Even though the current beam is used for event evaluation, the current beam could be changed relatively dynamically. For example, when the condition of the event (e.g. Event LTM2/3/5) is met for the first time, the UE is using the beam#1 of the serving cell. During the TTT, the current beam may be changed to another beam#2. A question is how to continue event evaluation for the serving cell in this case.

One way is to restart the event evaluation with resetting the TTT. This is simple and probably workable. However, if the current beam change happens relatively dynamically, the TTT may not be expired as expected and thus the beam failure or RLF may be declared before triggering LTM cell switch. To avoid unexpected cases, it is worth considering keeping TTT even after the current beam change as long as the new current beam also fulfils the event condition. Otherwise, the TTT should be stopped.

Proposal 1: For serving cell, the UE continues the TTT, if new current beam also fulfils the event condition when the current beam is changed.

For candidate cell, it was agreed to use any beam in candidate RS configuration. We understand that the event evaluation for e.g. Event LTM3 is performed for a pair of the current beam of the serving cell and each beam in the candidate RS configuration of the candidate cell. In this case, the TTT evaluation is also performed individually. Given that there are multiple beams per candidate cell, it would be better to consider a certain UE complexity reduction in event evaluation. For example, only best M beams of each candidate cell are to be the target of the event evaluation. If one or more beams among best M beams are replaced by other new beams, the event evaluation should be reset and restarted for those new beams.

For the Event LTM4/5, the UE compares the individual beams with absolute threshold. Similar to the Event LTM3, in order to reduce UE complexity, best M beams can be considered.

Proposal 2: RAN2 to discuss introducing an optimization to reduce UE complexity such that the UE uses only best M beams of the candidate cell for LTM event evaluation, where M is configured by the network explicitly.

2.2	Event triggered reporting

On top of the progress so far, there is one more aspect to be discussed. It is about need of event-triggered periodic L1 measurement reporting. The L3 measurement report is per periodic, event-triggered (one shot) and event-triggered periodic. We do not see any big difference between L3 based and L1 based with respect to the usefulness of the event-triggered periodic. We suggest supporting it also for L1 based measurements.

Proposal 3: RAN2 to support the event-triggered periodic L1 measurement reporting for LTM.

# 3. Conclusion

In this contribution we discussed some details on the L1 event evaluation and made the following proposals.

Proposal 1: For serving cell, the UE continues the TTT, if new current beam also fulfils the event condition when the current beam is changed.

Proposal 2: RAN2 to discuss introducing an optimization to reduce UE complexity such that the UE uses only best M beams of the candidate cell for LTM event evaluation, where M is configured by the network explicitly.

Proposal 3: RAN2 to support the event-triggered periodic L1 measurement reporting for LTM.

# References

[1] R2-2405701, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung)

[2] R2-2407571, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung)