3GPP TSG-RAN WG2 Meeting #127bis													                  R2-2408622

Hefei, China, Oct 14th – 18th, 2024                                                           Revision of R2-2406821

Agenda item:	8.9.2

Source:	MediaTek Inc.

Title:	RAN2 impact on S&F mode

Document for:	Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#105 [1], one of the objectives is to support S&F operation, as below:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

A study report, 23.700-29 [2], on the integration of satellite components in the 5G architecture Phase 3 has been released by 3GPP SA2. The conclusion for Key Issue #2: Support of Store and Forward Satellite operation has been made. Two types of system architecture can be adopted by operation deployment: a full CN onboard the satellite or a Split MME architecture.

In this contribution, we share our view on S&F mode, incorporating input from SA2.

# Discussion

S&F indication

In RAN2#125bis, it was agreed that IoT NTN network should be able to inform UE whether S&F Satellite operation is applied. Furthermore, in RAN2#126, it was agreed that the S&F indication can be provide by SIB, and no NAS indication is needed. In RAN2#127, the bit was defined to indicate the current feeder link status (i.e. whether the satellite is currently operating in S&F mode). Whether a static indication in general that the network supports the feature is FFS.

RAN2#125bis Agreement:

- An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this)

RAN2#126 Agreement:

- S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

RAN2#127 Agreement

- UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

An IoT NTN UE may mostly focus on delay-tolerant services. However, recently, the voice call has been gaining more interest in the industry. It will probably be within the scope of 3GPP Rel-20. The voice call requires a delay-sensitive transmission, which a cell in S&F mode cannot not provide. More assistance information on the feeder linker status can help in making decisions about the initiation of delay-sensitive transmissions.

A static indication in general that the network supports the feature can be used in the cell selection/reselection for R19 UEs. With this knowledge, a Rel-19 UE can select a preferred non-S&F cell during the cell selection or reselection if it may have a delay sensitive transmission, such as a voice call. Since the service type is out of AS scope, the function can be up to UE implementation.

Proposal 1: Introduce a static indication in system information indicating whether the cell support the S&F operation (different from whether it is currently operating in S&F mode).

In the RAN2#127, remaining time of the satellite’s current operation mode was discussed, but no agreement was reached.

Proposal 6 UE is informed of the remaining time of the satellite’s current operation mode (normal or S&F) via System Information broadcast. - Ericsson thinks this could be realized with the introduction of a t-service like parameter - QC is not sure how this would be used. Samsung agrees the UE behaviour is not clear. CATT agrees - ZTE is interested in this proposal - Sequans thinks we need an absolute time. Ericsson confirms this is the intention - IDC thinks this could be useful - Telit thinks that if the timer is shorter than t-service this would rather bring additional problems to the NW/system behaviour - Google thinks an indication could be beneficial also for R19 UUs not supporting S&F operation. Come back to this in the next meeting

It can also be beneficial in many aspects if the UE knows when the cell will gain or lose the feeder link. The device user can be notified of how long the feeder link will be available, allowing them to take the opportunity to start delay-sensitive transmissions. Alternatively, the device user can be aware of when the feeder link will become available, enabling them to catch the gap when the cell can provide real-time services. How to use this information is also up to the UE implementation.

Proposal 2: The times when the feeder link will become available or unavailable can be broadcast in an S&F operation-enabled cell via system information.

S&F barring bit

In terms of barring bit for legacy UE, RAN2 has made the following agreement:

RAN2#126 agreement:

- RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

RAN2#127 agreement:

- RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

- RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

According to the RAN2 agreement, the legacy cellBarred and cellBarred-NTN bits will be used to bar legacy UEs. Rel-19 UEs that support S&F mode should not be subject to the legacy barring bits.

Proposal 3: Rel-19 UEs that support S&F mode should ignore the legacy cellBarred and cellBarred-NTN bits.

Similar to the legacy barring bits, a new barring bit is needed to prevent UEs that support S&F mode from accessing the cell for the load balancing purpose.

Proposal 4: Introduce a new barring bit in the system information to bar the S&F mode enabled UEs.

The legacy barring bits in ASN.1 in 36.331 is shown as following:

SystemInformationBlockType1-NB message -- ASN1START SystemInformationBlockType1-NB ::= SEQUENCE { hyperSFN-MSB-r13 BIT STRING (SIZE (8)), cellAccessRelatedInfo-r13 SEQUENCE { plmn-IdentityList-r13 PLMN-IdentityList-NB-r13, trackingAreaCode-r13 TrackingAreaCode, cellIdentity-r13 CellIdentity, cellBarred-r13 ENUMERATED {barred, notBarred}, intraFreqReselection-r13 ENUMERATED {allowed, notAllowed} }, --omit— SystemInformationBlockType1-NB-v1700 ::= SEQUENCE { cellAccessRelatedInfo-NTN-r17 SEQUENCE { cellBarred-NTN-r17 ENUMERATED {barred, notBarred}, plmn-IdentityList-v1700 PLMN-IdentityList-NB-v1700 OPTIONAL -- Need OR } OPTIONAL, -- Need OR nonCriticalExtension SEQUENCE {} OPTIONAL } --omit—

cellBarred-NTN Barred means the cell is barred for connectivity to NTN, as defined in TS 36.304 [4]. E-UTRAN always includes cellBarred-NTN and sets cellBarred to 'barred' in an NTN cell.

We can see that when the bits are set to notBarred, the field cellBarred-NTN-r17 is still present. E-UTRAN always includes this field in an NTN cell. To align with legacy design, E-UTRAN should always include the new barring bit in an S&F mode enabled cell.

Additionally, follow the discussion in proposal 1, the present of this field could be used as an indication on whether the cell supports S&F mode.

Proposal 5: E-UTRAN should always include the new barring bit in an S&F mode enabled cell. This present of this field is used as an indication on whether the cell supports S&F operation.

Cell selection with satellite ID

In the study report [2], for the split MME architecture, a wait time and a list of satellite IDs will be provided to UE to re-attempt the Attach/TAU procedure.

23.700-29 v1.0.0 clause 8.2 b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

For the full CN onboard the satellite, a S&F monitoring list of satellites IDs will be optionally provided to the UE during the attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. Specifically, the S&F monitoring list may assist the UE in retrieving MT data.

23.700-29 v1.0.0 clause 8.2 - Optionally the MME provides the UE with a S&F monitoring list of satellites IDs, during attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. The S&F monitoring list can be determination by the CN. How network determines the S&F monitoring list is outside the scope of 3GPP in this release of specification. NOTE 8: The S&F monitoring list may assist the UE in retrieving MT data.

To support these functions, NAS can indicate a list of satellite IDs to AS during the PLMN selection and cell selection. Currently, SIB31, SIB32, and SIB33 contain satellite IDs. SIB32 includes a frequency list for a given satellite ID, and the corresponding frequency of the satellite ID in SIB33 can be found in SIB3 or SIB5. The frequency of satellite in SIB31 is the frequency of the serving cell. The UE can determine the frequency of the cell from the satellite ID. When NAS indicates the satellite ID to AS, the UE can first find the corresponding cell frequency from the SIBs and search the cell on that frequency. If a cell is found on that frequency, the UE may check the satellite ID to see if it matches the one requested by NAS.

Alternatively, NAS can request the UE to report the satellite ID of the found cell during the PLMN selection and cell selection.

The wait time can be used in NAS to trigger the PLMN selection or cell selection.

Proposal 6: NAS can indicate a list of satellite IDs to AS during the PLMN selection and cell selection. The UE AS will attempt to camp on the cell provided by satellite IDs.

Proposal 7: NAS can request UE to report the satellite ID of found cell during the PLMN selection and cell selection.

Proposal 8: Satellite IDs in SIB31, SIB32, and SIB33 can be reused for providing the mapping between satellite ID and frequency information.

To support cell search for a specific satellite when the UE is not camping on a cell, the UE may store all the known satellite ID and frequency mapping information for each PLMN.

Proposal 9: The UE stores satellite ID and frequency mapping information for each PLMN.

# Conclusion

S&F indication

Proposal 1: Introduce a static indication in system information indicating whether the cell support the S&F operation (different from whether it is currently operating in S&F mode).

Proposal 2: The times when the feeder link will become available or unavailable can be broadcast in an S&F operation-enabled cell via system information.

S&F barring bit

Proposal 3: Rel-19 UEs that support S&F mode should ignore the legacy cellBarred and cellBarred-NTN bits.

Proposal 4: Introduce a new barring bit in the system information to bar the S&F mode enabled UEs.

Proposal 5: E-UTRAN should always include the new barring bit in an S&F mode enabled cell. This present of this field is used as an indication on whether the cell supports S&F operation.

Cell selection with satellite ID

Proposal 6: NAS can indicate a list of satellite IDs to AS during the PLMN selection and cell selection. The UE AS will attempt to camp on the cell provided by satellite IDs.

Proposal 7: NAS can request UE to report the satellite ID of found cell during the PLMN selection and cell selection.

Proposal 8: Satellite IDs in SIB31, SIB32, and SIB33 can be reused for providing the mapping between satellite ID and frequency information.

Proposal 9: The UE stores satellite ID and frequency mapping information for each PLMN.

# References

RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)

TR 23.700-29 v.1.0.0. Study on integration of satellite components in the 5G architecture Phase 3