3GPP TSG-RAN WG2 Meeting #127bis      											                     R2-2408623

Hefei, China, Oct 14th – 18th, 2024                                                           Revision of R2-2406869

Agenda item:	8.9.3

Source:	MediaTek Inc.

Title:	Discussion on enhanced EDT

Document for:	Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#105 [1] and one of the objectives is to enhance EDT procedure:

- Support of Capacity enhancements for uplink

  - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

    - Msg3 transmission without msg1/ Random Access Response (RAR)

    - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In this contribution, we share our view on this enhanced EDT objective.

# Discussion

Details of Procedure

RAN2 agreed that the enhanced EDT will be a contention-based and EDT-like transaction. And before the enhanced EDT, the UE needs to verify/update the uplink synchronization. The procedural steps are FFS.

RAN2#126 Agreements: 1. RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). 2. If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

Self-evidently, the first step should be the acquisition of the satellite ephemeris and GNSS position to get the UL TA. Then the next step will be a contention based Msg3, followed by a Msg4 containing contention resolution information, DL RRC response message (optional), and (optional) DL user data.

Proposal 1: The steps of the enhanced EDT are as follows:

- Step 1: Acquire SIB31 and GNSS position.

- Step 2: A contention based Msg3.

- Step 3: A Msg4 contains contention resolution (+ DL response message) (+ DL user data).

Similar to legacy EDT, enhanced EDT can fallback to legacy connection establishment if there is more DL data to be transmitted.

Proposal 2: Enhanced EDT can fallback to legacy RRC connection establishment.

TA issue

RAN2 has asked RAN4 and RAN1 the following question in the LS [2] on UL synchronization for contention based Msg3 transmission without Msg1/Msg2:

Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2?

RAN4 and RAN1 replied in LS [3] and LS [4].

RAN1’s response: From RAN1 perspective, assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases: • NB-IoT with 3.75kHz SCS. • eMTC CE mode A. Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2 Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively. Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B

RAN4’s response: For Msg3 transmission without Msg1/Msg2, UE can meet UE UL transmit timing requirements defined for IoT NTN in TS 36.133[attached in Annex] from UE RRM requirement definition point of view. Note 1: RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2. RAN4 will follow the conclusion of the other working groups in the future, if any. Note 2: For NB-IoT with 15kHz SCS, the timing error may be larger than half CP.

Both RAN1 and RAN4 consider there will be no N_TA configured by the network, and the N_TA value should be considered as 0.

Proposal 3: The reference N_TA is not configured by the network for the Msg3 transmission without Msg1/Msg2.

However, we would like to introduce a new optional cell-specific TA adjustment to be compatible for the network-implemented TA margin solution. In this solution, to counter possible UE overcompensation, the network may configure a positive value in the TAC in RAR and subtract the same value from common TA. When the network needs to address the UE overcompensation, it simply decreases the positive value in the TAC in RAR for a specific UE. Without this solution, the network would have to put a negative value in the TAC in RAR which is not allowed in the current LTE/NR specification.

For enhanced EDT without Msg1/RAR, if this network-implemented TA margin method is still adopted, it may cause some problem to the UEs transmitting Msg3 without Msg1/Msg2. The common TA has been reduced by a certain value for the existing legacy UEs, it would be wrong for the new UEs doing TA pre-compensation based on the modified common TA value. We propose to add a cell-specific TA adjustment for the UEs transmitting Msg3 without RAR, so that the TA calculation would be remain correct.

A similar discussion occurred in the NR NTN support of regenerative payload on whether to introduce a negative value for common TA, where the common TA is supposed to be 0 so that it couldn’t be reduced any further. See the LS [5] for reference.

Proposal 4: Introduce a new optional cell-specific TA adjustment for the UEs transmitting Msg3 without Msg1/Msg2.

Details of Msg3

The aim of the Msg3 of the EDT is to convey the UL user data. Without Msg1 and Msg2, the difference will be the lack of the TAC, UL grant, and TC-RNTI.

The TAC may not be necessary. The UE can calculate a precise UE specific TA based on the latest satellite ephemeris and GNSS position. The accuracy should be sufficient without a UE specific network adjustment (TAC). Anyway, it is still subject to RAN1/RAN4 discussion. We can wait for the response to the outgoing LS R2-2405769.

The UL grant for Msg3 can be provided via system information or dedicated signalling. The difference is that dedicated signalling allows the network to control the number of UEs that use the resource, i.e., the PUSCH resource could be UE specific or a grouped UE specific. For the system information option, although the resources can be divided into groups to reduce the number of UEs using the shared resource, the total number cannot be controlled.

It can be complicated and overhead consuming to control the number of UEs for the network, but it can also help to reduce the collision chance. Without the RACH procedure, UE is going to send Msg3 on PUSCH directly, which increases the cost of UE collisions on shared resource. That is what we are trying to avoid.

On the other hand, the benefit of Msg3 PUSCH resource via system information is that UE can initiate enhanced EDT without having established a connection with the cell. For example, UE can initiate enhanced EDT right after a cell reselection.

Proposal 5: RAN2 to discuss the Msg3 PUSCH resource for enhanced EDT is provided via system information or RRC dedicated signalling.

If the Msg3 PUSCH resource for enhanced EDT provided via system information, the TC-RNTI should be derived by the index of resources. If Msg3 PUSCH resource for enhanced EDT provided via RRC dedicated signalling, the TC-RNTI can be assigned by network, or derived by the index of resources.

Proposal 6: RAN2 to discuss how to derive the TC-RNTI.

Regarding the signalling, we think the legacy RRC signalling can be reused.

Proposal 7: For CP solution, legacy RRCEarlyDataRequest(-NB) can be used to perform enhanced EDT as Msg3.

Proposal 8: For UP solution, legacy RRCConnectionResumeRequest(-NB) can be used to perform enhanced EDT as Msg3.

Details of Msg4

Because the Msg3 is contention based, contention must be resolved in Msg4. Hence, a contention resolution MAC CE should be included in the Msg4. The details of contention resolution MAC CE can be further discussed.

Proposal 9: A contention resolution MAC CE must be included in the Msg4 for enhanced EDT procedure.

Since the contention resolution is a must and cannot be done in DCI due to the size limitation, using the L1 ACK as the Msg4 is not suitable.

Proposal 10: Do not support using L1 ACK as the Msg4 for enhanced EDT procedure.

Regarding the RRC signalling, we think the legacy RRC signalling can be reused.

Proposal 11: For CP solution, legacy RRCEarlyDataComplete(-NB) can be used to perform enhanced EDT as Msg4. For UP solution, legacy RRCConnectionRelease(-NB) can be used to perform enhanced EDT as Msg4. For EDT fallback to legacy RRC connection establishment, the legacy RRCConnectionSetup(-NB) and RRCConnectionResume(-NB) can be used as Msg4.

When there is no DL response, there will be only a contention resolution MAC CE in the Msg4. There is enough room for other information. The signalling overhead can be improved. As one of the work item objectives, to improve the efficiency of Msg4, multiple Msg4s for different UEs can be included in one MAC PDU. The UEs aggregated in one MAC PDU can be from different Msg3 transmissions. The details can be further discussed.

Proposal 12: Multiple Msg4s for different UEs can be included in one MAC PDU.

DSA and CRDSA

In the previous RAN2 meeting, companies bring new methods for Msg3 transmission.

RAN2#126 agreements: 2. RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) RAN2#127 agreements: 1. RAN2 will continue working on a CB-msg3 EDT-like mechanism 2. RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts 3. In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1. 4. For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

Integration DSA and CR-DSA with repetition

UE needs to transmit multiple replicas in DSA and CR-DSA. Can the repetitions be evenly split among each replica?  If they are split and one replica collides, network cannot combine the repetitions, and will probably fail to successfully decode the packet. Hence, each replica should contain all the repetitions to meet the required SNR.

Observation 1: If DSA or CR-DSA is supported, each replica of DSA or CR-DSA should contain all the repetitions.

The purpose of the new methods, known as DSA and CR-DSA, is to reduce the collision probability and improve the transmission robustness.

The following simulation results (Figure 1) show the packet loss ratio based on Num of UEs per resource for SA, DSA, and CR-DSA.

The evaluation assumptions are:

- Number of transmissions K = 2

- Number of EDT occasions N_opp = 2

- Number of subcarriers N_sc = 48

- Number of stages in cancellation for CR-DSA N_ite = 4

Figure 1 packet loss ratio

For DSA, we can divide the diagram into three parts, as shown in Figure 2.

Figure 2 packet loss ratio distribution for DSA

In part 1, where the Num of UEs per resource is below 0.2, the packet loss ratio of DSA is lower than that of SA. However, the packet loss ratio of SA in part 1 is already low enough, so a lower value does bring significant difference.

In part 2, where the Num of UEs per resource is between 0.2 and 0.3, the packet loss ratio of DSA is lower than that of SA. Since the packet loss ratio of SA in part 2 is not quite low, the DSA shows an advantage over SA.

In part 3, where the Num of UEs per resource is greater than 0.3, the packet loss ratio of DSA is either greater than or similar to that of SA, so DSA does not show an advantage over SA.

We can see that the range where the DSA is better than the SA is quite limited, and DSA requires double transmission, which increase the power consumption on UE. It also brings extra specification changes. Therefore, we do not think DSA should be adopted.

Proposal 13: DSA is not adopted for Msg3 transmission without Msg1/Msg2.

For CR-DSA, we can also divide the diagram into three parts, as shown in Figure 3.

Figure 3 packet loss ratio distribution for CR-DSA

In part 1, where the Num of UEs per resource is below 0.2, packet loss ratio of CR-DSA is lower than that of SA. However, the packet loss ratio of SA in part 1 is already low enough, so a lower value does bring significant difference.

In part 2, where the Num of UEs per resource is between 0.2 and 0.9, the packet loss ratio of CR-DSA is lower than that of SA. Since the packet loss ratio of SA in part 2 is not quite low, the CR-DSA shows an advantage over SA.

In part 3, where the Num of UEs per resource is greater than 0.9, the packet loss ratio of CR-DSA is either greater than or similar to that of SA, so the CR-DSA does not show an advantage over SA.

The area where shows an advantage of CR-DSA over SA is broader than that of DSA.

We conducted an evaluation of the Num of UEs per resource, as shown in Table 1. The parameters such as UE density, cell coverage, and call rate per hour per UE are derived from the TR 36.763. The resource configuration of GSO (i.e., TBS, RU number, repetition number) is selected based on real field log. The resource configuration of LEO is inferred and is expected to be less than that of GSO. The EDT resource to total UL resource ratio is presumed to be 0.3.

Table 1 Num of UEs per resource evaluation

| Parameters | GEO/20 | GEO/40 | LEO/20 | LEO/400 |
| --- | --- | --- | --- | --- |
| UE density (UE/km2) | 20 | 400 | 20 | 400 |
| Cell coverage (radius, km) | 250 | 250 | 100 | 100 |
| UEs per cell | 3925000 | 78500000 | 628000 | 12560000 |
| call rate per hour per UE | 0.46 | 0.46 | 0.46 | 0.46 |
| EDT attempts per 1000ms | 501.5277778 | 10030.55556 | 80.24444444 | 1604.888889 |
| TBS | 144bits | 144bit | 144bit | 144bit |
| N_UL_slots (ms) | 32 | 32 | 32 | 32 |
| RU number | 3 | 3 | 2 | 2 |
| Repetition number | 2 | 2 | 1 | 1 |
| 3.75/15Khz | 3.75K | 3.75K | 3.75K | 3.75K |
| Single-Tone/Multi-tone | Single-Tone | Single-Tone | Single-Tone | Single-Tone |
| UL resources on frequency domain | 48 | 48 | 48 | 48 |
| EDT resource / all UL resource | 0.3 | 0.3 | 0.3 | 0.3 |
| allowed EDT attempts per 1000ms | 75 | 75 | 225 | 225 |
| Num of UEs / Resource | 6.687037037 | 133.7407407 | 0.356641975 | 7.132839506 |

However, according to the above table, the Num of UEs per resource is either much large than 1 or a small value where the packet loss ratio of SA is low. In real deployment, whether the CR-DSA is more beneficial than SA depends on multiple factors. It could be very difficult to keep the Num of UEs per resource in a certain range to let CR-DSA out-performing. Don’t forget the cost of CR-DSA. It also requires double transmission (or more) than SA, which increases the power consumption on UE. The network needs to perform the interference cancelling and buffer all the UL data for a certain time for that. The complexity on the network side is significant.

Furthermore, CR-DSA involves considerable RAN1 impacts as agreed in RAN2#127. It may need an extra TU budget in RAN1 to discuss this topic, given this feature is not within the scope of RAN1 according to the WID.

Therefore, we think CR-DSA should not be adopted, at least in this Rel-19.

Proposal 14: CR-DSA on Msg3 transmission without Msg1/Msg2 is not supported in Rel-19.

# Conclusion

Details of Procedure

Proposal 1: The steps of the enhanced EDT are as follows:

- Step 1: Acquire SIB31 and GNSS position.

- Step 2: A contention based Msg3.

- Step 3: A Msg4 contains contention resolution (+ DL response message) (+ DL user data).

Proposal 2: Enhanced EDT can fallback to legacy RRC connection establishment.

TA Issue

Proposal 3: The reference N_TA is not configured by the network for the Msg3 transmission without Msg1/Msg2.

Proposal 4: Introduce a new optional cell-specific TA adjustment for the UEs transmitting Msg3 without Msg1/Msg2.

Details of Msg3

Proposal 5: RAN2 to discuss the Msg3 PUSCH resource for enhanced EDT is provided via system information or RRC dedicated signalling.

Proposal 6: RAN2 to discuss how to derive the TC-RNTI.

Proposal 7: For CP solution, legacy RRCEarlyDataRequest(-NB) can be used to perform enhanced EDT as Msg3.

Proposal 8: For UP solution, legacy RRCConnectionResumeRequest(-NB) can be used to perform enhanced EDT as Msg3.

Details of Msg4

Proposal 9: A contention resolution MAC CE must be included in the Msg4 for enhanced EDT procedure.

Proposal 10: Do not support using L1 ACK as the Msg4 for enhanced EDT procedure.

Proposal 11: For CP solution, legacy RRCEarlyDataComplete(-NB) can be used to perform enhanced EDT as Msg4. For UP solution, legacy RRCConnectionRelease(-NB) can be used to perform enhanced EDT as Msg4. For EDT fallback to legacy RRC connection establishment, the legacy RRCConnectionSetup(-NB) and RRCConnectionResume(-NB) can be used as Msg4.

Proposal 12: Multiple Msg4s for different UEs can be included in one MAC PDU.

DSA and CRDSA

Observation 1: If DSA or CR-DSA is supported, each replica of DSA or CR-DSA should contain all the repetitions.

Proposal 13: DSA is not adopted for Msg3 transmission without Msg1/Msg2.

Proposal 14: CR-DSA on Msg3 transmission without Msg1/Msg2 is not supported in Rel-19.

# References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)

[2] R2-2405769, “LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, RAN2

[3] R4-2414114, “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, RAN4

[4] R1- 2407548, “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, RAN1

[5] R2-2407623, “LS on common TA in a regenerative payload scenario”, RAN2