# 3GPP TSG-RAN2 Meeting #127bis	R2-2408625

# Hefei, China, Oct 14-18, 2024

Agenda Item:	8.2.3

Study Item:	FS_Ambient_IoT_Solutions

Source: 	Qualcomm Incorporated

Title: 	Ambient IoT Paging

Document for:	Discussion/Decision

# Introduction

RAN#103 approved SI for Ambient IoT in [1]. RAN2#125bis discussed on paging aspects of AIoT and agreed the following:

Agreements

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

- RAN2 assumes that the device will not support tracking/RAN area update procedure.

- For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message.    FFS pending the details from SA2

RAN2#125bis discussed on stage-2/general aspects of AIOT and agreed the following:

Agreements - Unless explicitly stated all agreements apply to all device types and for both topologies. - From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2. - RAN2 will support two use cases, “inventory” and “command”. The definition, detailed wording is FFS - Baseline procedure: Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS Step C: The device may perform the data communication with the reader as needed,: Details FFS - We will study the support of both “inventory” and “command” in the same procedure. - FFS if Initial Trigger Message can also include “command”. - RAN2 will continue the study of ambient IoT assuming no support of AS security until SA3 provides further input.

RAN2#126 agreed the following:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

RAN2#127 had the following agreements on AIoT Paging:

Agreements:

- Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

This document provides more details about paging aspects for Ambient IoT, which are applicable to both topologies and all device types.

# Discussion

## Contents of initial trigger message (aka AIoT paging)

### Filter Criteria

In the inventory or command procedure, the trigger message may optionally include the filter criteria sent by the upper layer to identify the AIoT devices that are supposed to respond to the inventory request or command message. The Reader forwards the filter criteria in the inventory signaling to the AIoT devices. AIoT device is required to check the conditions whether the criteria are satisfied or not before initiating the AIoT access.

For the AIoT access, the AIoT device should check the filter criteria provided by Reader before performing the AIoT access. Only if the criteria are satisfied, the AIoT devices are allowed to perform the AIoT access to the Reader. What information are included in the filter criteria can be up to SA2 for further study.

Reader may include filter criteria (to identify the AIoT devices that are supposed to respond) in the Initial Trigger message (aka AIoT paging) sent towards the AIoT device(s). It is up to SA2 to define the details of the filter criteria.

RAN2 previously agreed that the AIoT paging message may or may not contain any device ID, and in the latter case, it is considered to be addressed for all AIoT devices within the coverage of the reader that can receive the AIoT message. If filter criteria is introduced, RAN2 should amend the previous agreement also considering the (absence of) filter criteria.

If neither a filter criterion nor a device ID is included in the initial trigger message, the message is considered as addressed for all devices that can receive the AIoT message.

### 2-step vs 4-step

In the past RAN2 meetings, the AIoT ‘4-step’ or 2-step random access was preliminary discussed. Since AIoT access is triggered by Reader, the Reader is responsible for providing the access resource considering the different service types, i.e., inventory request or command message. Reader may have preference on which type of AIoT access to be triggered to respond the service request.

For example, if Reader would like to only issue the ‘Write’ command to a small number of AIoT devices, a 2-step AIoT access can be selected for AIoT devices to feed back the simple confirmation after the command execution. In another example, Reader may indicate AIoT devices to perform so called ‘4-step’ AIoT access for the case that a large number of AIoT devices perform AIoT access by using the contention-based resource. Therefore, Reader should be able to indicate the type of AIoT access to AIoT devices in the trigger message.

Reader may indicate the type of AIoT access (e.g., 2-step or 4-step) to AIoT devices in the initial trigger message (aka AIoT paging).

### Subsequent R2D message

RAN2#126 discussed the so-called ‘Msg4’. RAN2 agreed to not use the term ‘msg4’ for further discussion of the AIoT random access. Instead, it could be called subsequent R2D message after D2R transmission. Such message may be a failure indication for Msg3 transmission or could be the acknowledgement like the Msg4 of NR random access.

While some scenarios would need the subsequent R2D message (e.g. for acknowledging the reception of the AIoT device ID and/or AIoT upper layer data sent in the previous D2R message or for triggering the next round of AIoT re-access or AIoT data communication), in other scenarios there would be no such subsequent R2D message (e.g. Reader requesting information from temperature sensors and no intention to ACK/NACK successful reception).

It is beneficial for AIoT device to have correct expectation on potential future steps after transmitting a D2R communication in response to the initial trigger/AIoT paging. To facilitate the AIoT device actions, the initial trigger message can indicate to the AIoT device whether a subsequent R2D message would be present.

The initial trigger message indicates the presence and information carried in the subsequent R2D message.

  - Multiple/Subsequent paging messages

In the previous meeting, RAN2 agreed to support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages associated with the same request from the CN.

In our view, how this is achieved is up to the reader implementation and deployment. Furthermore, in terms of design and aiming at reducing complexity, we think the subsequent AIoT trigger/paging message would be no different from the initial trigger message. Of course, what is actually signalled in the initial trigger vs the subsequent trigger can be different.

In terms of design, subsequent trigger/paging message is not different from initial trigger message.

Design of initial trigger message is reused for subsequent trigger message, i.e. a separate message is not introduced for subsequent trigger/paging message.

There were also proposals to introduce new messages indicating access occasion announcement messages. We do not see a need of such new messages. In our view, the resource allocation including whether there is one or multiple resource opportunities should be indicated by the initial trigger message.

The resource allocation including whether there is one or multiple resource opportunities should be indicated by the initial trigger message

Separate ‘access occasion announcement message’ is not needed.

  - On FFS if Initial Trigger Message can also include “command”

As shown in section 1, there is an FFS from RAN2#125bis.

5	We will study the support of both “inventory” and “command” in the same procedure.

6	FFS if Initial Trigger Message can also include “command”.

In our view, there may be valid use cases where the initial trigger message also includes command. E.g. suppose the AIoT application has some idea about the AIoT devices in a given area (e.g. based on deployment information within a warehouse) but does not have most up to date inventory information. In such case, the application may want to initiate a procedure which involves inventorying all the devices that satisfy certain criteria (e.g. all devices which are temperature sensors and belonging to booth # X). One might see the benefit of having ability to send a single message in the R2D link to indicate to the sensors (satisfying the criteria) to reply with their temperature values in the D2R. In such scenario, the device ID may be absent in the D2R message, which means including the device ID in D2R response should be optional. When to include it and when to leave it out should be indicated by the Reader in the initial trigger.

However, there may be some challenges to this. Firstly, it depends on how many devices would satisfy the criteria. It would be difficult if the reader does not have a good idea without prior inventory. However, in some use cases based on deployment information, the reader might already know how many devices may possibly respond to the initial trigger. Depending on the number of AIoT devices that need to respond, the reader can estimate the resources needed for the D2R link, e.g. whether the devices have to go through contention based random access procedure or whether they can be provided with contention free resources. In case of contention-based RA, the amount of potential collisions also depends on the potential number of devices that get triggered.

Therefore, in our view, it is ok to support both ‘inventory’ and ‘command’ in the same ‘request’ from the application to the Reader. In addition, in the air interface between the reader and the device, for certain use cases, using a single message for both ‘initial trigger’ and ‘command’ can also be supported.

As previously agreed, supporting both ‘inventory’ and ‘command’ based on a single request from application can be achieved as an inventory followed by command back-to-back as part of a single procedure.

Using a single message to initiate both initial trigger and command can add complexity if the device(s) were not already inventoried and may bring some challenges (e.g. how to provide resources for D2R depends on potential number of devices that gets triggered). However, for some use cases it can be beneficial.

In some scenarios, the device ID may be absent in the D2R message in response to the initial trigger. When to include it and when to leave it out should be indicated by the Reader in the initial trigger.

RAN2 will study solutions where Initial Trigger Message from the Reader also includes “command”.

The Initial Trigger Message indicates whether AIoT Device ID needs to be included in the D2R response.

## AIoT inventory and/or command towards the AIoT device

In both topologies, inventory refers to determining the identity of all or a subset of AIoT devices within the range of Reader(s). The AIoT application sends the inventory request optionally including the information such as a list of Reader IDs or area information to limit the inventory to specific group of Readers, the filter criteria to limit the inventory to certain AIoT devices matching the criteria. This is being discussed in SA2.

SA2 is discussing the details on the command protocol as well as the procedures for Ambient IoT Application Function to trigger AIoT inventory and AIoT command.

Each Reader that received the inventory request should trigger the inventory procedure to discover or trigger AIoT devices to perform AIoT access procedure. In inventory procedure, Reader attempts to discover all or a subset of AIoT devices in the range of Readers. In that sense, before the Reader performs the inventory, the Reader has no information on how many AIoT devices or which specific AIoT devices are or maybe nearby the Reader.

For AIoT command, the AIoT application function or AIoT Controller can indicate the last known reader to initiate AIoT command towards the AIoT device(s). However, the last known reader may not be able to reach the AIoT device(s). In such case, the AIoT AF or AIoT Controller may perform escalation techniques which are outside of RAN2 discussion scope. However, it should be noted that this should not mean a AIoT reader creates a cell.

RAN2 assumes the AIoT Application Function/AIoT Controller selects the appropriate reader to initiate AIoT inventory or command towards the AIoT device(s) (e.g. based on the last know reader).

# Summary

Based on the discussion above, we have following observations:

Observation 1.	In terms of design, subsequent trigger/paging message is not different from initial trigger message.

Observation 2.	The resource allocation including whether there is one or multiple resource opportunities should be indicated by the initial trigger message

Observation 3.	As previously agreed, supporting both ‘inventory’ and ‘command’ based on a single request from application can be achieved as an inventory followed by command back-to-back as part of a single procedure.

Observation 4.	Using a single message to initiate both initial trigger and command can add complexity if the device(s) were not already inventoried and may bring some challenges (e.g. how to provide resources for D2R depends on potential number of devices that gets triggered). However, for some use cases it can be beneficial.

Observation 5.	In some scenarios, the device ID may be absent in the D2R message in response to the initial trigger. When to include it and when to leave it out should be indicated by the Reader in the initial trigger.

Observation 6.	SA2 is discussing the details on the command protocol as well as the procedures for Ambient IoT Application Function to trigger AIoT inventory and AIoT command.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Reader may include filter criteria (to identify the AIoT devices that are supposed to respond) in the Initial Trigger message (aka AIoT paging) sent towards the AIoT device(s). It is up to SA2 to define the details of the filter criteria.

Proposal 2:	If neither a filter criterion nor a device ID is included in the initial trigger message, the message is considered as addressed for all devices that can receive the AIoT message.

Proposal 3:	Reader may indicate the type of AIoT access (e.g., 2-step or 4-step) to AIoT devices in the initial trigger message (aka AIoT paging).

Proposal 4:	The initial trigger message indicates the presence and information carried in the subsequent R2D message.

Proposal 5:	Design of initial trigger message is reused for subsequent trigger message, i.e. a separate message is not introduced for subsequent trigger/paging message.

Proposal 6:	Separate ‘access occasion announcement message’ is not needed.

Proposal 7:	RAN2 will study solutions where Initial Trigger Message from the Reader also includes “command”.

Proposal 8:	The Initial Trigger Message indicates whether AIoT Device ID needs to be included in the D2R response.

Proposal 9:	RAN2 assumes the AIoT Application Function/AIoT Controller selects the appropriate reader to initiate AIoT inventory or command towards the AIoT device(s) (e.g. based on the last know reader).

# References

[1] RP-240826, Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#103, March 2024

[2] TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, Release 18