# 3GPP TSG-RAN2 Meeting #127bis	R2-2408626

# Hefei, China, Oct 14-18, 2024

Agenda Item:	8.2.3

Study Item:	FS_Ambient_IoT_Solutions

Source: 	Qualcomm Incorporated

Title: 	Further aspects of Topology 2

Document for:	Discussion/Decision

# Introduction

RAN#103 approved SI for Ambient IoT in [1]. TR 38.848 [2] refers to the two topologies under consideration as shown below.

Figure 4.2.1.1-1: Topology 1

In Topology 1, the Ambient IoT device directly and bidirectionally communicates with a base station. The communication between the base station and the ambient IoT device includes Ambient IoT data and/or signalling.

Figure 4.2.1.2-1: Topology 2

In Topology 2, the Ambient IoT device communicates bidirectionally with an intermediate node between the device and base station. In this topology, typically the intermediate node would be a UE capable of functioning as Ambient IoT Reader. The Reader transfers Ambient IoT data and/or signalling between the base station and the Ambient IoT device.

RAN2#127 discussed the Topology 2 aspects and agreed the following:

Agreements

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

- A-IoT air interface  in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

- RAN2 assumes that intermediate UE authorization is performed by upper layers.  RAN2 impact can be studied after further SA/RAN3 progress.

- RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

- NW is in control of resources to be used by AIoT air interface.

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

This document discusses remaining aspects about topology 2 for Ambient IoT.

# Discussion

## Radio Resources for R2D and D2R

The WID indicates that “FR1 licensed spectrum in FDD” is to be assumed for the R2D and D2R link.

WID assumption indicates “FR1 licensed spectrum in FDD” for the R2D and D2R link regardless of the topology.

This means that, for topology 2, the scenarios discussion for radio resource allocation is somewhat similar to that for the NR sidelink discussions, e.g., how are the resources provided to the Reader/UE to use it in D2R/R2D (such as mode1/mode2), can the UE be in RRC_IDLE/INACTIVE or does it have to be in RRC_CONNECTED, etc.

## UE Reader’s RRC state

While one can argue that the UE reader should always be in RRC_CONNECTED state e.g. to get the commands from AIoT Controller, there can be valid scenarios where the UE Reader operates while being in other RRC states. E.g. the commands from the Controller arrive to the UE Reader at time t1 while it is in RRC_CONNECTED (or RRC_INACTIVE using MT SDT), and the UE Reader performs the actual inventory or command with the AIoT devices (tags) while being in RRC_IDLE or RRC_INACTIVE state, then UE Reader transitions to RRC_CONNECTED (or uses SDT from RRC_INACTIVE) after performing the inventory or command, to transmit the collected AIoT data to the AIoT Controller.

Note that the timescales for the AIoT operation and NR Uu operation are expected to be quite different. The NR Uu slots (and the potential resource allocation dynamicity) are in sub-milliseconds whereas AIoT operation can take multiple hundreds of milliseconds or multiple seconds. That means, while the UE reader is engaged in R2D/D2R, on the NR Uu side, it can reasonably go to RRC_IDLE or RRC_INACTIVE state saving on UE power.

UE Reader performing AIoT operations with the AIoT devices is not limited to RRC_CONNECTED state, i.e. the UE Reader can be in RRC_IDLE or RRC_INACTIVE state while performing R2D/D2R activities.

Therefore, we propose:

The UE Reader can be in any RRC State while performing R2D/D2R activities.

## Resource allocation for the Reader

Note that, in the following discussion, resource allocation refers to the allocation of the radio resources for the UE/Reader that can be used in R2D/D2R link, but not the allocation of resources by the reader for the R2D/D2R link.

We recognize that since licensed spectrum is to be used in D2R/R2D link, there needs to be coordination between the network and the Reader on which radio resources are to be used for D2R/R2D communication. Therefore RAN2 agreed:

5 NW is in control of resources to be used by AIoT air interface.

Borrowing the terminologies/discussion from NR Sidelink/V2X, there can be two modes of resource allocation [3, section 5.7.2]: mode 1 and mode 2. In mode 1, the resource allocation is provided by the network. In mode 2, UE decides the transmission resources from the (pre)configured resource pool(s).

SL-mode-2-like resource allocation is more suitable for scenarios where the resource scheduling for D2R and R2D link needs to be quick and made locally by the Reader. In addition, the resource pools allocation from the NW would be more static compared to mode-1, making it simpler.  In contrast, mode 1-like resource allocation can add more complexity to the Reader as well as the network, but can provide more dynamicity.

For the quick local decisions on D2R/R2D communication resources, SL mode-2-like resource allocation is more suitable.

Therefore, we propose:

gNB can configure a pool of resources for R2D/D2R communication and UE Reader can randomly select resource(s) among the configured pool(s).

With the SL mode-2-like resource allocation, current SL specification stipulates the UEs are to be in RRC_CONNECTED to use the resources. This will need to be updated to allow the UE readers in IDLE or INACTIVE state also be able to keep using the resources while the resource allocation is valid. In fact, regardless of mode-1-like or mode-2-like resource allocation, the resource validity should include potentially the time when UE reader is in IDLE/INACTIVE, i.e. moving to IDLE or INACTIVE should not be a sole reason to release AIoT resources and abandon AIoT operation.

UE reader being released to RRC_IDLE or RRC_INACTIVE shall not be a sole reason to require the Reader to abandon an ongoing AIoT operation and/or the AIoT resources allocated to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured or explicitly released the resource, etc.).

To keep the design simple, in the mode-2-like allocation, which is expected to be more static in nature, the resource pools to be allocated for AIoT operation can be determined by the gNB based on implementation (assistance information provided by the AIoT Controller).

On the other hand, if mode-1-like resource allocation from NW is preferred, which is more dynamic in terms of (re)configuration of resources, it would be beneficial for the NW to be aware of the radio resources needed by a certain reader at a certain time for the more efficient operation. Based on the messages from the AIoT Controller to the UE reader, e.g. to perform an inventory or a command, the reader UE can request the needed radio resources from the gNB. This request can help the gNB scheduler to efficiently allocate the radio resources for the UE reader for the AIoT operation. In addition, the AIoT resources that the UE reader is able to utilize may also depend on the deployment information where the reader may have better knowledge. For example, the tags deployed under a certain reader may be able to only work in a certain frequency band, and without such knowledge, gNB providing AIoT resources autonomously in another band will basically make the AIoT reader inoperable with the tags.

AIoT resource request from UE reader helps gNB for efficient resource allocation.

UE reader can request specific AIoT resources from the gNB. The actual resource allocation is decided by the gNB.

UE reader being released to RRC_IDLE or RRC_INACTIVE shall not be a sole reason to require the Reader to abandon an ongoing AIoT Operation and the AIoT resources allocated to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured or explicitly released the resource, etc.).

## UE Reader temporarily out-of-connection

While one can argue that the UE reader should always be in coverage of the network e.g. to get the commands from AIoT Controller, as already agreed by RAN2, there can be valid scenarios where the UE Reader is temporarily out of connection e.g. due to HO or RLF. E.g., the commands from the Controller arrive to the UE Reader at time t1 while it is in coverage; the UE reader starts to perform the actual inventory or command with the AIoT devices (tags) at t2, but during this time before the AIoT operation is complete (recall that AIoT operations can take much longer than couple of milliseconds), the UE Reader suffers RLF.

In such case, the UE reader should not be required to prematurely stop all the ongoing AIoT operations as long as the previously provided resource allocation is still valid. In other words, merely going through temporary interruption in Uu should not require the Reader to abandon AIoT resources already provided to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured and released the resource, etc.)

Temporary interruption in the Uu shall not require the Reader to abandon AIoT resources already provided to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured and released the resource, etc.).

Then, after performing the inventory or command, the Reader may come back to the coverage at t3 and can successfully transmit the collected AIoT data to the AIoT Controller.

As explained above, UE Reader performing AIoT operations with the AIoT devices while suffering from temporary interruption in Uu is a valid scenario. Therefore, we propose:

Temporary interruption in the Uu shall not require the Reader to abandon AIoT operation and/or the resources already allocated to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured and released the resource, etc.).

## AIoT Data transport from Reader to the network

It is our understanding that companies have proposed various options for AIoT data communication between the UE Reader and the AIoT CN.

RAN2 previously agreed:

4 RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

That means the following 3 options are on the table.

Option 1 (RRC-based): In this option, UE Reader uses its RRC connection with the gNB to send/receive AIoT messages explicitly using RRC. gNB then forwards the AIoT messages to the AIoT CN node explicitly over N2. No end-to-end protocol between the UE reader and the AIoT CN node (see R-AP in options 2 and 3).

Option 2 (NAS-based): In this option, UE Reader uses its NAS association with the AMF to send/receive AIoT messages as NAS PDUs. AMF can then forward the AIoT messages to the AIoT CN node (via SBI interfaces in the core).

In this solution, the AIoT enabled gNB is unaware of the contents of the AIoT messages as it is received as a NAS container.

Option 3 (PDU session-based): In this option, UE Reader uses user plane signalling (establishes a PDU session till UPF) to communicate with AIoT CN node. UPF can then forward the AIoT messages to the AIoT CN node (via SBI interfaces in the core).

In this solution, the AIoT enabled gNB is unaware of the contents of the AIoT messages as it is sent over user plane.

In the table below, we compare the three options by discussing the protocol stack, interface impacts and their pros vs. cons.

| RRC-based (option 1) | NAS-based (option 2) | PDU session-based (option 3) |
| --- | --- | --- |
| UE Reader uses its RRC connection with the gNB to send/receive AIoT messages explicitly in RRC. gNB then decodes the AIoT RRC messages and forwards the AIoT messages to the AIoT CN node explicitly over N2. | UE Reader uses its NAS association with the AMF to send/receive AIoT messages as NAS PDUs. AMF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core. | UE Reader uses user plane signaling (establishes a PDU session till UPF) to communicate with AIoT CN node. UPF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core. |
| AIoT enabled gNB is aware of the contents of the AIoT messages as it is received explicitly over RRC. However, it adds unnecessary RRC ASN.1 complexity. | AIoT enabled gNB is unaware of the contents of the AIoT messages as it is received as a NAS container AIoT enabled gNB can be made aware explicitly by the AIoT CN node providing assistance information. | AIoT enabled gNB is unaware of the contents of the AIoT messages as it is sent over user plane AIoT enabled gNB can be made aware explicitly by the AIoT CN node providing assistance information. |
| No end-to-end protocol between the UE reader and the AIoT CN node | There is an end-to-end protocol (R-AP) between the UE reader and the AIoT CN node. | There is an end-to-end protocol (R-AP) between the UE reader and the AIoT CN node |
| RRM like solution This is like how gNB can explicitly request RRM measurements to UE explicitly via RRC | LPP-like solution This is like how LPP is defined between UE and LMF and carried over NAS/NGAP/RRC | UPL like solution This is like how UPL (User Plane Location Protocol) is used to carry location between a UE and location server |
| UE Reader selection can be performed by gNB or AIoT CN node | UE Reader selection performed by AIoT CN node | UE Reader selection performed by AIoT CN node |
| Overload on AMF if there are lot of AIoT devices | Overload on AMF if there are lot of AIoT devices | UPF can handle IP traffic better. |
| RAN2 impact: High RAN2 would need to define new RRC signaling to support the reception/transmission of AIoT related messages from gNB to UE Reader RAN2 would need to define procedures when the UE Reader’s serving gNB changes (e.g., due to mobility) or RRC state changes | RAN2 impact: No/minimal | RAN2 impacts: No/minimal |
| RAN3 impact: High RAN3 would need to define explicit signaling over N2 for the AIoT messages In case the serving gNB of the UE Reader changes (e.g., due to mobility) or the RRC state of the UE reader changes, Xn coordination is needed to provide UE Reader’s context from source gNB to target gNB | RAN3 impact: No/minimal No new procedures needed when the UE Reader’s serving gNB changes or the UE Reader changes its RRC state | RAN3 impact: No/minimal No new procedures needed when the UE Reader’s serving gNB changes or the UE Reader changes its RRC state |
| SA2 impact: Medium SA2 needs to define the interface between AMF and AIoT CN node (Nac interface) | SA2 impact: Medium SA2 needs to define the interface between AMF and AIoT CN node (Nac interface) and support the end-to-end R-AP protocol | SA2 impact: Medium SA2 needs to define the interface between UPF and AIoT CN node (Nuc interface) and support the end-to-end R-AP protocol |
| Non-homogeneous control plane transport - RRC between UE Reader and gNB - N2 between gNB and AMF - API between AMF and AIoT CN node | Non-homogeneous control plane transport - NAS between UE Reader and AMF - API between AMF and AIoT CN node | Homogenous transport: End to-end IP - IP connectivity (PDU session) between UE Reader and UPF - IP connectivity between UPF and AIoT CN node |
| Needs support of RRC at UE Reader (already supported by UEs today) | Needs support of NAS at UE Reader (already supported by UEs today) | Needs support of HTTP client at UE Reader (already supported by UEs today) |
| Fragmented protocol stacks for Reader in T1 and T2 - AIoT RAN node in T1 will have R-AP over IP - UE Reader in T2 will receive signaling over RRC | Fragmented protocol stacks for Reader in T1 and T2 - AIoT RAN node in T1 will have R-AP over IP - UE Reader in T2 will have R-AP over NAS | Can support common SBI protocol stack for Reader in T1 and T2 - AIoT RAN node in T1 will have R-AP over IP - UE Reader in T2 will have R-AP over IP |

Considering there are high impacts to the UE reader, the AIoT enabled gNB, as well as the specification efforts for RAN2/RAN3 with Option 1 (RRC-based), it is our view that RAN2 should not prefer this option.

Comparing the pros and cons between the Option 2 (NAS-based) and Option 3 (PDU-based), it is clear that option 3 has less impacts to RAN2/3 work, can provide homogenous transport between different nodes (from Reader to the CN), and can also support common protocol stack for the Reader in both topologies. So, while our preference is clear that option 3 should be the way to go, we invite companies to study further the options 2 and 3.

Given the high impacts to RAN2/RAN3, RRC-based AIoT data transport between the Reader and the gNB is not considered.

For AIoT data transfer between the reader and the AIoT CN, PDU-session based AIoT data transfer has clear advantages over NAS-based AIoT data transfer.

Therefore, we propose:

PDU-session based AIoT data transfer between the reader and AIoT CN is studied. FFS NAS-based data transfer between Reader and AIoT CN.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	WID assumption indicates “FR1 licensed spectrum in FDD” for the R2D and D2R link regardless of the topology.

Observation 2.	UE Reader performing AIoT operations with the AIoT devices is not limited to RRC_CONNECTED state, i.e. the UE Reader can be in RRC_IDLE or RRC_INACTIVE state while performing R2D/D2R activities.

Observation 3.	For the quick local decisions on D2R/R2D communication resources, SL mode-2-like resource allocation is more suitable.

Observation 4.	UE reader being released to RRC_IDLE or RRC_INACTIVE shall not be a sole reason to require the Reader to abandon an ongoing AIoT operation and/or the AIoT resources allocated to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured or explicitly released the resource, etc.).

Observation 5.	AIoT resource request from UE reader helps gNB for efficient resource allocation.

Observation 6.	Temporary interruption in the Uu shall not require the Reader to abandon AIoT resources already provided to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured and released the resource, etc.).

Observation 7.	Given the high impacts to RAN2/RAN3, RRC-based AIoT data transport between the Reader and the gNB is not considered.

Observation 8.	For AIoT data transfer between the reader and the AIoT CN, PDU-session based AIoT data transfer has clear advantages over NAS-based AIoT data transfer.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	The UE Reader can be in any RRC State while performing R2D/D2R activities.

Proposal 2:	gNB can configure a pool of resources for R2D/D2R communication and UE Reader can randomly select resource(s) among the configured pool(s).

Proposal 3:	UE reader can request specific AIoT resources from the gNB. The actual resource allocation is decided by the gNB.

Proposal 4:	UE reader being released to RRC_IDLE or RRC_INACTIVE shall not be a sole reason to require the Reader to abandon an ongoing AIoT Operation and the AIoT resources allocated to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured or explicitly released the resource, etc.).

Proposal 5:	Temporary interruption in the Uu shall not require the Reader to abandon AIoT operation and/or the resources already allocated to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured and released the resource, etc.).

Proposal 6:	PDU-session based AIoT data transfer between the reader and AIoT CN is studied. FFS NAS-based data transfer between Reader and AIoT CN.

# References

[1] RP-240826, Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#103, March 2024

[2] TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, Release 18

[3] TS 38.300, NR and NG-RAN Overall Description; Stage 2