3GPP TSG-RAN WG2 Meeting  #127bis                                                                  R2-2408629

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.7.4

Source: 	Meta

Title:  	Discussion on DSR Enhancements

Document for:	Discussion and Decision

# Introduction

In the Revised WID on XR (eXtended Reality) for NR Phase 3, one of the objectives is to specify enhancements for scheduling [1]:

- Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Note: LCP implementation complexity need to be taken into account when evaluating solutions. - Note: Check in RAN#105

In RAN2 #126, the following agreements were made on DSR enhancements:

Agreement - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In this paper, we discuss our views on the DSR enhancements.

# Discussion

Release 18 introduced the Delay Status Report (DSR) to provide feedback on buffered data remaining time when a UE requests uplink resources, allowing gNB to perform delay-aware uplink scheduling. However, a limitation is that when multiple PDU sets in a LCG have different remaining times, only the shortest remaining time below the threshold is reported. To address this, consider enhancing DSR to report remaining time with finer granularity and potentially additional information such as multiple pairs of remaining time and buffer details. Meanwhile, it is important for DSR to avoid introducing excessive signal overhead.

In the RAN2 meeting, the inclusion of non-delay critical data that are in front of the delay critical data in the DSR data volume calculation is discussed. We think this can be addressed by UE implementation and there is no need to include non-delay critical data in the DSR data volume calculation.

Proposal 1:  No need to include non-delay critical data in the DSR data volume calculation.

In RAN2 meetings, incorporating the importance information in DSR was discussed. In Rel-18, PDU Set Importance (PSI) is used during congestion when performing discard operations. Given the unclear benefits of PSI in DSR and the desire of keeping delay-critical and importance requirements separate, it is not advisable to integrate PSI information within DSR.

Proposal 2: No need to enhance DSR to incorporate importance information.

# Conclusion

In this paper, we discuss our views on the DSR enhancements.

Proposal 1:  No need to include non-delay critical data in the DSR data volume calculation.

Proposal 2: No need to enhance DSR to incorporate importance information.

# References

[1] RP-241771, “Revised WID on XR (eXtended Reality) for NR Phase 3” Melbourne, Australia, September 9-12, 2024