3GPP TSG-RAN WG2 Meeting #127bis	R2-2408633

Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.7.5

Source:		Samsung

Title:			RLC Enhancements for XR

Document for:	Discussion & Decision

# 1 Introduction

In this contribution, we discuss RLC AM enhancements for XR, addressing the following aspects.

Objective: RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. Including aspects such as: - how to avoid unnecessary retransmissions, e.g. details of Tx, Rx and combined approaches, pros and cons comparison. - how to ensure timely RLC retransmissions for XR, e.g. - can existing mechanisms be reused or do we need enhancements? - what kind of enhancements are needed, e.g. autonomous retransmission, retransmission based on enhanced status report, retransmission based on enhanced polling. - details and pros and cons of different solutions.

# 2 Discussion

## 2.1 Unnecessary RLC AM Retransmission Avoidance

Table I presents a comparison of different approaches to avoid unnecessary RLC retransmissions:

TABLE I

| Approach | Attribute / Condition | Pros | Cons |
| --- | --- | --- | --- |
| Transmitter initiated | RLC SN gap reporting mechanism | - Tx-initiated approach can be effectively designed based on actual delay incurred by SDU - Rx operation is easy with just state variables update | - New signalling for RLC SN gap reporting (though it is quite aligned to existing PDCP SN gap reporting) |
| Receiver initiated | Indicate abandoned SDUs in status report upon t-Reassembly or a local timer expiry | - No impact on transmitter operation, as Tx updates based on status report as in legacy | - Rx operation becomes complex to determine and address abandoned SDU(s) - Does not allow for determining and allowing needed retransmissions - Receiver is not precisely aware about actual delay incurred by SDU - Status reporting is restricted by t-StatusProhibit |
| Combined Tx & Rx approach | Tx stops retransmission of obsolute SDUs based on discard indication and Rx stops receiving obsolute SDU based on local timer | - Tx can stop retransmission of obsolute SDUs | - Complex and severe impact on Rx - Same drawbacks as that for receiver initiated approach |

Observation 1: Receiver initiated approach does not allow for determining and allowing needed retransmissions and abandons the SDUs outrightly upon t-Reassembly expiry when reception failure for an AMD PDU is detected. Moreover, receiver is not precisely aware about the actual delay incurred by the SDU at the transmitter. Status reporting is also restricted by t-StatusProhibit timer.

Observation 2: Combined approach is quite complex and has severe impacts and suffer from same drawbacks as the receiver initiated approach.

Observation 3: Transmitter initiated approach is better aware about actual delay incurred by SDUs and can be effectively and accurately designed. A new signalling for RLC SN gap report would be required. Impact on Rx is also largely avoided.

Based on the comparison, the transmitter initiated approach seems quite preferable and hence it is proposed.

Proposal 1: Transmitter initiated approach is considered for unnecessary RLC retransmissions avoidance.

Note that in Release 18 XR, RAN2 has introduced PDCP SN gap reporting to reduce the reordering latency caused by already discarded PDCP SDUs, encountered at receiving PDCP entity.

In legacy RLC, an RLC SDU that has been discarded at PDCP, if it has been submitted to lower layer, will not be discarded at RLC. It seems unnecessary to retransmit such RLC SDUs, since there is no more value to deliver it. Such unnecessary RLC retransmission will delay the other RLC SDUs’ (re)transmissions, especially, for delay-critical RLC SDUs. Also, such operation will decrease the overall radio resource utilization from application point of view.

To avoid such unnecessary retransmissions, RAN2 should consider discarding RLC SDUs that have already been submitted to lower layer.

Proposal 2: Upon PDCP discard indication, RLC AM can discard RLC SDUs already submitted to lower layer, even if RLC SN gap can occur.

Since RLC AM is designed for loss-less delivery, legacy RLC AM will not work if discarding an RLC SDU already submitted to lower layer, which causes RLC SN gap, i.e., window stalling will occur at both transmitting and receiving sides of RLC AM.

To overcome window-stalling issue caused by RLC SN gap, RAN2 can consider introducing an RLC SN gap reporting mechanism to inform receiving side of RLC AM of the RLC SN gap. The approach can be quite aligned with PDCP SN gap reporting and can be greatly benefited w.r.t. the specification effort and detailing perspective.

Proposal 3: RLC SN gap reporting mechanism is considered for RLC AM to handle RLC SN gap.

## 2.2 Timely RLC AM Retransmissions

Table II presents a comparison of different approaches to enable timely RLC AM retransmissions:

TABLE II

| Approach | Attribute / Condition | Pros | Cons |
| --- | --- | --- | --- |
| Autonomous Retransmissions | Blind retransmission | - Quick retransmission | - In-efficient and huge impact on capacity i.e. resource wastage |
|  | Upon failure of a number of HARQ transmissions | - Early determination & Local NACK | - Not straight-forward to map a RLC PDU and HARQ TB - Involves complicated and cumber-some cross-layer interactions - Large specification impact |
|  | Upon remaining time of the PDU becomes lesser than a threshold | - Pre-emptive retransmission | - Unnecessary - same can be achieved with a suitable configuration of polling timers |
|  | Upon availability of spare transmisison resources | - Opportunistic approach | - Not full-proof and deterministic |
| Enhanced Status Report | Stopping StatusProhibit timer for small PDB | - Early status reporting for small PDB | - Unnecessary - same can be achieved with suitable status reporting configurations by NW |
| Enhanced polling | Dedicated polling thresholds and associated counters for delay critical RLC SDUs | - Polling enhancements for delay-critical SDUs - Differentiated treatment | - New configurations needed for delay critical but manageable |

Autonomous retransmission implies a transmitter can speed up retransmission without having to wait for receiving a status report from the receiver. Though the proposal looks attractive; however, there can be drastic impacts on the capacity as a large number of unnecessary and unregulated retransmissions could take place. As also described in Table II, it is not easy to regulate such autonomous transmissions based on specific conditions (e.g. HARQ transmisions, timers or spare transmission resources).

Observation 4: With autonomous retransmission, there can be drastic impacts on the capacity as a large number of unnecessary and unregulated retransmissions could take place. It is not easy to regulate such autonomous transmissions based on specific conditions (e.g. HARQ transmisions, timers or spare transmission resources).

Proposal 4: No enhancements are pursued for autonomous RLC AM retransmissions.

Enhanced status report requires the receiver should be able to provide a status report quickly to the transmitter. More particularly, this is to support stopping of t-StatusProhibit timer so as to be able to report status when small PDB is involved. However, such provisions can be easily achieved by the NW with suitable configurations of the status reporting e.g. t-StatusProhibit timer. Moreover, receiver can only determine about the need for timely retransmission from the perceived SN gap in the receiving window i.e. the receiver is not well aware about the retransmission requirements as compared to the transmitter.

Observation 5: Enhancements for status reporting for small PDB (e.g. stopping of t-StatusProhibit) are not needed as network configuration can take care with suitable configurations. Receiver is not well aware about the need for timely retransmissions as compared to the transmitter.

Proposal 5: No enhancements are pursued for status reporting in order to aid timely RLC AM retransmissions.

Further, we discuss the enhanced polling approach to facilitate timely RLC AM retransmissions.

As shown in Excerpt 1, there are three cases in legacy RLC AM to trigger a polling at the transmitting side:

- Case 1: the number of initially transmitted RLC SDUs or the bytes thereof, have been accumulated up to a certain thresholds (pollPDU or pollByte), respectively, whichever comes first

- Case 2: there is no data to (re)transmit

- Case 3: transmitting window stalling happens

Excerpt 1:

38.322 5.3.3 Polling ... Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below. Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); - include a poll in the AMD PDU as described below. ...

Among the three cases, a special care can be taken for Case 1, in reducing the polling latency, especially, when there are delay-critical RLC SDUs involved. E.g., dedicated thresholds (i.e., pollPDU and pollByte) for delay-critical RLC SDUs and the associated counters can be introduced, in order to reduce the polling latency for delay-critical RLC SDUs.

Proposal 6: Dedicated polling thresholds and the associated counters are introduced for delay-critical RLC SDUs.

# 3 Conclusion

Based on the discussion above, we have the following observations and proposals:

Unnecessary RLC AM Retransmission Avoidance

Observation 1: Receiver initiated approach does not allow for determining and allowing needed retransmissions and abandons the SDUs outrightly upon t-Reassembly expiry when reception failure for an AMD PDU is detected. Moreover, receiver is not precisely aware about the actual delay incurred by the SDU at the transmitter. Status reporting is also restricted by t-StatusProhibit timer.

Observation 2: Combined approach is quite complex and has severe impacts and suffer from same drawbacks as the receiver initiated approach.

Observation 3: Transmitter initiated approach is better aware about actual delay incurred by SDUs and can be effectively and accurately designed. A new signalling for RLC SN gap report would be required. Impact on Rx is also largely avoided.

Proposal 1: Transmitter initiated approach is considered for unnecessary RLC retransmissions avoidance.

Proposal 2: Upon PDCP discard indication, RLC AM can discard RLC SDUs already submitted to lower layer, even if RLC SN gap can occur.

Proposal 3: RLC SN gap reporting mechanism is considered for RLC AM to handle RLC SN gap.

Timely RLC AM Retransmissions

Observation 4: With autonomous retransmission, there can be drastic impacts on the capacity as a large number of unnecessary and unregulated retransmissions could take place. It is not easy to regulate such autonomous transmissions based on specific conditions (e.g. HARQ transmisions, timers or spare transmission resources).

Proposal 4: No enhancements are pursued for autonomous RLC AM retransmissions.

Observation 5: Enhancements for status reporting for small PDB (e.g. stopping of t-StatusProhibit) are not needed as network configuration can take care with suitable configurations. Receiver is not well aware about the need for timely retransmissions as compared to the transmitter.

Proposal 5: No enhancements are pursued for status reporting in order to aid timely RLC AM retransmissions.

Proposal 6: Dedicated polling thresholds and the associated counters are introduced for delay-critical RLC SDUs.