3GPP TSG-RAN WG2 Meeting #127bis	                                    R2-2408634

Hefei, China, Oct. 14th – Oct. 18th, 2024

Agenda item:		8.1.3

Source:			Sharp

Title:		Discussion on NW-side Data Collection

Document for:	 	Discussion

- Introduction

During Rel-18, data collection for the training of NW-side models was discussed with the following agreements in TR 38.843 [1]. The following principles apply to all the currently identified used cases (i.e., the beam management (BM), the positioning, and the CSI compression and prediction use cases).

- A set of general data collection principles is expected to be considered for network-side model training. These include: - UE to support data logging - UE to report the collected data periodically, event-based, and on-demand - The UE memory, processing power, energy consumption, signalling overhead should be considered

In the previous RAN2#127 meeting, the following agreements were made [2].

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support. - UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. - Measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. FFS whether the UE stops autonomously or if it reports to the network. - FFS whether AS buffer event based reporting is supported. FFS if we send availability indication or full report if it is supported - FFS on event based data collection/logging - On-demand request from the network is supported. FFS details on signalling

In our views, the agreements can be applied to both BM and positioning use cases, where RRC signalling is considered for the BM use cases and LPP is used for the positioning use cases. As a result, in this contribution, we would like to share our views on enhancing both RRC and LPP messages to support the data collection for NW-side models.

- Discussion

UE Capability for Data Collection

First, since the measurements are performed by the UE, the data collection for NW-side model needs both the consent and the capability of the UE. That is, whether the UE measures the SSB/CSI-RS (for BM use cases) and PRS (for positioning use cases) and reports the result for the purpose of data collection for NW-side data collection is based on the UE capability. The NW can obtain the measurement results for NW-side model only when the UE supports the function of data collection and is able to perform the function (e.g., has sufficient power and memory for data collection for NW-side model).

In our views, for BM use cases, the existing RRC messages UECapabilityEnquiry and UECapabilityInformation can be used to request/indicate the UE’s capabilities related to data collection for NW-side model, while the existing LPP messages ProvideCapabilities and RequestCapabilities can achieve the coordination. Specifically for the positioning use cases, the UE may not be capable of some UE-based positioning methods (e.g., DL TDoA) to acquire a ground truth label, but may be able to perform the measurement and collect the data (e.g., RSTD). In this case, the UE can provide such information in the corresponding IEs (e.g., DL-TDOA-ProvideCapabilities) to indicate the detailed capabilities. Alternatively, a new IE specifically for AI/ML can be considered. For example, the LMF and the UE can exchange the AI/ML-related capabilities via AIML-RequestCapabilities and AIML-ProvideCapabilities messages. In our views, we consider the new IE for AI/ML would be beneficial to incorporate all the capabilities related to different positioning methods.

Additionally, it was agreed to have a minimum AS layer memory size in the last meeting. Considering that the UE’s memory status configured for data collection may vary, we propose that the UE includes the memory size information in the RRC/LPP message providing the capability information. The memory size information can be an absolute value or can be an estimated value (e.g., the maximum number of instances that a UE can log).

Proposal 1: For BM use cases, the UECapabilityEnquiry and UECapabilityInformation messages are used to request and provide the UE capabilities for data collection for NW-side model.

Proposal 2: For Pos use cases, new IEs AIML-RequestCapabilities and AIML-ProvideCapabilities are used to request and provide the UE capabilities for data collection for NW-side model.

Proposal 3: The UE can include the AS layer memory size information in the RRC/LPP message providing the UE capability information.

Measurement Configuration

For BM use cases, the measurement configuration can take L1 CSI report framework as baseline. That is, the measurement is configured as a CSI-RS resource set with additional indication contained in the resource set configuration so that the UE can identify the resource set is for AI/ML purpose (regardless of inference, training, or monitoring). Additionally, the indication can be an associated ID to enable the network to collect datasets associated with multiple NW-side additional conditions.

Proposal 4: For BM use cases, the measurement is configured as a CSI-RS resource set with an associated ID.

Similarly, for positioning use cases, the measurement configuration can take the legacy ProvideAssistanceData message as a baseline. In our view, the LMF should be capable of flexibly switching among different NW-side models corresponding to different positioning methods (e.g., TDOA, AOD, Multi-RTT). As a result, LMF needs to collect data with different measurement quantities (e.g., RSTD, RSRP, RxTxTimeDiff, etc.) in different additional conditions, and the LMF needs to explicitly indicate the required measurement quantities in the measurement configuration. To allow the LMF to dynamically configure the measurement quantities, we propose to use a new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) to configure the UE what to measure. Similar to the legacy ProvideAssistanceData message, the AIML-ProvideAssistanceData message contains PRS resource configurations; additionally, it also configures whether one or more of the measurement quantities (i.e., RSTD, RSRP, RTT) need to be measured by the UE. In the AIML-ProvideAssistanceData message, the associated ID (or the area ID) which identifies the NW-side additional conditions can also be provided.

Observation 1: LMF should be capable of flexibly switching between different NW-side models leveraging different positioning methods, so it may need to collect data with different measurement quantities in different additional conditions.

Proposal 5: For Pos use cases, a new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) is used for the LMF to configure the measurements, where the configuration can include the required measurement quantity, the PRS resource set, and the associated ID (or area ID or Cell Global Identity).

Report Configuration

For BM use cases, there are two options for the report configuration.

        - L1-based report configuration (e.g., via the CSI-ReportConfig). This option is straightforward since the legacy CSI reporting works with the association between a CSI resource configuration and a CSI report configuration.

        - L3-based report configuration (e.g., via the ReportConfig). This option is also feasible since the legacy L3 report configuration allows the report of service cell beams by including the indication includeBeamMeasurements in the report configuration.

In our views, L3-based configuration is preferable since the latency requirement on data collection for NW-side model in training case is not stringent. Still, both the options can be considered and further discussion is needed.

The logging configuration can indicate how and when the UE logs the measurement results, and it can be configured in the report configuration after the L1/L3-based report configuration is determined. Regarding the buffer status, a counter can be configured to limit the number of logged instances to avoid overflow problem. In addition, in our views, the logging can be performed on an event-triggered manner; that is, a condition (e.g., the signal quality better/worse than a threshold) triggers the logging, and thus the measurement overhead can be reduced. Also, to support the on-demand report, an indication is needed in the report configuration for the UE to differentiate whether the report is a periodic report or an on-demand report.

Observation 2: For BM use cases, both the legacy L1-based and L3-based report configurations are feasible for data collection for NW-side model

Proposal 6: For BM use cases, RAN2 discuss the report configuration to be L1-based or L3-based.

Proposal 7: For BM use cases, the report configuration further contains: 1) a counter limiting the number of logged instances, 2) a condition that triggers the UE to start logging, 3) an indication whether the report is periodic or on-demand.

As for the positioning use cases, the legacy RequestLocationInformation message can be taken as a baseline. Considering some possible AI/ML-specific parameters that can be leveraged for the data collection, we propose a new type of RequestLocationInformation message (e.g., an AIML-RequestLocationInformation message) as the report configuration.  The AIML-RequestLocationInformation message) as the report configuration can include the following information:

- A counter to limit the number of logged instances.

- A condition to trigger the UE to start logging.

- An indication whether the report is periodic or on-demand.

- An indication whether the ground truth label obtained by the UE should be included in the report.

- The measurement quantities to be reported.

For the above mentioned information, the first three bullets are the same as in BM use cases for the same reason. Depending on the UE’s capability to obtain the ground truth label, the LMF can determine whether to include the indication to request that the UE reports the ground truth label along with the measurement results. Additionally, since multiple measurement quantities can be configured for the UE to measure, it would be flexible for the LMF to decide which measurement quantities are needed at some given time. In addition, since the time at which the UE obtains the measurement result (e.g., the RSTD) and the time at which the UE obtains the ground truth label (e.g., the location) may be different, the LMF can provide a validity period to the UE. The UE can then associate the measurement results with the ground truth label according to the validity period. By doing so, the timestamps of the measurement result and the ground truth label are not needed to be reported.

Observation 3: For Pos use cases, the UE may obtain the ground truth label by itself.

Proposal 8: For Pos use cases, use a new type of RequestLocationInformation message (e.g., an AIML-RequestLocationInformation message) as the report configuration.

Proposal 9: For Pos use cases, the report configuration further contains: 1) a counter limiting the number of logged instances, 2) a condition that triggers the UE to start logging, 3) an indication whether the report is periodic or on-demand, 4) an indication whether to report the ground truth label, and 5) the measurement quantities to be reported.

Proposal 10: For Pos use cases, the LMF provides a validity period for the UE to associate the measurement result and the ground truth label.

On-Demand Report

For BM use cases, the on-demand report can be triggered either by an RRC message (e.g., UEInformationRequest message) or by lower layer signallings (e.g., DCI or MAC CE). In our view, triggering by RRC signalling is preferable for two reasons. The first reason is the impact on specifications. On-demand report triggered by RRC signalling only requires the addition of an indication (e.g., aiml-MeasReport) in the UEInformationRequest message, while lower layer triggering requires new signalling design. The second reason is the latency requirement, since the data collection for NW-side model for training purpose is not an urgent use case, it is feasible to use L3 signalling to complete the whole on-demand report procedure.

In addition, if the network is only interested in a dataset associated with some specific NW side additional conditions, the network can further include the associated ID in the triggering signalling to filter the UE’s report.

Observation 4: For BM use cases, on-demand report triggered by L3 signalling has less spec impact and able to fulfill the latency requirements.

Proposal 11: For BM use cases, the on-demand report is triggered by L3 signalling.

Proposal 12: For BM use cases, the signalling triggering the on-demand report can include an associated ID.

For positioning use cases, the on-demand report can only be triggered by LPP messages (e.g., RequestLocationInformation message). As such, two options are identified for the triggering signalling.

- Option 1: Reuse existing RequestLocationInformation message. However, the UE needs to distinguish that the purpose of this message is to provide a report configuration or to request an on-demand report. A simple way is to add an indication in the message for the differentiation.

- Option 2: A new LPP message for on-demand report triggering. The option may have less spec impact since the message could be simple. Similar to the BM use cases, the message can further include the associated ID (or the area ID) to filter the UE’s report.

Proposal 13: For Pos use cases, RAN2 discuss whether to reuse legacy LPP signalling or define a new LPP signalling for the on-demand report triggering.

In a case that on-demand measurement report is supported, a UE may face the problem that the logged measurement results increase upon every logging instance while the network does not request the measurement results. To prevent the UE from the overwhelming size of logged measurement results, a dropping rule is needed. In our view, although this can be up to UE’s implementation, an upper bound of logging time (like 48 hours in logged MDT) can be discussed.

Observation 5: For both BM and Pos use cases, a dropping method is needed to prevent the UE from the overwhelming size of logged measurement results.

Proposal 14: An upper bound of logging time (e.g., 48 hours in logged MDT case) is specified for the use case of data collection for NW-side model.

Conclusion

Based on the above, we have the following observations and proposals.

Observation 1: LMF should be capable of flexibly switching between different NW-side models leveraging different positioning methods, so it may need to collect data with different measurement quantities in different additional conditions.

Observation 2: For BM use cases, both the legacy L1-based and L3-based report configurations are feasible for data collection for NW-side model

Observation 3: For Pos use cases, the UE may obtain the ground truth label by itself.

Observation 4: For BM use cases, on-demand report triggered by L3 signalling has less spec impact and is acceptable from latency aspect.

Observation 5: For both BM and Pos use cases, a dropping method is needed to prevent the UE from the overwhelming size of logged measurement results.

Proposal 1: For BM use cases, the UECapabilityEnquiry and UECapabilityInformation messages are used to request and provide the capabilities for data collection for NW-side model.

Proposal 2: For Pos use cases, new IEs AIML-RequestCapabilities and AIML-ProvideCapabilities are used to request and provide the capabilities for data collection for NW-side model.

Proposal 3: The UE can include the AS memory size information in the RRC/LPP message providing the capability information.

Proposal 4: For BM use cases, the measurement is configured as a CSI-RS resource set with additional indication, where the indication is an associated ID.

Proposal 5: For Pos use cases, a new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) is used for the LMF to configure the measurements, where the configuration can include the required measurement quantity, the PRS resource set, and the associated ID (or area ID).

Proposal 6: For BM use cases, RAN2 discuss the report configuration to be L1-based or L3-based.

Proposal 7: For BM use cases, the report configuration can further contain: 1) a counter limiting the number of logged instances, 2) a condition that triggers the UE to start logging, 3) an indication whether the report is periodic or on-demand.

Proposal 8: For Pos use cases, use a new type of RequestLocationInformation message (e.g., an AIML-RequestLocationInformation message) as the report configuration.

Proposal 9: For Pos use cases, the report configuration can further contain: 1) a counter limiting the number of logged instances, 2) a condition that triggers the UE to start logging, 3) an indication whether the report is periodic or on-demand, 4) an indication whether to report the ground truth label, 5) the measurement quantities to be reported.

Proposal 10: For Pos use cases, the LMF can provide a validity period for the UE to associate the measurement result and the ground truth label.

Proposal 11: For BM use cases, the on-demand report is triggered by L3 signalling.

Proposal 12: For BM use cases, the signalling triggering the on-demand report can include an associated ID.

Proposal 13: For Pos use cases, RAN2 discuss whether to reuse legacy LPP signalling or define a new LPP signalling for the on-demand report triggering.

Proposal 14: An upper bound of logging time (e.g., 48 hours in logged MDT case) is specified for the use case of data collection for NW-side model.

Reference

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, December 2023

- Report of 3GPP TSG RAN WG2 meeting#127, August 19-23, 2024