3GPP TSG-RAN WG2 Meeting #127bis	R2-2408642

Hefei, China, 14 – 18 October, 2024

Agenda Item	: 6.6 (NR_SL_enh-Core)

Source	: LG, OPPO, Ericsson

Title	: Discussion on MAC correction for IUC's resource selection

Document for	: Discussion and Decision

1.	Introduction

This contribution discusses MAC correction on resource (re)selection for IUC.

2.	Discussion

At the #127 meeting [1], RAN2 discussed as follows regarding the MAC correction on resource (re)selection for IUC.

R2-2406262 Correction on resource (re)selection for IUC OPPO CR Rel-17 38.321 17.9.0 1873 - F NR_SL_enh-Core R2-2406263 Correction on resource (re)selection for IUC OPPO CR Rel-18 38.321 18.2.0 1874 - A NR_SL_enh-Core For the first change: [LG]: Consider the current text has no problem. [OPPO]: The removed condition can be also applicable to the three conditions. It is not exclusive condition. That’s why “the latency requirement of the Sidelink Inter-UE Coordination Information transmission (if applicable)” is added for each three cases with the removal of the exclusive condition. [LG]: Current MAC specifies two parts of resource selections. One is the resource selection for the data and another one is explicitly for IUC Information. If we remove the condition, it becomes less clear for the resource selection for IUC. [Apple]: Share the same view as LG. [OPPO]: Want to have more time to discuss with other companies. Would like to revisit it next meeting. - Revisit it next meeting.

According to the current MAC specification, UE behavior for scenario 1 below is supported in resource (re)selection for IUC, but scenario 2 below is not supported.

- Scenario 1

- There are three UEs, UE 1, UE 2, and UE 3. UE 2 is operating UE-A (generating inter-UE coordination information) towards UE 1, while also operating as UE-B (who is to generate data or explicit Inter-UE Coordination Request message) towards UE 3.

- Scenario 2

- There are two UEs that have established a unicast connection, UE 1 is playing the role of UE-B (who is to generate data or explicit Inter-UE Coordination Request message) to transmit data or explicit Inter-UE Coordination Request to UE 2. And at the same time, the UE 1 can also perform the role of UE-A (generating inter-UE coordination information), which transmits inter-UE coordination information to the UE 2.

In other words, the current MAC specification does not support changing the IUC role of one UE. Therefore, the proponent of R2-2406262 proposed the TP to support the behaviour of a UE performing the role of UE-A/UE-B while changing its role to also perform the role of UE-B/UE-A.

The proponent of R2-2406262 proposed MAC correction to support the UE procedure of the above scenario 2 that is not supported in the current MAC specification.

- Example fo the TP (A version with some wording modified from the TP in R2-2406262)

3> if sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is configured by RRC and preferred resource set is not received from a UE: 4> if transmission based on random selection is configured by upper layers: 5> randomly select the time and frequency resources for one transmission opportunity from the resources pool, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, the latency requirement of the triggered Sidelink Inter-UE Coordination Information reporting (if applicable), and/or the latency requirement of the triggered SL CSI reporting. 4> else: 5> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, the latency requirement of the triggered Sidelink Inter-UE Coordination Information reporting (if applicable), and/or the latency requirement of the triggered SL CSI reporting. 3> if sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is configured by RRC and when the UE does not have own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity from the resources belonging to the received preferred resource set for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, the latency requirement of the triggered Sidelink Inter-UE Coordination Information reporting (if applicable), and/or the latency requirement of the triggered SL CSI reporting. 3> if sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is configured by RRC and when the UE has own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity within the intersection of the received preferred resource set and the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, the latency requirement of the triggered Sidelink Inter-UE Coordination Information reporting (if applicable), and/or the latency requirement of the triggered SL CSI reporting; 4> if there are no resources within the intersection that can be selected as the time and frequency resources for the one transmission opportunity according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 5> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, the latency requirement of the triggered Sidelink Inter-UE Coordination Information reporting (if applicable), and/or the latency requirement of the triggered SL CSI reporting.

RAN2 can consider the following two approaches as MAC correction to support the above scenario 2 for IUC.

Solution 1: Normative text approach

- Such as the TP suggested in R2-2406262. When discussing the CR, some wording can be updated.

Solution 2: Note approach

- Keep the existing normative text without modification and add NOTE to support the scenario 2. When discussing the CR, some wording can be updated.

- Example of NOTE

3> if sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is configured by RRC and preferred resource set is not received from a UE: 4> if transmission based on random selection is configured by upper layers: 5> randomly select the time and frequency resources for one transmission opportunity from the resources pool, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL CSI reporting. 4> else: 5> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL CSI reporting. 3> if sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is configured by RRC and when the UE does not have own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity from the resources belonging to the received preferred resource set for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL CSI reporting. 3> if sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is configured by RRC and when the UE has own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity within the intersection of the received preferred resource set and the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL CSI reporting; 4> if there are no resources within the intersection that can be selected as the time and frequency resources for the one transmission opportunity according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 5> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL CSI reporting. 3> if sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is configured by RRC and when the UE determines the resources for Sidelink Inter-UE Coordination Information transmission upon explicit request from a UE: 4> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources, the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL-CSI reporting and the latency requirement of the Sidelink Inter-UE Coordination Information transmission. NOTE X: A UE that transmits data or an explicit Inter-UE Coordination Request to a peer UE can simultaneously transmit Sidelink Inter-UE Coordination Information to the same peer UE.

Observation 1. Current MAC specification does not support the following IUC scenario.

- Scenario

- There are two UEs that have established a unicast connection, UE 1 is playing the role of UE-B (who is to generate data or explicit Inter-UE Coordination Request message) to transmit data or explicit Inter-UE Coordination Request to UE 2. And at the same time, the UE 1 can also perform the role of UE-A (generating inter-UE coordination information), which transmits inter-UE coordination information to the UE 2.

Proposal 1. To support the IUC scenario (i.e., A UE that simultaneously performs the role of UE-A and the role of UE-B) exampled in the observation 1, RAN2 can decide on one of the two approaches below. Detailed wording can be discussed in CR discussion.

- Solution 1: Normative text approach

- Solution 2: Note approach

3.	Conclusion

In this contribution, we share the solutions regarding the TP on resource selection for IUC. In summary, they are as follows:

Observation 1. Current MAC specification does not support the following IUC scenario.

- Scenario

- There are two UEs that have established a unicast connection, UE 1 is playing the role of UE-B (who is to generate data or explicit Inter-UE Coordination Request message) to transmit data or explicit Inter-UE Coordination Request to UE 2. And at the same time, the UE 1 can also perform the role of UE-A (generating inter-UE coordination information), which transmits inter-UE coordination information to the UE 2.

Proposal 1. To support the IUC scenario (i.e., A UE that simultaneously performs the role of UE-A and the role of UE-B) exampled in observation 1, RAN2 can decide on one of the two approaches below. Detailed wording can be discussed in CR discussion.

- Solution 1: Normative text approach

- Solution 2: Note approach

4.	Reference

[1] Session chair’s note for R18 SL-Evo of #127 meeting (Samsung)

[2] R2-2406262	Correction on resource (re)selection for IUC 	(OPPO)