3GPP TSG-RAN WG2 Meeting #127bis	                                                        R2-2408649

Hefei, China, 14 – 18 October 2024

Agenda item:	8.12.2

Source:	Nokia

Title:	RAN2 Aspects of the NR MIMO

WID/SID:	NR_MIMO_Ph5-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

MIMO is one of the key technologies in NR systems, and Rel-19, that started with RAN1#116, will bring important enhancements to both FR1 and FR2. Among the enhancements to be studied, the following is listed in the WID RP-242394.

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this document, we present Nokia’s views regarding requirements that RAN2 need to discuss in relation to the WID, in particular about:

- Objective 1: UE-initiated beam management;

- Objective 3: Non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions:

- Objective 5: Asymmetric DL sTRP/UL mTRP deployment scenarios.

# 2	UE-initiated beam management

The latest agreements from RAN1 on UEIBM related to RAN2 discussion are:

Agreement RAN1#117 Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Agreement RAN1#118 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following - Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. Agreement RAN1#118 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following - Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above is at least applied to a single CC case.

Rel-15 to Rel-18 has specified periodic, semi-persistent and aperiodic CSI reporting as described in TS 38.214. In the scope of Rel-19, some use cases where UE could benefit from initiating the beam management/reporting are being identified. The UE-initiated beam management (UEIBM) feature refers to the case where the UE may be configured with at least one event/condition, and then the UE may start beam reporting if this at least one event/condition occurs or is satisfied. The improvements aimed by this feature are twofold:

- Reduce overhead, such that beam reports are sent by the UE only when needed, avoiding unnecessary beam reports that we may have in case of periodic reporting configured with small periodicities.

- Reduce latency, such that, as soon as certain events/conditions are met, a beam report can be sent, to avoid long delay that we may have in case of periodic reporting configured with large periodicities.

From the RAN1 agreements, so far Event-2 has been agreed as an event that can trigger a UEIBM report. Event-2 is the event where the quality of at least one new beam becomes a “threshold value” better than the current beam. For example, the network may configure the UE via RRC with a certain threshold, e.g. 6 dB, and when the UE measures a new beam to have a L1-RSRP which is 6 dB better than the L1-RSRP of the current beam, then a UEIBM report is triggered.

RAN1 is discussing two procedures for actually sending back the UEIBM report:

- Mode A, where the second UL channel for the UEIBM report is dynamically scheduled by the gNB; in such procedure the following steps are implemented:

  - The UE sends in a first PUCCH channel an UL indication to request to the gNB resources in a second UL channel to carry the UEIBM report;

  - The gNB indicates via DCI to the UE a resource in a second UL channel to carry the UEIBM report;

  - The UE sends the UEIBM report on the second UL channel.

- Mode B, where the second UL channel for the UEIBM report is pre-configured by the gNB; in such procedure the following steps are implemented:

  - The UE sends in a first PUCCH channel an UL indication to notify to the gNB that a UEIBM report will be transmitted in a second UL channel;

  - UE sends the UEIBM report on the second UL channel.

RAN1 has defined Mode A as the baseline, whereas Mode B is optional and may be supported only by some UEs.

For both Modes A and B, the UE sends an UL indication in the first PUCCH channel once an event has been triggered. This indication is used in Mode A to request UL resources for the report, whereas in Mode B it simply notifies the gNB that the next preconfigured UL resource will be used for the UEIBM report.

Our research in [1] examined UEs configured with Event-2 and a threshold of 3 dB. When the UE averages beam measurements over X instances, with a sliding window size of X (resulting in a total averaging time of 20X ms due to a measurement periodicity of 20 ms), the findings indicate that the number of events decreases as the sliding window size increases. This confirms that filtering can effectively reduce beam reporting overhead. However, throughput results suggest that the optimal sliding window size can vary based on the UE's speed and location. The key takeaway from the study is that the ideal sliding window size is influenced by factors such as UE location (e.g., cell-edge versus cell median) and UE speed. Incorrectly selecting the sliding window size can lead to performance degradation, either through excessive overhead from a high number of triggered events/measurement reports or through a reduction in throughput. Therefore, we propose:

Proposal 1: Configure and indicate to UE a number of L1-RSRP measurement instances to be used for UEIBM event triggering, i.e., the network configures a counter/time window to control the number of L1-RSRP measurement instances to be averaged before triggering a given event.

When the UE sends a UEIBM report to the gNB triggered by an event, e.g., Event-2, in case the gNB does not do any beam switching after the UEIBM report sent by the UE, it may happen that the conditions triggering the same UEIBM report may persist over a period of time (the same Event-2, i.e., for same new beam(s) and for same conditions, e.g., same RSRP value(s) of the new beam(s) and of the current beam, keep repeating over time). In the current framework (even considering the enhancement where multiple Event-2 instances within a time window need to take place to trigger a UEIBM report) this causes the transmission of multiple consecutive UEIBM reports with same content, i.e., same top-N beams with same RSRP values, which bring no additional information to the network and just cause additional overhead (see also Figure 1 below).

Figure 1. Multiple consecutive UEIBM reports with same contents transmitted by the UE under the current framework.

Proposal 2: Introduce a prohibit timer that starts when a UEIBM report (on the second UL channel) is sent by the UE, such that the UE is not allowed to send a UEIBM report before the timer has expired when the Event, e.g., Event-2, conditions that triggered the last sent UEIBM report have not changed.

# 3	3-Tx UL codebook-based transmissions

The 3-port SRS resource is agreed to reuse the legacy 4-port SRS resource, where the 4th SRS port is muted, based on the agreement in RAN1 #117:

Agreement Update the agreement made in RAN1 #116bis as the following. Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, Option 3: Always a same port is muted, e.g., i.e., the 4th port.

Agreement

For a UE reporting UE capability of 3TX operation, support introduction of a new RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled.

For a UE with 4-port capability, the UE can support up to max 4-port SRS transmission. Usually, this type of UE can be configured to support 3-port uplink transmission. Therefore, 3-port SRS resource can be configured. Based on the 3-port SRS resource design, the legacy 4-port SRS resource would be configured for the UE. When the legacy 4-port SRS resource is configured, the UE won’t be able to differentiate whether the 3-port or the 4-port SRS resource shall be used.  Therefore, one RRC parameter would be useful to differentiate the 3-port SRS resource configuration. RAN1 has made following agreement in RAN1#118:

When configuring 3 Tx, you can choose to place the configuration in either SRS-Config or PUSCH-Config. While it may seem logical to associate the number of Tx chains with the number of SRS ports by adding "3 ports" in SRS-Config, this approach may not be suitable because we are not enhancing the SRS resource to support 3 ports in this work item, and dynamic switching of SRS resource size for codebook based operation isn’t supported for 3 Tx. PUSCH-Config, which has configurations that are closely related to SRS-Config, could be a better option. By considering 3 Tx as a variant of transmission using a 4-port SRS, configuring this in PUSCH-Config avoids the need to redefine the SRS. Alternatively, configuring it in SRS-Resource or SRS-ResourceSet could allow for future enhancements.

Proposal 3: RAN2 to  define one RRC parameter to differentiate 3-port SRS from 4-port SRS resource configuration. Discuss further if it is per SRS resource or SRS resource set vs. if it is per PUSCH.

# 4	Asymmetric DL sTRP/UL mTRP

Figure 2 shows examples of the asymmetric DL sTRP / UL mTRP scenario. In such scenario, the DL transmission is from only a single TRP, depicted in the figure as “DL/UL capable TRP”. When using this feature, one or more “UL-only TRPs” may be deployed, potentially close to the cell edge in order to improve coverage. A UE may be scheduled for UL transmissions toward more than one TRP. UL transmissions are scheduled by the DL/UL capable TRP, which is the only TRP sending data and control information to each UE. Moreover, there is no restriction for a certain UE on which TRPs the UE is scheduled for UL transmissions, as we may have with reference to the figure below:

- UE1/UE2 transmitting toward an UL-only TRP and the DL/UL capable TRP,

- UE3 transmitting only toward the DL/UL capable TRP (legacy UE),

- UE4 transmitting only to an UL-only TRP,

- UEs (not depicted in the figure) transmitting to even more than one UL-only TRP.

Figure 2. Asymmetric DL sTRP/UL mTRP scenario

An UL-only TRP exclusively receives signals and does not transmit any DL signals. In the absence of DL PL-RS transmission from an UL-only TRP, estimating the pathloss to the UL-only TRP within the current NR framework is not feasible for determining the UL transmit power needed for transmission to the UL-only TRP. However, a UE can estimate the pathloss to the UL-only TRP by using the pathloss of the Anchor TRP along with a pathloss offset specified by the NW. The NW indicates an uplink UL RSRP difference as a pathloss (PL) offset to assist the UE in estimating the pathloss between itself and the UL-only TRP. This specified PL offset is standard and typically applies to power control for all UL signals/channels (e.g., PUCCH, PUSCH, SRS, PRACH). During the RAN1#116 meeting, it was agreed that the initial PL offset would be conveyed to the UE through RRC signaling. Prior to establishing a connection with the UL-only TRP, the UL-only TRP may lack reliable uplink measurements, thus the NW can only provide the UE with an initial PL offset, ensuring a significant reduction in transmit power towards the UL-only TRP to mitigate potential interference. The UE utilizes this initial PL offset to adjust its initial transmission power to the UL-only TRP. After the first UL signal is received from the UE, the NW can precisely estimate the actual PL offset between the UL-only TRP and the Anchor TRP and update the PL offset through a MAC Control Element (CE) message. The UE should then revise the initially incorrect RRC-configured PL offset according to the updated, more accurate offset received from the NW measurement. In the RAN1#117 the following agreement is made on path-loss offset indication

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

“MAC-CE impact: MAC-CE updates the PL offset value(s) configured in one or more joint TCI states or UL TCI states”

In RAN 1 #118 following agreement is made:

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB - FFS: Extending the range to lower than -10dB - Step size is 4dB

Based on this agreement, we would need 5 bits to represent the path loss offset for the asymmetric DL sTRP/UL mTRP scenario with a range of [−10,60] dB  and a step size of 4 dB. This  PL offset indication will be signaled through a new MAC CE, distinct from existing TCI States Activation/Deactivation MAC CEs (e.g., Unified TCI States Activation/Deactivation MAC CE, Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint or Separate TCI states). This is because associated with each joint /UL  TCI state ID,  a 5-bit path-loss offset also required to be indicated. The reserved bits in the existing MAC CEs are not enough to convey this information.

Proposal 4: Design new MAC CE for path-loss offset indication.

# 5	Conclusion

Proposal 1: Configure and indicate to UE a number of L1-RSRP measurement instances to be used for UEIBM event triggering, i.e., the network configures a counter/time window to control the number of L1-RSRP measurement instances to be averaged before triggering a given event.

Proposal 2: Introduce a prohibit timer that starts when a UEIBM report (on the second UL channel) is sent by the UE, such that the UE is not allowed to send a UEIBM report before the timer has expired when the Event, e.g., Event-2, conditions that triggered the last sent UEIBM report have not changed.

Proposal 3: RAN2 to  define one RRC parameter to differentiate 3-port SRS from 4-port SRS resource configuration. Discuss further if it is per SRS resource or SRS resource set vs. if it is per PUSCH.

Proposal 4: Design new MAC CE for path-loss offset indication.

# References

- R1-2404918 ‘Enhancements to facilitate UE-initiated/event-driven beam management’, Nokia