3GPP TSG-RAN WG2 #127bis	R2-2408650

Hefei,  China, 14th – 18th Oct. 2024

Source:                    	DENSO CORPORATION

Title:  	Discussion on LCP enhancements for XR

Document for:        	Discussion and decision

Agenda Item:         	8.7.4.1		LCP enhancements

- Introduction

In RAN #105, revised WID on Rel-19 XR enhancements was approved [1], and the following objective was specified regarding scheduling enhancements.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

In this contribution, we provide the discussion on LCP enhancements for UL scheduling.

- Discussion

In Rel-18, Delay Status Report (DSR) was introduced to improve UL scheduling for XR services. DSR can provide a serving gNB with delay status including the remaining time and the total amount of delay-critical UL data. Based on the DSR, the gNB can schedule UL resources for UEs which have data with short remaining time to improve capacity. However, according to the current LCP procedure, the UE allocates the resources to each logical channel (LCH) based on its priority without considering the delay status. Therefore, the UE may delay the transmission of the delay-critical data if there is another data from LCHs with a higher priority. As a result, the delay-critical data may be discarded due to the timer expiry, even though the DSR has been reported.

To address the issue, RAN2 agreed that LCH priority adjustment based on the delay status can be supported in Rel-19. The adjustment to the higher priority level can provide more chances to transmit delay-critical data.

RAN2#126 For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to within the LCH or for the whole LCH LCP prioritization within a logical channel will not be considered in RAN2 discussions FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). RAN2#127 FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

One remaining issue would be how to configure an additional priority. Specifically, it is necessary to discuss whether the additional priority can be configured per LCH or in common for all LCHs. Considering the legacy priority configuration, it would be better to configure the additional one per LCH rather than across all LCHs. Furthermore, we need to consider how many additional priorities can be configured per LCH. If multiple additional priorities are introduced, the UE needs to determine which priority to use based on multiple delay status. For example, multiple remaining time thresholds may be needed for LCH priority adjustment, which complicated UE operation. For simplicity, only one additional priority per LCH would be sufficient.

Proposal 1:	Up to one additional priority can be configured for each LCH.

Another remaining issue would be how LCH priority adjustment can be triggered. In our opinion, if there is delay-critical data for a LCH, the additional priority can be used for the LCH instead of the legacy priority. Otherwise, the UE needs to fall back to the legacy priority so as not to prevent resource allocation to other LCHs. The definition of delay-critical data was already discussed in R18 DSR. This definition can be reused for delay-aware LCP. In other words, LCH priority adjustment can be applied if there is data with remaining time below the R18 threshold. In the previous meeting, it has been proposed to introduce a separate remaining time threshold for delay-aware LCP. However, from our perspective, it does not seem to bring many benefits. On the contrary, the separate threshold would have a specification impact in MAC but also in PDCP/RLC. Calculating delay-critical data based on two different thresholds may lead to unnecessary UE complexity.

Proposal 2:	If there is delay-critical data for a LCH, the additional priority can be used for the LCH instead of the legacy priority. Otherwise, the UE needs to fall back to the legacy priority.

Observation 1:	Introducing a separate remaining time threshold for delay-aware LCP would not bring many benefits and may lead to unnecessary UE complexity.

Proposal 3:	There is no need to introduce a separate remaining time threshold for delay-aware LCP. The remaining time threshold configured for R18 DSR can be reused for triggering LCH priority adjustments.

LCH priorities are not only used for LCP procedures but also for other purposes (e.g., BSR trigger or Intra-UE prioritization). Therefore, it needs to be discussed whether the additional LCH priority can be used for those purposes as well. For BSR trigger, there seems to be no advantage to using the additional priority. BSRs should always be triggered based on the legacy priority to avoid complexity.

Observation 2:	LCH priorities are not only used for LCP procedures but also for other purposes (e.g., BSR trigger or Intra-UE prioritization).

Observation 3:	For BSR trigger, there seems to be no advantage to using the additional priority.

Proposal 4:	BSRs should always be triggered based on the legacy priority even if the additional priority is configured.

On the other hand, for intra-UE prioritization, it may be useful to adjust the LCH priority based on the delay status. In intra-UE prioritization, if there are overlapping UL grants, only the UL grant with the highest priority is considered as a prioritized UL grant and can be used for transmission. The priority of each UL grant is determined by the highest priority among priorities of the LCHs that are multiplexed or have data available that can be multiplexed in the MAC PDU. This can lead to the same problem as in the LCP procedures. In other words, an UL grant for which delay-critical data from LCHs with a lower priority are multiplexed may be de-prioritized by another UL grant for which data from LCHs with a higher priority are multiplexed. Therefore, it needs to be considered to apply LCH priority adjustment to intra-UE prioritization as well.

Observation 4:	Intra-UE prioritization has the same problem as LCP procedure. In other words, an UL grant for which delay-critical data are multiplexed may be de-prioritized by another UL grant for which data from LCHs with a higher priority are multiplexed.

Proposal 5:	RAN2 to consider that LCH priority adjustment can also be applied to intra-UE prioritization.

- Summary and proposal

In this contribution, we provided the following observations and proposals.

Proposal 1:	Up to one additional priority can be configured for each LCH.

Proposal 2:	If there is delay-critical data for a LCH, the additional priority can be used for the LCH instead of the legacy priority. Otherwise, the UE needs to fall back to the legacy priority.

Observation 1:	Introducing a separate remaining time threshold for delay-aware LCP would not bring many benefits and may lead to unnecessary UE complexity.

Proposal 3:	There is no need to introduce a separate remaining time threshold for delay-aware LCP. The remaining time threshold configured for R18 DSR can be reused for triggering LCH priority adjustments.

Observation 2:	LCH priorities are not only used for LCP procedures but also for other purposes (e.g., BSR trigger or Intra-UE prioritization).

Observation 3:	For BSR trigger, there seems to be no advantage to using the additional priority.

Proposal 4:	BSRs should always be triggered based on the legacy priority even if the additional priority is configured.

Observation 4:	Intra-UE prioritization has the same problem as LCP procedure. In other words, an UL grant for which delay-critical data are multiplexed may be de-prioritized by another UL grant for which data from LCHs with a higher priority are multiplexed.

Proposal 5:	RAN2 to consider that LCH priority adjustment can also be applied to intra-UE prioritization.

- References

[1]	RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3