3GPP TSG-RAN WG2 #127-bis	R2-2408667

# Hefei, China, 14th – 18th October 2024

Agenda Item:	8.12.2

Source:	Ericsson

Title:	User plane and Control plane impacts from MIMO

Release:	19

Document for:	Discussion

# 1	Introduction

Release 19 MIMO with its justification and objective has been endorsed as one of the features for NR MIMO Phase 5 [1].

In this WI, looking at the objectives:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1

  - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

  - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook

    - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

    - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

For asymmetric DL sTRP/UL mTRP deployment scenarios RAN1 has sent a LS to RAN2 describing that an association between PL Offset and TCI-state has now been agreed for Rel-19 MIMO (Asymmetric mTRP), and that different PL offset values can be configured to separate UL or joint TCI-states. Additionally, a MAC-CE can update the PL Offset value for a TCI state or joint TCI-state (or PL offsets for TCI-states/joint TCI-states). [RAN2 Ref]

RAN1 has also provided an initial list of RRC parameters for MIMO (R1-2407285), with some parameters indicated to be up to RAN2 discussion.

This contribution discusses the user plane and control place impacts of the RAN1 input mentioned above and also consider some RAN1 related agreements w.r.t UE-initiated beam management.

# 2	Discussion

## 2.1	Background

UE-initiated beam management

Ever since Rel-15, NR supports intra-cell and inter-cell mobility. Inter-cell mobility, also known as L3 mobility, relies on event-driven driven reporting, which is used as a basis for HO signalling. The procedures related to intra-cell mobility are called beam management, and involve periodic, semi-persistent and aperiodic beam measurements, which are used as a basis for beam indication signalling from the NW to the UE.

For intra-cell mobility, there is no event-driven reporting specified. Event-driven, or UE-initiated, reporting could reduce the signalling overhead while maintaining reasonably fast discovery of changing propagation conditions resulting from UE movement across the Tx beam pattern. When the UE discovers that the propagation conditions have changed, the UE would notify the NW, and send a report. Based on the report, the NW would send a beam indication to the UE.

RAN1 has agreed to support the following events:

Event-1: Quality of the current beam is worse than a certain threshold.

Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality

RAN1 has defined “current beam” based on the indicated TCI state.

The main motivation for UE-initiated reporting is to reduce overhead, i.e., to avoid transmitting measurement reports that would not result in a beam switch: ideally, each transmitted report should result in a beam switch. This means there must be a mechanism to avoid transmitting reports based on measurement errors or fading dips/peaks. To avoid such spurious reports, RAN1 made the following agreement:

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

Asymmetric DL sTRP UL mTRP

Figure 1 illustrates an asymmetric DL sTRP UL mTRP deployment. It consists of an Anchor TRP (providing both DL and UL service), and a TRP which will be called an UL-only TRP (for ease of reference) to provide UL service, where the downlink reference signals, the downlink control channels, and the downlink data transmissions for a UE are only provided from the Anchor TRP.

Figure 1: UL-only TRP in an asymmetric DL sTRP UL mTRP deployment

The main motivation for asymmetric DL sTRP UL mTRP deployment is to improve UL throughput by UE transmitting UL data to the UL-only TRP. For example, the Anchor TRP in Figure 1 provides coverage with both DL and UL transmissions, and the UL-only TRP is deployed to improve UL performance. Other motivations would be to reduce network energy consumption, by avoid sending downlink data from the UL-only TRP. It is envisioned that a UE in the cell may first perform initial access via the Anchor TRP, and then, if the UE is closer to the UL-only TRP than to the Anchor TRP (i.e. better UL link budget), or if the NW decides to move the UE to an UL-only TRP, e.g., to reduce the UL interference for the Anchor TRP, or to improve resource usage efficiency, the NW may direct the UE to transmit towards the UL-only TRP.

Enhancement on power control is needed to support the asymmetric deployment. In existing NR multi-TRP operation, it is assumed that each TRP can be used for both DL transmission and UL reception. Uplink power control can thus be based on the pathloss measurement of DL PL-RS from the same TRP that provides the DL service. Different pathloss for uplink and downlink arises when the power for uplink transmission towards UL-only TRP is based on, however, pathloss measurement of DL PL-RS from the Anchor TRP.

Apart from power control, an important aspect with asymmetric mTRP deployment is how to set the UL transmission timing towards the Anchor TRP and UL-only TRP respectively. In a practical deployment the propagation delay differences and synchronization timing error between the Anchor TRP and the non-collocated UL-only TRP can be significantly large that will impact the performance of the asymmetric TRP deployment.

## 2.2	User Plane Impacts

### 2.2.3	UE-initiated beam management

Currently multiple beam operation including beam management is an important procedure to support mobility on beam level. The current functionality, like beam failure detection and recovery is a closely related set of PHY/MAC procedures. This includes counters for BFR where L1 provides indications to MAC which MAC uses to determine when beam failure should be declared. That is, currently associated actions for beam management are initiated in MAC in order to maintain TRP(s) and UL beams for DL/UL transmission and reception. In fact, the beam failure recovery procedure is a component of beam management itself.

The RAN1 agreement for Rel-19 beam management (see 2 Background above), when looking at the Event 2 basic function, where a better beam trigger is based on a counter, it seems natural that also that triggering event for the associated L1 indications can be captured in MAC similarly as e.g. beam failure in legacy etc.

In order to keep this existing separation in core specification, we propose:

The event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification.

RAN2 to inform RAN1 on its preference of specifying the triggering event determination for Event 2.

### 2.2.3		MAC-CE update of PL Offset value for Asymmetric DL sTRP UL mTRP

In RAN1 a PL Offset has been introduced to be configured per TCI State. This PL Offset is associated with one of the indicated joint/UL TCI State for the UL TRP in the unified TCI framework and is applied PDCCH order PRACH transmission. Here, details of the design is still to be defined.

As stated in the LS[ref], MAC-CE is assumed to be used to update the PL Offset value for a UL TCI state (or PL offsets for TCI-states/joint TCI-states). Note that the RAN1 agreement pointed out also assumes that the asymmetric DL sTRP/UL mTRP deployment scenario reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework.

In the details of the RAN1#117 meeting RAN1 concluded that:

--

For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework:

- When rel-17 unified TCI/ICBM is configured:

  - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied.

  - For FR2: {one DL TCI state + one UL TCI state} can be applied.

- When rel-18 unified TCI is configured:

  - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied.

  - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

--

To incorporate the associating PL offset with a TCI state for the asymmetric DL sTRP/ UL mTRP deployment RAN2 need to note what can be applied to FR1 and FR2. From the above agreement we see that when in Rel-18 and unified TCI is configured, for FR2, {one DL TCI state + up to two UL TCI states} can be applied.  As the agreement also is based on the reuse of the (Rel-17 and) Rel-18 unified TCI framework, and as the Rel-18 TCI framework currently only supports pairs of UL/DL TCI states (38.214, section 5.1.5), we assume here that for this case (FR2, when Rel-18 unified TCI is configured) and related to asymmetric DL sTRP/UL mTRP, " {one DL TCI state + up to two UL TCI states} can be applied “ then means that two separate DL/UL TCI state pairs can be configured and indicated, but a TCI codepoint associated with one DL TCI state is only used updating one of the two indicated DL TCI states for mTRP operation. As this interpretation may impact the MAC CE design, it can be useful though to have this understanding confirmed by RAN1.

RAN2 to confirm the detailed application of indicated TCI States in Rel-19 with RAN1

Currently, Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate or Unified TCI States are used for indicating Activation/Deactivation of TCI state(s) for PDCCH reception for a CORESET of a Serving Cell or a set of Serving Cells [38.321, 6.1.3.71]. As seen from the LS from RAN1, is that the PL offset can be associated to an UL TCI state, and that the UE should determine the output power for transmitted UL signals/channels associated with the UL TCI state by applying this PL offset to the open loop power control. As a result, upon activation/deactivation of TCI States, the legacy TCI States Activation/Deactivation MAC CE CEs could be reused by incorporating also an associated PL Offset.

The legacy Enhanced Unified TCI States Activation/Deactivation MAC CE could be extended for signalling of PL Offset (and PL RS)

A resulting MAC CE example using The Enhanced Unified TCI States Activation/Deactivation MAC CE CE for Separate TCI States can be found in fig. x. The Ti in this example would indicate the PL Offset for a TCI state with TCI-StateId “i” (c.f. 38.331) if needed. If the PL Offset are statically mapped to the TCI state, those may not be needed. Additionally, the number of bits for the PL Offset needs to be considered, i.e the granularity needed for the PL Offset signaling. The corresponding examples on additions using other legacy MAC-CE formats would apply.

Fig. 2

In the above, adding the PL Offset and additional fields to the TCI Activation/deactivation MAC CE results however in several new MAC CE formats for the UE to support along with possible large MAC CE sizes. Considering also that the PL Offset is initially RRC configured, it seems natural to agree to a base line that defines the PL Offset outside of the MAC CEs used for activation/deactivation of TCI States.

A PL Offset update MAC CE would have lower impact on MAC CE signalling.

Example of MAC CE to update PL offset for activated TCI states for mTRP operation:

Fig 3

The Ti in this example indicate if the UL or Joint TCI state with TCI-StateId “i” (c.f. 38.331) should have an updated pathloss offset or not. The format can of course be optimized, e.g. using R-bits, although a base line could be to represent a number N equal to the maximum number of Joint/UL TCI states configured. This also since it is yet not clear currently, what limitation exist in how many PL Offsets the UE will be able to maintain etc. For example, if the number of PL Offset is signalling is two (2), a MAC CE resulting example could be as in Fig x+3. Additionally, if the required bits for the PL Offset is lower, the TCI-StateId fields can be included in the PL Offset directly (not in example).

PL Offset update signalling MAC CE(s) is defined that includes a TCI-State Id association to activated joint/TCI States.

Fig 4

In the Unified TCI state framework, a path loss reference (PL-RS) can be associated with an UL TCI state through RRC configuration. The UE use this PL-RS associated with an UL TCI state when transmitting UL signals/channels associated with that UL TCI state. In an asymmetric deployment it is however assumed that uplink power control is based on the pathloss measurement using a DL PL-RS that may be the same as for a reference TRP; for instance the joint/UL TCI State of the TRP that provides the DL service. Considering mobility also in asymmetric Rel-19 deployments, the DL-TCI state towards the current serving TRP may thus need to be updated frequently. Similarly, the pathloss reference signal and PL Offset that the UE uses may also need to be updated frequently. Since the pathloss reference signal for the UL TCI state targeting the UL-only TRP is currently assumed to be RRC configured, this may require RRC signalling to frequently update the pathloss reference signal towards the serving TRP. As a result, the MAC CE used for updating the PL Offset may need to also include an association to a PL-RS.  It is however not clear yet in RAN1 what the details are w.r.t PL-RS, i.e. to what combination of e.g. PL offset, the PL offset association to an UL TCI state (UL) and joint DL TCI state (serving TRP) while the PL-RS is related to the joint TCI state of a serving TRP. Here, we suggest waiting for more progress in RAN1. The addition to MAC CE signalling can be rather straightforward and like that of PL Offsets, see example in fig 5.

Fig 5 MAC CE to update two pathloss offsets, and one single DL PL RS (common to the pathloss offsets)

The MAC CE used for updating the PL Offset, may need to include an association to (a) PL-RS as mobility may result in that the pathloss reference signal and PL Offset that the UE uses need to be update frequently

RAN2 to study MAC CE signalling options for PL Offset update while waiting with detailed design until RAN1 progress related topics sufficiently.

### 2.2.3	PHR

In asymmetric DL sTRP/ UL mTRP deployment an UL TRP is receive only and hence does not transmit any DL signals. Without a DL PL-RS (reference) transmission, the pathloss to the UL-only cannot be estimated to determine the UL transmit power for the transmission towards the UL-only TRP. However, it is possible for a UE to determine the pathloss based on the pathloss of the serving (DL) TRP using the pathloss offset (PL∆ [dB]) indicated by the NW (plOffset configured and subsequently subject to update through MAC CE, see previous section).

We can find the corresponding agreement in RAN1

--

RAN1 Agreements:

To support at least Type-1 PHR.

Proposal 1.3A:

For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH

Proposal 1.3B:

Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states.

- Continue to study whether to support including PL offset in the calculation of Type 3 PHR.

--

UL/DL TRP

UL only TRP

UE

PUSCH/PUCCH/SRS

DL-RS for path loss calculations

PL0

PL1

The current formula is thus updated to incorporate the determination for pathloss so that:

P=P0 +  α*(PLDL_RS+PL∆)+ PCL

PL∆=PL1-PL0

Fig. 6

Potential changes/impact to 38.321:

Concerning impact in 38.321, the PHR trigger has impact from the addition of a PL Offset. In 38.321 the current trigger includes a change to pathloss more than PowerFactorChange. For Rel-19, either a new threshold configuration may be needed, e.g.  “phr-Tx-PowerFactorChange-Asym” should there be a need for different threshold values, or if not, a possibility is to use a conditional presence of the PL Offset {plOffset} as different PL offset values may be configured to TCI-states acc. to the RAN1 agreement for asymmetric mTRP deployment, and due to that the triggering calculation is based on the use on the serving DL/UL TCI State and the corresponding reference PL-RS. That is, if plOffset is configured for an associated PL-RS, the path loss calculation follows the updated formula that includes a PL Offset (according to the configuration, or as updated with MAC CE signalling) and this path loss calculation result is then measured toward a PowerFactorChange threshold.

As RAN1 have not yet decided if other trigger then for PHR Type 1 is to be supported for UL Only, as well as details w.r.t the configurable combinations that include DL sTRP or UL mTRP scenarios (e.g. twoPHRMode, multipanelSchemeSDM, multipanelSchemeSFN etc), RAN2 should wait with the detailed actual procedure impacts. However, it may be beneficial to know to what extent or if the current trigger and conditions can be maintained from a RAN2 p.o.v. Below find some initial examples how the above could be specified.

Assume at least legacy triggers and thresholds can be reused for asymmetric mTRP.

Ran2 to wait with procedural updates until RAN1 concludes on PHR reporting.

38.321 (with using plOffset, this part can be left unchanged, :
phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB if configured for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

--

The paragraph above updates depends on the reference (RS) but as the plOffset has a reference to a UL/DL joint TCI State and also an explicit PL RS, the current wording may not need any update, or possibly very little changes. The preceding Note/Text in 38.321 however may need some update (or an additional new Note to be added) to clarify the use of the reference PL-RS related to the serving joint TCI State and not the UL only TRP:

--

38.321 preceding Note:

NOTE: 1 The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference (this would be the PL_DL_RS0)  and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, or based on a reference PUSCH on the pathloss reference in use at the time of transmission (this could be a PL_DL_RS1) if the TCI-State is configured with plOffset.

.--

## 2.3	Control plane impacts

An LS was received from RAN1 with RRC parameters for MIMO (R1-2407285).

This section discusses the impacts related to the LS.

### 2.3.1	UE Initiated Beam Reporting

The following parameter was left up to RAN2:

| enabledCurrentBeamReport-r19 | New |  | This parameter is to enable or disable whether current beam is always reported. - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported l - Note: The reported current beam is NOT counted in the N reported beams. - When disabled by RRC, N beams are reported. | Up to RAN2 |
| --- | --- | --- | --- | --- |

Since the parameter should have an enable/disable behaviour, this can be achieved in RRC by having an optional need R field where presence means the feature is enabled and absence means the feature is not enabled/released.

enabledCurrentBeamReport-r19 is defined as an optional need R field.

### 2.3.2	CSI-Type-I/II

For CSI-Type-I/II, multiple parameter details were left up to RAN2. Firstly, group based CBSR is described as below for CSI-typeI:

| n1-n2-typeI-r19 | new |  | Supported values of (N1,N2): 48 ports: (8,3), (6,4) 64 ports: (16,2), (8,4) 128 ports: (16,4), (8,8) | up to RAN2 |
| --- | --- | --- | --- | --- |
| typeI-CBSR-r19 | new |  | Group-based CBSR (hard restriction only): (N1O1N2O2)/(X1X2)-bit CBSR where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD basis vectors along the N2 direction - The CBSR is optional | up to RAN2, Bitmap size depends on (N1,N2) and (X1,X2) |
| valueOfX1-typeI-CBSR-r19 | new |  | X1 from one of the supported values of (X1,X2): (1,1), (2,1), (2,2), (4,1), (4,2), (4,4) | 1,2,4 |
| valueOfX2-typeI-CBSR-r19 | new |  | X2 from one of the supported values of (X1,X2): (1,1), (2,1), (2,2), (4,1), (4,2), (4,4) | 1,2,4 |

As already done for Rel-18, the CBSR configuration can be embedded in the N1-N2 configuration e.g.:

n1-n2-codebookSubsetRestriction-r19                 N1-N2-CBSR-r19

N1-N2-CBSR-r19 ::=     CHOICE {

eight-three-r19            CHOICE {no-cbsr-r19 NULL, X1-X2-Restriction1-r19},

six-four-r19               CHOICE {no-cbsr-r19 NULL, X1-X2-Restriction1-r19},

sixteen-two-r19            CHOICE {no-cbsr-r19 NULL, X1-X2-Restriction2-r19},

eight-four-r19             CHOICE {no-cbsr-r19 NULL, X1-X2-Restriction2-r19},

sixteen-four-r19           CHOICE {no-cbsr-r19 NULL, X1-X2-Restriction3-r19},

eight-eight-r19            CHOICE {no-cbsr-r19 NULL, X1-X2-Restriction3-r19}

}

To account for the X1-X2 grouping, different IEs can be used based on the bit string size required for each (N1, N2) pair e.g.:

X1-X2-Restriction1-r19 ::=     CHOICE {

one-one-r19            BIT STRING (SIZE (384)),

two-one-r19            BIT STRING (SIZE (192)),

two-two-r19            BIT STRING (SIZE (96)),

four-one-r19           BIT STRING (SIZE (96)),

four-two-r19           BIT STRING (SIZE (48)),

four-four-r19          BIT STRING (SIZE (24))

}

X1-X2-Restriction2-r19 ::=     CHOICE {

one-one-r19            BIT STRING (SIZE (512)),

two-one-r19            BIT STRING (SIZE (256)),

two-two-r19            BIT STRING (SIZE (128)),

four-one-r19           BIT STRING (SIZE (128)),

four-two-r19           BIT STRING (SIZE (64)),

four-four-r19          BIT STRING (SIZE (32))

}

X1-X2-Restriction3-r19 ::=     CHOICE {

one-one-r19            BIT STRING (SIZE (1024)),

two-one-r19            BIT STRING (SIZE (512)),

two-two-r19            BIT STRING (SIZE (256)),

four-one-r19           BIT STRING (SIZE (256)),

four-two-r19           BIT STRING (SIZE (128)),

four-four-r19          BIT STRING (SIZE (64))

}

For CSI-Type-I, CBSR configuration is embedded in the N1-N2 configuration. X1-X2 grouping is defined for each N1-N2 configuration.

For power scaling, a similar X1-X2 group handling was defined in RAN1:

| typeI-powerScaling-r19 | new |  | 3-bit scaling factor for each group of X1X2: 3(N1O1N2O2)/(X1X2)-bit power scaling factors in total - the power scaling scheme is optional | up to RAN2, Bitmap size depends on (N1,N2) and (X1,X2) |
| --- | --- | --- | --- | --- |
| valueOfX1-typeI-powerScaling-r19 | new |  | X1 from one of the supported values of (X1,X2): (2,1), (2,2), (4,1), (4,2), (4,4), (8,1) | 2,4,8 |
| valueOfX2-typeI-powerScaling-r19 | new |  | X2 from one of the supported values of (X1,X2): (2,1), (2,2), (4,1), (4,2), (4,4), (8,1) | 1,2,4,8 |

A similar structure as for CBSR grouping can be used, except that there is no need to configure N1-N2 again, and thus the choice structure within each field of N1-N2-CBSR-r19 is not needed.

typeI-powerScaling-r19                              TypeI-N1-N2-PowerScaling-r19           OPTIONAL  -- Need R

TypeI-N1-N2-PowerScaling-r19 ::=     CHOICE {

eight-three-r19            X1-X2-PowerScalingGroup1-r19,

six-four-r19               X1-X2-PowerScalingGroup1-r19,

sixteen-two-r19            X1-X2-PowerScalingGroup2-r19,

eight-four-r19             X1-X2-PowerScalingGroup2-r19,

sixteen-four-r19           X1-X2-PowerScalingGroup3-r19,

eight-eight-r19            X1-X2-PowerScalingGroup3-r19

}

X1-X2-PowerScalingGroup1-r19 ::=     CHOICE {

two-one-r19            BIT STRING (SIZE (576)),

two-two-r19            BIT STRING (SIZE (288)),

four-one-r19           BIT STRING (SIZE (288)),

four-two-r19           BIT STRING (SIZE (144)),

four-four-r19          BIT STRING (SIZE (72)),

eight-one-r19          BIT STRING (SIZE (144))

}

X1-X2-PowerScalingGroup2-r19 ::=     CHOICE {

two-one-r19            BIT STRING (SIZE (768)),

two-two-r19            BIT STRING (SIZE (384)),

four-one-r19           BIT STRING (SIZE (384)),

four-two-r19           BIT STRING (SIZE (192)),

four-four-r19          BIT STRING (SIZE (96)),

eight-one-r19          BIT STRING (SIZE (192))

}

X1-X2-PowerScalingGroup3-r19 ::=     CHOICE {

two-one-r19            BIT STRING (SIZE (1536)),

two-two-r19            BIT STRING (SIZE (768)),

four-one-r19           BIT STRING (SIZE (768)),

four-two-r19           BIT STRING (SIZE (384)),

four-four-r19          BIT STRING (SIZE (192)),

eight-one-r19          BIT STRING (SIZE (384))

}

For CSI-Type-I power scaling, X1-X2 grouping is defined for each N1-N2 configuration.

For ng-n1-n2, the L1 parameter tables states the following:

| ng-n1-n2-r19 | new |  | Supported values of (Ng, N1,N2): 48 ports: (2,4,3), (2,6,2),(2,12,1) 64 ports: (2,8,2), (2,16,1), (4,4,2), (2,4,4), (4,8,1) 128 ports: (4,4,4), (4,16,1), (4,8,2) | up to RAN2, (direct extension of the legacy design of ng-n1-n2 in R15 typeI-multiPanel) |
| --- | --- | --- | --- | --- |

As already stated in the L1 parameter table above, this parameter should be an extension of the legacy ng-n1-n2 in R15 typeI-multiPanel, hence the same structure can be used e.g.:

typeI-MultiPanel-r19                                    SEQUENCE {

ng-n1-n2                                                CHOICE {

two-four-three-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (192)),

two-six-two-TypeI-MultiPanel-Restriction                 BIT STRING (SIZE (192)),

two-twelve-one-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (48)),

two-eight-two-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (256)),

two-sixteen-one-TypeI-MultiPanel-Restriction             BIT STRING (SIZE (64)),

four-four-two-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (128)),

two-four-four-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (256)),

four-eight-one-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (32)),

four-four-four-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (256)),

four-sixteen-one-TypeI-MultiPanel-Restriction            BIT STRING (SIZE (64)),

four-eight-two-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (256))

}

}

The field ng-n1-n2-r19 is defined in the same way as ng-n1-n2 in R15 typeI-multiPanel.

For CSI-typeII, a group-based CBSR was also defined:

| n1-n2-typeII-r19 | new |  | Supported values of (N1,N2): 48 ports: (8,3), (6,4) 64 ports: (16,2), (8,4) 128 ports: (16,4), (8,8) | up to RAN2 |
| --- | --- | --- | --- | --- |
| typeII-CBSR-r19 | new |  | Group-based CBSR (hard restriction only): (N1N2)/(X1X2)-bit CBSR where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD basis vectors along the N2 direction - The CBSR is optional | up to RAN2, Bitmap size depends on (N1,N2) and (X1,X2) |
| valueOfX1-typeII-CBSR-r19 | new |  | X1 from one of the supported values of (X1,X2): (1,1), (2,1), (4,1) for (N1,N2)=(8,3), (1,1), (2,1), (2,2) for (N1,N2)=(6,4), or (1,1), (2,1), (2,2), (4,1), (4,2) for (N1,N2)=(16,2), (8,4), (16,4), or (8,8) | 1,2,4 |
| valueOfX2-typeII-CBSR-r19 | new |  | X2 from one of the supported values of (X1,X2): (1,1), (2,1), (4,1) for (N1,N2)=(8,3), (1,1), (2,1), (2,2) for (N1,N2)=(6,4), or (1,1), (2,1), (2,2), (4,1), (4,2) for (N1,N2)=(16,2), (8,4), (16,4), or (8,8) | 1,2 |

All N1/N2 values and X1/X2 values above are contained within those of CSI-typeI. Hence the same IEs defined for CSI-typeI can be used for CSI-typeII. However, CSI-typeII has further restrictions as described above i.e. X2 values can only include 1 or 2 (while CSI-typeI can have value 4) and not all X1, X2 groups are applicable to all N1, N2 configurations – such restrictions can be clarified in field description.

The same IE defined for CBSR CSI-typeI is also used for CBSR CSI-typeII.

### 2.3.3	CSI-CRI

It was left up to RAN2:

| cri-typeI-SinglePanel-ri-restriction-r19 | New |  | resource-specific RI restriction (8-bit bitmap per resource, where the 8-bit bitmap is similar to (or the same as) typeI-SinglePanel-ri-restriction) , Rel-15 Type-I-based | up to RAN2 |
| --- | --- | --- | --- | --- |
| cri-typeII-ri-restriction-r19 | New |  | resource-specific RI restriction (4-bit bitmap per resource, where the 4-bit bitmap is similar to (or the same as) typeII-ri-restriction) , Rel-16 eType-II based | up to RAN2 |

Those can be defined as legacy RI restrictions as also indicated in the L1 parameter list. They can also be bundled in a choice structure e.g.:

cri-RI-Restriction-r19              CHOICE {

typeI-SinglePanel-RI-Restriction-r19    BIT STRING (SIZE (8)),

typeII-r19-RI-Restriction-r19           BIT STRING (SIZE (4))

}                                                                                                           OPTIONAL   -- Need R

The fields cri-typeI-SinglePanel-ri-restriction-r19 and cri-typeII-ri-restriction-r19 are defined in the same way as typeI-SinglePanel-ri-restriction and typeII-ri-restriction, respectively.

### 2.3.4	CSI-CJTC

It was left up to RAN2:

| referenceAntennaPort | New |  | The UE antenna port for receiving the CSI-RS configured for phase offset measurement that is same as the UE antenna port(s) for transmitting the configured ports from the associated SRS resource | up to RAN2 |
| --- | --- | --- | --- | --- |
| subbandSize | New |  | Supported sub-band size(s) {1, 2, 4, 8, 16} PRB for SB PO reporting | {1,2,4,8,16} |
| po-ReportingBand | New |  | configuration of which NSB-P sub-band(s) that the UE reports | up to RAN2 |

It should be noted that referenceAntennaPort is still under discussion in RAN1, and thus there is no need to discuss it in RAN2 at this point. Similarly, po-ReportingBand is also still under discussion in RAN1 and can be postponed.

### 2.3.5	Asymmetric MTRP

It was left up to RAN2:

| plOffset | New |  | This parameter is used to inform pathloss offset value applied to the joint or UL TCI state. | up to RAN2 |
| --- | --- | --- | --- | --- |

For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB

- FFS: Extending the range to lower than -10dB

- Step size is 4dB

The values applicable for plOffset were agreed in RAN1 in RAN1#118:

It is clearer to simply add the applicable dB values in RRC. (if RAN1 decides to extend those values the field values can be updated as well).

plOffset ::=                     ENUMERATED {dB-10, dB-6, dB-2, dB2, dB6, dB10,

dB14, dB18, dB22, dB26, dB30, dB34, dB38,

dB42, dB46, dB50, dB54, dB58, dB60}

The applicable dB values for plOffset are defined in an ENUMERATED structure.

### 2.3.6	3Tx UL MIMO

The RRC parameter “[4portSRS_3TX]” is shown in the RAN1 LS on MIMO RRC parameters R1-2407285 as conditional upon whether the proposal in the comment field was agreed, as can be seen in the LS excerpt below. However, the agreement was made in RAN1#118, and so the parameter will be specified.  Exactly how it should be defined from a RAN2 perspective requires further discussion.

| Parameter name in the spec | New or existing? | Description | Value range | Comment |
| --- | --- | --- | --- | --- |
| [4portSRS_3TX] | New | [1 bit RRC parameter to indicate that the configured 4-port SRS resource is configured for a 3TX UE] | single bit | Note: The parameter is conditioned on endorsement of the following proposal,[Proposal: For a UE reporting a UE capability of 3TX operation, support introduction of a new RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled.] |

The Rel-19 MIMO WID objectives for 3 Tx (copied below) preclude enhancing SRS resources in order to minimize the specification impact and effort to implement the 3Tx feature.  Therefore, no 3 port SRS was defined; instead, a 4 port SRS resource is used and port 1003 (that can carry SRS and/or PUSCH) is disabled.  This means that the “[4portSRS_3TX]” parameter is configured together with a 4 port SRS to indicate that the UE should use 3 Tx UL MIMO, including transmitting only 3 of the 4 SRS ports, not transmitting PUSCH on port 1003, and using the 3 Tx codebook.

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

An SRS resource with nrofSRS-Ports = ports4 and the 3 Tx UL MIMO parameter  “[4portSRS_3TX]” should be both be configured to enable 3 Tx UL MIMO operation

The parameter “[4portSRS_3TX]” determines whether port 1003 is disabled for both PUSCH and SRS.

RAN1 did not suggest a parent IE for “[4portSRS_3TX]”, but since the parameter describes an SRS resource configuration, it should be a field within SRS-Resource, and not a value of (a Rel-19 version of) nrofSRS-Ports. RAN1 suggested a value range of 1 bit for “[4portSRS_3TX]”, however our understanding of the intention is to say if 3 Tx operation is configured or not, so the value range should be “ENUMERATED { enabled }”.

The L1 parameter [4portSRS_3TX] should be a field within SRS-Resource defined as “ENUMERATED { enabled }”. This field is only configured if nrofSRS-Ports is set to “ports4”.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The legacy Enhanced Unified TCI States Activation/Deactivation MAC CE could be extended for signalling of PL Offset (and PL RS)

Observation 2	A PL Offset update MAC CE would have lower impact on MAC CE signalling.

Observation 3	The MAC CE used for updating the PL Offset, may need to include an association to (a) PL-RS as mobility may result in that the pathloss reference signal and PL Offset that the UE uses need to be update frequently

Observation 4	An SRS resource with nrofSRS-Ports = ports4 and the 3 Tx UL MIMO parameter  “[4portSRS_3TX]” should be both be configured to enable 3 Tx UL MIMO operation

Observation 5	The parameter “[4portSRS_3TX]” determines whether port 1003 is disabled for both PUSCH and SRS.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification.

Proposal 2	RAN2 to inform RAN1 on its preference of specifying the triggering event determination for Event 2.

Proposal 3	RAN2 to confirm the detailed application of indicated TCI States in Rel-19 with RAN1

Proposal 4	PL Offset update signalling MAC CE(s) is defined that includes a TCI-State Id association to activated joint/TCI States.

Proposal 5	RAN2 to study MAC CE signalling options for PL Offset update while waiting with detailed design until RAN1 progress related topics sufficiently.

Proposal 6	Assume at least legacy triggers and thresholds can be reused for asymmetric mTRP.

Proposal 7	Ran2 to wait with procedural updates until RAN1 concludes on PHR reporting.

Proposal 8	enabledCurrentBeamReport-r19 is defined as an optional need R field.

Proposal 9	For CSI-Type-I, CBSR configuration is embedded in the N1-N2 configuration. X1-X2 grouping is defined for each N1-N2 configuration.

Proposal 10	For CSI-Type-I power scaling, X1-X2 grouping is defined for each N1-N2 configuration.

Proposal 11	The field ng-n1-n2-r19 is defined in the same way as ng-n1-n2 in R15 typeI-multiPanel.

Proposal 12	The same IE defined for CBSR CSI-typeI is also used for CBSR CSI-typeII.

Proposal 13	The fields cri-typeI-SinglePanel-ri-restriction-r19 and cri-typeII-ri-restriction-r19 are defined in the same way as typeI-SinglePanel-ri-restriction and typeII-ri-restriction, respectively.

Proposal 14	The applicable dB values for plOffset are defined in an ENUMERATED structure.

Proposal 15	The L1 parameter [4portSRS_3TX] should be a field within SRS-Resource defined as “ENUMERATED { enabled }”. This field is only configured if nrofSRS-Ports is set to “ports4”.

# 4	References

- RP-242394 NR MIMO Phase 5, Samsung (Moderator), RAN meeting #102 Scotland, December 11-15, 2023