3GPP TSG-RAN WG2 #127-bis	R2-2408668

Hefei, China, 14th – 18th October 2024

Agenda Item:	8.2.2

Source:	Ericsson

Title:	Functionality aspects for A-IoT

Document for:	Discussion/Decision

# 1	Introduction

This paper discusses aspects on the functionality required for A-IoT devices, including security related questions to SA3, the need for resource allocation, segmentation/reassembly (pending RAN1 progress), QoS handling, higher layer repetition, BSR and SR. Note that in parts were referring to “Reader”, this would apply to either a gNB or an Intermediate node as Reader, described in SID and elsewhere.

# 2	Discussion

## 2.x	Registration and Device Enabling/Disabling

Itis beneficial to have registration for A-IoT devices, if they are capable. Keeping device context helps avoid the need of setting up security, authentication, device ID allocation every time device accesses the network. Thus, we assume that A-IoT devices need to register to networkto get authorized to receive services associated with considered use cases and there is device context with a minimal set of key information established and stored at network and device side.

It is preferable to register device context information for subsequent communication as there is no need to perform identification, authentication, security setup every time when a service procedure (e.g., inventory or command) is initiated.

RAN2 assumes the existence of “registration procedure (e.g., authorization and authentication)” catering for A-IoT requirements on identification, authorization, and authentication.

The discussion in RAN2 has mainly been focusing on the two use cases of inventory and command. However, RAN2 impact from A-IoT involves other procedures including disabling devices and registration of devices, both of which are part of the key issue 2 of SA2 study (as in TR23.700-13). It is thus proposed that RAN2 study how to support procedures beyond inventory and command procedures including disabling and registration procedure.

RAN2 to study how to support other procedures than inventory and command over A-IoT air interface including registration and device enabling/disabling.

As for the registration procedure, from a RAN2’s perspective, similar to legacy 3GPP systems, it can be assumed that there is a need for signaling procedures over air interface to support device identification reporting from A-IoT device, provisioning of temporary CN allocated device ID, possible security information/setup including authentication. For example, RAN2 should ensure the three-step baseline procedure being defined in current TR38.760-13, clause 6.3 covers procedure steps for a registration procedure. The same applies to the device enabling/disabling.

RAN2 to discuss if the overall AS procedure (three-step baseline, in current TR38.760-13) can support procedure steps for registration and device enabling/disabling.

RAN2 can assume registration can be designed to be standalone or in connection to inventory or command procedures to reduce signaling overhead. For example, Figure 1 below shows an example of registration procedure which is performed in connection to the initial inventory procedure for reduced signaling.

RAN2 assumes registration can be performed standalone or in connection with inventory procedure.

RAN2 assumes CN determines trigger of registration based on device capability.

Figure 1: Message flow for registration procedure (in connection with inventory procedure)

Steps 1-6: Similar to inventory only procedure. However, there may need an indication in A-IoT paging to indicate to target devices to perform registration in connection with the inventory, if they are capable. We think this is applicable to other procedures/use cases as well.

As an example, device can report in step 5 its registration capability, which helps CN to determine if the registration in next steps 7-10 can be performed for the device. However, CN can also check device capability from e.g., UDM based on device ID provided in step 5. Hence, the registration procedure is not mandatory for all A-IoT devices, i.e., steps 7-10 are in dashed lines and textboxes.

Step 7: In case the device is registration capable, the CN perform authentication, temporary device ID allocation and security determination.

Step 8: the gNB/UE reader and device perform relevant procedures over air interface to exchange upper layers (e.g., A-IoT NAS) information related to authentication, security setup, and temporary device ID provisioning.

Step 9: the CN registers device information (e.g., into UDM).

Step 10: the device, the gNB/intermediate UE, CN store device context information for subsequent communications. As described in command only procedure, the temporary device ID can be used to start a subsequent command procedure and security can be used to protect command data without establishing/setup again.

Target device is indicated type of procedure the A-IoT paging is intended for (e.g., inventory, command, registration, enabling/disabling, etc.).

Send a LS to SA2, SA3 and CT1 asking for feedback while indicating the above statements captured in P1, P4, P5.

## 2.1	Protocol stack

In the RAN2#125bis, it was agreed that SDAP, PDCP, RRC and RLC will not be supported for A-IoT as in NR; nevertheless, it is still FFS whether and how functionalities typically associated to these layers should be supported. Specifically, AS security and segmentation.

In our understanding, this does not mean that functionality provided by the currently excluded protocol layers (c.f. NR), if needed, would necessarily be supported by MAC. Additionally, a separate layer may be defined, as features are FFS pending agreements in RAN2 and in other working groups (RAN1, SA3 and other).

Previous agreement on not supporting SDAP, PDCP, RRC, RLC does not prevent the definition of a new AS protocol layer in the protocol stack above MAC.

The resulting baseline protocol stack is shown in Figure 1.

Figure 1: Protocol stack for A-IoT

Specifically, regarding RRC layer, it was agreed that connection management, periodic System Information and that A-IoT devices will not support ASN.1 encoding/decoding.

If we assume that MAC will handle the set of legacy MAC functions which are also applicable for A-IoT (denoted as “MAC functions”) (e.g.: random access procedure, handling of logical channels or padding), there are still several functions for which a responsible entity needs to be defined.

The functions (denoted as “Device Control” functionality) currently may be:

- Handling of the inventory procedure at RAN level (i.e.: triggering multiple RA procedures, selecting the appropriate configuration for it, and determining when to terminate the inventory procedure)

- Delivery of PDUs from/to upper layers (e.g.: NAS)

- Control signalling to one group or a specific device (as needed)

- Handling of address (if needed, see Section 2.4)

In principle these “Device Control” functions could be incorporated with the MAC layer or alternatively separated into a different, upper layer, similar to what the RRC layer is in legacy NR. Both options have pros and cons that should be studied in RAN2.

Both “Device Control” and “MAC” functions will require control elements. In legacy MAC they are encoded through LCID code points (c.f. MAC CE), and in legacy RRC they are handled by ASN.1 encoding. As a result, we can imagine the method used for the “Device Control” signalling will be similar to the one used in MAC since ASN.1 UPER encoding will not be supported. If the number of needed “MAC” and “Device Control” control elements will be m and n respectively, having them in separate layers will require an overhead of at least log2m+1+log2n bits to encode them (+1 to handle a dedicated logical channel directed to the upper layer), while if they are all handled by the same layer, they would require an overhead of log2m+n. In general, handling control elements in a single layer will require a smaller number of bits for header/encoding.

Merging the “MAC” and “Device Control” functionalities in a single layer implies a smaller overhead in terms of encoding of control elements.

In terms of complexity, having many functions in the MAC will increase its complexity. Since this would cause a considerable re-design of MAC layer on network side as functions that typically are handled by other layers would be placed into MAC, whereas separating functional responsibility could simplify the design and specification of layers.

Merging the “MAC” and “Device Control” functionality in a single layer implies a higher complexity in terms of modelling and lower reusability of network-side implementation of such layer.

Study the pros and cons of merging “MAC” and “Device Control” functions into a single layer or in multiple layers.

## 2.2	Scheduling Request, Segmentation, and other indications

In previous meetings it was agreed that legacy NR BSR/SR will not be supported for A-IoT as well as any form of upper layer ARQ-like retransmission while it is still FFS how to handle segmentation depending on RAN1 agreements.

Nevertheless, the maximum supported TBS from the PHY layer in various coverage scenarios should still be determined. In most cases it is fine to assume that the UL data has a constant or at most predictable size so that the reader can allocate the appropriate TBS to complete the transmission without the need for segmentation, but it is unlikely that this mechanism will be sufficient in all possible scenarios. For instance, in the DL command case, the device reply may have unexpected size due to error on application side, or, depending on RAN1 agreements, it is possible that the maximum TBS that can be achieved in poor coverage conditions cannot satisfy all possible use cases.

Therefore, it is wise to consider a segmentation mechanism or a method for the device to request more resources to the reader for D2R transmission (similar to Scheduling Request).

Segmentation and/or scheduling request mechanisms are needed to satisfy all possible use cases depending on RAN1 agreements on maximum TBS at cell edge or SA2 agreements on maximum UL data size.

The complexity of this mechanism should be reduced to the minimum also in terms of control signaling overhead for obvious reasons.

In legacy NR there are several methods to implement behaviors coherent with the problem under exam:

- Scheduling Request (SR): a special PHY layer message, carried over PUCCH, that simply request an UL Grant to the gNB. If RRC Connection is not established the UE would start a RACH instead

- Buffer Status Report (BSR): a MAC CE with which a UE can inform the gNB on the current amount of pending data with more or less granularity. The gNB can then decide whether or not to provide an UL grant and how large.

- Segmentation: a RLC SDU can be split in multiple PDUs. Two control fields keep track of whether the PDU contains an entire SDU or the first, last or neither first nor last segment (SI field, 2 bits) and the start and final offset in the overall SDU of a specific PDU (SO field, 16 bit + 16 bit)

For A-IoT the most common scenario will be that the first UL grant provided by the reader is sufficient to deliver the whole SDU. For those cases in which the grant is not enough, the device should ask for further resources.

This can be achieved with a simple 1-bit indication in the simplest case with the function of a combined SR and BSR: “there is at least one more segment to be sent”. Note that this includes the “message size reporting” that is described in the latest agreements. Simply this reporting will be only 1 bit long.

The reader may append following transmission to form one single MAC PDU until this bit indication is 0.

Assuming that there is an error detection mechanism (e.g.: CRC) possible errors in the reception of one segment or the overall MAC PDU could be handled as a generic failure and then other policies will possibly trigger a retransmission of the whole multi-segment MAC PDU.

This mechanism is of course not very efficient, and it can be improved with more complex indications, but it is very simple.

Assuming segmentation is needed (pending RAN1/SA2 agreements) consider a simple mechanism with 1-bit indication signaling “more segments are pending”.
FFS if more complex mechanisms are required

In the previous meeting concern was raised regarding buffering of the MAC PDU while each segment is being transmitted. It is our understanding that even in the single segment case the data that is about to be transmitted needs to be stored temporarily in a buffer. It is obvious that this storage should be large enough to store the maximum possible UL data size (as per SA2 agreements). A simple segmentation mechanism as proposed above only needs an additional register to keep track of the offset reached within the buffer up to the previous segment, which seems a minimal additional complexity.

Segmentation does not require significantly larger buffer size or complexity with respect to no segmentation

Finally, it is worth noticing that passive devices that will likely have enough energy for the whole duration of the session will not have issues in losing the stored data due to lack of energy.

On the other hand, active devices may in fact lose energy during the transaction. In this case, assuming NVM should not be used for this feature, all information regarding the ongoing segmentation procedure will be lost. The reader will likely detect a failure and may decide to try again later. If the device is never able to transmit all the segments it may be considered simply out of coverage because unable to operate. Note that this will not be different than the case of a single segment (of the same size).

A device not able to complete the transmission of all the segments should be considered “out of coverage” or “unable to operate”.

## 2.3	Repetition

RAN1 has made the below agreement regarding repetition in previous meetings:

Agreement

Study D2R transmission in the physical layer using repetition

- Note: Discussions regarding higher-layer repetitions are up to RAN2.

Agreement

Define repetition types for study purposes as follows:

- Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times

- Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times

- Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times

- Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times

  - NOTE: Equivalent to extending the duration of each chip by Rchip times

Agreement

- For D2R, study at least block-level and bit-level repetition type 1 and type 2.

First, it is worth clarifying that repetitions in the context of RAN2 refers to the AS, excluding PHY layer, to autonomously decide to retransmit or repeat a MAC PDUs when necessary.

In other words, it means that the PHY layer may implement some forms of repetitions (as per agreements) which means that a certain success probability is guaranteed from the PHY layer. At the same time the layers above AS (e.g.: the application itself) may decide to improve reliability in other ways but this is out of 3GPP scope. Likewise other mechanisms managed by CN are out of RAN2 scope.

In addition, retransmissions or reattempts can be interpreted as a form of a repetition mechanism. This is an important aspect to note, especially given RAN2 has agreed to not support HARQ and ACK for A-IoT and thus an alternative mechanism may be required.

Depending on RAN1 decisions, a certain success probability is guaranteed in the worst-case coverage scenario (denoted afterwards as “PHY success probability”)

Repetitions to further improve reliability managed by application or CN are out of RAN2 scope.

Both blind repetitions and reattempts are considered as mechanisms to improve reliability

As a first step to discuss the need for repetitions we should first agree if a success probability target should be guaranteed by AS or not. Obviously, if it is not defined it can be argued that no repetition mechanism is needed on top of RAN1 repetition schemes. Otherwise, we can do a more proper analysis. This target may be more qualitative, but for the sake of discussion let’s assume it can be defined numerically (denoted afterwards as “AS success probability”.

The rationale to decide whether repetitions are needed or not should be that a device in the worst-case scenario (i.e.: cell edge and with the largest TBS) the success probability of a transmission should be equal or higher the “AS success probability”.

If the “PHY success probability” is greater or equal the “AS success probability” then further AS mechanisms to improve reliability are not needed. Otherwise, they are.

In case PHY layer does not implement repetitions, or the “PHY success probability” is lower than the “AS success probability” target, RAN2 should study a repetition mechanism to achieve the target.

Repetitions or Reattempts are needed in RAN2 scope if the AS success probability target is lower than the guaranteed PHY success probability in worst-case scenario.

It may be convenient to start studying these mechanisms while waiting for more detailed RAN1 evaluations and agreements on the matter.

RAN2 to study repetitions mechanism in the hypothesis that PHY layer repetitions are not enough to satisfy a success probability target at AS level.

Since data delivery will likely be part of RACH procedure, a simple RAN2 mechanism to implement repetitions may be performing multiple RACH attempts, similarly to what happens in legacy. Further mechanisms do not seem necessary now.

RAN2 to consider multiple RACH attempts as a mechanism to implement repetitions

## 2.4	Device Addressing and AS identifier handling

The UE should be able to determine if a message received on the shared DL channel is directed to itself or not.

In legacy this is achieved both by having a broadcast transport channel (BCH), or by scrambling the PDCCH associated with a DL transmission using an identifier (RNTI) that the MAC entity in the UE monitors (if the RNTI matches the ones monitored by the UE, the following DL transmission is directed to the UE).

A similar functionality should be supported for A-IoT since the DL channel is also shared, but it may be simplified considering the different transmission scheme.

Study how the A-IoT device determines if a DL transmission in the shared downlink channel (PRDCH) is directed to itself and whether it should be MAC or PHY responsibility.

For the remainder of the section, we call “address” the information needed to distinguish transmission from/to different devices from MAC perspective. (i.e.: it is out of scope higher-layer perspective such as NAS, Application or inventory as a whole).

There are two main problems to solve regarding the device addressing:

- Which information is used as address

- How the address is used

### 2.4.1	Which information is used as address

In general, the address should be a piece of information that is agreed between the reader and device (provisioning) and for which its validity is clear for both nodes.

In assessing each possible option, we have to consider the following aspects:

- Security and Privacy: can the information used as address be shared with the involved nodes, and should it be used in its encrypted form or can be used in clear text?

- Uniqueness: is the address unique within a certain area? Could there be collisions with other devices?

- Provisioning: how is the address provisioned (i.e.: agreed between device and network)

- Usage of memory: should the address be kept in volatile registers or written in NVM? How often? This affects energy consumption and complexity.

- Length: The longer is the ID the more signalling overhead is needed for correct operations

Determine which information should be used as address based on security, privacy, uniqueness, provisioning, length, and usage of NVM.
FFS other aspects.

As a general note regarding memory usage, whichever information is used, it should be stored either in volatile register or in non-volatile memory (NVM). The former implies that once the device loses energy, the address is no longer valid and should be provisioned again. The latter implies more or less frequent NVM write operation that may have a significant impact on complexity and energy consumption but would allow a longer validity of the address.

In general, we can consider three main options. The address could be the application ID (APP ID), an ID generated by the CN (CN ID) or an ID generated by the AS/RAN (AS ID).

The APP ID will likely be hardcoded in memory (i.e.: no writing operations) and has an infinite validity in time. Unless device vendors agree on how to assign these IDs uniquely there may be collision though. Depending on agreements on security, it may be forbidden to share the APP ID in clear text.

A solution to sending the APP ID in clear text is that device uses 3GPP encryption functionalities (e.g.: NAS) to encrypt the APP ID and use the ID as address in its encrypted form. Depending on the property of the encryption function, the resulting uniqueness will be identical to the APP ID in clear text. Nevertheless, the provisioning of the address will require security establishment, and therefore a new address should be stored after every security establishment procedure.

Within the family of CN ID, we have similar options. In case a static CN ID is hardcoded, we have similar concerns as for the APP ID in clear text (mainly security and privacy).

A CN ID may be assigned to the device during a registration procedure. Similarly to the 5G-TMSI, it is OK to transmit this ID in clear text over the air, but the ID should be provisioned and stored at every registration.

Similarly to the APP ID, an encrypted version of a CN ID may be used, but this requires storing and provisioning with every security establishment.

Finally, an ID can be generated by the AS. The intention is to have a shorter ID that has a valid scope only within the equivalent of the RNA area.

A popular option already discussed in other agenda items, is to promote the contention resolution ID to AS ID after the contention has been resolved. This ID, though, is randomly generated. So it is clear that there may be address collisions. Nevertheless, the provisioning procedure is very simple.

Alternatively, we can use a mechanism like in legacy where a temporary ID is assigned by the reader during CBRA and then promoted to address once contention has been resolved (similar to TC-RNTI promoted to C-RNTI). This would guarantee a unique address but requires the transmission of the temporary ID during CBRA.

Finally, the address used in the AS context, can be part or a function of a CN ID. This requires the reader to know at least part of the CN ID but, assuming the CN ID is unique, using only part of it does not guarantee uniqueness in the address regardless of how this part is selected.

The following table summarizes all these options.

It is not clear at this stage which are the requirements that should be strictly followed; therefore, we propose to capture in the TR this list, possibly other options, and leave for the WI phase the decision on which option should be specified.

Capture in TR and study the options mentioned above and postpone to WI phase the decision on which one(s) to adopt
FFS other options

### 2.4.2	Methods to support addressing

Assuming that an address has been agreed in this section we discuss how it may be used to univocally determine which device is supposed to decode a certain R2D transmission or which device transmitted in a certain D2R resource.

There are two general sets of methods. With explicit addressing a data transmission will contain the address as part of control information. The receiver of this message will decode the entire message, including the address control information. Otherwise with implicit addressing some separate signalling, containing the address, will indicate which device is supposed to receive/use some upcoming resources. In this case the data transmission itself will not contain the address.

Which option is more efficient depends on the expected message exchange. For instance, explicit addressing requires larger data transmission but no additional signalling, while the opposite is true for implicit addressing, therefore explicit addressing may be more suited for short sessions, while implicit for longer sessions.

Within the explicit addressing methods, the address may be included at different layers. For instance, if PHY layer will include a control part, this could contain the address. Otherwise, the address may be included as a MAC CE or part of the MAC subheader or even added at upper layers.

Within the implicit addressing methods, we distinguish one-shot and persistent implicit addressing.

One-shot implicit addressing consists in providing an UL/DL Grant containing the address and a description of a specific resource which will be used by the indicated device for a single transmission. This is analogous of legacy PDCCH followed by a PDSCH or PUSCH transmission.

Figure : Basic functioning of one-shot implicit addressing method.

Persistent implicit addressing consists in a DL control signal blocking until further notice all the resources (or all resources in a specific frequency range in case of FDMA) for a specific device. In this case all data transmissions in this reserved band will belong to the same device. This option is more similar to how RFID works where once a slot begins with QueryRep all transmissions will belong to the same user until the following QueryRep starts a new slot.

Figure : Basic functioning of persistent implicit addressing method.

Considering current progress on other agenda items, such as RACH, it is likely that either one-shot or persistent implicit methods can fit nicely in the agreed mechanism. So, we propose to focus more on these.

Capture in the TR and study the two implicit addressing options (see Text Proposal in Annex B)

### 2.4.3	Notes on mobility

It has been already agreed that mobility (at least cell (re)selection) is not supported. Nevertheless, it is important to highlight that once an address is being defined, this is valid only within a certain scope (e.g.: an area handled by the entity allocating the address). If a device exits this scope, either a procedure should provision a new address, transfer the existing address to the node managing the new scope or simply a fallback mechanism should be considered. The extension of the scope depends on which address is considered. For instance, in case of APP ID the address is valid everywhere, while in case of AS ID the address is valid only within the reader coverage area.

Whether or not a context transfer mechanism should be defined also depends on whether the address is stored in NVM or not. Assuming a device moves to different scopes only when in sleep mode, this means that once it wakes up again in the new scope, either a new address is provided, or the old address is transferred.

Considering that writing operation in NVM are likely too energy consuming, the natural assumption is that the address becomes invalid once the device enters a new scope.

Of course, at least the network or the device should be able to determine that this mobility happened so that the old address can be ignored and a new one can be provisioned.

The address in use is valid only within a certain scope (e.g.: area handled by the entity which assigns the address). It becomes invalid once the device exits its scope.

Study how a device or the network determines if a device exited the previous scope and how a new address is provisioned.

### 2.4.4	Note on Logical Channels and modelling

RAN1 has agreed that PHY layer will support PRDCH for DL data transmission, PDRCH for UL data transmission, and not to support a broadcast physical channel (PBCH). This does not necessarily imply a certain set of supported transport channels are concluded for use in the interface between PHY and MAC layers.

In legacy NR the MAC PDUs received from the BCH are processed assuming a transparent MAC PDU format which does not add any overhead information. This is typically used for MIB delivery. In addition, in NR, PCH is used for paging message delivery. Whether the BCH and the PCH is needed would be depending on RAN1 decision.

DL-SCH (mapped to PRDCH) and UL-SCH (mapped to PDRCH) is the minimum set of transport channels provided by PHY layer.

It is important to notice, though, that some messages in DL should be transmitted in a broadcast manner. An example of this is the initial trigger message targeting all devices in reader coverage. For modelling purposes, with no necessary implication on device complexity, it is convenient to define a Broadcast control channel (BCCH). For messages delivered through BCCH the addressing problem explained in this section does not apply as all devices in coverage area are supposed to decode the message. Notice that even if a message via BCCH is broadcast from MAC point of view, the NAS PDU may still contain a command directed to a specific device. Simply, if an address is not available at the moment, all devices should decode the message and determine if they are the intended receiver at a higher layer than MAC.

DL-DTCH (mapped to DL-SCH), BCCH (mapped to DL-SCH), UL-DTCH (mapped to UL-SCH) logical channels are defined at least for modelling purposes (not necessarily with complexity impact).

Only messages delivered through DTCH are affected by the addressing problem from MAC perspective.

## 2.5	Other MAC aspects

### 2.5.1	Energy Status Report

In the last meeting it was agreed to study the need and use of an Energy Status Report (ESR) for the device to inform the reader that it has not enough energy for further message exchanges. Moreover in [R2-2406590] some use cases and other aspects are discussed.

In terms of “need” we can think about what would happen if the ESR is not specified and, in particular, how the reader/network would behave differently compared to simply considering the device out of coverage.

Overall, there are four possible cases regarding the behaviour of the device and network combined: the reader can determine if the transmission failed or not and the device can decide or not to continue with the transmission after it detects low energy (and possibly informed the network with the ESR).

The table below describes the overall expected behaviour in case ESR is used or not (assuming that network and device agrees on the expected behaviour in case ESR is sent).

| Behaviour after ESR transmission | Network is able to determine transmission failed | Network is unable to determine transmission failed |
| --- | --- | --- |
| Device continues after it detects low energy | No ESR: Network treats the failure assuming out of coverage and possibly reattempts later. With ESR: The network is advised that a possible reason for the failure is out-of-energy (but it cannot be sure), possibly reattempts later. | No ESR: Network considers (wrongly) the transmission as successful. With ESR: Network is advised that the transmission may have failed, but it will still not try again in case it was in fact successful (wrongly) |
| Device stops after it detects low energy | No ESR: Network treats the device as out of coverage despite the device not even trying to transmit. Possibly reattempts later. With ESR: Network avoids allocating resources for the transmission after ESR is received. Possibly reattempts later. | No ESR: Network considers (wrongly) the transmission as successful. With ESR: Network avoids allocating resources for the transmission after ESR is received. Possibly reattempts later. |

The only notable difference in network behaviour and end result is highlighted in green, and it consists in avoiding a waste of radio resources once ESR has been received. Nevertheless, it’s worth mentioning that this is truly convenient if the energy measurement in the device is precise enough to activate the ESR only if the next message cannot be transmitted. Otherwise pausing prematurely, the session and starting a new one later may consume more resources than not having ESR.

ESR may be useful in case the specified behaviour is to stop the ongoing session after ESR has been sent and under the assumption that the energy measurement is precise enough to send the ESR when in fact there is no energy to transmit the following message

In case ESR is studied, the expected behaviour should be to stop the session after ESR has been transmitted

Moreover, if energy is depleted before security keys and/or a connection context is established, it’s obvious that at the next round when energy is restored the communication should start over from the beginning and having an ESR does not helps other than what mentioned above. This is because if the network is not able to determine which device is trying to access, it cannot contact it directly once energy is available and it likely would have to restart the whole procedure.

As an example, if energy is depleted after Msg1 transmission, the only information that the network has is a random identifier that the device will not likely store in NVM and so it cannot be used in the next round, and the device would have to transmit a new random ID anyway.

If energy is depleted before security or RAN context establishment the next session should start over and ESR provides limited benefit

If the energy is depleted after security and AS context is established, at the next round it may be possible to restart the connection using CFRA which is leaner and would allow to advance more in the session. Nevertheless, besides the benefit mentioned above, the network behaviour would not be different compared to the case of out of coverage failure, thus ESR does not provide any additional benefit.

If energy is depleted after security or RAN context establishment the next session could start with a leaner CFRA, but this would not be different than the case of out-of-coverage failure, thus ESR does not provide any additional benefit.

There might be corner cases or optimizations that would make the ESR more useful. For instance, if ACKs can be enabled or disabled, the reader may choose to enable them only when the device energy is low, but at this stage we should not prioritize this kind of optimizations and focus on core functionalities.

Another possible interesting case is determining when the inventory procedure is completed (i.e.: all devices in coverage replied). If a device was not able to end the inventory because of lack of energy it will be queried by the network later again. Nevertheless, if the inventory was not completed the network has no way to determine if the same device was successful later as the AS context will be different. Therefore, the information provided by ESR has again no real impact on network behaviour.

It is also worth mentioning that the benefit largely depends on how precisely the device can determine the amount of energy left and how much energy these measurements consume (which could be not negligible) compared to just let the device work until it switches off. It is very likely that the small loss due to the lack ESR is compensated by avoiding stopping prematurely or doing frequent measurements.

How precisely the energy measurement is performed and how much energy it consumes may have a relevant impact on the usefulness of ESR.

As an example, shown in Figure , if performing energy measurements consumes a relevant amount of energy, there might be cases where a device may not be able to complete a session whereas if ESR and relative measurements are ignored that would be possible.

Figure : Example of case where not implementing ESR is more convenient than doing so

Finally, it is worth reflecting on whether a device with not enough energy to at least complete a minimum set of operations should be allowed in the network. The harvesting conditions of a device will not likely change a lot between different sessions, so it is very likely that a device that failed a minimum set of operations will fail again multiple times, consuming radio resources. It is possible that a device in such conditions should be considered as “out of coverage” despite being technically in radio coverage.

Study ESR assuming network and device agree that a device pauses the ongoing session after ESR has been sent.

Study whether a device without enough energy to complete a minimum set of operations should be considered “out of coverage” or not, and what this minimum set of operations is.

### 2.5.2	Octet vs. Bit aligned MAC PDU

In the previous meeting some companies proposed the concept of not having octet aligned MAC PDU (or upper layer SDUs). It may be that the amount of control information to be sent is very low, just a few bits. In such case it might be beneficial to avoid octet alignment to avoid padding bits.

This discussion of course depends on how flexible and generic the TBS allocation is expected to be, also in future extensions.

Nevertheless, typically the length of a SDU is given in octet in NR. Avoiding octet-alignment means that now the length should be given in bits, which would increase the amount of signaling bits for indicating the TBS size.

- As a result, It is not obvious, without other assumptions, to determine if bit-alignment would lead to a more efficient MAC PDU encoding than octet-alignment.

Moreover, considering device limitations, memory works by addressing words of various length, typically a byte. Therefore, it might create excessive complexity on device side to have bit-aligned buffers and stored data.

Also, current network design assumes octet alignment. Changing this may create excessive implementation effort on network side.

Bit-alignment may cause excessive complexity and implementation effort on both device and network size due to the normal functioning of memory and existing implementations.

Include in the TR that bit-aligned MAC PDUs is out of the scope of the study.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	It is preferable to register device context information for subsequent communication as there is no need to perform identification, authentication, security setup every time when a service procedure (e.g., inventory or command) is initiated.

Observation 2	Previous agreement on not supporting SDAP, PDCP, RRC, RLC does not prevent the definition of a new AS protocol layer in the protocol stack above MAC.

Observation 3	Merging the “MAC” and “Device Control” functionalities in a single layer implies a smaller overhead in terms of encoding of control elements.

Observation 4	Segmentation and/or scheduling request mechanisms are needed to satisfy all possible use cases depending on RAN1 agreements on maximum TBS at cell edge or SA2 agreements on maximum UL data size.

Observation 5	Segmentation does not require significantly larger buffer size or complexity with respect to no segmentation

Observation 6	Depending on RAN1 decisions, a certain success probability is guaranteed in the worst-case coverage scenario (denoted afterwards as “PHY success probability”)

Observation 7	Repetitions to further improve reliability managed by application or CN are out of RAN2 scope.

Observation 8	Repetitions or Reattempts are needed in RAN2 scope if the AS success probability target is lower than the guaranteed PHY success probability in worst-case scenario.

Observation 9	Only messages delivered through DTCH are affected by the addressing problem from MAC perspective.

Observation 10	ESR may be useful in case the specified behaviour is to stop the ongoing session after ESR has been sent and under the assumption that the energy measurement is precise enough to send the ESR when in fact there is no energy to transmit the following messge

Observation 11	If energy is depleted before security or RAN context establishment the next session should start over and ESR provides limited benefit

Observation 12	If energy is depleted after security or RAN context establishment the next session could start with a leaner CFRA, but this would not be different than the case of out-of-coverage failure, thus ESR does not provide any additional benefit.

Observation 13	How precisely the energy measurement is performed and how much energy it consumes may have a relevant impact on the usefulness of ESR.

As a result, It is not obvious, without other assumptions, to determine if bit-alignment would lead to a more efficient MAC PDU encoding than octet-alignment.

Observation 14	Bit-alignment may cause excessive complexity and implementation effort on both device and network size due to the normal functioning of memory and existing implementations.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 assumes the existence of “registration procedure (e.g., authorization and authentication)” catering for A-IoT requirements on identification, authorization, and authentication.

Proposal 2	RAN2 to study how to support other procedures than inventory and command over A-IoT air interface including registration and device enabling/disabling.

Proposal 3	RAN2 to discuss if the overall AS procedure (three-step baseline, in current TR38.760-13) can support procedure steps for registration and device enabling/disabling.

Proposal 4	RAN2 assumes registration can be performed standalone or in connection with inventory procedure.

Proposal 5	RAN2 assumes CN determines trigger of registration based on device capability.

Proposal 6	Target device is indicated type of procedure the A-IoT paging is intended for (e.g., inventory, command, registration, enabling/disabling, etc.).

Proposal 7	Send a LS to SA2, SA3 and CT1 asking for feedback while indicating the above statements captured in P1, P4, P5.

Proposal 8	Merging the “MAC” and “Device Control” functionality in a single layer implies a higher complexity in terms of modelling and lower reusability of network-side implementation of such layer.

Proposal 9	Study the pros and cons of merging “MAC” and “Device Control” functions into a single layer or in multiple layers.

Proposal 10	Assuming segmentation is needed (pending RAN1/SA2 agreements) consider a simple mechanism with 1-bit indication signaling “more segments are pending”. FFS if more complex mechanisms are required

Proposal 11	A device not able to complete the transmission of all the segments should be considered “out of coverage” or “unable to operate”.

Proposal 12	Both blind repetitions and reattempts are considered as mechanisms to improve reliability

Proposal 13	RAN2 to study repetitions mechanism in the hypothesis that PHY layer repetitions are not enough to satisfy a success probability target at AS level.

Proposal 14	RAN2 to consider multiple RACH attempts as a mechanism to implement repetitions

Proposal 15	Study how the A-IoT device determines if a DL transmission in the shared downlink channel (PRDCH) is directed to itself and whether it should be MAC or PHY responsibility.

Proposal 16	Determine which information should be used as address based on security, privacy, uniqueness, provisioning, length, and usage of NVM. FFS other aspects.

Proposal 17	Capture in TR and study the options mentioned above and postpone to WI phase the decision on which one(s) to adopt FFS other options

Proposal 18	Capture in the TR and study the two implicit addressing options (see Text Proposal in Annex B)

Proposal 19	The address in use is valid only within a certain scope (e.g.: area handled by the entity which assigns the address). It becomes invalid once the device exits its scope.

Proposal 20	Study how a device or the network determines if a device exited the previous scope and how a new address is provisioned.

Proposal 21	DL-SCH (mapped to PRDCH) and UL-SCH (mapped to PDRCH) is the minimum set of transport channels provided by PHY layer.

Proposal 22	DL-DTCH (mapped to DL-SCH), BCCH (mapped to DL-SCH), UL-DTCH (mapped to UL-SCH) logical channels are defined at least for modelling purposes (not necessarily with complexity impact).

Proposal 23	In case ESR is studied, the expected behaviour should be to stop the session after ESR has been transmitted

Proposal 24	Study ESR assuming network and device agree that a device pauses the ongoing session after ESR has been sent.

Proposal 25	Study whether a device without enough energy to complete a minimum set of operations should be considered “out of coverage” or not, and what this minimum set of operations is.

Proposal 26	Include in the TR that bit-aligned MAC PDUs is out of the scope of the study.

# References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, Maastricht, Netherlands, March 18-21, 2024

RAN2#125bis Chair Notes, April 2024

RAN1#116bis R19 Ambient IoT Chair notes, April 2024

TS 22.369, “Service requirements for ambient power-enabled IoT”, 3GPP, March 2024

R3-24xxxx, “AIoT RAN CN interface details”, Ericsson, August 2024

# Annex A.	 Current Agreements

RAN2 has made the below relevant agreements in RAN2#125-bis

Agreements (AI 8.2.2 Stage 2 General aspects)

- Unless explicitly stated all agreements apply to all device types and for both topologies.

- From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2.

- RAN2 will support two use cases, “inventory” and “command”.  The definition, detailed wording is FFS

- Baseline procedure:
Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS
Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS
Step C: The device may perform the data communication with the reader as needed,: Details FFS

- We will study the support of both “inventory” and “command” in the same procedure.

- FFS if Initial Trigger Message can also include “command”.

- RAN2 will continue the study of ambient IoT assuming no support of AS security until SA3 provides further input.

Agreement (AI 8.2.3.2 User Plane)

- SDAP is not supported for UP protocol stack.

- PDCP layer is not needed.  FFS how to handle AS security (if needed pending SA3 dicsussion) and any other really needed functionalities.

- RLC layer is not needed.   FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size).  RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed.

- No HARQ and RLC AM

- FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

- RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL).  FFS how to handle the general QoS requirements from SA2

Agreement (AI 8.2.5 Random Access)

- RAN2 confirms slotted-ALOHA is the baseline for Ambient IoT random access

- We will study the support for access triggering for a single device, group of devices, or all devices.    RAN2 to discuss the contention-based and contention-free access procedures and detailed solutions.

- Random Access is triggered by the reader

- Reader provides the information that the device needs to respond to the random access trigger.  FFS what those parameters are

- Study the solution and benefits of both 2-step like random access procedure and 4-step like random access procedure.  FFS the details on each procedure and how we call it.

- Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access.  FFS details

- For the very first access message from the device to reader in random access an ID is included.  RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well).

Agreement for CP aspects

- RRC connection management is not supported.FFS how the resource configuration is provided to the device (if needed based on RAN1 progress)

- RRM L3 measurement reporting is not supported by Ambient IoT devices.

- RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding.

- Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals.

- RAN2 assumes that RRC layer is not necessary between the reader and the device.   RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

RAN2#126 agreements:

Agreements

- As baseline, the “inventory only” case is supported by the procedure:
Step A: A-IoT paging;
Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS

- As baseline, the “inventory and command” case is supported by the procedure:
Step A: A-IoT paging;
Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS 
Step C: reader to device data transmission (e.g. the R2D command), and
Step D: corresponding device to reader data transmission (e.g. the feedback).  FFS whether this is optional, pending other WG discussions.   
Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message.  This doesn’t mean that inventory and command are received by the reader at the same time from upper layer.

- From RAN2 point of view we will study “Command only” use case.  
FFS the options on how to support it :
Initial trigger message from the reader contains the command.  Final feasibility depends on SA2 and SA3 work/conclusions.

Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective.   FFS how we handle segmentation case (if needed)

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. - What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.   “Msg2” usage/presence can be further discussed

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

# Annex B.	Text Proposal for TR 38.769

Start of Change

### 6.3.5	A-IoT data transmission

Editor’s Note:	 This clause is to capture the studies related to the A-IoT data transmission functionalities (also to give the reference to clause 6.3.2 on the protocol stack), e.g., AS ID for scheduling purposes, ... (omitted).

#### 6.3.5.x	Device Addressing

The purpose of device addressing function is for the reader to determine which device has transmitted in a specific set of PDRCH resources, and for the device to determine if it is the intended receiver of a transmission over PRDCH.

(omitted description of options for which ID should be used as address, See Section 2.4.1)

For A-IoT the address used for scheduling purposes shall not be included in the data transmission, but in a separate control signalling instead.

A first method is denoted as “One-shot implicit addressing” where a grant transmitted by the reader contains the device address and a specific resource definition (e.g.: time, frequency, MCS, TBS, etc). A new grant is provided at every new data transmission.

Figure 6.3.5.x-a: Basic functioning of one-shot implicit addressing method.

Note: FFS further conclusions and consideration on one-shot implicit addressing method

Alternatively, a second method denoted as “persistent implicit addressing” can be defined where a grant provided by the reader starts a persistent reservation of a subset of periodic resources until further notice. The further notice may be a new grant or a specific “end” command.

Figure 6.3.5.x-b: Basic functioning of persistent implicit addressing method.

Note: FFS further conclusions and consideration on persistent implicit addressing method

Note: FFS how to handle error cases (e.g.: missed reception of Reservation Start/End or Grant)

End of Change