3GPP TSG RAN WG2 #127bis                                                                     R2-2408670

Hefei, China, Oct 14th – 18th, 2024

Source:	TCL

Title:	Discussion on NW side data collection for positioning

Agenda Item:	8.1.3

Document for:	Discussion and Decision

- Introduction

Data collection is a very important function in the AI/ML-based LCM, which is used to provide the input data to the different LCM aspects (i.e., model inference, monitoring, and training). In the recent RAN2#127 meeting, there were full discussions on starting/stopping of data collection, event-based/on-demand reporting and handling of large logs. Some conclusions were drawn as follows [1].

Agreements - As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support. - UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. - Measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. FFS whether the UE stops autonomously or if it reports to the network . - FFS whether AS buffer event based reporting is supported. FFS if we send availability indication or full report if it is supported - FFS on event based data collection/logging - On-demand request from the network is supported. FFS details on signalling Agreements - The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements) - Data collection report will not be transmitted over SRB1. FFS which SRB is used.

In this contribution, we provide our views on NW side data collection for the positioning accuracy enhancement use case, focusing on the first priority Case 3a and Case 3b.

- Discussion

2.1 Case 3a

The training data can be autonomously collected by gNB or OAM for Case 3a. As for gNB-centric data collection, SRS is configured reusing the legacy RRC message for measurement data collection. The label data (intermediate positioning measurement) can be calculated based on the UE location information, which can be contained in the RRM measurement report associated with the corresponding SRS configuration for data collection to match the measurement data and label data. The SRS configuration can be contained in the RRM measurement report or associated to the RRM measurement report via a measurement ID.

Proposal 1: The RRM measurement report containing UE location information can be associated with the corresponding SRS configuration for label data collection for Case 3a.

The label data can also be generated by LMF based on the UE location information estimated by traditional uplink positioning method. In this way, the NRPPa protocol can be extended to support the transfer of label data. For example, gNB can request the label data from LMF along with the time stamp (e.g., the SRS transmission time) and then LMF transfers the label data corresponding to the SRS measurement data to gNB. The NRPPa protocol extension can be further discussed in RAN3.

Proposal 2: The NRPPa protocol can be extended to support the transfer of label data generated by LMF for Case 3a and it can be further discussed in RAN3.

As for OAM-centric data collection, the Immediate MDT with RAN measurement configuration can be extended. The channel measurement (e.g., DP, PDP, CIR) by RAN should be additionally supported for Immediate MDT so that OAM can collect the measurement data. The measurement collection can be triggered by event or at the end of measurement collection period, and the trigger events can be further discussed. Besides, the UE location information provision is supported in the legacy Immediate MDT so there should be no extension for the label data collection.

Proposal 3: The Immediate MDT with NW-side channel measurements, e.g., DP, PDP, CIR, can be introduced to support OAM-centric training data collection for Case 3a.

2.2 Case 3b

The training data can be collected by LMF for Case 3b. LMF collects the measurement data by indicating the SRS configuration for data collection to gNB. Considering that gNB transfers the measurement data rather than intermediate positioning measurements to LMF for training data collection, the NRPPa protocol can be extended by introducing the indication of data collection in order to make gNB aware of the transferred data format. The detailed data collection configuration can be further discussed in RAN3.

Proposal 4: The NRPPa protocol can be extended to support the measurement data transfer between gNB and LMF for Case 3b, which can be further discussed in RAN3.

The label data can be generated by PRU, non-PRU UE and LMF, which is captured as follows [2].

Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed.

If the label data is generated by PRU and LMF, LMF can easily match the measurement data and label data. However, the non-PRU UE may generate the label data via PRS. In this case, the UE may move within the time gap between the PRS reception and SRS transmission, which leads to the mismatch between the measurement data and label data. Hence, the data association should be considered to guarantee the reliability of model training. Intuitively, LMF can request the label data from UE via LPP signaling and then UE sends the label data to LMF to check validity or sends the label data after self-validation along with the assistance information. The assistance information for data association waits for RAN1 discussion.

Proposal 5: The data association procedure should be considered for Case 3b and the assistance information for data association waits for RAN1 discussion.

- Conclusion

We have the following proposals:

Proposal 1: The RRM measurement report containing UE location information can be associated with the corresponding SRS configuration for label data collection for Case 3a.

Proposal 2: The NRPPa protocol can be extended to support the transfer of label data generated by LMF for Case 3a and it can be further discussed in RAN3.

Proposal 3: The Immediate MDT with NW-side channel measurements, e.g., DP, PDP, CIR, can be introduced to support OAM-centric training data collection for Case 3a.

Proposal 4: The NRPPa protocol can be extended to support the measurement data transfer between gNB and LMF for Case 3b, which can be further discussed in RAN3.

Proposal 5: The data association procedure should be considered for Case 3b and the assistance information for data association waits for RAN1 discussion.

- References

- RAN2 #127 Chair Notes

- RAN1 #117 Chair Notes