3GPP TSG-RAN WG2 #127bis 	R2-2408673

Hefei, China, Oct 14th – 18rd, 2024

Agenda Item:	8.7.5  RLC AM Enhancements

Source:	NEC

Title:	Further discussion on RLC AM enhancements

Document for:	Discussion, Decision

# 1.Introduction

We made good progress in past RAN2 meetings on how to enhance RLC AM mode to avoid unnecessary retransmission and to retransmit data timely:

- To avoid unnecessary retransmissions (when it is too late)

Following approaches has been agreed: - Rx initiated approach - RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - Tx initiated approach - Tx side stops retransmit obsolete SDUs - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Rx side updates state variables according to the information from Tx side - A combined Rx and Tx approach - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

In this document, we provide a comparison of these approaches as above

- To Achieve timely retransmissions (before it is too late)

RAN2 agree to consider the following options: • Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) • Retransmission based on enhanced status report • Retransmission based on enhanced polling FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

In this document, we will share our view on above options to achieve timely retransmission before it is too late.

# 2 Discussion

## 2.1 Avoid unnecessary retransmissions

Three approaches including Tx, Rx and combine have been listed to avoid unnecessary retransmissions when it is too late. Our previous document in [R2-2407015] has discussed the details on how each approach will work.

In table below, for each approach we summarize the impacts to Tx and Rx side operation of RLC entity respectively, we also list the main concern on each approach. This will give an overall idea on the difference, the complexity, and concerns of each approach:

Table 1 Specification impact of different approaches

|  | Impact to Tx side | Impact to Rx side | Main concerns |
| --- | --- | --- | --- |
| Rx initiated approach | No impact Or Impact: update existing variables and timers to advance the Tx window based on received “Window Advance Command” , if we introduce new signalling rather than reuse legacy “status report” | - introduce a new timer and new variable for triggering to abandon SDU(s), this includes to specify the initiation/update condition of the new timer and variable (same/similar as RLC UM) - update existing variables and timers to advance the Rx window considering the abandoned SDUs. - Reuse “Status Report”to send ACK for abandoned SDUs or introduce “window advance command” | - Longer delay , because Tx side cannot move forward till receive fake ACK - Fake ACK in RLC layer may impact PDCP operation, a separate “window advance command” rather than legacy “status report” might be needed |
| Tx initiated approach | - specify the trigger to abandon a SDU e.g., discard indication from PDCP - update existing variables and timers to advance the Tx window considering abandoned SDUs - introduce RLC SN gap report including its triggers, format and (re)transmission. | - update existing variables and timers to advance the Rx window upon receiving the RLC SN gap report. | - Complication to introduce “RLC SN gap report” and to guarantee the robustness of its delivery as we experienced in UMTS - This violate the principle that Tx window always advance after Rx window. |
| Independent Rx and Tx approach NOTE: synchronization between Rx and Tx window is guaranteed by proper configuration | - specify the trigger to abandon a SDU e.g., discard indication from PDCP - update existing variables and timers to advance the Tx window considering abandoned SDUs | - introduce a new timer and new variable for triggering to abandon SDU(s), this includes to specify the initiation/update condition of the new timer and variable (same/similar as RLC UM) - update existing variables and timers to advance the Rx window considering the abandoned SDUs. | - Synchronization between Rx and Tx window may be lost. However considering the short latency, the “discard timer” and “giving up receiving timer” would not be long, window stalling or window Insync unlikely happen |

With all approaches, There are common/similar specification impact including:

- New specification to advance the Tx window , either triggered by local indication (in Rx-based) or triggered by peer entity at Rx side

- New specification to advance the Rx window, either based on new local timer/variable or based on “SN gap report“ from Tx

The main difference between three approaches:

- Tx-based approach has to specify “SN gap report” and guarantee the robustness of its delivery, or make sure there is not severe impact if the “SN gap report” is lost

- Rx-based approach has to introduce new local timer and variable, but we believe this is not an issue since we already have similar thing in PDCP/RLC UM. But if a separate “window advance command” rather than legacy “status report” will be needed, then the specification impact level starts to similar to “SN gap report” in Rx-based approach

We think the combined approach is most simple and could be considered unless issue is identified.

Proposal 1 RAN2 choose one approach considering the specification impact and concerns summarized in the table

## 2.2 Timely retransmission

To achieve timely RLC retransmission for XR, following possible solutions have been identified in last meeting:

•	Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered)

•	Retransmission based on enhanced status report

•	Retransmission based on enhanced polling

As shown in below the copied RLC specification, STATUS reporting will be triggered either by polling from its peer AM RLC entity or detection of reception failure of an AMD PDU, and STATUS report transmission is further subject to t-StatusProhibit timer:

Triggers to initiate STATUS reporting include: - Polling from its peer AM RLC entity: - When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall: - if the AMD PDU is to be discarded as specified in clause 5.2.3.2.2; or - if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size: - trigger a STATUS report. - else: - delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size. NOTE 1: This ensures that the RLC Status report is transmitted after HARQ reordering. - Detection of reception failure of an AMD PDU - The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires. NOTE 2: The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated. When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall: - if t-StatusProhibit is not running: - at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer. - else: - at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer. When a STATUS PDU has been submitted to lower layer, the receiving side of an AM RLC entity shall: - start t-StatusProhibit.

- For polling, transmitting side can send polling bit after number of PDUs or number of bytes transmitted out , this is fully under NW configuration via parameters pollPDU and pollByte, they can be as small as 4PDUs and 1000bytes.

- For detection of reception failure of an AMD PDU, this is controlled by parameter t-Reassembly. t-Reassembly starts as long as a receiving SN gap appears, the value of t-Reassemby is configured by network, it can be as small as 0, in which case, STATUS report will be triggered right after detection a reception gap

- In addition, a new STATUS report transmission is not allowed during the time window following the previous STATUS report transmission. This is controlled by parameter t-StatusProhibit, to avoid too frequent signalling. but this parameter can be configured as small as 0.

And the possible configurable value of these parameters

PollPDU ::= ENUMERATED { p4, p8, p16, p32, p64, p128, p256, p512, p1024, p2048, p4096, p6144, p8192, p12288, p16384,p20480, p24576, p28672, p32768, p40960, p49152, p57344, p65536, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1} PollByte ::= ENUMERATED { kB1, kB2, kB5, kB8, kB10, kB15, kB25, kB50, kB75, kB100, kB125, kB250, kB375, kB500, kB750, kB1000, kB1250, kB1500, kB2000, kB3000, kB4000, kB4500, kB5000, kB5500, kB6000, kB6500, kB7000, kB7500, mB8, mB9, mB10, mB11, mB12, mB13, mB14, mB15, mB16, mB17, mB18, mB20, mB25, mB30, mB40, infinity, spare20, spare19, spare18, spare17, spare16, spare15, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1} T-Reassembly ::= ENUMERATED { ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35, ms40, ms45, ms50, ms55, ms60, ms65, ms70, ms75, ms80, ms85, ms90, ms95, ms100, ms110, ms120, ms130, ms140, ms150, ms160, ms170, ms180, ms190, ms200, spare1} T-StatusProhibit ::= ENUMERATED { ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35, ms40, ms45, ms50, ms55, ms60, ms65, ms70, ms75, ms80, ms85, ms90, ms95, ms100, ms105, ms110, ms115, ms120, ms125, ms130, ms135, ms140, ms145, ms150, ms155, ms160, ms165, ms170, ms175, ms180, ms185, ms190, ms195, ms200, ms205, ms210, ms215, ms220, ms225, ms230, ms235, ms240, ms245, ms250, ms300, ms350, ms400, ms450, ms500, ms800, ms1000, ms1200, ms1600, ms2000, ms2400, spare2, spare1}

NW could make the feedback procedure as fast as possible by configuring small value for these parameters, with the cost of very frequent STATUS report signalling from Rx to Tx. For example, with T-StatusProhibit=0, T-Reassembly=0 and PollPDU=4, Status Report will be sent in following conditions:

- Upon receive every 4th PDU, due to Rx side sets the polling bit at every 4th PDU transmission. In this case  Status report may or may not be necessarily needed.

- Upon detect a receiving SN gap, this is necessary and timely

One possible enhancement is to introduce remaining time based polling. This means Rx will poll only if the remaining time is short

Proposal 2: RAN2 consider remaining time based polling.

For blind retransmission, we can see following slightly different options:

- Timer based: If the remaining time of a RLC PDU has been less than a configured threshold, retransmission can be triggered

- Counter-based: If a RLC PDU has been retransmitted for certain number of times, a retransmission can be triggered

Counter-based solution seems not solve the issue fully. A given number of retransmission could last longer or shorter , which depends on the variable round trip delay, consequently counter-based solution cannot guarantee timely retransmission. Remaining time based solution sounds more reasonable to us.

Proposal 3: RAN2 consider to introduce autonomous retransmission based on remaining time

## 2.3 Disable RLF trigger from RLC AM

There is also proposal to not trigger RLF when maximum ARQ retransmission reaches.

Maximum times of ARQ retransmissions reaches, it means a RLC PDU cannot be successfully received after (x times of ARQ retransmissions * y time of HARQ retransmissions) . This was considered as a rare case if we assume that x and y would not be small numbers. With this assumption, RLF can be triggered.

However, if the event of reaching the maximum ARQ retransmissions become very common e.g., configured maximum retransmission times are very small due to the small PDB. we believe there is no point to trigger RLF.

At the same time, If we terminate ARQ retransmission based on PDB, it may stop to reach the maximum retransmissions. If we speed up retransmission or introduce other trigger for retransmission, it may impact how fast the maximum retransmissions will reaches. so we propose:

Proposal 4: Delay discussion on disabling RLF declaration at RLC layer, till other RLC AM enhancements become clear.

# 3 Summary

In this contribution, firstly we shared our comparison and summarized the main concern on the three approaches including Tx initiated, Rx initiated and a combined Rx and Tx approach on RLC AM window operation enhancement. In our view, the combined approach is most simple and could be considered unless issue is identified. Nevertheless,  we are open to hear more opinion:

Proposal 1 RAN2 choose one approach considering the specification impacts and main concerns summarized in the table

Table 1 Specification impact of different approaches

|  | Impact to Tx side | Impact to Rx side | Main concerns |
| --- | --- | --- | --- |
| Rx initiated approach | No impact Or Impact: update existing variables and timers to advance the Tx window based on received “Window Advance Command” , if we introduce new signalling rather than reuse legacy “status report” | - introduce a new timer and new variable for triggering to abandon SDU(s), this includes to specify the initiation/update condition of the new timer and variable (same/similar as RLC UM) - update existing variables and timers to advance the Rx window considering the abandoned SDUs. - Reuse “Status Report”to send ACK for abandoned SDUs or introduce “window advance command” | - Longer delay , because Tx side cannot move forward till receive fake ACK - Fake ACK in RLC layer may impact PDCP operation, a separate “window advance command” rather than legacy “status report” might be needed |
| Tx initiated approach | - specify the trigger to abandon a SDU e.g., discard indication from PDCP - update existing variables and timers to advance the Tx window considering abandoned SDUs - introduce RLC SN gap report including its triggers, format and (re)transmission. | - update existing variables and timers to advance the Rx window upon receiving the RLC SN gap report. | - Complication to introduce “RLC SN gap report” and to guarantee the robustness of its delivery as we experienced in UMTS - This violate the principle that Tx window always advance after Rx window. |
| Independent Rx and Tx approach NOTE: synchronization between Rx and Tx window is guaranteed by proper configuration | - specify the trigger to abandon a SDU e.g., discard indication from PDCP - update existing variables and timers to advance the Tx window considering abandoned SDUs | - introduce a new timer and new variable for triggering to abandon SDU(s), this includes to specify the initiation/update condition of the new timer and variable (same/similar as RLC UM) - update existing variables and timers to advance the Rx window considering the abandoned SDUs. | - Synchronization between Rx and Tx window may be lost. However considering the short latency, the “discard timer” and “giving up receiving timer” would not be long, window stalling or window Insync unlikely happen |

Secondary, we shared our view on timely retransmission and propose following:

Proposal 2: RAN2 consider remaining time based polling.

Proposal 3: RAN2 consider to introduce autonomous retransmission based on remaining time

Finally, we discussed the possibility to disable RLF trigger from RLC AM:

Proposal4: Delay discussion on disabling RLF declaration at RLC layer, till other RLC AM enhancements become clear.