3GPP TSG-RAN WG2 #127bis	R2-2408674

Hefei, China, 14th – 18rd Oct 2024

Agenda Item:	8.8.6 LTE to NR NTN mobility

Source:	NEC

Title:	Remaining Issues on NR Satellite Info Provision in LTE TN cell

Document for:	Discussion, Decision

# 1.Introduction

To support LTE TN to NR NTN idle mode mobility, NR satellite information has been introduced in SIB33 and could be linked to inter-RAT frequencies. CRs have been endorsed: stage 2 CR in R2-2407616 , RRC CR in R2-2407617 and 36.306 CR in R2-2407618.

In this contribution we discuss how to signal satelliteID considering the scenario that the same satellite provides both NR NTN and IOT NTN neighbouring cell (called it share satellite or NR+IOT satellite).

# 2 Discussion

As indicated in below, it has been not fully concluded how to address the scenario where same satellite provides both IOT NTN and NR NTN cell, and we believe this should be discussed together with whether IOT and NR satellites is identified with the same satelliteID-r18 or identified separately/independently.

Proposal 3: RAN2 down-selects following alternatives to identify the NR satellite assistance information: - Alternative 1: Reuse the SatelliteId-r18 to identify either an NR satellite or an IoT NTN satellite; - Alternative 2: Define NR specific satellite ID, e.g. SatelliteId-NR-r19, to identify NR satellite specifically. - Samsung thinks we can go for Alt1 unless we find problems We go for Alt1 We consider a solution that avoids repeating the ephemeris for a satellite which provides both IoT NTN and NR NTN cells R2-2407812 Summary of [AT127][305][LTE_TN_NR_NTN_mob] RRC CR CATT disc Rel-19 LTE_TN_NR_NTN_mob Proposal 1: RAN2 agrees signaling Alternative 1 to indicate ephemeris data for a satellite supporting both NR NTN and IoT NTN, i.e. introducing a Satellite reference ID that refers to the ephemeris data of an IoT NTN satellite in the existing list neighSatelliteInfoList-r18, and using the "CHOICE" signaling structure including this Satellite reference ID and explicit ephemeris data configuration as the two choices. Come back in the next meeting

RAN2 firstly agreed to have separate LTE and NR neighbouring satellites list in the same SIB33, i.e., neighSatelliteInfo-List and neighSatelliteInfoNR-List. Moreover, we agreed to reuse SatelliteId-r18 to identify either an NR satellite or an IoT NTN satellite. During the email discussion and endorsed CR implementation stage, company proposed to add a SatelliteRefID to refer to a IOT NTN satellite to avoid repeating the same ephemeris of the same satellite. This combined solution can be summarized as below and IE relationship is shown in following figure:

Option 0: same SatelliteId-r18 + with SatelliteRefID

By reusing the same SatelliteId-r18, satelliteID will uniquely identify all satellites including IOT NTN satellite in neighSatelliteInfo-List and NR NTN satellite in neighSatelliteInfoNR-List. For example a ID value #10 identifying a satellite providing IOT NTN cell, shall not be used to identify another satellite providing NR NTN cell.

Together with SatelliteRefID , it starts to be strange to have SAT#q in NR satellite list referring to SAT#x in IOT satellite list, and SAT#q and SAT#x are the same satellite proving both IOT NTN and NR NTN cells. Instead, the straight forward way is to use the same satellite ID for the shared satellite which is listed in both IOT satellite list (neighSatelliteInfo-List) and NR satellite list ( neighSatelliteInfoNR-List). SatelliteRefID is not needed. This combination (called option1) is shown as in following figure:

Option1: same SatelliteId-r18 + w/o SatelliteRefID

Same satelliteID value will implicitly indicate that this is the same satellite. Because the ephemerisInfo appeares in IOT NTN satellite list, to avoid repeating it in NR satellite list, ephemerisInfo in neighSatelliteInfoNR-List is optional. And make the IE description clear that If a satelliteID in neighSatelliteInfoNR-List match a satellite ID listed in neighSatelliteInfo-List, reuse the corresponding ephemerisInfo from neighSatelliteInfo-List.

As shown in above figure, satellites listed in neighSatelliteInfo-List are a IOT satellites and could be linked to a intra/inter frequency listed in SIB3 and SIB5; satellites listed in neighSatelliteInfoNR-List are NR NTN satellites and could be linked to an inter-RAT frequency listed SIB24. This two lists are independently and respectively cited by different SIB. Hence the satellite ID value range could be independent too, i.e., NW only need to make sure uniqueness of the satellite ID within each list. alternative 2 (separate SatelliteId-NR-r19) works well. Instead alternative 1(reuse SatelliteId-r18) creates unnecessary correlation between two lists. If we go with alternative 2, in case of shared satellite scenario (a satellite provides both NR and IOT NTN), it make sense to introduce a Satellite reference ID referring to the satellite ID listed in neighSatelliteInfo-List. This combination (called option2) is shown in figure below:

Option2: Separate SatelliteIdNR-r19 + with SatelliteRefID

In our understanding, either option 1 or option 2 above is better than option 0 which is currently implemented in the endorsed CR .

Proposal 1 to adopt one of following options to identify IOT satellites, NR satellites and a NR+IOT satellites (i.e., a shared satellite proving both IOT NTN and NR NTN cells):

- Option 1: reuse SatelliteId-r18 to uniquely identify all types of satellites. Same satellite ID value will be used in neighSatelliteInfo-List and neighSatelliteInfoNR-List for a NR+IOT satellite, and make ephemerisInfo-r19 IE as optional

- Option 2: Define separate SatelliteIdNR-r19 to uniquely identify NR satellites. Introduce a SatelliteRefID referring to a IoT NTN satellite listed in neighSatelliteInfoList-r18, in case of a NR+IOT satellite

# 3 Summary

This contribution we propose:

Proposal 1 to adopt one of following options to identify IOT satellites, NR satellites and a NR+IOT satellites (i.e., a shared satellite proving both IOT NTN and NR NTN cells):

- Option 1: reuse SatelliteId-r18 to uniquely identify all types of satellites. Same satellite ID value will be used in neighSatelliteInfo-List and neighSatelliteInfoNR-List for a NR+IOT satellite, and make ephemerisInfo-r19 IE as optional

- Option 2: Define separate SatelliteIdNR-r19 to uniquely identify NR satellites. Introduce a SatelliteRefID referring to a IoT NTN satellite listed in neighSatelliteInfoList-r18, in case of a NR+IOT satellite

# 3 Reference

[1] Report from Break-out session on NR-NTN and IoT-NTN RAN2#127

[2] R2-2407617	Introduction of LTE TN to NR NTN IDLE mode mobility	CATT	draftCR	Rel-19	36.331	18.2.0	B	LTE_TN_NR_NTN_mob