3GPP TSG-RAN WG2 Meeting #127bis	R2-2408683

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.7.4.2

Source:	ITRI

Title:	Discussion on enhanced DSR for XR

Document for:	Discussion and Decision

1 Introduction

In RAN2 #127, some consensus of enhanced DSR had been reached as below.

- Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG.

- For enhanced DSR:

    - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds

    - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR

    - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In this document, we discuss the detailed impact of enhaced DSR with multiple reporting thresholds, and whether PSI reporting in DSR is needed.

2 Discussion

2.1 Delay-critical data definition

The two agreements about multiple reporting thresholds in the DSR in RAN2 #127:

- There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds

- FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR

From our point of view, no need to modify the delay-critical data definition. First, DSR is for urgent data, so we think that DSR should not be triggered very frequently in a normal condition. Secondly, the multiple reporting thresholds in the DSR agreed in Rel-19 are used to provide more buffer information with delay status, so the gNB can proceed the UL resource allocation for multiple UEs more efficiently. Therefore, the single triggering threshold is assumed to be a smaller value compared to the multiple reporting thresholds of the LCG. For example, the multiple reporting thresholds may be 20ms, 40ms, 60ms, and so on, and the single triggering threshold can be the smallest value of threshold for DSR reporting of an LCG, e.g, 20ms.

Based on the above discussion, we clarify that only the single triggering threshold is related to the delay-critical data definition, and the multiple reporting thresholds are related to detailed buffer information with delay status in DSR. No need to modify the delay-critical data definition in TS 38.323.

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

Proposal 1: No need to modify the delay-critical data definition.

2.2 Buffer size calculation for DSR

The agreement about buffer size calculation for DSR in RAN2 #127:

- FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

From our point of view, although the logical channel buffer at MAC layer is FIFO (first in, first out) as default, it is reasonable that the packets can be assumed to be sorted properly by UE implementation for XR application. Also, since multiple reporting thresholds in the DSR had been supported in Rel-19, we think that there is no need to include non-delay critical data specifically in the buffer size calculation.

Proposal 2: No need to include non-delay critical data specifically in the buffer size calculation for DSR.

2.3 PSI reporting in DSR

From our point of view, PSI reporting in DSR can provide more detailed information of logical channel buffers to gNB. For example, gNB can allocate more UL resource to the UE(s) with urgent and important data than other UE(s) with urgent but low-important data. That means the gNB can proceed the UL resource allocation for multiple UEs more efficiently. Also, according to the DSR MAC CE, there is one reserved bit which can be used to indicate the importance of the associated data, i.e., the importance field set to 1 indicates that the associated data is high importance, while the importance field set to 0 indicates that the associated data is low importance. The identification of PSI of a PDU Set and determination of low importance PDU Set are left up to UE implementation, which is the same as TS 38.323.

Proposal 3: RAN2 is suggested to consider PSI reporting in DSR to enhance the scheduling efficiency by Network for XR.

3 Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: No need to modify the delay-critical data definition.

Proposal 2: No need to include non-delay critical data specifically in the buffer size calculation for DSR.

Proposal 3: RAN2 is suggested to consider PSI reporting in DSR to enhance the scheduling efficiency by Network for XR.

4 References

- R2_127 R18 MBS, R18 QoE and R19 XR session notes (Dawid)_EOM_cl.docx