3GPP TSG RAN WG2#127bis	R2-2408685

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.8.4

Source: 	ITRI

Title: 	Discussions on MCCH reacquiring

Document for:	Discussion and Decision

1	Introduction

RAN2 agreed that the intended service area provided for MBS broadcast service needs to be associated with MBS session, and UE may not need to (re)acquire up-to-date MCCH when the UE is not in any intended service areas of its interested MBS broadcast services.

Related RAN2 agreements are [1][2]:

RAN2 #126 Agreements: 1. For MBS broadcast service, both EFC and EMC are supported. 2. RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. 3. MBS broadcast intended service area is provided via system information 4. For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS 5. When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

RAN2 #127 Agreements:

1.	The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

2.	RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

3.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

4.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

This contribution discusses the method of updating MBS broadcast configuration (including the intended service areas as well as the association between intended service areas and MBS sessions) without impacting UEs not interested in MBS broadcast services.

2	Discussion

2.1 	Configuration of intended service areas

In RAN2 #126, it is agreed that for NTN, the intended service area provided for MBS broadcast service should be associated with MBS session [1]. In addition, RAN2 agreed that when a UE is not in any intended service area of its interested MBS broadcast services, the UE may not need to (re)acquire up-to-date MCCH [2]. It could be understood as the intended service area needs to be associated with MBS broadcast session information.

The UE capable of MBS broadcast which is receiving or interested to receive MBS broadcast service(s) via a broadcast MRB shall ensure having a valid version of SIB20, regardless of the RRC state the UE is in [3]. A UE interested in MBS broadcast services would acquire SIB20 for the MCCH configuration and obtain the information of MBS sessions from the MBSBroadcastConfiguration message carried on MCCH.

The UE not capable of MBS broadcast or not interested in MBS broadcast services are not required to obtain SIB20.

Observation 1: UE capable of MBS broadcast or interested in MBS broadcast services would acquire SIB20, MCCH configuration, and the MBSBroadcastConfiguration message transmitted on MCCH.

Observation 2: A UE not capable of MBS broadcast or not interested in MBS broadcast services needs not to acquire SIB20.

A UE only needs to check whether it is located in an intended service area when the MBS session associated with the interested MBS broadcast service is provided in the current cell. That is, after obtained the information of the MBS sessions from the MBSBroadcastConfiguration message carried on MCCH, the UE could be capable to check whether it is located in the intended service areas of the interested MBS sessions.

Observation 3: A UE is capable to check whether it is located in the intended service areas after successfully decoded the MBSBroadcastConfiguration message carried on MCCH.

From procedure viewpoint, the association between the intended service areas and the interested MBS session could be included in the MBSBroadcastConfiguration message.

Proposal 1: It is proposed to include the information of intended service areas in SIB20, and include the association of intended service areas and the MBS session in MBSBroadcastConfiguration message carried on MCCH.

2.2 	Impacts to non-MBS UE

The information of intended service area (e.g., geographical area/TN coverage) only applies to UE interested in MBS broadcast services. However, network may change the information of intended service areas or may update the association between the intended service areas and the MBS sessions. It is expected the update should not impact non-MBS UE.

If the information change of the intended service area results in System Information modification procedure and UE could not identify which SIB is changed, both MBS UE and non-MBS UE should re-acquire System Information.

If the information of intended service areas is included in SIB20, regarding specification, it is possible that the information update of intended service areas would not result in System Information modification procedure. In that case, non-MBS UE would not be impacted by the update of intended service area. On the other hand, MBS UE would obtain up-to-date SIB20 and the information of intended service areas when (re)acquiring System Information of the current cell or when initiating a MRB establishment procedure.

Observation 4: If the intended service area information is included in SIB20 and the information update of intended service area would not result in System Information modification procedure, non-MBS UE would not be impacted by the change of intended service area information.

Proposal 2: It is proposed that the update of intended service areas should not result in System Information modification procedure.

2.3 	Procedure of MCCH (re)acquiring

For (re)acquiring up-to-date MCCH, the following cases are discussed.

- Case 1: none of the interested MBS services is broadcast in the current cell

Based on the MBSBroadcastConfiguration message carried on MCCH, when none of the interested MBS sessions is broadcast in the current cell, a UE needs not to check the intended service areas.

Considering the MBS session information in the current cell may be changed by the network (e.g., change the mapping of frequencies and MBS sessions, or change the associated intended service area), a UE interested in MBS broadcast services may need to reacquire MCCH (e.g., each MCCH modification period) for MBS session information of the cell.

If network changes the information of MBS sessions to be provided in the current cell, UE interested in MBS broadcast services without the latest MBS session information may not establish MRB for the interested MBS session timely. However, the UE could obtain the updated information of MBS sessions when reacquiring SIB20.

- Case 2: UE location is beyond the intended service areas

When the interested MBS session is broadcast in the current cell but the UE is not in any intended service areas of its interested broadcast service, the UE needs not to (re)acquire the up-to-date MCCH configuration since the UE is not allowed to establish MRB for the MBS session.

UE may reacquire SIB20 and MCCH according to the validity of System Information, and may re-evaluate whether it is allowed to access interested MBS broadcast services when (re)acquire GNSS information of the UE location.

- Case 3: UE location is within the intended service areas

When the interested MBS session is broadcast in the current cell and the UE location is within the intended service areas of its interested broadcast service, the UE needs to (re)acquire the up-to-date MCCH configuration (e.g., each MCCH modification period, or when acquiring GNSS information of UE location) for receiving the content of MBS broadcast services transmitted on MTCH.

Observation 5: For a UE interested in MBS services and the UE location is beyond intended service area, UE may reacquire SIB20 and MCCH according to the validity of System Information, and may re-evaluate whether it is allowed to access interested MBS broadcast services when (re)acquire GNSS information of the UE location.

3	Conclusion

Based on the analysis in this contribution, the observations are listed in the following:

Observation 1: UE capable of MBS broadcast or interested in MBS broadcast services would acquire SIB20, MCCH configuration, and the MBSBroadcastConfiguration message transmitted on MCCH.

Observation 2: A UE not capable of MBS broadcast or not interested in MBS broadcast services needs not to acquire SIB20.

Observation 3: A UE is capable to check whether it is located in the intended service areas after successfully decoded the MBSBroadcastConfiguration message carried on MCCH.

Observation 4: If the intended service area information is included in SIB20 and the information update of intended service area would not result in System Information modification procedure, non-MBS UE would not be impacted by the change of intended service area information.

Observation 5: For a UE interested in MBS services and the UE location is beyond intended service area, UE may reacquire SIB20 and MCCH according to the validity of System Information, and may re-evaluate whether it is allowed to access interested MBS broadcast services when (re)acquire GNSS information of the UE location.

The proposals are:

Proposal 1: It is proposed to include the information of intended service areas in SIB20, and include the association of intended service areas and the MBS session in MBSBroadcastConfiguration message carried on MCCH.

Proposal 2: It is proposed that the update of intended service areas should not result in System Information modification procedure.

4	References

[1] R2-2406202	Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, 2024.

[2] R2-24079xx	Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands, 2024

[3] 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".