3GPP TSG-RAN WG2 Meeting #127bis	R2-2408689

Hefei, China, Oct 14th – 18th, 2024

Agenda Item	:	8.7.3

Title	:	RRM Measurement Gaps/Restrictions related enhancements for XR

Source	:	Google

Document for	:	Discussion and Decision

# Introduction

A Rel-19 Work item for XR enhancement has been approved in RAN#102 [2]. The following objective for transmission and reception during measurement gaps has been captured as below:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

- Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In this Tdoc, we propose some techniques to specify the above enhancement.

# RRM Measurement Gaps/Restrictions related enhancements for XR

In RAN1#118, the following agreement has been reached:

Agreement

For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select one among the following options:

Option 1: Support Alt. 1-1:

- Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements.

  - Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s);

    - Indication is included as part of scheduling DCI:

      - FFS: Bit-field size is one bit;

      - FFS: Bit-field size is >1 bit;

    - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced.

  - FFS: DCI format, DCI content, DCI bit-field size;

  - FFS: Whether indication is for one or more occasions;

  - FFS: How to consider time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped.

Option 2: Support Alt. 3-1:

- Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements.

  - Alt 3-1: Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions;

    - FFS: Details of pattern:

      - FFS: Pattern is based on periodicity, offset and duration;

      - FFS: Pattern is based on a bitmap;

    - FFS: whether a pattern is applied to all or subset of configured MG configurations/scheduling restrictions.

Also, the following working assumption was made:

Working Assumption

For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option:

- Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements.

  - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction;

    - Indication is included as part of scheduling DCI:

      - Bit-field size is one bit;

        - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Final LS in R1-2407561.

While a unified solution for dynamic and configured grant scheduling is desirable to minimize specification effort and implementation complexity, distinct approaches in this case may be necessary for dynamic scheduling and semi-static scheduling. Alternative 1-1, where the network explicitly signals gap/restriction skipping, aligns well with the dynamic nature of dynamic scheduling. However, applying this to semi-static scheduling would require continuous PDCCH monitoring by the UE, even when unnecessary, creating inefficiency. Therefore, adopting Alternative 3-1 specifically for semi-static scheduling offers a more efficient solution. This dual approach optimizes performance for both scheduling types with not much additional signalling or complexity.

Proposal 1: Adopt Option 1 for dynamic scheduling and Option 2 for semi-static scheduling. Send LS to RAN1/RAN4 to share our views.

The MAC specification for the handling of measurement gaps (section 5.14 in 38.321) needs to be updated to take into consideration the DCI signalling the skipping of measurement gaps.

Proposal 2: The MAC specification for the handling of measurement gaps (section 5.14 in 38.321) needs to be updated to take into consideration the DCI signalling the skipping of measurement gaps.

The PDU Set Importance (PSI) can be leveraged to make informed decisions regarding the skipping of measurement gaps/restrictions, thereby optimizing radio resource utilization and enhancing XR application performance. The PSI serves as an indicator of the relative importance of a PDU Set within the XR traffic flow.

When an UL transmission coincides with a scheduled measurement gap/restriction, the UE can dynamically assess the PSI of the pending data to determine if its transmission warrants skipping the measurement gap.

To facilitate this mechanism, the gNB configures the UE with a predefined PSI threshold. If the PSI of the data awaiting UL transmission surpasses this configured threshold, the UE prioritizes the transmission and skips the measurement gap/restriction, ensuring timely delivery of critical XR data. Conversely, if the PSI of the data falls below the threshold, the UE adheres to the measurement procedure, potentially delaying or dropping the less critical data transmission.

This adaptive approach limits the detrimental impact on RRM performance while affording the network granular control over the trade-off between measurement accuracy and XR data delivery. By dynamically adjusting the PSI threshold, the network can fine-tune this balance according to specific needs and evolving network conditions.

Proposal 3: Skip a measurement gap for a CG transmission based on the priority of the XR data. E.g., skip the measurement gap/restriction if the PSI of the overlapping transmission is above a configured PSI threshold.

Also, not all CG configurations are used to transmit critical XR data. CG configurations carrying non-critical data are not required to be configured with measurement gaps/restrictions skipping and this will allow to reduce the impact on RRM performance. Therefore, the skipping of measurement gaps/restrictions could be activated/deactivated per Configured Grant configuration.

Proposal 4: The skipping of measurement gaps/restrictions could be activated/deactivated per Configured Grant configuration

In Rel-18, multi-PUSCH CG configuration has been defined to better support XR traffic. It can be specified to skip measurement gaps/restrictions only if the UL CG transmission is taking place on a CG occasion of the multi-PUSCH CG resources and overlapping with the measurement gap/restriction.

Proposal 5:  The measurement gaps/restrictions skipping could be specified to be supported only on multi-PUSCH CG configurations.

A similar challenge arises with SPS where the UE cannot preemptively determine whether an SPS occasion coinciding with a measurement gap/restriction will be utilized for transmission by the gNB. Employing DCI to convey this information for each overlapping instance proves inefficient from a resource utilization standpoint. Furthermore, it necessitates continuous PDCCH monitoring by the UE, which contradicts the principle of SPS operation, designed to reduce signaling overhead and UE PDCCH monitoring.

Proposal 6: The skipping of measurement gaps/restrictions could be activated/deactivated per SPS configuration

Measurement gaps/restrictions priority has been introduced in Rel-17. A new parameter GapPriority-r17 has been included in GapConfig-r17 to indicate the priority of the measurement gap/restriction. The intention of defining this priority field included in Rel-17 is to address the overlapping between different measurement gaps/restrictions and allow the UE to prioritize the measurement gap/restriction of high priority when multiple measurement gaps/restrictions are overlapping.

The priority field can also be used when the measurement gap/restriction is overlapping with XR traffic.  The gNB can configure the UE 1with a measurement gaps/restrictions priority threshold GapPriorityThrshld. If the UE has an UL transmission overlapping with a measurement gap/restriction, the UE checks if the priority of the measurement gap/restriction is equal or above a specific configured priority threshold.  If the measurement gap/restriction priority is equal or above a specific configured priority threshold, the UE skips the measurement gap/restriction and transmits the UL data. If the measurement gap/restriction is below the specific configured threshold, the UE carries the measurement and drops/delays the XR data transmission. The existing priority indication of measurement gaps/restrictions introduced in Rel-17 can be re-used or a new priority indication for the measurement gap/restriction can be introduced specifically for this purpose.

Proposal 7: Priority can be specified for the UE to decide between the XR data transmission or the measurement gap/restriction.

If priority is given to CG PUSCH occasions of the multi-PUSCH CG configuration when overlapping with measurement gaps/restrictions, and if a CG PUSCH occasion overlapping with a measurement gap/restriction is indicated as cancelled by the UTO-UCI (Unused Transmit Occasion – UCI) and the measurement gap/restriction has no longer any other overlapping with a CG PUSCH occasion of the multi-PUSCH CG configuration then the measurement gap/restriction is not skipped in this case and the measurement is carried by the UE

In scenarios employing multi-PUSCH CG configurations, prioritized handling of CG PUSCH occasions during overlap with measurement gaps/restrictions is essential. To further optimize this process, the following rule is proposed:

When a CG PUSCH occasion overlapping with a measurement gap/restriction is cancelled as indicated by the UTO-UCI, and no other CG PUSCH occasion within the same multi-PUSCH CG configuration overlaps with that measurement gap/restriction, the UE should not skip the measurement gap. Instead, the UE should perform the scheduled measurement.

This refined approach ensures that measurement gaps are utilized effectively when specific CG PUSCH occasions are cancelled.

Proposal 8: UE should not skip the measurement gap/restriction if the overlapping CG PUSCH occasion within the multi-PUSCH CG configuration is cancelled by the UTO-UCI

# 3		Conclusions

In conclusion, we have the following proposals:

Proposal 1: Adopt Option 1 for dynamic scheduling and Option 2 for semi-static scheduling. Send LS to RAN1/RAN4 to share our views.

Proposal 2: The MAC specification for the handling of measurement gaps (section 5.14 in 38.321) needs to be updated to take into consideration the DCI signalling the skipping of measurement gaps.

Proposal 3: Skip a measurement gap for a CG transmission based on the priority of the XR data. E.g., skip the measurement gap/restriction if the PSI of the overlapping transmission is above a configured PSI threshold.

Proposal 4: The skipping of measurement gaps/restrictions could be activated/deactivated per Configured Grant configuration

Proposal 5:  The measurement gaps/restrictions skipping could be specified to be supported only on multi-PUSCH CG configurations.

Proposal 6: The skipping of measurement gaps/restrictions could be activated/deactivated per SPS configuration

Proposal 7: Priority can be specified for the UE to decide between the XR data transmission or the measurement gap/restriction.

Proposal 8: UE should not skip the measurement gap/restriction if the overlapping CG PUSCH occasion within the multi-PUSCH CG configuration is cancelled by the UTO-UCI

# 4		References

- TS 38.133 V17.11.0

- RP-234014 ‘New WID: XR (eXtended Reality) for NR Phase 3’, MediaTek (Moderator), RAN#102, Edinburgh, Scotland, December 11-15, 2023