3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408692

Hefei, China, 14 – 18 October, 2024

Agenda item:	8.4.4

Source:	InterDigital

Title:	LP-WUS in CONNECTED mode

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This TDoc discusses the issues with LP-WUS in RRC_CONNECTED mode.

# 2	Discussion

## 2.1	LP-WUS capability

As it has been agreed that the LP-WUS monitoring is configured for the UE through dedicated RRC signaling, it is evident that UE capability is required for gNB to be able to trigger the configuration for a UE.

Proposal 1: UE capability for supporting LP-WUS in RRC_CONNECTED is supported.

## 2.2	Entry/exit conditions

RAN2 agreed the following baseline for entry/exit conditions for RRC_IDLE/INACTIVE mode in RAN2#126 and RAN2#127 meetings, respectively:

RAN2#126:

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

RAN2#127:

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

This baseline seems to work as a reasonable foundation for entry/exit conditions also for RRC_CONNECTED mode where the UE monitors PDCCH solely based on C-DRX when the serving cell quality does not satisfy the condition to monitor LP-WUS (entry condition not satisfied); and where the UE monitors PDCCH based on both the LP-WUS and C-DRX when the serving cell quality satisfies the entry condition. Given the dedicated configuration of the LP-WUS monitoring for RRC_CONNECTED mode UE agreed, it seems reasonable to allow the configuration of these thresholds separately for RRC_CONNECTED UE from RRC_IDLE/INACTIVE conditions, or the same can be applied when no dedicated configuration is provided. To avoid ping-pong effect at the UE (especially, if there would be a requirement to report out-of-coverage events to the NW), UE could be required to have a threshold number of measurement events (in or out-of-coverage) before entering/leaving the LP-WUS monitoring.

Proposal 2: As a baseline, support the same entry/exit condition criteria based on serving cell quality to activate/deactivate LP-WUS monitoring in RRC_CONNECTED mode as agreed for RRC_IDLE/INACTIVE mode.

Proposal 3: UE can be configured with a threshold number of LP-WUS receptions for determining LP-WUS in coverage (e.g., successful reception) and out-of-coverage (e.g., reception failure).

It is worth discussing whether only the serving cell quality based entry/exit condition is enough to maintain synchronization between UE and the gNB and avoid any service interruptions if the UE cannot receive the LP-WUS signal regardless of entry/exit condition being satisfied. Similarly to data inactivity timer (which protects the system from RRC mismatch), a timer could be exploited to avoid an issue where the entry/exit condition is satisfied but the UE cannot receive the LP-WUS signal while gNB tries to wake up the UE. While the timer is running (and entry/exit condition satisfied), the UE can activate LP-WUS monitoring whereas when the timer expires, the UE deactivates LP-WUS monitoring and monitors PDCCH based on the other rules in the specification (i.e., C-DRX).

Proposal 4: Support a timer as additional condition to activate/deactivate LP-WUS monitoring.

While the UE activating the LP-WUS monitoring based on the entry/exit condition being satisfied doesn’t need to be known to the gNB, the UE deactivating the LP-WUS monitoring based on the events as above would seem reasonable to be made known to the gNB, especially, if it happens through the timer while the entry/exit condition was still satisfied.

Proposal 5: UE reports the events of autonomous deactivation of LP-WUS monitoring to the gNB.

## 2.3	PDCCH monitoring triggered by LP-WUS

In the previous RAN2#127 meeting, the following was agreed wrt. the LP-WUS operation and C-DRX:

For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

Only meaningful legacy timers to be considered for the purpose would be the drx-onDurationTimer and drx-InactivityTimer.

The drx-onDurationTimer is defined to be a duration at the start of the DRX cycle, hence, using this timer for the purpose would mean that the definition would not apply anymore. On the other hand, issues could be foreseen where the timer duration would overlap with the start of the DRX cycle where the drx-onDurationTimer should be started (based on potentially another LP-WUS trigger).

The drx-InactivityTimer is not good for the purpose given its expiry will lead to start or restart of the drx-ShortCycleTimer (ie. restart of the short DRX cycle) while the LP-WUS signal could have been used to wake up another UE to receive DL transmission, but this UE just happened to be in the same subgroup with the intended UE.

Observation 1: Existing DRX timers are not good to be used for LP-WUS triggered PDCCH monitoring for Option 1-2.

Proposal 6: New timer is used for UE to monitor PDCCH with Option 1-2 LP-WUS.

RAN1#118 made the following working assumption:

Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

Depending on whether RAN1 confirms the WA, the main issue to discuss is whether to allow simultaneous configuration of Option 1-1 and 1-2. As RAN1 agreed that C-DRX configuration is required with the LP-WUS configuration in RRC_CONNECTED mode, from power saving perspective, it seems not useful to configure Option 1-2 with C-DRX. This would only enable waking up the UE with LP-WUS outside the PDCCH monitoring dictated by the C-DRX timers, ie., in this case Option 1-2 would only enable additional PDCCH monitoring to be triggered outside the Active time, and hence, would not enable any power saving. Allowing simultaneous configuration of Option 1-1 and 1-2 would allow maximal power saving while still allowing flexible waking up the UE even outside the defined DRX cycles.

Proposal 7: Support simultaneous configuration of Option 1-1 and Option 1-2.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: UE capability for supporting LP-WUS in RRC_CONNECTED is supported.

Proposal 2: As a baseline, support the same entry/exit condition criteria based on serving cell quality to activate/deactivate LP-WUS monitoring in RRC_CONNECTED mode as agreed for RRC_IDLE/INACTIVE mode.

Proposal 3: UE can be configured with a threshold number of LP-WUS receptions for determining LP-WUS in coverage (e.g., successful reception) and out-of-coverage (e.g., reception failure).

Proposal 4: Support a timer as additional condition to activate/deactivate LP-WUS monitoring.

Proposal 5: UE reports the events of autonomous deactivation of LP-WUS monitoring to the gNB.

Observation 1: Existing DRX timers are not good to be used for LP-WUS triggered PDCCH monitoring for Option 1-2.

Proposal 6: New timer is used for UE to monitor PDCCH with Option 1-2 LP-WUS.

Proposal 7: Support simultaneous configuration of Option 1-1 and Option 1-2.