3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408694

Hefei, China, 14 – 18 October, 2024

Agenda item:	8.7.4.1

Source:	InterDigital

Title:	LCP enhancements for UL scheduling

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The WID objective wrt. LCP enhancements was focused as follows [RP-241771]:

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets;

This TDoc discusses the LCP enhancements.

# 2	Discussion

The following agreements on LCP enhancements were made in RAN2#126 [2]:

LCH prioritization Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. The solution should consider impact on UE complexity (as already indicated in SI objective description) For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH Granularity of LCH enhancements LCP prioritization within a logical channel will not be considered in RAN2 discussions FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

RAN2 agreed in the previous meetings to override/adjust the priority of a logical channel based on delay/deadline information as a baseline, i.e., to use an additional priority configured to the logical channels in case the logical channels were to have in their buffer delay-critical data. Such a reprioritization would allow LCP to also consider the remaining time of the buffered data w.r.t. their respective delay budgets enabling urgent packets to be prioritized when they belong to low-priority logical channels. The UE would be configured to prioritize a logical channel carrying data whose remaining time falls below a remaining time threshold, over a logical channel of higher priority if the remaining time of the data from that channel is larger than the remaining time compared to the data in the lower priority logical channel.

Addressing the FFS on whether such reprioritization only applies to delay-critical data within the LCH or for the whole LCH, an agreement was made later on in the same meeting to not consider LCP prioritization within a logical channel, essentially implying that any LCP prioritization should apply to the whole LCH. In light of this agreement, any reprioritization should apply to the whole LCH. Furthermore, how the UE selects SDUs within a logical channel to multiplex into a transport block, i.e., whether or not it is on a first-in-first-out basis, is currently left to UE implementation.

One may argue that given that both delay-critical and non-delay-critical data may now be buffered into a logical channel, there is value in the priority only applying to delay-critical data within the LCH given that the UE reported the amount of delay-critical data in the DSR and the network may have provided only enough grant for the delay-critical data to be multiplexed in the MAC PDU. However, there are implications to this approach, for e.g., stalling at PDCP if the non-delay-critical data arrived before the delay-critical data and has a sequence number already assigned, which if properly addressed will lead to high spec impact. For the sake of simplicity, the LCH prioritization for delay-aware LCP should apply to the whole LCH.

Proposal 1: In light of the agreement to not consider LCP prioritization within a logical channel and for simplicity, LCH prioritization for delay-aware LCP should apply to the whole LCH.

The other FFS addressed whether a separate remaining time threshold can be configured for delay aware LCP, i.e., different from the one used for Release 18 DSR. The current agreed solution of overriding/adjusting the priority of a logical channel based on delay/deadline information by using an additional priority configured to LCHs in case they carry delay-critical data may have some impact on other logical channels that may be carrying high priority data with a longer remaining time. Such a reprioritization should be limited to only when strictly needed. We hence think that to prevent the dynamic prioritization from happening too often, a critical remaining time threshold should be defined at a value lower than the Rel-18 remaining time threshold configured for DSR. The UE is configured to only prioritize data from a lower priority logical channel at the expense of a higher priority logical channel if the remaining time of the data in the lower priority logical channel is below the critical remaining time threshold.

Proposal 2: A separate remaining time threshold is configured for delay aware LCP (i.e., with a value smaller than the one used for Release 18 DSR).

# 3	Conclusion

This document has made the following proposals:

Proposal 1: In light of the agreement to not consider LCP prioritization within a logical channel and for simplicity, LCH prioritization for delay-aware LCP should apply to the whole LCH.

Proposal 2: A separate remaining time threshold is configured for delay aware LCP (i.e., with a value smaller than the one used for Release 18 DSR).