3GPP TSG-RAN WG2 Meeting #127bis	R2-2408698

Hefei, China, 14 – 18 October 2024

Agenda item:	8.2.3

Source:	Nokia

Title:	Paging procedures for Ambient IoT

WID/SID:	FS_Ambient_IoT_solutions – Rel-19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#103, the study item on Ambient Internet of Things (AIoT) solutions in NR was revised in RP-240826. In RAN2#126, the following agreements were made related to Ambient IoT paging.

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

The following agreements were also made in RAN2 #126 with respect to Stage 2 general aspects of Ambient IoT.

Agreements 1 As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS 2 As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

In RAN2#127, the following agreements were made related to Ambient IoT paging.

Agreements:

1	Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

In this contribution, we further discuss the considerations on paging for Ambient IoT and make some observations and proposals focusing on Topology 1.

# 2	Discussion

## 2.1	AIoT paging procedures

There are two types of traffic currently within scope, as defined in the general scope of the study item: DO-DTT (device-originated – device-terminated-triggered) and DT (device-terminated). In the case of DO-DTT traffic type, the device has data that it must send to the network; however, the device cannot initiate communication with the network on its own, but it can send data only in response to a trigger from the network. As defined by the scope of the study item, there is no consideration of RRC states for AIoT devices. Therefore, the term “paging” in the context of AIoT devices has no relationship with RRC states.

In RAN #126, some agreements (noted in Section 1) were made regarding the procedure to supporting the above traffic types in different application scenarios. The following use cases are considered.

Use Case: “Inventory only”

In this use case, associated with DO-DTT traffic, the base station triggers one or more AIoT devices to respond with their respective device IDs. Here it is agreed that the first step will consist of AIoT paging, which will be followed by a step consisting of device ID transmission. Therefore, the AIoT paging message is the initial trigger message for the procedure and indicates which device or devices should respond, and the devices then respond with an indication of the respective device ID(s). There are no further steps associated with this procedure.

Use Case: “Inventory and command”

According to the current agreements noted in Section 1, the steps of this procedure consist of the steps in Case A for “inventory” followed by additional steps associated with a “command”. Thus, the first two steps consist of an AIoT paging message followed by device ID transmission. In the subsequent step, the base station sends data or a command to one or more devices. In the last step, the device sends a corresponding response. The AIoT paging message is the initial trigger message for this procedure as well and indicates which device or devices should respond, and the devices then respond with an indication of the respective device ID(s). Since the “command” (which may be either “read” or “write”) steps follow the “inventory” steps, the information from the “inventory” steps may be used in the “command”, i.e., the IDs of devices that responded to the paging message may be indicated in the command message.

In both the above procedures, inventory is performed wherein devices indicate their respective IDs in response to the AIoT paging. This response may consist of multiple devices performing random access, where each device selects a random occasion of a set of occasions provided by the reader. This procedure may cause contention between multiple AIoT devices trying to use the same access occasions and thus collisions within message transmission(s) and potential loss of data. Here either a 1-step or 3-step access procedure are possible. In the 1-step case, the device sends its device ID directly in the initial response message. In the 3-step case, the device sends only a random ID in the initial response and sends its device ID in the subsequent message (which is actually the first step of contention resolution). Thus, the AIoT paging message should indicate whether the devices shall respond using 1-step or 3-step access and accordingly determine whether to include the device ID in the initial response to the AIoT paging message.

Proposal 1: In case of “inventory only” and “inventory and command” procedures, the AIoT paging message indicates whether the initial response shall contain device ID.

Use Case: “Command only”

There are no agreements yet on the steps associated with the procedure for this use case other than that RAN2 will study this use case. However, our understanding is that in this case, there are no steps associated with “inventory”. Therefore, the base station may directly send a command and/or data to one or more devices in the initial trigger message.

It is clear from the above cases that the “inventory and command” procedure can be distinguished from the “inventory only” procedure through adding the steps associated with sending a command or data to the device(s) and subsequently receiving data back from the devices. Therefore, in the “command only” use case, there is no need for separate steps associated with the “inventory” procedure. In other words, in this use case the network may want to directly send commands to devices without performing an inventory. For example, devices in this use case may be stationary or always within range of the base station. Therefore, it is not necessary to have separate steps for the purpose of obtaining device ID(s).

Proposal 2: RAN2 to assume the “command only” procedure does not include the steps associated with device ID delivery during “inventory” / “inventory & command” scenarios.

In the paging scenarios in cellular network, the RAN does not maintain any timer to track the device responses to AIoT paging. It is up to CN to retransmit the paging message with different escalation levels such as extending paging over larger area. For AIoT scenarios, the device mobility is not a prime consideration, but the reachability may be an issue to get a response to the AIoT paging message. Since the reader may have only a rough estimate of the number of devices that are to be addressed by an AIoT paging message, it may not be possible to always allocate sufficient resources for successful access by all the devices. As a result, contention among devices may cause failure of paging responses regardless of the type of procedure or service request. When the reader detects that such contention among devices in has caused failure of responses to a paging request, it should be able to subsequently send out AIoT paging retransmissions for the same paging request to trigger devices that experienced the failure to respond again. As discussed in Section Whether the reader can attempt a specific number of AIoT paging retransmissions before the AMF attempts retransmission needs to be discussed.

Proposal 3: RAN2 supports AIoT paging retransmissions by a reader to trigger AIoT devices to respond again to a paging request. FFS whether/how the reader can attempt a specific number of AIoT paging retransmissions before the AMF attempts retransmission.

In Topology 1, where the gNB itself is the reader, the reader has the flexibility to modify the resources for paging response. In Topology 2, where an assisting node is the reader, the gNB may assign a resource pool to the reader and the reader may assign resources from this pool for a particular AIoT paging transmission. For subsequent AIoT paging retransmissions, the reader may assign a different amount of resources from the pool in order to reduce the contention among devices. A simple approach might involve increasing the resources assigned for random access until all (or most) devices are able to respond successfully. However, there are two issues with this. The first is that the reader is constrained by the pool of resources allocated by the gNB and therefore cannot arbitrarily increase the resources that it assigns to devices for random access. The second is that it is wasteful to request duplicate paging responses from devices that have already successfully performed access. Therefore, it is necessary to optimize the procedures associated with subsequent AIoT paging retransmissions. The reader can take into consideration information related to responses that it has already received to a paging transmission when it sends a paging retransmission.

Observation 1: For paging transmissions and retransmissions in Topology 2, the reader is constrained by the resources allocated by the gNB.

Observation 2: It is wasteful to seek a response to an AIoT paging retransmission from devices whose responses have previously been successfully received.

Proposal 4: RAN2 to study methods for AIoT paging retransmissions to utilize information related to responses to prior AIoT paging (re)transmissions.

## 2.2	AIoT paging message

There are two aspects involved in the triggering of communication by the reader through AIoT paging:

- Indication of paged device(s);

- Other information like

  - controlling access related parameters

  - data to be written by the device

We can again consider the three scenarios discussed in Section 2.1 for the above two aspects.

### 2.2.1	Indication of device ID(s):

Based on current agreements, the following cases can be studied for the AIoT paging message:

- A message containing the ID of a single device.

  - The network may send an AIoT paging message to inventory a single device, e.g., for the purpose of proximity detection.

- A message containing a group ID that maps to multiple devices.

  - The network may send an AIoT paging message to only a specific group of devices either for the purpose of inventory only to perform an inventory and subsequently send a command to only the devices that responded. It is understood that the group ID may be obtained through a mask of the device ID or some method of pre-configuration.

- A message that does not contain an ID, in which case it is addressed to all devices that can receive the AIoT message.

  - Similar to the previous case, the network may send an AIoT paging message to all devices that can receive the message either for the purpose of inventory only to perform an inventory and subsequently send a command to only the devices that responded. Details of the indication in the AIoT paging message for this case should be studied further.

- A message containing a fixed and/or a variable-length list of multiple IDs of AIoT devices. The need for this case should be confirmed based on SA2 discussion.

  - From RAN2 perspective, this is similar to the second case with the exception that multiple devices that have not been pre-configured with a group ID and hence has similar applicability. Thus, subject to SA2 confirmation for the need for this case, it should be supported.

Thus, in our view, each of the above cases is valid for consideration by RAN2. The network may need to send an AIoT paging message to one or more devices to detect their presence or initiate communication with them.

One issue that is still open is option 3, i.e., whether/how to indicate that the paging is addressing all devices with no restriction. From a unified design point of view, we believe that the simplest approach would be to always include the ID, but setting is to 0 in case all devices are addressed. Although this may cost bits, it will allow for easier decoding at device side.

Proposal 5: When an AIoT paging addresses all devices, the group/device ID is still contained in the paging message but set to 0.

The use of full-length device identifiers in AIoT paging messages may cost a high overhead, especially when listing a large number of identifiers in the paging message. Therefore, it would be beneficial to consider short identifiers for devices. Two options for how short identifiers are assigned to devices are: (a) the short identifiers are generated by the devices and shared with the reader (b) the short identifiers are generated by the reader and assigned to devices.  and details of how they can be used can be studied.

Regarding RAN2 assuming a permanent ID, even though this will be different from the full ID, we believe that such an agreement may cause both privacy and security risk, and should not be agreed without consulting SA3.

Observation 3: A permanent ID is not suitable for use as it provides privacy and security risk.

Proposal 6: RAN2 not to study permanent ID without consulting SA3.

Proposal 7: RAN2 to study how to support the use of short device IDs for future access related procedures. FFS whether the ID is allocated by either the reader or device. FFS whether it is the short random ID.

Proposal 8: The paging message needs to contain at least an ID type field indicating the type of identification.

### 2.2.2	Resource and power aware retransmission control

For the paging retransmissions procedure to effectively use the allocates resources it can be considered to preventing devices that have previously successfully responded to an AIoT paging message for an inventory or other procedure from responding to a retransmission of the paging message for the same paging request.

Therefore, from a practical point of view, it may be also beneficial to provide an indication in an AIoT paging (re)transmission message related to the devices excluded from the list of paged devices which are not / no more expected to respond (e.g., since their response was successfully received in the past or is of lower priority) Additionally, methods for indication to trigger only those devices  whose responses were not received in the past can also be studied. Identifying devices through the access occasions previously used by them would reduce the signaling overhead.

Proposal 9: RAN2 to discuss whether to an AIoT paging message may also provide an indication to identify

- AIoT devices that are excluded from responding to a given AIoT paging request, i.e. devices that shall ignore the AIoT paging message.

- AIoT devices that shall respond to the AIoT paging message.

FFS whether there is a need for mapping to previous access occasions.

Proposal 10: The paging message may indicate a duration in which the device is excluded from responding e.g. a number of access occasions, number of paging occasions, or a time reference points.

### 2.2.3	Other information

To ensure that devices respond using the appropriate form of random access (contention-based or contention-free) as well as the correct resource, access type and the associated resources shall be indicated. For example, when contention-free access is selected by the reader, resource indication may indicate the granted resource (e.g., time slot) relatively to current time (e.g., by using nextSlotOffset information). Alternatively, when pure random access is selected, a specific set of suitable resources (e.g., the next N slots for an initial response attempt) or the appropriate random access configuration (e.g., backoff parameters). For AIoT paging of a single device, there is no need for consideration of random access since access resource can be granted to the device and hence contention-free access can be the only type of access supported in this case. Likewise, for AIoT paging of a device group consisting of an arbitrary number of devices, contention-free access is not an option and hence contention-based random access can be the only type of access in this case. If AIoT paging with indication of multiple device IDs is supported, both contention-free access and contention-based random access can be considered. While indication of contention-free resources may be efficient for a small number of devices, such indication may result in high overhead in the case of a large number of devices. Therefore, contention-based random access should be considered as an option in this case.

Similarly, service type may need to be also indicate the procedure type and length (e.g., number of subsequent steps until service termination). For example, the “command” scenario requires more steps compared to pure “inventory” use case.

Proposal 11: To simplify signaling, RAN2 to agrees that

- an AIoT paging of a single-device means “Contention-free access”,

- an AIoT paging of a device group means “Contention-based random access”.

Proposal 12: For the case of an AIoT paging indication to multiple device IDs (if supported), RAN2 to consider supporting both contention-free access and contention-based random access.

Proposal 13: RAN2 to agree that an AIoT paging message contains at least one of the following

- access type, in case RAN2 agrees on multiple targets for CFA (“Contention-based random access”, “Contention-free access”)

- service type (FFS options “Inventory only”, “Command only”, “Inventory & command”)

- associated response resources and related parameters (FFS details).

As part of the “write” command service, dynamic configuration updates could be sent to one or more devices. The information sent may comprise the full configuration or only the part of the configuration that has changed (differential or “delta” information). The information can include parameters for device access in response to inventory or “read” command requests. In case of paging multiple devices, parameters for random access (e.g., similar to a Q-parameter in the RFID standard) can be provided. Alternatively, when a single device is paged for a “read” command service request, scheduling parameters for fixed resource allocation can be provided. If dynamic configuration update through the paging procedure is supported, further study is needed on the frequency of such updates.

Proposal 14: RAN2 to discuss the signaling of MAC parameters/configuration/updates during paging procedures, including the parameters needed to determine backoff time (if needed).

As discussed above, a paging request may be retransmitted. It is further observed that a paging request may be reconfigured. For example, the targeted devices may be changed (e.g. based on priority if the response time is protracted due large-scale contention implying long / frequent backoff) or MAC parameters may be updated (e.g., backoff time is shortened if contention is low).

To this end, it would be beneficial to uniquely identify individual paging requests, e.g. by using a paging index or ID that is preferably unique over some time. Alternatively, single-bit indication of a new paging occasion may be used, similarly to New Data Indication.

Paging request identification is also needed when multiple paging requests occur simultaneously (be it by originating from the same reader, or multiple independent readers).

Proposal 15: RAN2 to agree that individual paging requests are identified for retransmission or reconfiguration purposes as well as to permit the execution of multiple parallel paging requests. FFS whether multi-bit identifiers (e.g., random ID or sequence number) or single-bit flags (e.g., similarly to NDI) are used.

Alternatively, a paging request may be terminated if a sufficient number of devices have responded, or the available resources will become unavailable (e.g., the need to reserve them for SL communications). Also, massive contention and contention may result in excessive response times. If an upper-layer deadline elapses, there is no need to further collect and forward data from AIoT devices.

Proposal 16: RAN2 to study how to terminate ongoing responses from AIoT devices to a given paging request.

A related scenario for consideration is one where the AIoT device is within the range of multiple readers.  In our view, this may be a common scenario, especially in Topology 2, where readers may not always be deployed in a carefully planned manner. In such scenarios, an AIoT device may receive AIoT paging messages associated with the same service request (i.e., with the same paging identifier) from multiple readers. In this case, the device behavior could be similar to that when multiple paging (re)transmissions are received from the same reader. That is, it may not be relevant to the device to know whether the AIoT paging messages are received from the same reader or different readers as long as the paging identifier is included in the AIoT paging messages. Therefore, it is necessary to further study whether reader ID should be included in the AIoT paging messages.

Observation 4: Deployment of readers cannot always be planned to avoid the possibility that an AIoT device is within range of multiple readers, especially in Topology 2.

Proposal 17: RAN2 to further study whether it is necessary to include reader ID/paging identifier in the AIoT paging message to support the scenario where the AIoT device receives AIoT paging messages from multiple readers with the same paging identifier.

As discussed in Section 2.1, there is no command or other data sent in the AIoT paging message in the “inventory only” and “inventory and command” use cases. For the inventory step, only device ID(s) may be indicated based on which AIoT devices send response messages indicating their respective IDs. While no data or command is included in the AIoT paging message in these cases, control information related to the inventory procedure (e.g., random access parameters) may be included.

The “inventory and command” procedure employs what may be called a two-step approach. In this approach, the indication of device/group ID is provided in an AIoT paging message in the first step (corresponding to inventory), and indication of information related to data communication is provided in a separate command message in the second step. This information may comprise not only a command but also DT data (in the case of “write” command) specific to one or more devices.

On the other hand, the “command only” procedure is a one-step approach for communication with AIoT devices. In this approach, both the device/group ID indication and the indication of the information related to data communication (e.g., “command”, DT data) are provided in the initial trigger message. In many use cases, the “command only” procedure for communicating with devices may be preferable since a separate inventory step may be unnecessary and would increase the signaling overhead.

However, including the DT traffic data in the initial trigger message may lead to a failure to deliver the data if the device is not reachable. Moreover, delivering the traffic directly in the initial trigger message may lead to some security issues and it may be necessary for SA3 to analyze the security issues associated with this approach.

Proposal 18: RAN2 to consider inclusion of the network information such as DT traffic data and/or command in the initial trigger message.

Proposal 19: If inclusion of network information in initial trigger message is agreed, RAN2 to inform SA3 to analyze the security issues related to this option.

## 2.4	Consideration of different Ambient IoT devices for paging

We adopt the following terminologies agreed in RAN1 for the devices that are within the scope of the study.

- Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device.Devices 1 and 2a are the same in terms of basic capabilities – both perform uplink transmission only via backscattering – with the difference between them being that Device 2a is also capable of DL and/or UL amplification. On the other hand, Device 2b is capable of generating an uplink transmission internally. Nevertheless, the uplink transmission time is under network control for all devices. While the backscattering transmission time for Devices 1 and 2a is determined by the activation signal, the uplink transmission time for Devices 2b may be scheduled or configured by the network.

In the legacy paging procedure, the AMF may initiate the procedure by sending a paging message to the gNB of the cell where the UE is expected to be camping. The gNB then pages the UE. If the UE is expected to respond to the paging, it does so by initiating the random access procedure. In the case of Device 2b, it may be possible to use the legacy random access procedure, whereas in the case of Devices 1 and 2a, the network controls the paging response procedure. The objective is to develop a harmonized air interface design. It would be possible to harmonize the paging procedure for the different Ambient IoT devices in terms of the high-level messages.

However, the differences in the capabilities of the Ambient IoT devices may result in some differences in how devices access the network in response to paging. For example, given that uplink transmission of Device 2b is internally generated rather than backscattered, the transmission can be delayed with respect to the triggering message. As a result, if the network has prior knowledge on the type of device being paged, configuration and control information related to random access or resource allocation that is contained in the paging message may be different.

Therefore, the content of paging message may depend on the targeted device when the device type is known to the reader. However, there may scenarios in which the network has no prior knowledge of the device. In this case, a common paging message can be used for all devices. Subsequent message content can be different based on the device.

Observation 5: Paging procedure for different Ambient IoT devices (1, 2a, 2b) can be harmonized, but content of paging message can be device dependent when prior knowledge of device type is available to the reader.

Proposal 20: RAN2 to study differences in AIoT paging messages for Device 1/2a and Device 2b when prior knowledge of device type is available to the reader.

## 2.6	AIoT Device Availability

To ensure the availability of the AIoT device during the transmission and reception procedures, the following agreement was made during the RAN#103 meeting in RP-240854 (Section 8):

- The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study [RAN2, RAN1]

  - Duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds

    - Note: this value can be revisited in future RAN plenary meetings, if necessary

In general, an AIoT device may experience failure during the random-access procedure due to their need to harvest energy between transmissions. In particular, AIoT devices may not harvest enough energy between two successive transmissions of the D2R access messages, or before the start and the completion of access (whether contention-based or contention-free) procedures. Not having enough energy for completing the access procedure may result in unexpected behaviour and can cause radio link failures.

In addition, AIoT devices (e.g., device 1 and device 2a) may not have the capability to send their D2R transmissions with constant transmission power which means that the maximum transmission power they could achieve at any given moment could vary. Backscattering the D2R signals with variable transmission power may negatively affect the access procedure for A-IoT devices. For example, AIoT devices may not have sufficient power to backscatter Msg1 with successive increases in power. Failure to complete a successful access procedure not only impacts the network performance but also creates unnecessary collisions and spectral inefficiency. It may also lead to excessive drain of stored energy at A-IoT devices.

Proposal 21: RAN2 to study how AIoT devices can successfully complete the access procedure under the constraint of device energy availability.

In the cellular network, A-IoT devices will be distributed throughout the network area. Some A-IoT devices will be far from the reader, while others will be closer. For AIoT devices (e.g., device 1 and device 2a) that are far away (e.g., at the cell edge in Topology 1), to backscatter a signal that can be successfully received and decoded by the reader, the A-IoT device needs to be provided with a carrier wave that is transmitted at high power levels.

Activating AIoT devices with high-power levels may create severe interference and may also cause excessive energy waste for near AIoT devices. For this reason, farther AIoT devices would have to harvest more energy to transmit a signal that is sufficient for the distant reader to successfully receive the AIoT backscattering.

Proposal 22: RAN2 to consider a measure of proximity (e.g., expressed as signal strength) between an AIoT device and the reader and/or CW node when studying AIoT access procedure.

# 3	Conclusion

In this contribution, the considerations on paging for Ambient IoT are discussed. The following observations and proposals are made.

Proposal 1: In case of “inventory only” and “inventory and command” procedures, the AIoT paging message indicates whether the initial response shall contain device ID.

Proposal 2: RAN2 to assume the “command only” procedure does not include the steps associated with device ID delivery during “inventory” / “inventory & command” scenarios.

Proposal 3: RAN2 supports AIoT paging retransmissions by a reader to trigger AIoT devices to respond again to a paging request. FFS whether/how the reader can attempt a specific number of AIoT paging retransmissions before the AMF attempts retransmission.

Observation 1: For paging transmissions and retransmissions in Topology 2, the reader is constrained by the resources allocated by the gNB.

Observation 2: It is wasteful to seek a response to an AIoT paging retransmission from devices whose responses have previously been successfully received.

Proposal 4: RAN2 to study methods for AIoT paging retransmissions to utilize information related to responses to prior AIoT paging (re)transmissions.

Proposal 5: When an AIoT paging addresses all devices, the group/device ID is still contained in the paging message but set to 0.

Observation 3: A permanent ID is not suitable for use as it provides privacy and security risk.

Proposal 6: RAN2 not to study permanent ID without consulting SA3.

Proposal 7: RAN2 to study how to support the use of short device IDs for future access related procedures. FFS whether the ID is allocated by either the reader or device. FFS whether it is the short random ID.

Proposal 8: The paging message needs to contain at least an ID type field indicating the type of identification.

Proposal 9: RAN2 to discuss whether to an AIoT paging message may also provide an indication to identify

- AIoT devices that are excluded from responding to a given AIoT paging request, i.e. devices that shall ignore the AIoT paging message.

- AIoT devices that shall respond to the AIoT paging message.

FFS whether there is a need for mapping to previous access occasions.

Proposal 10: The paging message may indicate a duration in which the device is excluded from responding e.g. a number of access occasions, number of paging occasions, or a time reference points.

Proposal 11: To simplify signaling, RAN2 to agrees that

- an AIoT paging of a single-device means “Contention-free access”,

- an AIoT paging of a device group means “Contention-based random access”.

Proposal 12: For the case of an AIoT paging indication to multiple device IDs (if supported), RAN2 to consider supporting both contention-free access and contention-based random access.

Proposal 13: RAN2 to agree that an AIoT paging message contains at least one of the following

- access type, in case RAN2 agrees on multiple targets for CFA (“Contention-based random access”, “Contention-free access”)

- service type (FFS options “Inventory only”, “Command only”, “Inventory & command”)

- associated response resources and related parameters (FFS details).

Proposal 14: RAN2 to discuss the signaling of MAC parameters / configuration / updates during paging procedures, including the parameters needed to determine backoff time (if needed).

Proposal 15: RAN2 to agree that individual paging requests are identified for retransmission or reconfiguration purposes as well as to permit the execution of multiple parallel paging requests. FFS whether multi-bit identifiers (e.g., random ID or sequence number) or single-bit flags (e.g., similarly to NDI) are used.

Proposal 16: RAN2 to study how to terminate ongoing responses from AIoT devices to a given paging request.

Observation 4: Deployment of readers cannot always be planned to avoid the possibility that an AIoT device is within range of multiple readers, especially in Topology 2.

Proposal 17: RAN2 to further study whether it is necessary to include reader ID/paging identifier in the AIoT paging message to support the scenario where the AIoT device receives AIoT paging messages from multiple readers with the same paging identifier.

Proposal 18: RAN2 to consider inclusion of the network information such as DT traffic data and/or command in the initial trigger message.

Proposal 19: If inclusion of network information in initial trigger message is agreed, RAN2 to inform SA3 to analyze the security issues related to this option.

Observation 5: Paging procedure for different Ambient IoT devices (1, 2a, 2b) can be harmonized, but content of paging message can be device dependent when prior knowledge of device type is available to the reader.

Proposal 20: RAN2 to study differences in AIoT paging messages for Device 1/2a and Device 2b when prior knowledge of device type is available to the reader.

Proposal 21: RAN2 to study how AIoT devices can successfully complete the access procedure under the constraint of device energy availability.

Proposal 22: RAN2 to consider a measure of proximity (e.g., expressed as signal strength) between an AIoT device and the reader and/or CW node when studying AIoT access procedure.